3GPP TSG-CT WG1 Meeting #157	C1-256697

Sophia Antipolis, France, 13-17 October 2025	was C1-256081

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.379 | CR | 1034 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on call forwarding outcome |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc19 |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The descrption of the condition in clause 11.1.9.2.2 Client procedures for handling incoming private call forwarding request when the SIP MESSAGE indicating the outcome of the call forwarding is sent does not contain all the necessary details. This CR adds more details |  |  |  |  |  |  |  |  |
| Summary of change: |  | 11.1.9.2.2 - add more details to the sentence that describes the condition when the SIP MESSAGE indicating the outcome of the call forwarding is sent |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The exact condition when the message indicating the outcome of a call forwarding remains unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 11.1.9.2.2	Client procedures for handling incoming private call forwarding request

Upon receiving a "SIP MESSAGE request for forwarding private call request for terminating client", the MCPTT client:

1)	should indicate to the forwarded MCPTT user that a request to forward the previously initiated call to a new target MCPTT user has been received;

2)	shall extract the MCPTT ID of the target MCPTT user from the <forwarding-target-id> element contained in the in the <anyExt> element of the <mcptt-Params> element of the <mcpttinfo> element contained in the application/vnd.3gpp.mcptt-info+xml MIME body contained in the received SIP MESSAGE request;

3)	if according to local policy on-demand sessions are to be used for forwarding of private calls, shall invoke the procedures of clause 11.1.1.2.1.1 to originate an MCPTT private call to the target MCPTT user; and

4)	if according to local policy pre-established sessions are to be used for forwarding of private calls and a pre-established session is available, shall invoke the procedures of clause 11.1.1.2.2.1 to originate an MCPTT private call to the target MCPTT user.

Once the procedures of clause 11.1.1.2.1.1 or clause 11.1.1.2.2.1 resulted in receiving a response:

-	SIP 180 (Ringing), SIP 183 (Session Progress), SIP 480 (Temporarily Unavailable) including the warning text set to "175 call is forwarded", SIP 480 (Temporarily Unavailable) including the warning text set to "191 call forwarding due to migration", or a SIP final success response (2xx) indicate that the call forwarding was successful; or

-	another final response indicates failure of the call forwarding;

the MCPTT client:

1)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33] with the following clarifications:

a)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP MESSAGE request;

b)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

c)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [4];

d)	shall include in a "uri" attribute of <entry> element of a <list> element of the <resource-lists> element of an application/resource-lists+xml MIME body the MCPTT ID contained in the forwarded-by-mcptt-id> element in the application/ vnd.3gpp.mcptt-info+xml MIME body of the received SIP MESSAGE request; and

e)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element containing:

i)	the <mcptt-called-party-id> set to the MCPTT ID of the forwarded-to-client called by theforwarded-calling-client; and

ii)	an <anyExt> element containing:

A)	the <response-type> element set to a value of "forwarding-private-call-response";

B)	if the procedures of clause 11.1.1.2.1.1 or clause 11.1.1.2.2.1 were successful in originating an MCPTT private call to the identified MCPTT user, a <forwarding-call-outcome> element set to a value of "success"; and

C)	if the procedures of clause 11.1.1.2.1.1 or clause 11.1.1.2.2.1 were not successful in originating an MCPTT private call to the identified MCPTT user, a <forwarding-call-outcome> element set to a value of "fail";

2)	shall set the Request-URI to the public service identity identifying the participating MCPTT function serving the forwarded-by-client; and

3)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [4].

* * * Next Change * * *

* * * End of Changes * * *