3GPP TSG-CT WG1 Meeting #157	C1-256596

# Sophia Antipolis, France 13 – 17 October 2025	was C1-256282

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7030 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on handling of exemptions for high priority access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc19 |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The term "HPAOP" (High Priority Access Operator Policy) is misaligned with its actual use which is indicating support for operator policy for high priority access exemptions for service area restrictions as defined in 5GMM capability IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the abbreviation "HPAOP (High Priority Access Operator Policy)" to "HPAEOP (High Priority Access Exemption Operator Policy)" Change the naming of the "HPAOP bit" to "OPHPAE bit" (operator policy for high priority access exemptions for service area restrictions)". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The term which does not represent the actual indication is misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.3.5.1, 5.4.4.2, 5.5.1.2.2, 5.5.1.3.2, 9.11.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

4G-GUTI	4G-Globally Unique Temporary Identifier

5GCN	5G Core Network

5G-GUTI	5G-Globally Unique Temporary Identifier

5GMM	5GS Mobility Management

5G-RG	5G Residential Gateway

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5GS	5G System

5GSM	5GS Session Management

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G-TMSI	5G Temporary Mobile Subscription Identifier

5QI	5G QoS Identifier

ABBA	Anti-Bidding down Between Architectures

ACS	Auto-Configuration Server

AKA	Authentication and Key Agreement

AKMA	Authentication and Key Management for Applications

A-KID	AKMA Key Identifier

A-TID	AKMA Temporary Identifier

AMBR	Aggregate Maximum Bit Rate

AMF	Access and Mobility Management Function

ANDSP	Access Network Discovery and Selection Policy

APN	Access Point Name

AS	Access stratum

ATSSS	Access Traffic Steering, Switching and Splitting

ATSSS-LL	ATSSS Low-Layer

AUN3	Authenticable Non-3GPP

AUSF	Authentication Server Function

AVC	Advanced Video Coding

BH	BackHaul

CAG	Closed access group

CGI	Cell Global Identity

CHAP	Challenge Handshake Authentication Protocol

DDX	Downlink Data Expected

DEI	Drop Eligible Indicator

DL	Downlink

DN	Data Network

DNN	Data Network Name

DNS	Domain Name System

DS-TT	Device-Side TSN Translator

eDRX	Extended DRX cycle

ePDG	Evolved Packet Data Gateway

EUI	Extended Unique Identifier

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EAC	Early Admission Control

EAP-AKA'	Improved Extensible Authentication Protocol method for 3rd generation Authentication and Key Agreement

EAS	Edge Application Server

EASDF	Edge Application Server Discovery Function

ECIES	Elliptic Curve Integrated Encryption Scheme

ECN	Explicit Congestion Notification

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDC	Edge DNS Client

EEC	Edge Enabler Client

EPD	Extended Protocol Discriminator

EMM	EPS Mobility Management

EPC	Evolved Packet Core Network

EPS	Evolved Packet System

EPS-UPIP	User-plane integrity protection in EPS

ESM	EPS Session Management

FN-RG	Fixed Network RG

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FQDN	Fully Qualified Domain Name

Gbps	Gigabits per second

GEO	Geostationary Orbit

GFBR	Guaranteed Flow Bit Rate

GUAMI	Globally Unique AMF Identifier

HPAEOP	High Priority Access Exemption Operator Policy

HEVC	High Efficiency Video Coding

IAB	Integrated access and backhaul

IMEI	International Mobile station Equipment Identity

IMEISV	International Mobile station Equipment Identity and Software Version number

IMSI	International Mobile Subscriber Identity

IP-CAN	IP-Connectivity Access Network

KSI	Key Set Identifier

L4S	Low Latency, Low Loss and Scalable Throughput

LADN	Local Area Data Network

LCS	LoCation Services

LCS-UPP	Location Services User Plane Protocol

LEO	Low Earth Orbit

LMF	Location Management Function

LPP	LTE Positioning Protocol

LP-WUSPS	Low Power Wake Up Signal with Paging Subgrouping

MAC	Message Authentication Code

MA PDU	Multi-Access PDU

MBS	Multicast/Broadcast Services

MBSR	Mobile Base Station Relay

Mbps	Megabits per second

MCS	Mission Critical Service

MEO	Medium Earth Orbit

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MID	Media Identification

MINT	Minimization of Service Interruption

MPQUIC	Multi-Path QUIC

MPQUIC-E	MPQUIC – Ethernet

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP

MSK	MBS Service Key

MTK	MBS Traffic Key

MUSIM	Multi-USIM

MWAB	Mobile gNB with Wireless Access Backhauling

N3IWF	Non-3GPP Inter-Working Function

N3QAI	Non-3GPP QoS Assistance Information

N5CW	Non-5G-Capable over WLAN

N5GC	Non-5G Capable

NAI	Network Access Identifier

NAUN3	Non-Authenticable Non-3GPP

NITZ	Network Identity and Time Zone

ngKSI	Key Set Identifier for Next Generation Radio Access Network

NPN	Non-public network

NR	New Radio

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

NSAG	Network slice AS group

NS-AoS	Network slice area of service

NSSAA	Network slice-specific authentication and authorization

NSSAAF	Network Slice-Specific and SNPN authentication and authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSRG	Network Slice Simultaneous Registration Group

NTZ	No-Transmit Zone

ON-SNPN	Onboarding Standalone Non-Public Network

OS	Operating System

OS Id	OS Identity

PAP	Password Authentication Protocol

PCO	Protocol Configuration Option

PCP	Priority Code Point

PEI	Permanent Equipment Identifier

PEIPS	Paging Early Indication with Paging Subgrouping

PEGC	PIN Element with Gateway Capability

PEMC	PIN Element with Management Capability

PIN	Personal IoT Network

PINE	PIN Element

PMF	Performance Measurement Function

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity based Services

ProSeP	5G ProSe policy

PG	PLMN Generic

PTI	Procedure Transaction Identity

PTP	Precision Time Protocol

PVS	Provisioning Server

QFI	QoS Flow Identifier

QoS	Quality of Service

QRI	QoS Rule Identifier

RACS	Radio Capability Signalling Optimisation

(R)AN	(Radio) Access Network

RFSP	RAT Frequency Selection Priority

RG	Residential Gateway

RPLMN	Registered PLMN

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSC	Relay Service Code

RSN	Redundancy Sequence Number

RSNPN	Registered SNPN

RTT	Round Trip Time

S-NSSAI	Single NSSAI

SA	Security Association

SDF	Service Data Flow

SDNAEPC	Secondary DN authentication and authorization over EPC

SDT	Small Data Transmission

SMF	Session Management Function

SGC	Service Gap Control

SLPP	SideLink Positioning Protocol

SNN	Serving Network Name

SNPN	Stand-alone Non-Public Network

SOR	Steering of Roaming

SOR-CMCI	Steering of Roaming Connected Mode Control Information

SRTP	Secure Real-time Transport Protocol

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TA	Tracking Area

TAC	Tracking Area Code

TAI	Tracking Area Identity

Tbps	Terabits per second

TMGI	Temporary Mobile Group Identity

TNAN	Trusted Non-3GPP Access Network

TNGF	Trusted Non-3GPP Gateway Function

TSC	Time Sensitive Communication

TSCTSF	Time Sensitive Communication and Time Synchronization Function

TWIF	Trusted WLAN Interworking Function

TSN	Time-Sensitive Networking

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

UAV-C	Uncrewed Aerial Vehicle-Controller

UDM	Unified Data Management

UL	Uplink

UPDS	UE policy delivery service

UPF	User Plane Function

UP-PRUK	User Plane ProSe Remote User Key

UPP-CMI	User Plane Positioning Connection Management Information

UPSC	UE Policy Section Code

UPSI	UE Policy Section Identifier

URN	Uniform Resource Name

URSP	UE Route Selection Policy

USS	UAS Service Supplier

UUAA	USS UAV Authorization/Authentication

V2X	Vehicle-to-Everything

V2XP	V2X policy

VID	VLAN Identifier

VPS	VPLMN Specific

W-AGF	Wireline Access Gateway Function

WLAN	Wireless Local Area Network

WUS	Wake-Up Signal

* * * Next Change * * * *

#### 5.3.5.1	General

Service area restrictions are applicable only to 3GPP access and to wireline access.

Subclause 5.3.5.2 applies when the UE accesses 5GCN over 3GPP access.

Subclause 5.3.5.3 applies when the 5G-RG or the W-AGF acting on behalf of an FN-CRG (or on behalf of the N5GC device) access 5GCN over wireline access.

NOTE:	Service area restrictions are not applicable for the W-AGF acting on behalf of the FN-BRG.

The network can optionally convey operator policy for service area restriction exemptions for UEs configured for high priority access in selected PLMN or SNPN using the HPASE bit of the Feature authorization indication IE in the REGISTRATION ACCEPT message or the HPASE bit in the Feature authorization indication IE in the CONFIGURATION UPDATE COMMAND message, if the UE supports HPAEP.

* * * Next Change * * * *

#### 5.4.4.2	Generic UE configuration update procedure initiated by the network

The AMF shall initiate the generic UE configuration update procedure by sending the CONFIGURATION UPDATE COMMAND message to the UE.

The AMF shall in the CONFIGURATION UPDATE COMMAND message either:

a)	include one or more of the following parameters: 5G-GUTI, TAI list, allowed NSSAI that may include the mapped S-NSSAI(s), LADN information, extended LADN information, service area list, MICO indication, NITZ information, configured NSSAI that may include the mapped S-NSSAI(s), NSSRG information, S-NSSAI location validity information, S-NSSAI time validity information, rejected S-NSSAI(s) in the Rejected NSSAI IE or in the Extended rejected NSSAI IE, network slicing subscription change indication, operator-defined access category definitions, SMS indication, "CAG information list", UE radio capability ID, 5GS registration result, UE radio capability ID deletion indication, truncated 5G-S-TMSI configuration, T3447 value, "list of PLMN(s) to be used in disaster condition", disaster roaming wait range, disaster return wait range, PEIPS assistance information, the priority indicator, service-level-AA container, the NSAG information, alternative NSSAI, partially allowed NSSAI, partially rejected NSSAI, on-demand NSSAI, feature authorization indication, discontinuous coverage maximum time offset, access technology utilization control information or LP-WUSPS assistance information;

b)	include the Configuration update indication IE with the Registration requested bit set to "registration requested"; or

c)	include a combination of both a) and b).

If the UE is registering or registered for onboarding services in SNPN, the serving SNPN shall not provide the configured NSSAI, the allowed NSSAI or the rejected NSSAI to the UE.

If the UE supports extended rejected NSSAI, the rejected S-NSSAI(s) shall be included in the Extended rejected NSSAI IE. Otherwise, the rejected S-NSSAI(s) shall be included in the Rejected NSSAI IE.

If the Extended rejected NSSAI IE is included in the CONFIGURATION UPDATE COMMAND message and mapped S-NSSAI(s) are required as specified in subclause 4.6.1, the AMF shall provide mapped S-NSSAI(s) for the rejected NSSAI.

If an acknowledgement from the UE is requested, the AMF shall indicate "acknowledgement requested" in the Acknowledgement bit of the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message and shall start timer T3555. Acknowledgement shall be requested for all parameters except when only NITZ information is included.

To initiate parameter re-negotiation between the UE and network, the AMF shall indicate "registration requested" in the Registration requested bit of the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message.

NOTE 1:	Generic UE configuration update procedure can be initiated by the AMF for updating the emergency number list, the extended emergency number list or both by indicating "registration requested" in the Registration requested bit of the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message to the UE.

If a new allowed NSSAI information or AMF re-configuration of supported S-NSSAIs requires an AMF relocation, the AMF shall indicate "registration requested" in the Registration requested bit of the Configuration update indication IE and include the Allowed NSSAI IE in the CONFIGURATION UPDATE COMMAND message.

If the AMF includes a new allowed NSSAI in the CONFIGURATION UPDATE COMMAND message and the subscription information includes the NSSRG information, then the S-NSSAIs of the allowed NSSAI shall be associated with at least one common NSSRG value. If the network has pending NSSAI, the S-NSSAIs in the pending NSSAI and allowed NSSAI shall be associated with at least one common NSSRG value.

If the AMF includes a new partially allowed NSSAI and the new partially allowed NSSAI requires an AMF relocation, the AMF shall indicate "registration requested" in the Registration requested bit of the Configuration update indication IE and include the Partially allowed NSSAI IE in the CONFIGURATION UPDATE COMMAND message.

If the AMF includes a new configured NSSAI in the CONFIGURATION UPDATE COMMAND message and the new configured NSSAI requires an AMF relocation as specified in 3GPP TS 23.501 [8], the AMF shall indicate "registration requested" in the Registration requested bit of the Configuration update indication IE in the message.

If the AMF includes a new configured NSSAI in the CONFIGURATION UPDATE COMMAND message, the subscription information includes the NSSRG information, and the UE has set the NSSRG bit in the 5GMM capability IE of the REGISTRATION REQUEST message to:

a)	"NSSRG supported", then the AMF shall include the NSSRG information in the CONFIGURATION UPDATE COMMAND message; or

b)	"NSSRG not supported", then the configured NSSAI shall include one or more S-NSSAIs each of which is associated with all the NSSRG value(s) of the default S-NSSAI(s), or the configured NSSAI shall include, based on the indication received from the UDM as specified in 3GPP TS 23.501 [8], all subscribed S-NSSAIs even if these S-NSSAIs do not share any common NSSRG value.

If the AMF needs to update the NSSRG information and the UE has set the NSSRG bit to "NSSRG supported" in the 5GMM capability IE of the REGISTRATION REQUEST message, then the AMF shall include the new NSSRG information in the CONFIGURATION UPDATE COMMAND message.

If the AMF includes a new NSSRG information in the CONFIGURATION UPDATE COMMAND message and the AMF determines that the UE needs to provide a new requested NSSAI due to no NSSRG value common to all the S-NSSAI(s) of the allowed NSSAI based on the new NSSRG information, then the CONFIGURATION UPDATE COMMAND message shall indicate "registration requested" in the Registration requested bit of the Configuration update indication IE.

If the UE supports S-NSSAI time validity information and the AMF needs to update the S-NSSAI time validity information, then the AMF shall include the new S-NSSAI time validity information in the CONFIGURATION UPDATE COMMAND message.

If the AMF needs to update the S-NSSAI location validity information toward a UE which has set the SLVI bit to "S-NSSAI location validity information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message, then the AMF shall include the new S-NSSAI location validity information in the CONFIGURATION UPDATE COMMAND message.

If the CONFIGURATION UPDATE COMMAND message is initiated only due to changes to the allowed NSSAI and these changes require the UE to initiate a registration procedure, but the AMF is unable to determine an allowed NSSAI for the UE as specified in 3GPP TS 23.501 [8], then the CONFIGURATION UPDATE COMMAND message shall indicate "registration requested" in the Registration requested bit of the Configuration update indication IE, and shall not contain any other parameters.

If the slice deregistration inactivity timer has expired, the UE does not support network slice usage control and the AMF removes the on-demand S-NSSAI from the allowed NSSAI over the corresponding access type, the AMF shall include the new allowed NSSAI in the CONFIGURATION UPDATE COMMAND message.

If:

-	the AMF needs to enforce a change in the restriction on the use of enhanced coverage or use of CE mode B as described in subclause 5.3.18;

-	the AMF decides to inform a UE in 5GMM-CONNECTED mode and registered for disaster roaming services, that a disaster condition is no longer applicable; or

-	the AMF needs to redirect the UE to EPC as described in subclause 4.8.4A.2;

NOTE 1A:	The case of the AMF triggering a generic UE configuration update procedure to inform a UE registered for disaster roaming services that a disaster condition is no longer applicable, is only applicable for a UE already in 5GMM-CONNECTED mode.

the AMF shall indicate "registration requested" in the Registration requested bit of the Configuration update indication IE and "release of N1 NAS signalling connection not required" in the Signalling connection maintain request bit of the Additional configuration indication IE in the CONFIGURATION UPDATE COMMAND message.

If the AMF needs to change the SMSF associated with the UE, the AMF may indicate "registration requested" in the Registration requested bit of the Configuration update indication IE and "release of N1 NAS signalling connection not required" in the Signalling connection maintain request bit of the Additional configuration indication IE in the CONFIGURATION UPDATE COMMAND message.

If a network slice-specific authentication and authorization procedure for an S-NSSAI is completed as a:

a)	success, the AMF shall include this S-NSSAI in the allowed NSSAI over the same access of the requested S-NSSAI or in the partially allowed NSSAI over 3GPP access; or

b)	failure, the AMF shall include this S-NSSAI in the rejected NSSAI for the failed or revoked NSSAA with the rejection cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" over either 3GPP access or non-3GPP access.

If authorization is revoked for an S-NSSAI that is in the current allowed NSSAI for an access type, the AMF shall:

a)	provide a new allowed NSSAI to the UE, excluding the S-NSSAI for which authorization is revoked; and

b)	provide a new rejected NSSAI for the failed or revoked NSSAA, including the S-NSSAI in the rejected NSSAI for which the authorization is revoked, with the rejection cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization".

The allowed NSSAI, the partially allowed NSSAI and the rejected NSSAI shall be included in the CONFIGURATION UPDATE COMMAND message to reflect the result of the procedures subject to network slice-specific authentication and authorization.

NOTE 2:	If there are multiple S-NSSAIs subject to network slice-specific authentication and authorization, it is implementation specific if the AMF informs the UE about the outcome of the procedures in one or more CONFIGURATION UPDATE COMMAND messages.

If the AMF includes the Network slicing indication IE in the CONFIGURATION UPDATE COMMAND message with the Network slicing subscription change indication set to "Network slicing subscription changed", and changes to the allowed NSSAI require the UE to initiate a registration procedure, but the AMF is unable to determine an allowed NSSAI for the UE as specified in 3GPP TS 23.501 [8], then the CONFIGURATION UPDATE COMMAND message shall additionally indicate "registration requested" in the Registration requested bit of the Configuration update indication IE and shall not include an allowed NSSAI.

If EAC mode is activated for an S-NSSAI, the AMF shall perform NSAC for the S-NSSAI subject to NSAC before such S-NSSAI is included in the allowed NSSAI in the CONFIGURATION UPDATE COMMAND message. If EAC mode is deactivated for an S-NSSAI, the AMF shall perform NSAC for the S-NSSAI subject to NSAC after such S-NSSAI is included in the allowed NSSAI in the CONFIGURATION UPDATE COMMAND message.

If the UE supports extended rejected NSSAI and the AMF determines that maximum number of UEs reached for one or more S-NSSAI(s) in the allowed NSSAI as specified in subclause 4.6.2.5, the AMF shall include the rejected NSSAI containing one or more S-NSSAIs with the rejection cause "S-NSSAI not available due to maximum number of UEs reached" in the Extended rejected NSSAI IE in the CONFIGURATION UPDATE COMMAND message. In addition, the AMF may include a back-off timer value for each S-NSSAI with the rejection cause "S-NSSAI not available due to maximum number of UEs reached" included in the Extended rejected NSSAI IE of the CONFIGURATION UPDATE COMMAND message. To avoid that large numbers of UEs simultaneously initiate deferred requests, the network should select the value for the backoff timer for each S-NSSAI for the informed UEs so that timeouts are not synchronised.

If the UE does not indicate support for extended rejected NSSAI and the maximum number of UEs has been reached, the AMF should include the rejected NSSAI containing one or more S-NSSAIs with the rejection cause "S-NSSAI not available in the current registration area" in the Rejected NSSAI IE and should not include these S-NSSAIs in the allowed NSSAI in the CONFIGURATION UPDATE COMMAND message. In addition, the AMF may based on the network policies start a local implementation specific timer for the UE per rejected S-NSSAI and upon expiration of the local implementation specific timer, the AMF may remove the rejected S-NSSAI from the rejected NSSAI and update to the UE by initiating the generic UE configuration update procedure.

NOTE 3:	Based on network policies, the AMF can include the S-NSSAI(s) for which the maximum number of UEs has been reached in the rejected NSSAI with rejection causes other than "S-NSSAI not available in the current registration area".

If the UE has set the NSAG bit to "NSAG supported" in the 5GMM capability IE of the REGISTRATION REQUEST message, the AMF may include the NSAG information IE in the CONFIGURATION UPDATE COMMAND message. Up to 4 NSAG entries are allowed to be associated with a TAI list in the NSAG information IE.

NOTE 3A:	How the AMF selects NSAG entries to be included in the NSAG information IE is implementation specific, e.g. take the NSAG priority and the current registration area into account.

NOTE 3B:	If the NSAG for the PLMN and its equivalent PLMN(s) have different associations with S-NSSAIs, then the AMF includes a TAI list for the NSAG entry in the NSAG information IE.

If the UE supports network slice replacement and the AMF determines to provide the mapping information between the S-NSSAI to be replaced and the alternative S-NSSAI to the UE, then the AMF shall include the Alternative NSSAI IE, the Allowed NSSAI IE including the alternative S-NSSAI, if not included in the current allowed NSSAI, and the Configured NSSAI IE including the alternative S-NSSAI, if not included in the current configured NSSAI, in the CONFIGURATION UPDATE COMMAND message. If the AMF determines that the replaced S-NSSAI is available, then the AMF shall provide the updated alternative NSSAI excluding the replaced S-NSSAI and the corresponding alternative S-NSSAI in the Alternative NSSAI IE in the CONFIGURATION UPDATE COMMAND message. If the AMF determines that all the replaced S-NSSAI(s) are available, then the AMF shall provide the Alternative NSSAI IE with Length of Alternative NSSAI contents set to 0 in the CONFIGURATION UPDATE COMMAND message. If the AMF determines that the replaced S-NSSAI is not supported due to the UE moving outside of NS-AoS of the S-NSSAI while the alternative S-NSSAI is available, then the AMF shall provide the updated allowed NSSAI and partially allowed NSSAI, if available, excluding the replaced S-NSSAI, if included, in the allowed NSSAI or partially allowed NSSAI in the CONFIGURATION UPDATE COMMAND message.

If the UE supports partial network slice and the AMF needs to update the partially allowed NSSAI, partially rejected NSSAI or both, then the AMF shall include the Partially allowed NSSAI IE, the Partially rejected NSSAI IE or both, in the CONFIGURATION UPDATE COMMAND message.

If the UE supports network slice usage control and the AMF needs to update the on-demand NSSAI, the AMF shall include the On-demand NSSAI IE in the CONFIGURATION UPDATE COMMAND message.

If the AMF needs to update the LADN information, the AMF shall include the LADN information in the LADN information IE of the CONFIGURATION UPDATE COMMAND message.

If the UE supports LADN per DNN and S-NSSAI and the AMF needs to update the extended LADN information, the AMF shall include the extended LADN information in the Extended LADN information IE of the CONFIGURATION UPDATE COMMAND message.

If:

-	the UE does not support LADN per DNN and S-NSSAI;

-	the UE is subscribed to the LADN DNN for a single S-NSSAI only; and

-	the AMF only has the extended LADN information;

the AMF may decide to provide the LADN service area for that LADN DNN of the extended LADN information as the LADN information and include the LADN information in the LADN information IE of the CONFIGURATION UPDATE COMMAND message.

NOTE 3C:	If the LADN service area is configured per DNN and S-NSSAI, in order to serve the UEs that do not support LADN per DNN and S-NSSAI, it is recommended that the LADN DNN is only served by a single S-NSSAI.

NOTE 3D:	In case of the UE is subscribed to the LADN DNN for multiple S-NSSAIs, the AMF can treat this as no extended LADN information is available.

NOTE 3E:	The AMF allocates the LADN service area and the TAI list associated with the S-NSSAI in the partially allowed NSSAI independently, if applicable.

If the UE does not support LADN per DNN and S-NSSAI and the AMF has neither the LADN information nor the extended LADN information, the AMF shall not provide any LADN information to the UE.

If the AMF needs to update the "CAG information list", the AMF shall include the CAG information list IE or the Extended CAG information list IE in the CONFIGURATION UPDATE COMMAND message.

NOTE 4:	If the UE supports extended CAG information list, the CAG information list can be included either in the CAG information list IE or Extended CAG information list IE.

If the UE does not support extended CAG information list, the CAG information list shall not be included in the Extended CAG information list IE.

If the AMF needs to update the "CAG information list", the UE has an emergency PDU session, and the AMF can determine that the UE is in

a)	a CAG cell and none of the CAG-ID(s) supported by the CAG cell is authorized based on the "allowed CAG list" for the current PLMN in the updated "CAG information list"; or

b)	a non-CAG cell and the entry for the current PLMN in the updated "CAG information list" includes an "indication that the UE is only allowed to access 5GS via CAG cells";

the AMF may indicate to the SMF to perform a local release of:

a)	all non-emergency single access PDU sessions associated with 3GPP access;

b)	all MA PDU sessions without a PDN connection established as a user-plane resource and without user plane resources established on non-3GPP access; and

c)	the 3GPP access user plane resources of all those MA PDU sessions with user plane resources established on both accesses.

The AMF shall not indicate to the SMF to release the emergency PDU session. If the AMF indicated to the SMF to perform a local release of:

a)	all single access non-emergency PDU sessions associated with 3GPP access;

b)	all MA PDU sessions without a PDN connection established as a user-plane resource and without user plane resources established on non-3GPP access; and

c)	the 3GPP access user plane resources of all those MA PDU sessions with user plane resources established on both accesses;

the network shall behave as if the UE is registered for emergency services over 3GPP access and shall set the emergency registered bit of the 5GS registration result IE to "Registered for emergency services" in the CONFIGURATION UPDATE COMMAND message.

If the AMF is initiating the generic UE configuration update procedure to indicate to a UE which is registered for disaster roaming services, and which has an ongoing emergency PDU session, that the UE is registered for emergency services as described in subclause 4.24, the AMF shall set the emergency registered bit of the 5GS registration result IE to "Registered for emergency services" in the CONFIGURATION UPDATE COMMAND message.

If the AMF:

-	updated the "CAG information list" to remove one or more CAG-ID(s) authorized based on the Allowed CAG list for the serving PLMN or an equivalent PLMN; or

-	updated the "CAG information list" to set the "indication that the UE is only allowed to access 5GS via CAG cells" for the serving PLMN or an equivalent PLMN which was not set before,

then upon completion of the configuration update procedure and if the UE does not have an emergency PDU session, the AMF shall initiate the release of the N1 NAS signalling connection according to subclause 5.3.1.3.

If the AMF needs to update the truncated 5G-S-TMSI configuration for a UE in NB-N1 mode using control plane CIoT 5GS optimization, the AMF shall include the Truncated 5G-S-TMSI configuration IE in the CONFIGURATION UPDATE COMMAND message.

If the AMF includes a UE radio capability ID deletion indication IE in the CONFIGURATION UPDATE COMMAND message, the AMF shall indicate "registration requested" in the Registration requested bit of the Configuration update indication IE.

If the UE is not in NB-N1 mode and the UE supports RACS, the AMF may include either a UE radio capability ID IE or a UE radio capability ID deletion indication IE in the CONFIGURATION UPDATE COMMAND message.

During an established 5GMM context, the network may send none, one, or more CONFIGURATION UPDATE COMMAND messages to the UE. If more than one CONFIGURATION UPDATE COMMAND message is sent, the messages need not have the same content.

Upon receipt of the result of the UUAA-MM procedure from the UAS-NF, the AMF shall include:

a)	the service-level-AA response with the SLAR field set to:

1)	"Service level authentication and authorization was successful" if the AMF detects the UUAA-MM procedure has succeeded; or

2)	"Service level authentication and authorization was not successful or service level authorization is revoked" if the AMF detects the UUAA-MM procedure has failed;

b)	if the CAA-Level UAV ID is provided by the UAS-NF, the service-level device ID with the value set to the CAA-Level UAV ID; and;

c)	if a payload is received from the UAS-NF:

1)	the service-level-AA payload with the value set to the payload; and

2)	if a payload type associated with the payload is received, the service-level-AA payload type with the values set to the payload type; and

3)	if the payload type associated with the payload is for C2 authorization payload, the service-level-AA response with C2AR field set to either "C2 authorization was successful" or "C2 authorization was not successful or C2 authorization is revoked" according to the authorization result received from the UAS-NF;

in the Service-level-AA container IE of the CONFIGURATION UPDATE COMMAND message.

NOTE 5:	UAS security information can be included in the UUAA payload by the USS as specified in 3GPP TS 33.256 [24B].

NOTE 5A0:	A list of USS addresses, where each entry contains a USS address and a corresponding geographical area, can be included in the UUAA payload by the USS as specified in 3GPP TS 23.256 [6AB].

NOTE 5A:	The service-level-AA payload associated with the payload type "C2 authorization payload" can include pairing information for the direct C2 communication, or the security information as specified in TS 33.256 [24B], or both.

NOTE 6:	If the AMF receives the HTTP code set to "4xx" or "5xx" as specified in 3GPP TS 29.500 [20AA] or the AMF detects that the UUAA-MM failure as specified in 3GPP TS 29.256 [21B], then the AMF considers the UUAA-MM procedure has failed.

If the AMF needs to deliver to the UE the UUAA revocation notification received from the UAS-NF, the AMF shall include the service-level-AA response with SLAR set to "Service level authentication and authorization was not successful or service level authorization is revoked" in the Service-level-AA container IE of the CONFIGURATION UPDATE COMMAND message.

If the UE supports UAS services and UAS services become enabled for the UE (e.g. because of the aerial subscription becomes a part of the UE subscription data retrieved from the UDM), the AMF may include the service-level-AA service status indication with UAS field set to "UAS services enabled" in the Service-level-AA container IE of the CONFIGURATION UPDATE COMMAND message.

If the UE supports MINT, MINT-EPS or both, the AMF may include the List of PLMNs to be used in disaster condition IE in the CONFIGURATION UPDATE COMMAND message.

If the UE supports MINT, MINT-EPS or both, the AMF may include the Disaster roaming wait range IE in the CONFIGURATION UPDATE COMMAND message.

If the UE supports MINT, MINT-EPS or both, the AMF may include the Disaster return wait range IE in the CONFIGURATION UPDATE COMMAND message.

NOTE 7:	The AMF can determine the content of the "list of PLMN(s) to be used in disaster condition", the value of the disaster roaming wait range and the value of the disaster return wait range based on the network local configuration.

If the UE supports discontinuous coverage, the AMF may include the Discontinuous coverage maximum time offset IE in the CONFIGURATION UPDATE COMMAND message.

If the UE supports and the network supports and accepts the use of the PEIPS assistance information, and the AMF needs to update the PEIPS assistance information, the AMF shall include the PEIPS assistance information in the Updated PEIPS assistance information IE of the CONFIGURATION UPDATE COMMAND message.

If the AMF needs to inform the UE that the use of access identity 1 is valid or is no longer valid, then,

1)	if the UE supports MPS indicator update via the UE configuration update procedure, the AMF:

a)	informs the UE by setting the MPS indicator bit of the Priority indicator IE to "Access identity 1 valid" or "Access identity 1 not valid" respectively, in the CONFIGURATION UPDATE COMMAND message. Based on operator policy, the AMF sets the MPS indicator bit in the CONFIGURATION UPDATE COMMAND message based on the MPS priority information in the user's subscription context obtained from the UDM; or

b)	indicates "registration requested" in the Registration requested bit of the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message; or

2)	if the UE supports MCS indicator update via the UE configuration update procedure, the AMF:

a)	informs the UE by setting the MCS indicator bit of the Priority indicator IE to "Access identity 2 valid" or "Access identity 2 not valid" respectively, in the CONFIGURATION UPDATE COMMAND message. Based on operator policy, the AMF sets the MCS indicator bit in the CONFIGURATION UPDATE COMMAND message based on the MCS priority information in the user's subscription context obtained from the UDM; or

b)	indicates "registration requested" in the Registration requested bit of the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message; or

3)	otherwise, the AMF shall indicate "registration requested" in the Registration requested bit of the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message.

If the UE has set the Reconnection to the network due to RAN timing synchronization status change (RANtiming) bit to "Reconnection to the network due to RAN timing synchronization status change supported" in the 5GMM capability IE of the REGISTRATION REQUEST message, the AMF may include the RAN timing synchronization IE with the RecReq bit set to "Reconnection requested" in the CONFIGURATION UPDATE COMMAND message.

When the UE is operating as an MBSR and its status of the MBSR authorization changes to not authorized for MBSR operation based on the UE subscription and local policy and the UE is allowed to operate as a UE, then the AMF shall include the Feature authorization indication IE in the CONFIGURATION UPDATE COMMAND message and shall set the MBSRAI field to "not authorized to operate as MBSR but allowed to operate as a UE", and the AMF can request the SMF to release the PDU session for OAM access (see 3GPP TS 23.501 [8]) provided in the serving PLMN. When the UE that was not allowed for MBSR operation becomes to be allowed for MBSR operation based on the UE subscription and local policy, then the AMF shall include the Feature authorization indication IE in the CONFIGURATION UPDATE COMMAND message and shall set the MBSRAI field to "authorized to operate as MBSR".

If the UE supports access technology utilization control and the AMF needs to update the access technology utilization control information, the AMF shall include the Access technology utilization control IE with the length of access technology utilization control contents field set to a non-zero value in the CONFIGURATION UPDATE COMMAND message. If the UE supports access technology utilization control and the AMF needs to indicate to the UE to remove the access technology utilization control information, the AMF shall include the Access technology utilization control IE with the length of access technology utilization control contents field set to zero in the CONFIGURATION UPDATE COMMAND message.

NOTE 8:	As an implementation option, the type of access technology utilization control is not set to "current PLMN and its equivalent PLMN(s)" by the AMF in the Access technology utilization control IE if the AMF does not include a list of equivalent PLMNs in the last REGISTRATION ACCEPT message.

If the UE supports and the network supports and accepts the use of the LP-WUSPS assistance information, and the AMF needs to update the LP-WUSPS assistance information, the AMF shall include the LP-WUSPS assistance information in the Updated LP-WUSPS assistance information IE with the length of LP-WUSPS assistance information contents field set to a non-zero value in the CONFIGURATION UPDATE COMMAND message. If the UE supports the LP-WUSPS assistance information and the AMF needs to remove the LP-WUSPS assistance information from the UE, the AMF shall include the Updated LP-WUSPS assistance information IE with the length of LP-WUSPS assistance information contents field set to zero in the CONFIGURATION UPDATE COMMAND message.

If the AMF needs to inform the UE of the operator policy on UE exemption for service area restrictions, and if the UE supports HPAEP, the AMF may do so by setting the HPASE bit of the Feature authorization indication IE in the CONFIGURATION UPDATE COMMAND message.

* * * Next Change * * * *

##### 5.5.1.2.2	Initial registration initiation

The UE in state 5GMM-DEREGISTERED shall initiate the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF,

a)	when the UE performs initial registration for 5GS services;

b)	when the UE performs initial registration for emergency services;

c)	when the UE performs initial registration for SMS over NAS;

d)	when the UE moves from GERAN to NG-RAN coverage or the UE moves from a UTRAN to NG-RAN coverage and the following applies:

1)	the UE initiated a GPRS attach or routing area updating procedure while in A/Gb mode or Iu mode; or

2)	the UE has performed 5G-SRVCC from NG-RAN to UTRAN as specified in 3GPP TS 23.216 [6A],

and since then the UE did not perform a successful EPS attach or tracking area updating procedure in S1 mode or registration procedure in N1 mode;

e)	when the UE performs initial registration for onboarding services in SNPN;

f)	when the UE performs initial registration for disaster roaming services; and

g)	when the UE performs initial registration to come out of unavailability period and resume normal services;

with the following clarifications to initial registration for emergency services:

a)	the UE shall not initiate an initial registration for emergency services over the current access, if the UE is already registered for emergency services over the non-current access, unless the initial registration has to be initiated to perform handover of an existing emergency PDU session from the non-current access to the current access; and

NOTE 1:	Transfer of an existing emergency PDU session between 3GPP access and non-3GPP access is needed e.g. if the UE determines that the current access is no longer available.

b)	the UE can only initiate an initial registration for emergency services over non-3GPP access if it cannot register for emergency services over 3GPP access.

The UE initiates the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF, starting timer T3510. If timer T3502 is currently running, the UE shall stop timer T3502. If timer T3511 is currently running, the UE shall stop timer T3511.

During initial registration the UE handles the 5GS mobile identity IE in the following order:

a)	if:

1)	the UE:

i)	was previously registered in S1 mode before entering state EMM-DEREGISTERED; and

ii)	has received an "interworking without N26 interface not supported" indication from the network; and

2)	EPS security context and a valid native 4G-GUTI are available;

then the UE shall create a 5G-GUTI mapped from the valid native 4G-GUTI as specified in 3GPP TS 23.003 [4] and indicate the mapped 5G-GUTI in the 5GS mobile identity IE. The UE shall include the UE status IE with the EMM registration status set to "UE is not in EMM-REGISTERED state" and shall include an ATTACH REQUEST message as specified in 3GPP TS 24.301 [15] in the EPS NAS message container IE.

Additionally, if the UE holds a valid 5G-GUTI, the UE shall include the 5G-GUTI in the Additional GUTI IE in the REGISTRATION REQUEST message in the following order:

1)	a valid 5G-GUTI that was previously assigned by the same PLMN with which the UE is performing the registration, if available;

2)	a valid 5G-GUTI that was previously assigned by an equivalent PLMN, if available; and

3)	a valid 5G-GUTI that was previously assigned by any other PLMN, if available;

b)	if:

1)	the UE is registering with a PLMN and the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP access, by the same PLMN with which the UE is performing the registration, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE; or

2)	the UE is registering with a SNPN, the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP access, by the same SNPN with which the UE is performing the registration, and the UE is not initiating the initial registration for onboarding services in SNPN, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE;

c)	if:

1)	the UE is registering with a PLMN and the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP access, by an equivalent PLMN, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE; or

2)	the UE is registering with an SNPN, the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP access, by an equivalent SNPN identified by a globally unique SNPN identity, and the UE is not initiating the initial registration for onboarding services in SNPN, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE and shall additionally include the NID of the equivalent SNPN in the NID IE;

d)	if:

1)	the UE is registering with a PLMN and the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP access, by any other PLMN, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE; or

2)	the UE is registering with an SNPN, the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP access, by any other SNPN identified by a globally unique SNPN identity, and the UE is not initiating the initial registration for onboarding services in SNPN, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE and shall additionally include the NID of the other SNPN in the NID IE;

e)	if a SUCI other than an onboarding SUCI is available, and the UE is not initiating the initial registration for onboarding services in SNPN, the UE shall include the SUCI other than an onboarding SUCI in the 5GS mobile identity IE;

f)	if the UE does not hold a valid 5G-GUTI or SUCI other than an onboarding SUCI, and is initiating the initial registration for emergency services, the PEI shall be included in the 5GS mobile identity IE; and

g)	if the UE is initiating the initial registration for onboarding services in SNPN, an onboarding SUCI shall be included in the 5GS mobile identity IE.

NOTE 2:	The AMF in ON-SNPN uses the onboarding SUCI as specified in 3GPP TS 23.501 [8].

If the SUCI is included in the 5GS mobile identity IE and the timer T3519 is not running, the UE shall start timer T3519 and store the value of the SUCI sent in the REGISTRATION REQUEST message. The UE shall include the stored SUCI in the REGISTRATION REQUEST message while timer T3519 is running.

If the UE is operating in the dual-registration mode and it is in EMM state EMM-REGISTERED, the UE shall include the UE status IE with the EMM registration status set to "UE is in EMM-REGISTERED state".

NOTE 3:	Inclusion of the UE status IE with this setting corresponds to the indication that the UE is "moving from EPC" as specified in 3GPP TS 23.502 [9].

NOTE 4:	The value of the 5GMM registration status included by the UE in the UE status IE is not used by the AMF.

If the last visited registered TAI is available, the UE shall include the last visited registered TAI in the REGISTRATION REQUEST message.

NOTE 5:	The AMF can use the last visited registered TAI included in the REGISTRATION REQUEST message, if available, in the procedure of slice-based N3IWF selection as specified in 3GPP TS 23.502 [9] when the registration procedure is performed over non-3gpp access.

If the UE requests the use of SMS over NAS, the UE shall include the 5GS update type IE in the REGISTRATION REQUEST message with the SMS requested bit set to "SMS over NAS supported". When the 5GS update type IE is included in the REGISTRATION REQUEST for reasons other than requesting the use of SMS over NAS, and the UE does not need to register for SMS over NAS, the UE shall set the SMS requested bit of the 5GS update type IE to "SMS over NAS not supported" in the REGISTRATION REQUEST message.

If the UE supports MICO mode and requests the use of MICO mode, then the UE shall include the MICO indication IE in the REGISTRATION REQUEST message. If the UE requests to use an active time value, it shall include the active time value in the T3324 IE in the REGISTRATION REQUEST message. If the UE includes the T3324 IE, it may also request a particular T3512 value by including the Requested T3512 IE in the REGISTRATION REQUEST message. Additionally, if the UE supports strictly periodic registration timer, the UE shall set the Strictly Periodic Registration Timer Indication bit of the MICO indication IE in the REGISTRATION REQUEST message to "strictly periodic registration timer supported".

If the UE needs to use the UE specific DRX parameters, the UE shall include the Requested DRX parameters IE in the REGISTRATION REQUEST message.

If the UE is in NB-N1 mode and if the UE needs to use the UE specific DRX parameters for NB-N1 mode, the UE shall include the Requested NB-N1 mode DRX parameters IE in the REGISTRATION REQUEST message.

If the UE supports eDRX and requests the use of eDRX, the UE shall include the Requested extended DRX parameters IE in the REGISTRATION REQUEST message.

If the UE needs to request LADN information for specific LADN DNN(s) or indicates a request for LADN information as specified in 3GPP TS 23.501 [8], the UE shall include the LADN indication IE in the REGISTRATION REQUEST message and:

a)	request specific LADN DNNs by including a LADN DNN value in the LADN indication IE for each LADN DNN for which the UE requests LADN information; or

b)	to indicate a request for LADN information by not including any LADN DNN value in the LADN indication IE.

The UE shall include the requested NSSAI containing the S-NSSAI(s) corresponding to the slice(s) to which the UE intends to register with and shall include the mapped S-NSSAI(s) for the requested NSSAI, if mapped S-NSSAI(s) were provided by the network, in the REGISTRATION REQUEST message. If the UE has allowed NSSAI, partially allowed NSSAI or configured NSSAI for the current PLMN or SNPN, the requested NSSAI shall be either:

a)	the configured NSSAI for the current PLMN or SNPN, or a subset thereof as described below;

b)	the allowed NSSAI or partially allowed NSSAI for the current PLMN or SNPN, or a subset thereof as described below; or

c)	the allowed NSSAI or partially allowed NSSAI for the current PLMN or SNPN, or a subset thereof as described below, plus the configured NSSAI for the current PLMN or SNPN, or a subset thereof as described below.

If both the S-NSSAI to be replaced and the alternative S-NSSAI are included in the configured NSSAI, and the UE needs to request the S-NSSAI to be replaced, the UE shall include the S-NSSAI to be replaced in the requested NSSAI.

If the UE supports the S-NSSAI time validity information, S-NSSAI time validity information is available for an S-NSSAI, and the S-NSSAI time validity information indicates that the S-NSSAI is not available, the UE shall not include the S-NSSAI in the Requested NSSAI IE of the REGISTRATION REQUEST message. If the UE has S-NSSAI time validity information over the other access in the same PLMN and the S-NSSAI time validity information indicates that the S-NSSAI is not available, the UE shall not include the S-NSSAI in the Requested NSSAI IE of the REGISTRATION REQUEST message for the current access type.

If the UE has neither allowed NSSAI, partially allowed NSSAI for the current PLMN or SNPN nor configured NSSAI for the current PLMN or SNPN and has a default configured NSSAI, the UE shall:

a)	include the S-NSSAI(s) in the Requested NSSAI IE of the REGISTRATION REQUEST message using the default configured NSSAI; and

b)	include the Network slicing indication IE with the Default configured NSSAI indication bit set to "Requested NSSAI created from default configured NSSAI" in the REGISTRATION REQUEST message.

If the UE has no allowed NSSAI or partially allowed NSSAI for the current PLMN or SNPN, no configured NSSAI for the current PLMN or SNPN, and no default configured NSSAI, the UE shall not include a requested NSSAI in the REGISTRATION REQUEST message.

If all the S-NSSAI(s) corresponding to the slice(s) to which the UE intends to register are included in the pending NSSAI, the UE shall not include a requested NSSAI in the REGISTRATION REQUEST message.

The subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to the current PLMN or SNPN, where any included S-NSSAI is:

a)	neither in the rejected NSSAI nor associated to an S-NSSAI in the rejected NSSAI. If the UE is inside the NS-AoS of an S-NSSAI in the rejected NSSAI with a rejection cause value set to "S-NSSAI not available in the current registration area", the S-NSSAI may be included in the requested NSSAI; or

b)	in the partially rejected NSSAI and the current TAI is not in the list of TAs for which the S-NSSAI is rejected. If the UE is inside the NS-AoS of an S-NSSAI in the partially rejected NSSAI and the current TAI is in the list of TAs for which the S-NSSAI is rejected, the S-NSSAI may be included in the requested NSSAI.

In addition, if the NSSRG information is available, the subset of configured NSSAI provided in the requested NSSAI shall be associated with at least one common NSSRG value. The UE may also include in the requested NSSAI, the S-NSSAI(s) which were added to configured NSSAI in S1 mode and for which the associated NSSRG information is not available. If the UE is in 5GMM-REGISTERED state over the other access and has already an allowed NSSAI for the other access in the same PLMN or in different PLMNs, all the S-NSSAI(s) in the requested NSSAI for the current access shall share at least an NSSRG value common to all the S-NSSAI(s) of the allowed NSSAI for the other access. If the UE is simultaneously performing the registration procedure on the other access in different PLMNs, the UE shall include S-NSSAIs that share at least a common NSSRG value across all access types. If the UE has pending NSSAI which the UE is still interested in using, then S-NSSAIs in the pending NSSAI and requested NSSAI shall be associated with at least one common NSSRG value.

NOTE 6:	If the UE has stored mapped S-NSSAI(s) for the rejected NSSAI, and one or more S-NSSAIs in the stored mapped S-NSSAI(s) for the configured NSSAI are not included in the stored mapped S-NSSAI(s) for the rejected NSSAI, then a S-NSSAI in the configured NSSAI associated to one or more of these mapped S-NSSAI(s) for the configured NSSAI are available to be included in the requested NSSAI together with their mapped S-NSSAI (if the mapped S-NSSAI was provided by the network).

NOTE 7:	If one or more mapped S-NSSAIs in the stored mapped S-NSSAI(s) for the configured NSSAI are not included in the stored rejected NSSAI for the failed or revoked NSSAA, a S-NSSAI in the configured NSSAI associated to one or more of these mapped S-NSSAI(s) for the configured NSSAI are available to be included in the registration request together with their mapped S-NSSAI (if the mapped S-NSSAI was provided by the network).

NOTE 8:	There is no need to consider the case that the UE is simultaneously performing the registration procedure on the other access in the same PLMN, due to that the UE is not allowed to initiate the registration procedure over one access when the registration over the other access to the same PLMN is going on.

The subset of allowed NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the allowed NSSAI or partially allowed NSSAI for the current PLMN.

NOTE 9:	How the UE selects the subset of configured NSSAI or allowed NSSAI or partially allowed NSSAI to be provided in the requested NSSAI is implementation specific. The UE can take preferences indicated by the upper layers (e.g. policies like URSP, applications) and UE local configuration into account.

NOTE 10:	The number of S-NSSAI(s) included in the requested NSSAI cannot exceed eight.

If the UE initiates an initial registration for onboarding services in SNPN, the UE shall not include the Requested NSSAI IE in the REGISTRATION REQUEST message.

If the UE supports NSAG, the UE shall set the NSAG bit to "NSAG supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports sending of REGISTRATION COMPLETE message for acknowledging the reception of NSAG information IE in the REGISTRATION ACCEPT message, the UE shall set the RCMAN bit to "Sending of REGISTRATION COMPLETE message for NSAG information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE initiates an initial registration for emergency services or needs to prolong the established NAS signalling connection after the completion of the initial registration procedure (e.g. due to uplink signalling pending), the UE shall set the Follow-on request indicator to "Follow-on request pending".

NOTE 11:	The UE does not have to set the Follow-on request indicator to 1, even if the UE has to request resources for V2X communication over PC5 reference point, 5G ProSe direct discovery over PC5, 5G ProSe direct communication over PC5 or A2X communication over PC5 reference point.

If the UE supports S1 mode and the UE has not disabled its E-UTRA capability and the 5GS registration type IE in the REGISTRATION REQUEST message is not set to "disaster roaming initial registration", the UE shall:

-	set the S1 mode bit to "S1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message;

-	include the S1 UE network capability IE in the REGISTRATION REQUEST message; additionally, if the UE supports EPS-UPIP, the UE shall set the EPS-UPIP bit to "EPS-UPIP supported" in the S1 UE network capability IE in the REGISTRATION REQUEST message; and

-	if the UE supports sending an ATTACH REQUEST message containing a PDN CONNECTIVITY REQUEST message with request type set to "handover" to transfer a PDU session from N1 mode to S1 mode, set the HO attach bit to "attach request message containing PDN connectivity request with request type set to handover to transfer PDU session from N1 mode to S1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the LTE positioning protocol (LPP) in N1 mode as specified in 3GPP TS 37.355 [26], the UE shall set the LPP bit to "LPP in N1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the Location Services (LCS) notification mechanisms in N1 mode as specified in 3GPP TS 23.273 [6B], the UE shall set the 5G-LCS bit to "LCS notification mechanisms supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the user plane positioning using LCS-UPP as specified in 3GPP TS 23.273 [6B], the UE shall set the LCS-UPP bit to "LCS-UPP user plane positioning supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports multiple LCS secured user plane connections as specified in 3GPP TS 23.273 [6B], the UE shall set the MLCSUP bit to "multiple LCS secured user plane connections supported" and the LCS-UPP bit to "LCS-UPP user plane positioning supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the user plane positioning using SUPL as described in 3GPP TS 38.305 [67] and 3GPP TS 23.271 [68], the UE shall set the SUPL bit to "SUPL user plane positioning supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports network verified UE location over satellite NG-RAN as specified in 3GPP TS 23.501 [8], the UE shall set the NVL-SATNR bit to "Network verified UE location over satellite NG-RAN supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE is in NB-N1 mode, then the UE shall set the Control plane CIoT 5GS optimization bit to "Control plane CIoT 5GS optimization supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE is capable of NB-S1 mode, then the UE shall set the Control plane CIoT EPS optimization bit to "Control plane CIoT EPS optimization supported" in the S1 UE network capability IE of the REGISTRATION REQUEST message.

If the UE supports N3 data transfer and multiple user-plane resources in NB-N1 mode (see 3GPP TS 36.306 [25D], 3GPP TS 36.331 [25A]), then the UE shall set the Multiple user-plane resources support bit to "Multiple user-plane resources supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports 5G-SRVCC from NG-RAN to UTRAN as specified in 3GPP TS 23.216 [6A], the UE shall:

-	set the 5G-SRVCC from NG-RAN to UTRAN capability bit to "5G-SRVCC from NG-RAN to UTRAN supported" in the 5GMM capability IE of the REGISTRATION REQUEST message; and

-	include the Mobile station classmark 2 IE and the Supported codecs IE in the REGISTRATION REQUEST message.

If the UE supports service gap control, then the UE shall set the SGC bit to "service gap control supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the restriction on use of enhanced coverage, the UE shall set the RestrictEC bit to "Restriction on use of enhanced coverage supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports network slice-specific authentication and authorization, the UE shall set the NSSAA bit to "network slice-specific authentication and authorization supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports CAG feature, the UE shall set the CAG bit to "CAG Supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports extended CAG information list, the UE shall set the Ex-CAG bit to "Extended CAG information list supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports enhanced CAG information, the UE shall set the ECI bit to "enhanced CAG information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports network slice replacement, the UE shall set the NSR bit to "network slice replacement supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports sending of REGISTRATION COMPLETE message for acknowledging the reception of Negotiated PEIPS assistance information IE in the REGISTRATION ACCEPT message, the UE shall set the RCMP bit to "Sending of REGISTRATION COMPLETE message for negotiated PEIPS assistance information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

When the UE is not in NB-N1 mode, if the UE supports RACS, the UE shall:

a)	set the RACS bit to "RACS supported" in the 5GMM capability IE of the REGISTRATION REQUEST message;

b)	if the UE has an applicable network-assigned UE radio capability ID for the current UE radio configuration in the selected PLMN or SNPN, include the applicable network-assigned UE radio capability ID in the UE radio capability ID IE of the REGISTRATION REQUEST message; and

c)	if the UE:

1)	does not have an applicable network-assigned UE radio capability ID for the current UE radio configuration in the selected PLMN or SNPN; and

2)	has an applicable manufacturer-assigned UE radio capability ID for the current UE radio configuration,

include the applicable manufacturer-assigned UE radio capability ID in the UE radio capability ID IE of the REGISTRATION REQUEST message.

If the UE has one or more stored UE policy sections:

-	identified by a UPSI with the PLMN ID part indicating the HPLMN or the selected PLMN; or

-	identified by a UPSI with the PLMN ID part indicating the PLMN ID part of the SNPN identity of the selected SNPN and associated with the NID of the selected SNPN;

then the UE shall set the Payload container type IE to "UE policy container" and include the UE STATE INDICATION message (see annex D) in the Payload container IE of the REGISTRATION REQUEST message.

If the UE does not have any stored UE policy section:

-	identified by a UPSI with the PLMN ID part indicating the HPLMN or the selected PLMN; or

-	identified by a UPSI with the PLMN ID part indicating the PLMN ID part of the SNPN identity of the selected SNPN and associated with the NID of the selected SNPN;

and the UE needs to send a UE policy container to the network, then the UE shall set the Payload container type IE to "UE policy container" and include the UE STATE INDICATION message (see annex D) in the Payload container IE of the REGISTRATION REQUEST message.

NOTE 12:	In this version of the protocol, the UE can only include the Payload container IE in the REGISTRATION REQUEST message to carry a payload of type "UE policy container".

If the UE does not have a valid 5G NAS security context, the UE shall send the REGISTRATION REQUEST message without including the NAS message container IE. The UE shall include the entire REGISTRATION REQUEST message (i.e. containing cleartext IEs and non-cleartext IEs, if any) in the NAS message container IE that is sent as part of the SECURITY MODE COMPLETE message as described in subclauses 4.4.6 and 5.4.2.3.

If the UE has a valid 5G NAS security context and the UE needs to send non-cleartext IEs, the UE shall send a REGISTRATION REQUEST message including the NAS message container IE as described in subclause 4.4.6. If the UE does not need to send non-cleartext IEs, the UE shall send a REGISTRATION REQUEST message without including the NAS message container IE.

If the UE supports ciphered broadcast assistance data and needs to obtain new ciphering keys, the UE shall include the Additional information requested IE with the CipherKey bit set to "ciphering keys for ciphered broadcast assistance data requested" in the REGISTRATION REQUEST message.

The UE shall set the WUSA bit to "WUS assistance information reception supported" in the 5GMM capability IE if the UE supports WUS assistance information. The UE may include its UE paging probability information in the Requested WUS assistance information IE if the UE has set the WUSA bit to "WUS assistance information reception supported" in the 5GMM capability IE and the UE is not performing the initial registration for emergency services.

The UE shall set the NR-PSSI bit to "NR paging subgrouping supported" in the 5GMM capability IE if the UE supports PEIPS assistance information and the 5GS registration type IE in the REGISTRATION REQUEST message is not set to "emergency registration". The UE may include its UE paging probability information in the Requested PEIPS assistance information IE if the UE has set the NR-PSSI bit to "NR paging subgrouping supported" in the 5GMM capability IE.

The UE shall set the LP-WUSPSAI-SI bit to "LP-WUSPS assistance information supported" in the 5GMM capability IE if the UE supports the LP-WUSPS assistance information. The UE may include its UE paging probability information in the Requested LP-WUSPS assistance information IE if the UE has set the LP-WUSPSAI-SI bit to "LP-WUSPS assistance information supported" in the 5GMM capability IE.

If the REGISTRATION REQUEST message includes a NAS message container IE, the AMF shall process the REGISTRATION REQUEST message that is obtained from the NAS message container IE as described in subclause 4.4.6.

If the UE supports V2X as specified in 3GPP TS 24.587 [19B], the UE shall set the V2X bit to "V2X supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports V2X communication over E-UTRA-PC5 as specified in 3GPP TS 24.587 [19B], the UE shall set the V2XCEPC5 bit to "V2X communication over E-UTRA-PC5 supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports V2X communication over NR-PC5 as specified in 3GPP TS 24.587 [19B], the UE shall set the V2XCNPC5 bit to "V2X communication over NR-PC5 supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

The UE shall set the ER-NSSAI bit to "Extended rejected NSSAI supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the NSSRG, then the UE shall set the NSSRG bit to "NSSRG supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the W-AGF acting on behalf of an N5GC device initiates an initial registration as specified in 3GPP TS 23.316 [6D], the W-AGF acting on behalf of the N5GC device shall include the N5GC indication IE with the N5GC device indication bit set to "N5GC device registration is requested" in the REGISTRATION REQUEST message.

If the 5G-RG acting on behalf of an AUN3 device initiates an initial registration as specified in 3GPP TS 23.316 [6D], the 5G-RG acting on behalf of an AUN3 device shall include the AUN3 indication IE with the AUN3 device indication bit set to "AUN3 device registration is requested" in the REGISTRATION REQUEST message.

If the UE supports UAS services, the UE shall set the UAS bit to "UAS services supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports A2X over E-UTRA-PC5 as specified in 3GPP TS 24.577 [60], the UE shall set the A2XEPC5 bit to "A2X over E-UTRA-PC5 supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports A2X over NR-PC5 as specified in 3GPP TS 24.577 [60], the UE shall set the A2XNPC5 bit to "A2X over NR-PC5 supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports A2X over Uu as specified in 3GPP TS 24.577 [60], the UE shall set the A2X-Uu bit to "A2X over Uu supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

When the UE supporting UAS services initiates an initial registration for UAS services, the UE shall include the service-level device ID in the Service-level-AA container IE of the REGISTRATION REQUEST message and set the value to the CAA-level UAV ID. The UE shall include the service-level-AA server address in the Service-level-AA container IE of the REGISTRATION REQUEST message and set the value to the USS address, if it is provided by the upper layers. The UE shall include the service-level-AA payload in the Service-level-AA container IE of the REGISTRATION REQUEST message and shall set the service-level-AA payload type, if the service-level-AA payload is provided by upper layers.

NOTE 13:	The service-level-AA payload can be of type "C2 authorization payload". The C2 authorization payload can include one or both of an indication of the request for direct C2 communication and pairing information for direct C2 communication.

If the UE supports 5G ProSe direct discovery as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-dd bit to "5G ProSe direct discovery supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports 5G ProSe direct communication as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-dc bit to "5G ProSe direct communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-2 UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2relay bit to "Acting as a 5G ProSe layer-2 UE-to-network relay UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-3 UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3relay bit to "Acting as a 5G ProSe layer-3 UE-to-network relay UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports acting as 5G ProSe layer-2 UE-to-network remote UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2rmt bit to "Acting as a 5G ProSe layer-2 UE-to-network remote UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-3 UE-to-network remote UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3rmt bit to "Acting as a 5G ProSe layer-3 UE-to-network remote UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-2 UE-to-UE relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2U2U relay bit to "Acting as a 5G ProSe layer-2 UE-to-UE relay UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-3 UE-to-UE relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3U2U relay bit to "Acting as a 5G ProSe layer-3 UE-to-UE relay UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-2 end UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2end bit to "Acting as a 5G ProSe layer-2 end UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-3 end UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3end bit to "Acting as a 5G ProSe layer-3 end UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports acting as 5G ProSe multi-hop layer-3 UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3relay bit to "Acting as a 5G ProSe layer-3 UE-to-network relay UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe multi-hop layer-3 UE-to-network remote UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3rmt bit to "Acting as a 5G ProSe layer-3 UE-to-network remote UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe multi-hop layer-2 UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2relay bit to "Acting as a 5G ProSe layer-2 UE-to-network relay UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe multi-hop layer-2 UE-to-network remote UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2rmt bit to "Acting as a 5G ProSe layer-2 UE-to-network remote UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe multi-hop layer-3 UE-to-UE relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3U2U relay bit to "Acting as a 5G ProSe layer-3 UE-to-UE relay UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe multi-hop layer-3 end UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3end bit to "Acting as a 5G ProSe layer-3 end UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-3 intermediate UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-L3IMrelay bit to "Acting as a 5G ProSe layer-3 intermediate UE-to-network relay UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-2 intermediate UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-L2IMrelay bit to "Acting as a 5G ProSe layer-2 intermediate UE-to-network relay UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the MUSIM UE supports the N1 NAS signalling connection release, then the UE shall set the N1 NAS signalling connection release bit to "N1 NAS signalling connection release supported" in the 5GMM capability IE of the REGISTRATION REQUEST message otherwise the UE shall not set the N1 NAS signalling connection release bit to "N1 NAS signalling connection release supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the MUSIM UE supports the paging indication for voice services, then the UE shall set the paging indication for voice services bit to "paging indication for voice services supported" in the 5GMM capability IE of the REGISTRATION REQUEST message otherwise the UE shall not set the paging indication for voice services bit to "paging indication for voice services supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the MUSIM UE supports the reject paging request, then the UE shall set the reject paging request bit to "reject paging request supported" in the 5GMM capability IE of the REGISTRATION REQUEST message otherwise the UE shall not set the reject paging request bit to "reject paging request supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the MUSIM UE sets:

-	the reject paging request bit to "reject paging request supported";

-	the N1 NAS signalling connection release bit to "N1 NAS signalling connection release supported"; or

-	both of them;

and supports the paging restriction, then the UE shall set the paging restriction bit to "paging restriction supported" in the 5GMM capability IE of the REGISTRATION REQUEST message otherwise the UE shall not set the paging restriction bit to "paging restriction supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports MINT, the UE shall set the MINT bit to "MINT supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports MINT-EPS, the UE shall set the MINT-EPS bit to "MINT-EPS supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports slice-based N3IWF selection, the UE shall set the SBNS bit to "Slice-based N3IWF selection supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports slice-based TNGF selection, the UE shall set the SBTS bit to "Slice-based TNGF selection supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE initiates the registration procedure for disaster roaming services, the UE has determined the UE determined PLMN with disaster condition as specified in 3GPP TS 23.122 [5] and:

a)	the UE determined PLMN with disaster condition is the HPLMN and:

1)	the Additional GUTI IE is included in the REGISTRATION REQUEST message and does not contain a valid 5G-GUTI that was previously assigned by the HPLMN; or

2)	the Additional GUTI IE is not included in the REGISTRATION REQUEST message and the 5GS mobile identity IE contains neither the SUCI nor a valid 5G-GUTI that was previously assigned by the HPLMN; or

b)	the UE determined PLMN with disaster condition is not the HPLMN and:

1)	the Additional GUTI IE is included in the REGISTRATION REQUEST message and does not contain a valid 5G-GUTI that was previously assigned by the UE determined PLMN with disaster condition; or

2)	the Additional GUTI IE is not included in the REGISTRATION REQUEST message and the 5GS mobile identity IE does not contain a valid 5G-GUTI that was previously assigned by the UE determined PLMN with disaster condition;

the UE shall include in the REGISTRATION REQUEST message the UE determined PLMN with disaster condition IE indicating the UE determined PLMN with disaster condition.

NOTE 14:		If the UE initiates the registration procedure for disaster roaming services, and the UE determined PLMN with disaster condition cannot be determined when an NG-RAN cell of the PLMN broadcasts the disaster related indication as specified in 3GPP TS 23.122 [5], the UE does not include in the REGISTRATION REQUEST message the UE determined PLMN with disaster condition IE but includes the Additional GUTI IE or the 5GS mobile identity IE or both as specified in subclauses 5.5.1.2.2.

If the UE supports event notification, the UE shall set the EventNotification bit to "Event notification supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports access to an SNPN using credentials from a credentials holder and:

a)	the UE is in its HPLMN or EHPLMN or the subscribed SNPN; or

b)	the UE is in a non-subscribed SNPN and supports equivalent SNPNs;

the UE shall set the SSNPNSI bit to "SOR-SNPN-SI supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports equivalent SNPNs, the UE shall set the ESI bit to "equivalent SNPNs supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the unavailability period, the UE shall set the UN-PER bit to "unavailability period supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the reconnection to the network due to RAN timing synchronization status change, the UE shall set the Reconnection to the network due to RAN timing synchronization status change (RANtiming) bit to "Reconnection to the network due to RAN timing synchronization status change supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports LADN per DNN and S-NSSAI, the UE shall set the LADN-DS bit to "LADN per DNN and S-NSSAI supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports MPS indicator update via the UE configuration update procedure, the UE shall set the MPSIU bit to "MPS indicator update supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports MCS indicator update via the UE configuration update procedure, the UE shall set the MCSIU bit to "MCS indicator update supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports ranging and sidelink positioning as specified in 3GPP TS 24.514 [62] and supports:

a)	V2X communication over PC5 as specified in 3GPP TS 24.587 [19B];

b)	5G ProSe direct discovery and 5G ProSe direct communication as specified in 3GPP TS 24.554 [19E]; or

c)	both a) and b),

the UE shall set

a)	the RSLP bit to "Ranging and sidelink positioning supported" if the UE supports ranging and sidelink positioning as target UE or SL reference UE or both;

b)	the RSLPL bit to "Ranging and sidelink positioning for located UE supported" if the UE supports ranging and sidelink positioning as located UE;

c)	the RSLPS bit to "Ranging and sidelink positioning for SL positioning server UE supported" if the UE supports ranging and sidelink positioning as SL positioning server UE; or

d)	any combination of a), b) and c), in the 5GMM capability IE of the REGISTRATION REQUEST message;

and in addition:

a)	if the UE supports ranging and sidelink positioning with V2X capable UE, the UE shall set the RSLPVU bit to "Ranging and sidelink positioning with V2X capable UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message; and

b)	if the UE supports ranging and sidelink positioning with 5G ProSe capable UE, the UE shall set the RSLPPU bit to "Ranging and sidelink positioning with 5G ProSe capable UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the partial network slice, the UE shall set the PNS bit to "Partial network slice supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports network slice usage control, the UE shall set the NSUC bit to "Network slice usage control supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the S-NSSAI time validity information, the UE shall set the TempNS bit to "S-NSSAI time validity information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the S-NSSAI location validity information, the UE shall set the SLVI bit to "S-NSSAI location validity information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports access technology utilization control, the UE shall set the ATUC bit to "access technology utilization control supported" in the 5GMM capability IE of the REGISTRATION REQUEST message over 3GPP access.

If the UE supports HPAEOP, the UE shall set the OPHPAE bit to "Operator policy for high priority access exemption for service area restrictions is supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

Figure 5.5.1.2.2.1: Registration procedure for initial registration

* * * Next Change * * * *

##### 5.5.1.3.2	Mobility and periodic registration update initiation

The UE in state 5GMM-REGISTERED shall initiate the registration procedure for mobility and periodic registration update by sending a REGISTRATION REQUEST message to the AMF,

a)	when the UE detects that the current TAI is not in the list of tracking areas that the UE previously registered in the AMF;

b)	when the periodic registration updating timer T3512 expires in 5GMM-IDLE mode and the UE is not registered for emergency services (see subclause 5.3.7);

c)	when the UE receives a CONFIGURATION UPDATE COMMAND message indicating "registration requested" in the Registration requested bit of the Configuration update indication IE as specified in subclauses 5.4.4.3;

d)	when the UE in state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE either receives a paging or the UE receives a NOTIFICATION message with access type indicating 3GPP access over the non-3GPP access for PDU sessions associated with 3GPP access;

NOTE 1:	As an implementation option, MUSIM UE is allowed to not respond to paging based on the information available in the paging message, e.g. voice service indication.

e)	upon inter-system change from S1 mode to N1 mode and if the UE previously had initiated an attach procedure or a tracking area updating procedure when in S1 mode;

f)	when the UE receives an indication of "RRC Connection failure" from the lower layers and does not have signalling pending (i.e. when the lower layer requests NAS signalling connection recovery);

g)	when the UE changes the 5GMM capability or the S1 UE network capability or both;

h)	when the UE's usage setting changes;

i)	when the UE needs to change the slice(s) it is currently registered to;

NOTE 1A:	The UE can after the completion of the ongoing registration procedure, initiate another registration procedure for mobility registration update to request more slices.

j)	when the UE changes the UE specific DRX parameters;

k)	when the UE in state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE receives a request from the upper layers to establish an emergency PDU session or perform emergency services fallback;

l)	when the UE needs to register for SMS over NAS, indicate a change in the requirements to use SMS over NAS, or de-register from SMS over NAS;

m)	when the UE needs to indicate PDU session status to the network after performing a local release of PDU session(s) as specified in subclauses 6.4.1.5 and 6.4.3.5;

n)	when the UE in 5GMM-IDLE mode changes the radio capability for NG-RAN or E-UTRAN;

o)	when the UE receives a fallback indication from the lower layers and does not have signalling pending, see subclauses 5.3.1.4 and 5.3.1.2);

p)	void;

q)	when the UE needs to request new LADN information;

r)	when the UE needs to request the use of MICO mode or needs to stop the use of MICO mode or to request the use of new T3324 value or new T3512 value;

s)	when the UE in 5GMM-CONNECTED mode with RRC inactive indication enters a cell in the current registration area belonging to an equivalent PLMN of the registered PLMN and not belonging to the registered PLMN;

t)	when the UE receives over 3GPP access a SERVICE REJECT message or a DL NAS TRANSPORT message, with the 5GMM cause value set to #28 "Restricted service area";

u)	when the UE needs to request the use of eDRX, when a change in the eDRX usage conditions at the UE requires different extended DRX parameters, or needs to stop the use of eDRX;

NOTE 2:	A change in the eDRX usage conditions at the UE can include e.g. a change in the UE configuration, a change in requirements from upper layers or the battery running low at the UE.

v)	when the UE supporting 5G-SRVCC from NG-RAN to UTRAN changes the mobile station classmark 2 or the supported codecs;

w)	when the UE in state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE decides to request new network slices after being rejected due to no allowed network slices requested, or request S-NSSAI(s) which have been removed from the rejected NSSAI for the maximum number of UEs reached;

x)	when the UE is not in NB-N1 mode and the UE has received a UE radio capability ID deletion indication IE set to "Network-assigned UE radio capability IDs deletion requested";

y)	when the UE receives a REGISTRATION REJECT message with 5GMM cause values #3, #6 or #7 without integrity protection over another access;

z)	when the UE needs to request new ciphering keys for ciphered broadcast assistance data;

za)	when due to manual CAG selection the UE has selected a CAG-ID which is not a CAG-ID authorized based on the "allowed CAG list" for the selected PLMN or a CAG-ID in a PLMN for which the entry in the "CAG information list" does not exist or when the UE has selected, without selecting a CAG-ID, a PLMN for which the entry in the "CAG information list" includes an "indication that the UE is only allowed to access 5GS via CAG cells";

zb)	when the UE needs to start, stop or change the conditions for using the WUS assistance information, the PEIPS assistance information, or the LP-WUSPS assistance information;

zc)	when the UE changes the UE specific DRX parameters in NB-N1 mode;

zd)	when the UE in 5GMM-CONNECTED mode with RRC inactive indication enters a new cell with different RAT in current TAI list or not in current TAI list;

ze)	when the UE enters state 5GMM-REGISTERED.NORMAL-SERVICE or 5GMM-REGISTERED.NON-ALLOWED-SERVICE (as described in subclause 5.3.5.2) over 3GPP access after the UE has sent a NOTIFICATION RESPONSE message over non-3GPP access in response to reception of a NOTIFICATION message over non-3GPP access as specified in subclause 5.6.3.1;

zf) when the UE supporting UAS services is not registered for UAS services and needs to register to the 5GS for UAS services;

zg)	when the UE supporting MINT needs to perform the registration procedure for mobility and periodic registration update to register to the PLMN offering disaster roaming;

zh)	when the MUSIM UE supporting the paging timing collision control needs to request a new 5G-GUTI assignment and the UE is not registered for emergency services;

NOTE 3:	Based on implementation, the MUSIM UE can request a new 5G-GUTI assignment (e.g. when the lower layers request to modify the timing of the paging occasions).

zi)	when the network supports the paging restriction and the MUSIM UE in state 5GMM-REGISTERED.NON-ALLOWED-SERVICE needs to requests the network to remove the paging restriction;

zj)	when the UE changes the 5GS Preferred CIoT network behaviour or the EPS Preferred CIoT network behaviour;

zk)	when the UE that has entered 5GMM-REGISTERED.NO-CELL-AVAILABLE and it has one or more S-NSSAI(s) in pending NSSAI, finds a suitable cell according to 3GPP TS 38.304 [28];

zl)	when the UE is registered for disaster roaming services and receives a request from the upper layers to establish an emergency PDU session or perform emergency services fallback;

zm1)	when the UE needs to provide the unavailability information or to update the unavailability information;

zm2)	void;

NOTE 3A: How UE determines that it is about to lose satellite coverage is an implementation option.

zn)	when the UE needs to come out of unavailability period and resume normal services;

zo)	when the UE in state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE, the UE supports the reconnection to the network due to RAN timing synchronization status change has been requested to reconnect to the network upon receiving an indication of a change in the RAN timing synchronization status (see subclauses 5.4.4.2, 5.5.1.2.4, and 5.5.1.3.4), and the UE receives an indication of a change in the RAN timing synchronization status;

zp)	when the UE that supports non-3GPP access path switching needs to trigger non-3GPP access path switching from the old non-3GPP access to the new non-3GPP access that is in the same PLMN; or

NOTE 4:	Non-3GPP access path switching from a non-3GPP access to a wireline access, or from a wireline access to a non-3GPP access, is not specified in this release of the specification.

zq)	if the UE moves from a tracking area for which the TAI is configured for partially rejected NSSAI to another tracking area within the registration area with aTAI for which the S-NSSAI(s) is supported and the UE still needs to request that S-NSSAI(s).

NOTE 4A:	The UEs that do not initiate the registration procedure for mobility and periodic registration update in the case above will not receive service for the S-NSSAI(s) that are configured to support the S-NSSAI(s) in the tracking areas supporting the S-NSSAI(s). Operators are recomended to consider the impact of such a configuration, of having partially rejected NSSAI applicable in some tracking areas within a registration area, as it requires the UE to perform mobility and periodic registration update procedure upon mobility within the same registration area.

If case b is the only reason for initiating the registration procedure for mobility and periodic registration update, the UE shall indicate "periodic registration updating" in the 5GS registration type IE; otherwise, if the UE initiates the registration procedure for mobility and periodic registration update due to case zg, the UE shall indicate "disaster roaming mobility registration updating" in the 5GS registration type IE; otherwise the UE shall indicate "mobility registration updating".

If case zl is the reason for initiating the registration procedure for mobility and periodic registration update and if the UE supports S1 mode and the UE has not disabled its E-UTRA capability, the UE shall:

-	set the S1 mode bit to "S1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message; and

-	include the S1 UE network capability IE in the REGISTRATION REQUEST message;

If the UE which is not registered for disaster roaming services indicates "mobility registration updating" in the 5GS registration type IE and the UE supports S1 mode and the UE has not disabled its E-UTRA capability, the UE shall:

-	set the S1 mode bit to "S1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message;

-	include the S1 UE network capability IE in the REGISTRATION REQUEST message additionally, if the UE supports EPS-UPIP, the UE shall set the EPS-UPIP bit to "EPS-UPIP supported" in the S1 UE network capability IE in the REGISTRATION REQUEST message; and

-	if the UE supports sending an ATTACH REQUEST message containing a PDN CONNECTIVITY REQUEST message with request type set to "handover" to transfer a PDU session from N1 mode to S1 mode, set the HO attach bit to "attach request message containing PDN connectivity request with request type set to handover to transfer PDU session from N1 mode to S1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the LTE positioning protocol (LPP) in N1 mode as specified in 3GPP TS 37.355 [26], the UE shall set the LPP bit to "LPP in N1 mode supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports the Location Services (LCS) notification mechanisms in N1 mode as specified in 3GPP TS 23.273 [6B], the UE shall set the 5G-LCS bit to "LCS notification mechanisms supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports the user plane positioning using LCS-UPP as specified in 3GPP TS 23.273 [6B], the UE shall set the LCS-UPP bit to "LCS-UPP user plane positioning supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports multiple LCS secured user plane connections as specified in 3GPP TS 23.273 [6B], the UE shall set the MLCSUP bit to "multiple LCS secured user plane connections supported" and the LCS-UPP bit to "LCS-UPP user plane positioning supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports the user plane positioning using SUPL as specified in 3GPP TS 38.305 [67] and 3GPP TS 23.271 [68], the UE shall set the SUPL bit to "SUPL user plane positioning supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports network verified UE location over satellite NG-RAN as specified in 3GPP TS 23.501 [8], the UE shall set the NVL-SATNR bit to "Network verified UE location over satellite NG-RAN supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

For all cases except case b, when the UE is not in NB-N1 mode and the UE supports RACS, the UE shall set the RACS bit to "RACS supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports 5G-SRVCC from NG-RAN to UTRAN as specified in 3GPP TS 23.216 [6A], the UE shall set:

-	the 5G-SRVCC from NG-RAN to UTRAN capability bit to "5G-SRVCC from NG-RAN to UTRAN supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b; and

-	include the Mobile station classmark 2 IE and the Supported codecs IE in the REGISTRATION REQUEST message for all cases except case b.

If the UE supports the restriction on use of enhanced coverage, the UE shall set the RestrictEC bit to "Restriction on use of enhanced coverage supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports network slice-specific authentication and authorization, the UE shall set the NSSAA bit to "network slice-specific authentication and authorization supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports CAG feature, the UE shall set the CAG bit to "CAG Supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports extended CAG information list, the UE shall set the Ex-CAG bit to "Extended CAG information list supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports enhanced CAG information, the UE shall set the ECI bit to "enhanced CAG information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports sending of REGISTRATION COMPLETE message for acknowledging the reception of Negotiated PEIPS assistance inforation IE, the UE shall set the RCMAP bit to "Sending of REGISTRATION COMPLETE message for negotiated PEIPS assistance information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE operating in the single-registration mode performs inter-system change from S1 mode to N1 mode and:

a)	has one or more stored UE policy sections identified by a UPSI with the PLMN ID part indicating the HPLMN or the selected PLMN, the UE shall set the Payload container type IE to "UE policy container" and include the UE STATE INDICATION message (see annex D) in the Payload container IE of the REGISTRATION REQUEST message; or

b)	does not have any stored UE policy section identified by a UPSI with the PLMN ID part indicating the HPLMN or the selected PLMN, and the UE needs to send a UE policy container to the network, the UE shall set the Payload container type IE to "UE policy container" and include the UE STATE INDICATION message (see annex D) in the Payload container IE of the REGISTRATION REQUEST message.

NOTE 5:	In this version of the protocol, the UE can only include the Payload container IE in the REGISTRATION REQUEST message to carry a payload of type "UE policy container".

The UE in state 5GMM-REGISTERED shall initiate the registration procedure for mobility and periodic registration update by sending a REGISTRATION REQUEST message to the AMF when the UE needs to request the use of SMS over NAS transport or the current requirements to use SMS over NAS transport change in the UE. The UE shall set the SMS requested bit of the 5GS update type IE in the REGISTRATION REQUEST message as specified in subclause 5.5.1.2.2.

When initiating a registration procedure for mobility and periodic registration update and the UE needs to send the 5GS update type IE for a reason different than indicating a change in requirement to use SMS over NAS, the UE shall set the SMS requested bit of the 5GS update type IE in the REGISTRATION REQUEST message to the same value as indicated by the UE in the last REGISTRATION REQUEST message.

If the UE no longer requires the use of SMS over NAS, then the UE shall include the 5GS update type IE in the REGISTRATION REQUEST message with the SMS requested bit set to "SMS over NAS not supported".

After sending the REGISTRATION REQUEST message to the AMF the UE shall start timer T3510. If timer T3502 is currently running, the UE shall stop timer T3502. If timer T3511 is currently running, the UE shall stop timer T3511.

If the last visited registered TAI is available, the UE shall include the last visited registered TAI in the REGISTRATION REQUEST message.

The UE shall handle the 5GS mobile identity IE in the REGISTRATION REQUEST message as follows:

a)	if the UE is operating in the single-registration mode, performs inter-system change from S1 mode to N1 mode, and the UE holds a valid native 4G-GUTI, the UE shall create a 5G-GUTI mapped from the valid native 4G-GUTI as specified in 3GPP TS 23.003 [4] and indicate the mapped 5G-GUTI in the 5GS mobile identity IE. Additionally, if the UE holds a valid 5G-GUTI, the UE shall include the 5G-GUTI in the Additional GUTI IE in the REGISTRATION REQUEST message in the following order:

1)	a valid 5G-GUTI that was previously assigned by the same PLMN with which the UE is performing the registration, if available;

2)	a valid 5G-GUTI that was previously assigned by an equivalent PLMN, if available; and

3)	a valid 5G-GUTI that was previously assigned by any other PLMN, if available; and

NOTE 6:	The 5G-GUTI included in the Additional GUTI IE is a native 5G-GUTI.

b)	for all other cases, if the UE holds a valid 5G-GUTI, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE. If the UE is registering with an SNPN and the valid 5G-GUTI was previously assigned by another SNPN, the UE shall additionally include the NID of the other SNPN in the NID IE.

If the UE does not operate in SNPN access operation mode, holds two valid native 5G-GUTIs assigned by PLMNs and:

1)	one of the valid native 5G-GUTI was assigned by the PLMN with which the UE is performing the registration, then the UE shall indicate the valid native 5G-GUTI assigned by the PLMN with which the UE is performing the registration. In addition, the UE shall include the other valid native 5G-GUTI in the Additional GUTI IE; or

2)	none of the valid native 5G-GUTI was assigned by the PLMN with which the UE is performing the registration, then the UE shall indicate the valid native 5G-GUTI assigned over the same access via which the UE is performing the registration.

If the UE supports MICO mode and requests the use of MICO mode, then the UE shall include the MICO indication IE in the REGISTRATION REQUEST message. If the UE requests to use an active time value, it shall include the active time value in the T3324 IE in the REGISTRATION REQUEST message. If the UE includes the T3324 IE, it may also request a particular T3512 value by including the Requested T3512 value IE in the REGISTRATION REQUEST message. Additionally, if the UE supports strictly periodic registration timer, the UE shall set the Strictly Periodic Registration Timer Indication bit of the MICO indication IE in the REGISTRATION REQUEST message to "strictly periodic registration timer supported". If the UE needs to stop the use of MICO mode, then the UE shall not include the MICO indication IE in the REGISTRATION REQUEST message.

If the UE needs to use or change the UE specific DRX parameters, the UE shall include the Requested DRX parameters IE in the REGISTRATION REQUEST message for all cases except case b.

If the UE is in NB-N1 mode and if the UE needs to use or change the UE specific DRX parameters for NB-N1 mode, the UE shall include the Requested NB-N1 mode DRX parameters IE in the REGISTRATION REQUEST message for all cases except case b.

If the UE supports eDRX and requests the use of eDRX, the UE shall include the Requested extended DRX parameters IE in the REGISTRATION REQUEST message.

For all cases except case b if the UE needs to request LADN information for specific LADN DNN(s) or indicates a request for LADN information as specified in 3GPP TS 23.501 [8], the UE shall include the LADN indication IE in the REGISTRATION REQUEST message and:

-	request specific LADN DNNs by including a LADN DNN value in the LADN indication IE for each LADN DNN for which the UE requests LADN information; or

-	to indicate a request for LADN information by not including any LADN DNN value in the LADN indication IE.

For all cases except case b if the UE is initiating the registration procedure for mobility and periodic registration update, the UE may include the Uplink data status IE to indicate which PDU session(s) is:

-	not associated with control plane only indication;

-	associated with the access type the REGISTRATION REQUEST message is sent over; and

-	have pending user data to be sent over user plane or are associated with active multicast MBS session(s).

If the UE has one or more active always-on PDU sessions associated with the access type over which the REGISTRATION REQUEST message is sent and the user-plane resources for these PDU sessions are not established, and for cases triggering the REGISTRATION REQUEST message except b), the UE shall include the Uplink data status IE and indicate that the UE has pending user data to be sent for those PDU sessions. If the UE is located outside the LADN service area and inside the registration area assigned by the network, the UE shall not include the PDU session for LADN in the Uplink data status IE. If the S-NSSAI associated with the PDU session is:

a)	included in the partially allowed NSSAI and the TA where the UE is currently camped is not in list of TAs for which the S-NSSAI is allowed; or

b)	included in the S-NSSAI location validity information and the UE is outside the NS-AoS of the S-NSSAI;

then the UE shall not indicate those PDU session(s), including always-on PDU session(s), in Uplink data status IE in the REGISTRATION REQUEST message. If the UE is in a non-allowed area or is not in an allowed area as specified in subclause 5.3.5, and the UE is in the registration area assigned by the network, the UE shall not include the Uplink data status IE except for emergency services or for high priority access. If the MUSIM UE requests the network to release the NAS signalling connection, the UE shall not include the Uplink data status IE in the REGISTRATION REQUEST message.

If the UE has one or more active PDU sessions which are not accepted by the network as always-on PDU sessions and no uplink user data pending to be sent for those PDU sessions, the UE shall not include those PDU sessions in the Uplink data status IE in the REGISTRATION REQUEST message.

When the registration procedure for mobility and periodic registration update is initiated in 5GMM-IDLE mode, the UE may include a PDU session status IE in the REGISTRATION REQUEST message, indicating:

a)	which single access PDU sessions associated with the access type the REGISTRATION REQUEST message is sent over are not inactive in the UE; and

b)	which MA PDU sessions are not inactive and having the corresponding user plane resources being established or established in the UE on the access the REGISTRATION REQUEST message is sent over.

If the UE received a paging message with the access type indicating non-3GPP access, the UE shall include the Allowed PDU session status IE in the REGISTRATION REQUEST message. If the UE has PDU session(s) over non-3GPP access, where

a)	the associated S-NSSAI(s) are included in the allowed NSSAI for 3GPP access or the partially allowed NSSAI for 3GPP access and the current TAI is in the list of TAs for which the S-NSSAI is allowed; and

b)	the UE is currently located inside the NS-AoS of the S-NSSAI, if the S-NSSAI location validity information is available,

the UE shall indicate the PDU session(s) for which the UE allows to re-establish the user-plane resources over 3GPP access in the Allowed PDU session status IE; otherwise, the UE shall not indicate any PDU session(s) in the Allowed PDU session status IE. If the UE is in a non-allowed area or the UE is not in an allowed area, the UE shall set the Allowed PDU session status IE as specified in subclause 5.3.5.2.

When the Allowed PDU session status IE is included in the REGISTRATION REQUEST message, the UE shall indicate that a PDU session is not allowed to be transferred to the 3GPP access if the 3GPP PS data off UE status is "activated" for the corresponding PDU session and the UE is not using the PDU session to send uplink IP packets for any of the 3GPP PS data off exempt services (see subclause 6.2.10).

If the UE operating in the single-registration mode performs inter-system change from S1 mode to N1 mode, the UE:

a)	shall include the UE status IE with the EMM registration status set to "UE is in EMM-REGISTERED state" in the REGISTRATION REQUEST message;

NOTE 7:	Inclusion of the UE status IE with this setting corresponds to the indication that the UE is "moving from EPC" as specified in 3GPP TS 23.502 [9], subclause 4.11.1.3.3 and 4.11.2.3.

NOTE 8:	The value of the 5GMM registration status included by the UE in the UE status IE is not used by the AMF.

b)	may include the PDU session status IE in the REGISTRATION REQUEST message indicating the status of the PDU session(s) mapped during the inter-system change from S1 mode to N1 mode from the PDN connection(s) for which the EPS indicated that interworking to 5GS is supported, if any (see subclause 6.1.4.1);

c)	shall include a TRACKING AREA UPDATE REQUEST message as specified in 3GPP TS 24.301 [15] in the EPS NAS message container IE in the REGISTRATION REQUEST message if the registration procedure is initiated in 5GMM-IDLE mode and the UE has received an "interworking without N26 interface not supported" indication from the network;

c1)	may include a TRACKING AREA UPDATE REQUEST message as specified in 3GPP TS 24.301 [15] in the EPS NAS message container IE in the REGISTRATION REQUEST message if the registration procedure is initiated in 5GMM-IDLE mode and the UE has received an "interworking without N26 interface supported" indication from the network; and

d)	shall include an EPS bearer context status IE in the REGISTRATION REQUEST message indicating which EPS bearer contexts are active in the UE, if the UE has locally deactivated EPS bearer context(s) for which interworking to 5GS is supported while the UE was in S1 mode without notifying the network.

For a REGISTRATION REQUEST message with a 5GS registration type IE indicating "mobility registration updating", if the UE:

a)	is in NB-N1 mode and:

1)	the UE needs to change the slice(s) it is currently registered to within the same registration area; or

2)	the UE has entered a new registration area; or

b)	is not in NB-N1 mode and is not registered for onboarding services in SNPN;

the UE shall include the Requested NSSAI IE containing the S-NSSAI(s) corresponding to the network slices to which the UE intends to register and associated mapped S-NSSAI(s), if provided by the network, in the REGISTRATION REQUEST message as described in this subclause. When the UE is entering a visited PLMN and intends to register to the slices for which the UE has only HPLMN S-NSSAI(s) available, the UE shall include these HPLMN S-NSSAI(s) in the Requested mapped NSSAI IE. When the UE is entering an EHPLMN whose PLMN code is not derived from the IMSI and intends to register to the slices for which the UE has only HPLMN S-NSSAI(s) available, the UE shall include HPLMN S-NSSAI(s) in the Requested mapped NSSAI IE. The sum of the number of S-NSSAI values in the Requested NSSAI IE and the number of S-NSSAI values in the Requested mapped NSSAI IE shall not exceed eight.

NOTE 9:	The REGISTRATION REQUEST message can include both the Requested NSSAI IE and the Requested mapped NSSAI IE as described below.

If the UE is registered for onboarding services in SNPN, the UE shall not include the Requested NSSAI IE in the REGISTRATION REQUEST message.

For case b, the UE shall not include the Requested NSSAI IE in the REGISTRATION REQUEST message.

If the UE has allowed NSSAI, partially allowed NSSAI or configured NSSAI for the current PLMN, the Requested NSSAI IE shall include either:

a)	the configured NSSAI for the current PLMN or SNPN, or a subset thereof as described below;

b)	the allowed NSSAI or partially allowed NSSAI for the current PLMN or SNPN, or a subset thereof as described below; or

c)	the allowed NSSAI or partially allowed NSSAI for the current PLMN or SNPN, or a subset thereof as described below, plus the configured NSSAI for the current PLMN or SNPN, or a subset thereof as described below.

In addition, the Requested NSSAI IE shall include S-NSSAI(s) applicable in the current PLMN or SNPN and, if provided by the network, the associated mapped S-NSSAI(s) for:

a)	each PDN connection that is established in S1 mode when the UE is operating in the single-registration mode and the UE is performing an inter-system change from S1 mode to N1 mode; or

b)	each active PDU session.

If the UE does not have S-NSSAI(s) applicable in the current PLMN or SNPN, then the Requested mapped NSSAI IE shall include HPLMN S-NSSAI(s) (e.g. mapped S-NSSAI(s), if available) for:

a)	each PDN connection established in S1 mode when the UE is operating in the single-registration mode and the UE is performing an inter-system change from S1 mode to N1 mode to a visited PLMN; or

b)	each active PDU session when the UE is performing mobility from N1 mode to N1 mode to a visited PLMN.

NOTE 10:	The Requested NSSAI IE is used instead of Requested mapped NSSAI IE in REGISTRATION REQUEST message when the UE enters HPLMN and the EHPLMN list is not present or is empty; or when the UE enters a PLMN whose PLMN code is derived from the IMSI and the EHPLMN list is not empty. The Requested mapped NSSAI IE is used when the UE enters an EHPLMN whose PLMN code is not derived from the IMSI.

If both the S-NSSAI to be replaced and the alternative S-NSSAI are included in the configured NSSAI, and the UE needs to request the S-NSSAI to be replaced, the UE shall include the S-NSSAI to be replaced in the Requested NSSAI IE or the Requested mapped NSSAI IE.

For a REGISTRATION REQUEST message with a 5GS registration type IE indicating "mobility registration updating", if the UE is in NB-N1 mode and the procedure is initiated for all cases except case a), c), e), i), s), t), w), and x), the REGISTRATION REQUEST message shall not include the Requested NSSAI IE.

If the UE has:

-	no allowed NSSAI or partially allowed NSSAI for the current PLMN or SNPN;

-	no configured NSSAI for the current PLMN or SNPN;

-	neither active PDU session(s) nor PDN connection(s) to transfer associated with an S-NSSAI applicable in the current PLMN or SNPN; and

-	neither active PDU session(s) nor PDN connection(s) to transfer associated with mapped S-NSSAI(s);

and has a default configured NSSAI, then the UE shall:

a)	include the S-NSSAI(s) in the Requested NSSAI IE of the REGISTRATION REQUEST message using the default configured NSSAI; and

b)	include the Network slicing indication IE with the Default configured NSSAI indication bit set to "Requested NSSAI created from default configured NSSAI" in the REGISTRATION REQUEST message.

If the UE has:

-	no allowed NSSAI or partially allowed NSSAI for the current PLMN or SNPN;

-	no configured NSSAI for the current PLMN or SNPN;

-	neither active PDU session(s) nor PDN connection(s) to transfer associated with an S-NSSAI applicable in the current PLMN or SNPN

-	neither active PDU session(s) nor PDN connection(s) to transfer associated with mapped S-NSSAI(s); and

-	no default configured NSSAI,

the UE shall include neither Requested NSSAI IE nor Requested mapped NSSAI IE in the REGISTRATION REQUEST message.

If all the S-NSSAI(s) corresponding to the slice(s) to which the UE intends to register are included in the pending NSSAI, the UE shall not include a requested NSSAI in the REGISTRATION REQUEST message.

When the UE storing a pending NSSAI intends to register to additional S-NSSAI(s) over the same access type, the UE shall send the requested NSSAI containing the additional S-NSSAI(s) that the UE intends to register to in the REGISTRATION REQUEST message. The requested NSSAI shall not include any S-NSSAI from the pending NSSAI.

The subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to the current PLMN or SNPN, where any included S-NSSAI is neither in the rejected NSSAI nor associated to an S-NSSAI in the rejected NSSAI. If the UE is inside the NS-AoS of an S-NSSAI in the rejected NSSAI with a rejection cause value set to "S-NSSAI not available in the current registration area", the S-NSSAI may be included in the requested NSSAI.

For case zq, the subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to the current PLMN or SNPN, where any included S-NSSAI is in the partially rejected NSSAI and the current TAI is in the list of TAs for which the S-NSSAI is not rejected. If the UE is inside the NS-AoS of an S-NSSAI in the partially rejected NSSAI and the current TAI is in the list of TAs for which the S-NSSAI is rejected, the S-NSSAI may be included in the requested NSSAI.

In addition, if the NSSRG information is available, the subset of configured NSSAI provided in the requested NSSAI shall be associated with at least one common NSSRG value. The UE may also include in the requested NSSAI included in the Requested NSSAI IE or the Requested mapped NSSAI IE or both, the S-NSSAI(s) which were added to configured NSSAI in S1 mode and for which the associated NSSRG information is not available. If the UE is in 5GMM-REGISTERED state over the other access and has already an allowed NSSAI for the other access in the same PLMN or in different PLMNs, all the S-NSSAI(s) in the requested NSSAI included in the Requested NSSAI IE or the Requested mapped NSSAI IE or both for the current access shall share at least an NSSRG value common to all the S-NSSAI(s) of the allowed NSSAI for the other access. If the UE is simultaneously performing the registration procedure on the other access in different PLMNs, the UE shall include S-NSSAIs that share at least a common NSSRG value across all access types. If the UE has pending NSSAI which the UE is still interested in using, then S-NSSAIs in the pending NSSAI and requested NSSAI shall be associated with at least one common NSSRG value.

NOTE 11:	If the UE has stored mapped S-NSSAI(s) for the rejected NSSAI, and one or more S-NSSAIs in the stored mapped S-NSSAI(s) for the configured NSSAI are not included in the stored mapped S-NSSAI(s) for the rejected NSSAI, then a S-NSSAI in the configured NSSAI associated to one or more of these mapped S-NSSAI(s) for the configured NSSAI are available to be included in the requested NSSAI together with their mapped S-NSSAI (if the mapped S-NSSAI was provided by the network).

NOTE 12:	If one or more mapped S-NSSAIs in the stored mapped S-NSSAI(s) for the configured NSSAI are not included in the stored rejected NSSAI for the failed or revoked NSSAA, a S-NSSAI in the configured NSSAI associated to one or more of these mapped S-NSSAI(s) for the configured NSSAI are available to be included in the registration request together with their mapped S-NSSAI (if the mapped S-NSSAI was provided by the network).

NOTE 13:	There is no need to consider the case that the UE is simultaneously performing the registration procedure on the other access in the same PLMN, due to that the UE is not allowed to initiate the registration procedure over one access when the registration over the other access to the same PLMN is going on.

If:

a)	the UE is registered to current PLMN over the other access and has NSSRG information available;

b)	the UE is attempting mobility registration to the same current PLMN from other PLMN in the current access; and

c)	the UE has PDU session(s) or PDN connection(s) associated with NSSAI not sharing part of NSSRG available of the current PLMN;

then the UE locally releases these PDU session(s) or PDN connection(s), as the NSSAI for these PDU session(s) or PDN connection(s) will not be included in the requested or the requested mapped NSSAI in the current PLMN due to its lack of association to the common NSSRG of the current PLMN.

The subset of allowed NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the allowed NSSAI or partially allowed NSSAI for this PLMN.

If the UE supports the S-NSSAI time validity information, S-NSSAI time validity information is available for an S-NSSAI, and the S-NSSAI time validity information indicates that the S-NSSAI is not available, the UE shall not include the S-NSSAI in the Requested NSSAI IE of the REGISTRATION REQUEST message. If the UE has S-NSSAI time validity information over the other access in the same PLMN and the S-NSSAI time validity information indicates that the S-NSSAI is not available, the UE shall not include the S-NSSAI in the Requested NSSAI IE of the REGISTRATION REQUEST message for the current access type.

NOTE 14:	How the UE selects the subset of configured NSSAI or allowed NSSAI or partially allowed NSSAI to be provided in the requested NSSAI is implementation specific. The UE can take preferences indicated by the upper layers (e.g. policies like URSP, applications) and UE local configuration into account.

NOTE 14A:	If the UE determines the on-demand S-NSSAI for a PDU session establishment as specified in subclause 4.2.2 of 3GPP TS 24.526 [19], the UE includes the on-demand S-NSSAI in the requested NSSAI during the registration procedure.

NOTE 15:	The number of S-NSSAI(s) included in the requested NSSAI cannot exceed eight.

If the UE supports NSAG, the UE shall set the NSAG bit to "NSAG supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b. If the UE supports sending of REGISTRATION COMPLETE message for acknowledging the reception of NSAG information IE in the REGISTRATION ACCEPT message, the UE shall set the RCMAN bit to "Sending of REGISTRATION COMPLETE message for NSAG information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports the unavailability period, the UE shall set the UN-PER bit to "unavailability period supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports network slice replacement, the UE shall set the NSR bit to "network slice replacement supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

For case zm1, if the network indicated support for the unavailability period in the last registration procedure, the UE shall include the Unavailability information IE in the REGISTRATION REQUEST message. If the UE did not include a start of the unavailability period in the Unavailability information IE, the UE shall set the Follow-on request indicator to "No follow-on request pending" in the REGISTRATION REQUEST message and shall not include the Uplink data status IE or the Allowed PDU session status IE in the REGISTRATION REQUEST message even if the UE has one or more active always-on PDU sessions associated with the 3GPP access. If the UE includes the Unavailability information IE to indicate the type of the unavailability and the UE will be unavailable due to NR satellite access discontinuous coverage, the UE shall set the Unavailability type bit to "unavailability due to discontinuous coverage" in the Unavailability information IE.

For case zm1, the UE should initiate the registration procedure for mobility and periodic registration update only if the UE can determine, based on its implementation, that there is enough time to complete the procedure before the start of the unavailability period.

NOTE 15A:	If the UE is unable to store its 5GMM and 5GSM contexts, the UE triggers the de-registration procedure. Ability to store the 5GMM information for UEs not operating in SNPN access operation mode as described in subclause C.1 does not imply the ability to store the 5GMM and 5GSM contexts.

NOTE 15B:	If the UE is able to store its 5GMM and 5GSM contexts, the UE can store the 5GMM and 5GSM contexts even if the registration procedure for mobility and periodic registration update is not completed successfully.

The UE shall set the Follow-on request indicator to "Follow-on request pending", if the UE:

a)	initiates the registration procedure for mobility and periodic registration update upon request of the upper layers to establish an emergency PDU session;

b)	initiates the registration procedure for mobility and periodic registration update upon receiving a request from the upper layers to perform emergency services fallback; or

c)	needs to prolong the established NAS signalling connection after the completion of the registration procedure for mobility and periodic registration update (e.g. due to uplink signalling pending but no user data pending).

NOTE 16:	The UE does not have to set the Follow-on request indicator to 1 even if the UE has to request resources for V2X communication over PC5 reference point, 5G ProSe direct discovery over PC5, 5G ProSe direct communication over PC5 or ranging and sidelink positioning or A2X communication over PC5 reference point.

For case n, the UE shall include the 5GS update type IE in the REGISTRATION REQUEST message with the NG-RAN-RCU bit set to "UE radio capability update needed". Additionally, if the UE is not in NB-N1 mode, the UE supports RACS and the UE has an applicable UE radio capability ID for the new UE radio configuration in the serving PLMN or SNPN, the UE shall include the applicable UE radio capability ID in the UE radio capability ID of the REGISTRATION REQUEST message.

NOTE 16A:	For cases n, if the UE supports RACS irrespective whether the UE has an applicable UE radio capability ID for the new UE radio configuration in the selected PLMN the 5GS update type IE in the REGISTRATION REQUEST message with the NG-RAN-RCU bit is set to "UE radio capability update needed".

If the UE is in the 5GMM-CONNECTED mode and the UE changes the radio capability for NG-RAN or E-UTRAN, the UE may locally release the established N1 NAS signalling connection and enter the 5GMM-IDLE mode. Then, the UE shall initiate the registration procedure for mobility and periodic registration update including the 5GS update type IE in the REGISTRATION REQUEST message with the NG-RAN-RCU bit set to " UE radio capability update needed".

For case o, the UE shall include the Uplink data status IE in the REGISTRATION REQUEST message indicating the PDU session(s) without active user-plane resources for which the UE has pending user data to be sent, if any, and the PDU session(s) for which user-plane resources were active prior to receiving the fallback indication, if any. If the UE has joined one or more multicast MBS session and was in 5GMM-CONNECTED mode with RRC inactive indication before receiving the fallback indication from the lower layers, the UE shall include the Uplink data status IE in the REGISTRATION REQUEST message indicating the PDU session(s) that are associated to the one or more multicast MBS session. If the UE is in a non-allowed area or if the UE is not in allowed area, the UE shall not include the Uplink data status IE in REGISTRATION REQUEST message, except if the PDU session for which user-plane resources were active prior to receiving the fallback indication is an emergency PDU session, or if the UE is configured for high priority access in the selected PLMN or SNPN as specified in subclause 5.3.5.

For case f, the UE shall include the Uplink data status IE in the REGISTRATION REQUEST message indicating the PDU session(s) for which the UE has uplink user data pending and the PDU session(s) for which user-plane resources were active prior to receiving "RRC Connection failure" indication from the lower layers, if any. If the UE has joined one or more multicast MBS session and was in 5GMM-CONNECTED mode with RRC inactive indication before receiving the indication of "RRC Connection failure" from the lower layers or before receiving the indication that the resumption of the RRC connection has failed from the lower layers, the UE shall include the Uplink data status IE in the REGISTRATION REQUEST message indicating the PDU session(s) that are associated to the one or more multicast MBS session. If the UE is in non-allowed area or not in allowed area, the UE shall not include the Uplink data status IE in REGISTRATION REQUEST message, except that the PDU session for which user-plane resources were active prior to receiving the "RRC Connection failure"indication is emergency PDU session, or that the UE is configured for high priority access in selected PLMN or SNPN, as specified in subclause 5.3.5.

If the UE supports service gap control, then the UE shall set the SGC bit to "service gap control supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

For cases a, x or if the UE operating in the single-registration mode performs inter-system change from S1 mode to N1 mode, the UE shall:

a)	if the UE has an applicable network-assigned UE radio capability ID for the current UE radio configuration in the selected PLMN or SNPN, include the applicable network-assigned UE radio capability ID in the UE radio capability ID IE of the REGISTRATION REQUEST message; and

b)	if the UE:

1)	does not have an applicable network-assigned UE radio capability ID for the current UE radio configuration in the selected PLMN or SNPN; and

2)	has an applicable manufacturer-assigned UE radio capability ID for the current UE radio configuration,

include the applicable manufacturer-assigned UE radio capability ID in the UE radio capability ID IE of the REGISTRATION REQUEST message.

For all cases except cases b and z, if the UE supports ciphered broadcast assistance data and the UE needs to obtain new ciphering keys, the UE shall include the Additional information requested IE with the CipherKey bit set to "ciphering keys for ciphered broadcast assistance data requested" in the REGISTRATION REQUEST message.

For case z, the UE shall include the Additional information requested IE with the CipherKey bit set to "ciphering keys for ciphered broadcast assistance data requested" in the REGISTRATION REQUEST message.

For case a, if the UE supports ciphered broadcast assistance data and the UE detects that one or more ciphering keys stored at the UE is not applicable in the current TAI, the UE should include the Additional information requested IE with the CipherKey bit set to "ciphering keys for ciphered broadcast assistance data requested" in the REGISTRATION REQUEST message.

For case b, if the UE supports ciphered broadcast assistance data and the remaining validity time for one or more ciphering keys stored at the UE is less than timer T3512, the UE should include the Additional information requested IE with the CipherKey bit set to "ciphering keys for ciphered broadcast assistance data requested" in the REGISTRATION REQUEST message as specified in subclause 6.14.2 in 3GPP TS 23.273 [6B].

The UE shall set the WUSA bit to "WUS assistance information reception supported" in the 5GMM capability IE for all cases except case b, if the UE supports WUS assistance information. The UE may include its UE paging probability information in the Requested WUS assistance information IE if the UE has set the WUSA bit to "WUS assistance information reception supported" in the 5GMM capability IE and does not have an active emergency PDU session.

The UE shall set the NR-PSSI bit to "NR paging subgrouping supported" in the 5GMM capability IE for all cases except case b, if the UE supports PEIPS assistance information, is not registered for emergency services and does not have an active emergency PDU session. The UE may include its UE paging probability information in the Requested PEIPS assistance information IE if the UE has set the NR-PSSI bit to "NR paging subgrouping supported" in the 5GMM capability IE.

The UE shall set the LP-WUSPSAI-SI bit to "LP-WUSPS assistance information supported" in the 5GMM capability IE for all cases except case b, if the UE supports the LP-WUSPS assistance information. The UE may include its UE paging probability information in the Requested LP-WUSPS assistance information IE if the UE has set the LP-WUSPSAI-SI bit to "LP-WUSPS assistance information supported" in the 5GMM capability IE.

If the network supports the N1 NAS signalling connection release, and the MUSIM UE requests the network to release the NAS signalling connection, the UE shall set Request type to "NAS signalling connection release" in the UE request type IE, set the Follow-on request indicator to "No follow-on request pending" and, if the network supports the paging restriction, may set the paging restriction preference in the Paging restriction IE in the REGISTRATION REQUEST message. In addition, the UE shall not include the Uplink data status IE or the Allowed PDU session status IE in the REGISTRATION REQUEST message even if the UE has one or more active always-on PDU sessions associated with the 3GPP access.

NOTE 17:	If the network has already indicated support for N1 NAS signalling connection release in the current stored registration area and the UE doesn't have an emergency PDU session established, the MUSIM UE is allowed to request the network to release the NAS signalling connection during registration procedure for mobility and periodic registration update that is due to mobility outside the registration area even before detecting whether the network supports the N1 NAS signalling connection release in the current TAI.

NOTE 18:	If the network has already indicated support for paging restriction in the current stored registration area and the UE doesn't have an emergency PDU session established, the MUSIM UE is allowed to include paging restriction together with the request to the network to release the NAS signalling connection during registration procedure for mobility and periodic registration update that is due to mobility outside the registration area even before detecting whether the network supports the paging restriction in the current TAI.

For case zi, the UE shall not include the Paging restriction IE in the REGISTRATION REQUEST message. If the UE is in 5GMM-IDLE mode and the network supports the N1 NAS signalling connection release, the UE may include the UE request type IE and set Request type to "NAS signalling connection release" to remove the paging restriction and request the release of the NAS signalling connection at the same time. In addition, the UE shall not include the Uplink data status IE in the REGISTRATION REQUEST message.

If the UE does not have a valid 5G NAS security context and the UE is sending the REGISTRATION REQUEST message after an inter-system change from S1 mode to N1 mode in 5GMM-IDLE mode, the UE shall send the REGISTRATION REQUEST message without including the NAS message container IE. The UE shall include the entire REGISTRATION REQUEST message (i.e. containing cleartext IEs and non-cleartext IEs, if any) in the NAS message container IE that is sent as part of the SECURITY MODE COMPLETE message as described in subclauses 4.4.6 and 5.4.2.3.

If the UE indicates "mobility registration updating" in the 5GS registration type IE and supports V2X as specified in 3GPP TS 24.587 [19B], the UE shall set the V2X bit to "V2X supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE indicates "mobility registration updating" in the 5GS registration type IE and supports V2X communication over E-UTRA-PC5 as specified in 3GPP TS 24.587 [19B], the UE shall set the V2XCEPC5 bit to "V2X communication over E-UTRA-PC5 supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE indicates "mobility registration updating" in the 5GS registration type IE and supports V2X communication over NR-PC5 as specified in 3GPP TS 24.587 [19B], the UE shall set the V2XCNPC5 bit to "V2X communication over NR-PC5 supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

The UE shall send the REGISTRATION REQUEST message including the NAS message container IE as described in subclause 4.4.6:

a)	when the UE is sending the message from 5GMM-IDLE mode, the UE has a valid 5G NAS security context, and needs to send non-cleartext IEs; or

b)	when the UE is sending the message after an inter-system change from S1 mode to N1 mode in 5GMM-IDLE mode and the UE has a valid 5G NAS security context and needs to send non-cleartext IEs.

The UE with a valid 5G NAS security context shall send the REGISTRATION REQUEST message without including the NAS message container IE when the UE does not need to send non-cleartext IEs and the UE is sending the message:

a)	from 5GMM-IDLE mode; or

b)	after an inter-system change from S1 mode to N1 mode in 5GMM-IDLE mode.

If the UE is sending the REGISTRATION REQUEST message after an inter-system change from S1 mode to N1 mode in 5GMM-CONNECTED mode and the UE needs to send non-cleartext IEs, the UE shall cipher the NAS message container IE using the mapped 5G NAS security context and send the REGISTRATION REQUEST message including the NAS message container IE as described in subclause 4.4.6. If the UE does not need to send non-cleartext IEs, the UE shall send the REGISTRATION REQUEST message without including the NAS message container IE.

If the REGISTRATION REQUEST message includes a NAS message container IE, the AMF shall process the REGISTRATION REQUEST message that is obtained from the NAS message container IE as described in subclause 4.4.6.

If the UE is in NB-N1 mode, then the UE shall set the Control plane CIoT 5GS optimization bit to "Control plane CIoT 5GS optimization supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b. For all cases except case b, if the UE is capable of NB-S1 mode, then the UE shall set the Control plane CIoT EPS optimization bit to "Control plane CIoT EPS optimization supported" in the S1 UE network capability IE of the REGISTRATION REQUEST message.

If the registration procedure for mobility and periodic registration update is initiated and there is request from the upper layers to perform "emergency services fallback" pending, the UE shall send a REGISTRATION REQUEST message without an Uplink data status IE.

If the UE supports N3 data transfer and multiple user-plane resources in NB-N1 mode (see 3GPP TS 36.306 [25D], 3GPP TS 36.331 [25A]), then the UE shall set the Multiple user-plane resources support bit to "Multiple user-plane resources supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

The UE shall set the ER-NSSAI bit to "Extended rejected NSSAI supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports the NSSRG, then the UE shall set the NSSRG bit to "NSSRG supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

For case zf, the UE shall include the service-level device ID in the Service-level-AA container IE of the REGISTRATION REQUEST message and set the value to the CAA-level UAV ID. The UE shall include the service-level-AA server address in the Service-level-AA container IE of the REGISTRATION REQUEST message and set the value to the USS address, if it is provided by the upper layers. The UE shall include the service-level-AA payload in the Service-level-AA container IE of the REGISTRATION REQUEST message and shall set the service-level-AA payload type, if the service-level-AA payload is provided by upper layers.

NOTE 18:	The service-level-AA payload can be of type "C2 authorization payload". The C2 authorization payload can include one or both of an indication of the request for direct C2 communication and pairing information for direct C2 communication.

For all cases except case b, then:

-	if the UE supports 5G ProSe direct discovery as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-dd bit to "5G ProSe direct discovery supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports 5G ProSe direct communication as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-dc bit to "5G ProSe direct communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-2 UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2relay bit to "Acting as a 5G ProSe layer-2 UE-to-network relay UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-3 UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3relay bit to "Acting as a 5G ProSe layer-3 UE-to-network relay UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-2 UE-to-network remote UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2rmt bit to "Acting as a 5G ProSe layer-2 UE-to-network remote UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-3 UE-to-network remote UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3rmt bit to "Acting as a 5G ProSe layer-3 UE-to-network remote UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-2 UE-to-UE relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2U2U relay bit to "Acting as a 5G ProSe layer-2 UE-to-UE relay UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-3 UE-to-UE relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3U2U relay bit to "Acting as a 5G ProSe layer-3 UE-to-UE relay UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-2 end UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2end bit to "Acting as a 5G ProSe layer-2 end UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-3 end UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3end bit to "Acting as a 5G ProSe layer-3 end UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

-	If the UE supports acting as 5G ProSe multi-hop layer-3 UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3relay bit to "Acting as a 5G ProSe layer-3 UE-to-network relay UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe multi-hop layer-3 UE-to-network remote UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3rmt bit to "Acting as a 5G ProSe layer-3 UE-to-network remote UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe multi-hop layer-2 UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2relay bit to "Acting as a 5G ProSe layer-2 UE-to-network relay UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe multi-hop layer-2 UE-to-network remote UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l2rmt bit to "Acting as a 5G ProSe layer-2 UE-to-network remote UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe multi-hop layer-3 UE-to-UE relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3U2U relay bit to "Acting as a 5G ProSe layer-3 UE-to-UE relay UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe multi-hop layer-3 end UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-l3end bit to "Acting as a 5G ProSe layer-3 end UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-3 intermediate UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-L3IMrelay bit to "Acting as a 5G ProSe layer-3 intermediate UE-to-network relay UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports acting as 5G ProSe layer-2 intermediate UE-to-network relay UE as specified in 3GPP TS 24.554 [19E], the UE shall set the 5G ProSe-L2IMrelay bit to "Acting as a 5G ProSe layer-2 intermediate UE-to-network relay UE supported" and set 5G ProSe-MCI bit to "Multi-hop relay communication supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

For all cases except case b, if the MUSIM UE supports the N1 NAS signalling connection release, then the UE shall set the N1 NAS signalling connection release bit to "N1 NAS signalling connection release supported" in the 5GMM capability IE of the REGISTRATION REQUEST message otherwise the UE shall not set the N1 NAS signalling connection release bit to "N1 NAS signalling connection release supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

For all cases except case b, if the MUSIM UE supports the paging indication for voice services, then the UE shall set the paging indication for voice services bit to "paging indication for voice services supported" in the 5GMM capability IE of the REGISTRATION REQUEST message otherwise the UE shall not set the paging indication for voice services bit to "paging indication for voice services supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

For all cases except case b, if the MUSIM UE supports the reject paging request, then the UE shall set the reject paging request bit to "reject paging request supported" in the 5GMM capability IE of the REGISTRATION REQUEST message otherwise the UE shall not set the reject paging request bit to "reject paging request supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

For all cases except case b, if the MUSIM UE sets:

-	the reject paging request bit to "reject paging request supported";

-	the N1 NAS signalling connection release bit to "N1 NAS signalling connection release supported"; or

-	both of them;

and supports the paging restriction, then the UE shall set the paging restriction bit to "paging restriction supported" in the 5GMM capability IE of the REGISTRATION REQUEST message otherwise the UE shall not set the paging restriction bit to "paging restriction supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports MINT, the UE shall set the MINT bit to "MINT supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports MINT-EPS, the UE shall set the MINT-EPS bit to "MINT-EPS supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports slice-based N3IWF selection, the UE shall set the SBNS bit to "Slice-based N3IWF selection supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports slice-based TNGF selection, the UE shall set the SBTS bit to "Slice-based TNGF selection supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

For all cases except case b, then:

-	if the UE supports UAS services, the UE shall set the UAS bit to "UAS services supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports A2X over E-UTRA-PC5 as specified in 3GPP TS 24.577 [60], the UE shall set the A2XEPC5 bit to "A2X over E-UTRA-PC5 supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports A2X over NR-PC5 as specified in 3GPP TS 24.577 [60], the UE shall set the A2XNPC5 bit to "A2X over NR-PC5 supported" in the 5GMM capability IE of the REGISTRATION REQUEST message. If the UE supports A2X over Uu as specified in 3GPP TS 24.577 [60], the UE shall set the A2X-Uu bit to "A2X over Uu supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

For case zg, if the UE has determined the UE determined PLMN with disaster condition as specified in 3GPP TS 23.122 [5], and:

a)	the UE determined PLMN with disaster condition is the HPLMN and:

1)	the Additional GUTI IE is included in the REGISTRATION REQUEST message and does not contain a valid 5G-GUTI that was previously assigned by the HPLMN; or

2)	the Additional GUTI IE is not included in the REGISTRATION REQUEST message and the 5GS mobile identity IE contains neither the SUCI nor a valid 5G-GUTI that was previously assigned by the HPLMN; or

b)	the UE determined PLMN with disaster condition is not the HPLMN and:

1)	the Additional GUTI IE is included in the REGISTRATION REQUEST message and does not contain a valid 5G-GUTI that was previously assigned by the UE determined PLMN with disaster condition; or

2)	the Additional GUTI IE is not included in the REGISTRATION REQUEST message and the 5GS mobile identity IE does not contain a valid 5G-GUTI that was previously assigned by the UE determined PLMN with disaster condition;

the UE shall include in the REGISTRATION REQUEST message the UE determined PLMN with disaster condition IE indicating the UE determined PLMN with disaster condition.

NOTE 19:	If the UE initiates the registration procedure for disaster roaming services, and the UE determined PLMN with disaster condition cannot be determined when an NG-RAN cell of the PLMN broadcasts the disaster related indication as specified in 3GPP TS 23.122 [5], the UE does not include in the REGISTRATION REQUEST message the UE determined PLMN with disaster condition IE but includes the Additional GUTI IE or the 5GS mobile identity IE or both as specified in subclauses 5.5.1.2.2.

For case zh the UE shall indicate "mobility registration updating" in the 5GS registration type IE of the REGISTRATION REQUEST message.

For case zp, the UE shall send the REGISTRATION REQUEST message over the new non-3GPP access. The UE shall include the Uplink data status IE in the REGISTRATION REQUEST message indicating the MA PDU session ID(s) or the single access PDU session ID(s) whose user plan resources are to be switched from the old non-3GPP access to the new non-3GPP access or to be established over the new non-3GPP access, if any. If the UE requests the network to keep using the user plane resources of the old non-3GPP access during path switching to the new non-3GPP access, the UE shall include the Non-3GPP path switching information IE in the REGISTRATION REQUEST message and set the NSONR bit to "non-3GPP path switching while using old non-3GPP resources requested".

If the UE supports event notification, the UE shall set the EventNotification bit to "Event notification supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports access to an SNPN using credentials from a credentials holder and:

a)	the UE is in its HPLMN or EHPLMN or the subscribed SNPN; or

b)	the UE is in a non-subscribed SNPN and supports equivalent SNPNs;

the UE shall set the SSNPNSI bit to "SOR-SNPN-SI supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports equivalent SNPNs, the UE shall set the ESI bit to "equivalent SNPNs supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b. If the UE supports LADN per DNN and S-NSSAI, the UE shall set the LADN-DS bit to "LADN per DNN and S-NSSAI supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports the reconnection to the network due to RAN timing synchronization status change, the UE shall set the Reconnection to the network due to RAN timing synchronization status change (RANtiming) bit to "Reconnection to the network due to RAN timing synchronization status change supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports MPS indicator update via the UE configuration update procedure, the UE shall set the MPSIU bit to "MPS indicator update supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports MCS indicator update via the UE configuration update procedure, the UE shall set the MCSIU bit to "MCS indicator update supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

For all cases except case b, if the UE supports ranging and sidelink positioning as specified in 3GPP TS 24.514 [62] and supports:

a)	V2X communication over PC5 as specified in 3GPP TS 24.587 [19B];

b)	5G ProSe direct discovery and 5G ProSe direct communication as specified in 3GPP TS 24.554 [19E]; or

c)	both a) and b),

the UE shall set

a)	the RSLP bit to "Ranging and sidelink positioning supported" if the UE supports ranging and sidelink positioning as target UE or SL reference UE or both;

b)	the RSLPL bit to "Ranging and sidelink positioning for located UE supported" if the UE supports ranging and sidelink positioning as located UE;

c)	the RSLPS bit to "Ranging and sidelink positioning for SL positioning server UE supported" if the UE supports ranging and sidelink positioning as SL positioning server UE; or

d)	any combination of a), b) and c), in the 5GMM capability IE of the REGISTRATION REQUEST message;

and in addition:

a)	if the UE supports ranging and sidelink positioning with V2X capable UE, the UE shall set the RSLPVU bit to "Ranging and sidelink positioning with V2X capable UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message; and

b)	if the UE supports ranging and sidelink positioning with 5G ProSe capable UE, the UE shall set the RSLPPU bit to "Ranging and sidelink positioning with 5G ProSe capable UE supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

If the UE supports the partial network slice, the UE shall set the PNS bit to "Partial network slice supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports network slice usage control, the UE shall set the NSUC bit to "Network slice usage control supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports the S-NSSAI time validity information, the UE shall set the TempNS bit to "S-NSSAI time validity information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports the S-NSSAI location validity information, the UE shall set the SLVI bit to "S-NSSAI location validity information supported" in the 5GMM capability IE of the REGISTRATION REQUEST message for all cases except case b.

If the UE supports access technology utilization control, the UE shall set the ATUC bit to "access technology utilization control supported" in the 5GMM capability IE of the REGISTRATION REQUEST message over 3GPP access for all cases except case b.

If the UE supports HPAEOP, the UE shall set the OPHPAE bit to "Operator policy for high priority access exemption for service area restrictions is supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

Figure 5.5.1.3.2.1: Registration procedure for mobility and periodic registration update

* * * Next Change * * * *

#### 9.11.3.1	5GMM capability

The purpose of the 5GMM capability information element is to provide the network with information concerning aspects of the UE related to the 5GCN or interworking with the EPS. The contents might affect the manner in which the network handles the operation of the UE.

The 5GMM capability information element is coded as shown in figure 9.11.3.1.1 and table 9.11.3.1.1.

The 5GMM capability is a type 4 information element with a minimum length of 3 octets and a maximum length of 15 octets.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GMM capability IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of 5GMM capability contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| SGC |  | 5G-IPHC-CP CIoT |  | N3 data |  | 5G-CP CIoT |  | RestrictEC |  | LPP |  | HO attach |  | S1 mode |  | octet 3 |  |
| RACS |  | NSSAA |  | 5G-LCS |  | V2XCNPC5 |  | V2XCEPC5 |  | V2X |  | 5G-UP CIoT |  | 5GSRVCC |  | octet 4* |  |
| 5G ProSe-l2relay |  | 5G ProSe-dc |  | 5G ProSe-dd |  | ER-NSSAI |  | 5G-EHC-CP CIoT |  | multipleUP |  | WUSA |  | CAG |  | octet 5* |  |
| PR |  | RPR |  | PIV |  | NCR |  | NR-PSSI |  | 5G ProSe-l3rmt |  | 5G ProSe-l2rmt |  | 5G ProSe-l3relay |  | octet 6* |  |
| MPSIU |  | UAS |  | NSAG |  | Ex-CAG |  | SSNPNSI |  | EventNotification |  | MINT |  | NSSRG |  | octet 7* |  |
| SBTS |  | NSR |  | LADN-DS |  | RAN timing |  | ECI |  | ESI |  | RCMAN |  | RCMAP |  | octet 8* |  |
| 5G ProSe-l2end |  | 5G ProSe-l3U2U relay |  | 5G ProSe-l2U2U relay |  | RSLPS |  | SBNS |  | UN-PER |  | A2XNPC5 |  | A2XEPC5 |  | octet 9* |  |
| A2X-Uu |  | SLVI |  | TempNS |  | SUPL |  | LCS-UPP |  | PNS |  | RSLP |  | 5G ProSe-l3end |  | octet 10* |  |
| LP-WUSPSAI-SI |  | ATUC |  | RSLPPU |  | RSLPVU |  | NSUC |  | RSLPL |  | NVL-SATNR |  | MCSIU |  | octet 11* |  |
| 0 spare |  | 0 spare |  | MINT-EPS |  | 5G ProSe-L2IMrelay |  | 5G ProSe-L3IMrelay |  | MLCSUP |  | 5G ProSe-MCI |  | OPHPAE |  | octet 12* |  |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | octet 13*-15* |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 9.11.3.1.1: 5GMM capability information element

Table 9.11.3.1.1: 5GMM capability information element

| EPC NAS supported (S1 mode) (octet 3, bit 1) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  | S1 mode not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  | S1 mode supported |  |  |
| ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message for handover support (HO attach) (octet 3, bit 2) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message with request type set to "handover" or "handover of emergency bearer services" to transfer PDU session from N1 mode to S1 mode not supported |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message with request type set to "handover" or "handover of emergency bearer services" to transfer PDU session from N1 mode to S1 mode supported |  |  |  |
| LTE Positioning Protocol (LPP) capability (octet 3, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support LTE Positioning Protocol (LPP) (see 3GPP TS 37.355 [26]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  | LPP in N1 mode not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  | LPP in N1 mode supported |  |  |
| Restriction on use of enhanced coverage support (RestrictEC) (octet 3, bit 4) This bit indicates the capability to support restriction on use of enhanced coverage. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  | Restriction on use of enhanced coverage not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  | Restriction on use of enhanced coverage supported |  |  |
| Control plane CIoT 5GS optimization (5G-CP CIoT) (octet 3, bit 5) This bit indicates the capability for control plane CIoT 5GS optimization. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | Control plane CIoT 5GS optimization not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Control plane CIoT 5GS optimization supported |  |  |
| N3 data transfer (N3 data) (octet 3, bit 6) This bit indicates the capability for N3 data transfer. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | N3 data transfer supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | N3 data transfer not supported |  |  |
| IP header compression for control plane CIoT 5GS optimization (5G-IPHC-CP CIoT) (octet 3, bit 7) This bit indicates the capability for IP header compression for control plane CIoT 5GS optimization. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | IP header compression for control plane CIoT 5GS optimization not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | IP header compression for control plane CIoT 5GS optimization supported |  |  |
| Service gap control (SGC) (octet 3, bit 8) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  | service gap control not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  | service gap control supported |  |  |
| 5G-SRVCC from NG-RAN to UTRAN (5GSRVCC) capability (octet 4, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability for 5G-SRVCC from NG-RAN to UTRAN (5GSRVCC) (see 3GPP TS 23.216 [6A]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  | 5G-SRVCC from NG-RAN to UTRAN not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  | 5G-SRVCC from NG-RAN to UTRAN supported |  |  |
| User plane CIoT 5GS optimization (5G-UP CIoT) (octet 4, bit 2) This bit indicates the capability for user plane CIoT 5GS optimization. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane CIoT 5GS optimization not supported |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane CIoT 5GS optimization supported |  |  |
| V2X capability (V2X) (octet 4, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability for V2X, as specified in 3GPP TS 24.587 [19B]. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | V2X not supported |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | V2X supported |  |  |  |
| V2X communication over E-UTRA-PC5 capability (V2XCEPC5) (octet 4, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability for V2X communication over E-UTRA-PC5, as specified in 3GPP TS 24.587 [19B]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | V2X communication over E-UTRA-PC5 not supported |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | V2X communication over E-UTRA-PC5 supported |  |  |  |
| V2X communication over NR-PC5 capability (V2XCNPC5) (octet 4, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability for V2X communication over NR-PC5, as specified in 3GPP TS 24.587 [19B]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | V2X communication over NR-PC5 not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | V2X communication over NR-PC5 supported |
| Location Services (5G-LCS) notification mechanisms capability (octet 4, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support Location Services (5G-LCS) notification mechanisms (see 3GPP TS 23.273 [6B]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LCS notification mechanisms not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LCS notification mechanisms supported |
| Network slice-specific authentication and authorization (NSSAA) (octet 4, bit 7) This bit indicates the capability to support network slice-specific authentication and authorization. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice-specific authentication and authorization not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice-specific authentication and authorization supported |
| Radio capability signalling optimisation (RACS) capability (octet 4, bit 8) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RACS not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RACS supported |
| Closed Access Group (CAG) capability (octet 5, bit 1) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CAG not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CAG supported |
| WUS assistance (WUSA) information reception capability (octet 5, bit 2) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WUS assistance information reception not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WUS assistance information reception supported |
| Multiple user-plane resources support (multipleUP) (octet 5, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support multiple user-plane resources in NB-N1 mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multiple user-plane resources not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multiple user-plane resources supported |
| Ethernet header compression for control plane CIoT 5GS optimization (5G-EHC-CP CIoT) (octet 5, bit 4) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ethernet header compression for control plane CIoT 5GS optimization not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ethernet header compression for control plane CIoT 5GS optimization supported |
| Extended rejected NSSAI support (ER-NSSAI) (octet 5, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support extended rejected NSSAI. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Extended rejected NSSAI not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Extended rejected NSSAI supported |
| 5G ProSe direct discovery (5G ProSe-dd) (octet 5, bit 6) This bit indicates the capability for 5G ProSe direct discovery. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5G ProSe direct discovery not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5G ProSe direct discovery supported |
| 5G ProSe direct communication (5G ProSe-dc) (octet 5, bit 7) This bit indicates the capability for 5G ProSe direct communication. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5G ProSe direct communication not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5G ProSe direct communication supported |
| 5G ProSe layer-2 UE-to-network-relay (5G ProSe-l2relay) (octet 5, bit 8) This bit indicates the capability to act as a 5G ProSe layer-2 UE-to-network relay UE. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-network relay UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-network relay UE supported |
| 5G ProSe layer-3 UE-to-network-relay (5G ProSe-l3relay) (octet 6, bit 1) This bit indicates the capability to act as a 5G ProSe layer-3 UE-to-network relay UE Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-network relay UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-network relay UE supported |
| 5G ProSe layer-2 UE-to-network-remote (5G ProSe-l2rmt) (octet 6, bit 2) This bit indicates the capability to act as a 5G ProSe layer-2 UE-to-network remote UE Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-network remote UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-network remote UE supported |
| 5G ProSe layer-3 UE-to-network-remote (5G ProSe-l3rmt) (octet 6, bit 3) This bit indicates the capability to act as a 5G ProSe layer-3 UE-to-network remote UE Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-network remote UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-network remote UE supported |
| NR paging subgroup support indication (NR-PSSI) (octet 6, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of PEIPS assistance information for NR paging subgrouping |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR paging subgrouping not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR paging subgrouping supported |
| N1 NAS signalling connection release (NCR) (octet 6, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates whether N1 NAS signalling connection release is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N1 NAS signalling connection release not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N1 NAS signalling connection release supported |
| Paging indication for voice services (PIV) (octet 6, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates whether paging indication for voice services is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | paging indication for voice services not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | paging indication for voice services supported |
| Reject paging request (RPR) (octet 6, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates whether reject paging request is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | reject paging request not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | reject paging request supported |
| Paging restriction (PR) (octet 6, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates whether paging restriction is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | paging restriction not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | paging restriction supported |
| NSSRG (octet 7, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support the NSSRG. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NSSRG not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NSSRG supported |
| Minimization of service interruption (MINT) (octet 7, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support Minimization of service interruption (MINT) Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MINT not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MINT supported |
| Event notification (EventNotification) (octet 7, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support event notification for upper layers Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Event notification not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Event notification supported |
| SOR-SNPN-SI (SOR SNPN SI) (octet 7, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support SOR-SNPN-SI. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SOR-SNPN-SI not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SOR-SNPN-SI supported |
| Extended CAG information list support (Ex-CAG) (octet 7, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support extended CAG information list. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Extended CAG information list not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Extended CAG information list supported |
| NSAG (octet 7, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support NSAG. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NSAG not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NSAG supported |
| UAS (octet 7, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support UAS services. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | UAS services not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | UAS services supported |
| MPS indicator update (MPSIU) (octet 7, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support MPS indicator update via the UE configuration update procedure. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MPS indicator update not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MPS indicator update supported |
| Registration complete message for acknowledging negotiated PEIPS assistance information (RCMAP) (octet 8, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability for sending REGISTRATION COMPLETE message when Negotiated PEIPS assistance information IE is included in the REGISTRATION ACCEPT message. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sending of REGISTRATION COMPLETE message for negotiated PEIPS assistance information not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sending of REGISTRATION COMPLETE message for negotiated PEIPS assistance information supported |
| Registration complete message for acknowledging NSAG information (RCMAN) (octet 8, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability for sending REGISTRATION COMPLETE message when NSAG information IE is included in the REGISTRATION ACCEPT message. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sending of REGISTRATION COMPLETE message for NSAG information not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sending of REGISTRATION COMPLETE message for NSAG information supported |
| Equivalent SNPNs indicator (ESI) (octet 8, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support equivalent SNPNs. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Equivalent SNPNs not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Equivalent SNPNs supported |
| Enhanced CAG information (ECI) (octet 8, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support enhanced CAG information. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Enhanced CAG information not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Enhanced CAG information supported |
| Reconnection to the network due to RAN timing synchronization status change (RANtiming) (octet 8, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support Reconnection to the network due to RAN timing synchronization status change. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reconnection to the network due to RAN timing synchronization status change not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reconnection to the network due to RAN timing synchronization status change supported |
| LADN per DNN and S-NSSAI support (LADN-DS) (octet 8, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support LADN per DNN and S-NSSAI. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LADN per DNN and S-NSSAI not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LADN per DNN and S-NSSAI supported |
| Network slice replacement (NSR) (octet 8, bit 7) This bit indicates the capability to support network slice replacement. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice replacement not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice replacement supported |
| Slice-based TNGF selection support (SBTS) (octet 8, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support slice-based TNGF selection. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Slice-based TNGF selection not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Slice-based TNGF selection supported |
| A2X over E-UTRA-PC5 (A2XEPC5) (octet 9, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability for A2X over E-UTRA-PC5, as specified in 3GPP TS 24.577 [60]. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over E-UTRA-PC5 not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over E-UTRA-PC5 supported |
| A2X over NR-PC5 (A2XNPC5) (octet 9, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability for A2X over NR-PC5, as specified in 3GPP TS 24.577 [60]. Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over NR-PC5 not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over NR-PC5 supported |
| Unavailability period (UN-PER) (octet 9, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support unavailability period. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Unavailability period not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Unavailability period supported |
| Slice-based N3IWFselection support (SBNS) (octet 9, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support slide-based N3IWF selection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Slice-based N3IWF selection not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Slice-based N3IWF selection supported |
| SL positioning server UE (RSLPS) (octet 9, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability for SL positioning server UE, as specified in 3GPP TS 24.586 [ts23586]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning for SL positioning server UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning for SL positioning server UE supported |
| 5G ProSe layer-2 UE-to-UE relay (5G ProSe-l2U2U relay) (octet 9, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to act as a 5G ProSe layer-2 UE-to-UE relay UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-UE relay UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-UE relay UE supported |
| 5G ProSe layer-3 UE-to-UE relay (5G ProSe-l3U2U relay) (octet 9, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to act as a 5G ProSe layer-3 UE-to-UE relay UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-UE relay UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 UE-to-UE relay UE supported |
| 5G ProSe layer-2 end UE (5G ProSe-l2end) (octet 9, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to act as a 5G ProSe layer-2 end UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-UE relay end UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 UE-to-UE relay end UE supported |
| 5G ProSe layer-3 end UE (5G ProSe-l3end) (octet 10, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to act as a 5G ProSe layer-3 end UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 end UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 end UE supported |
| Ranging and sidelink positioning support (RSLP) (octet 10, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning supported |
| Partial network slice (PNS) (octet 10, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates whether the UE support partial network slice in the registration area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Partial network slice not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Partial network slice supported |
| LCS-UPP user plane positioning (LCS-UPP) (octet 10, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support LCS-UPP user plane positioning (see 3GPP TS 23.273 [6B]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane positioning using LCS-UPP not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane positioning using LCS-UPP supported |
| SUPL user plane positioning (SUPL) (octet 10, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support SUPL user plane positioning (see 3GPP TS 38.305 [67] and 3GPP TS 23.271 [68]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane positioning using SUPL not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | User plane positioning using SUPL supported |
| S-NSSAI time validity information (TempNS) (octet 10, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support the S-NSSAI time validity information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S-NSSAI time validity information not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S-NSSAI time validity information supported |
| S-NSSAI location validity information (SLVI) (octet 10, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support S-NSSAI location validity information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S-NSSAI location validity information not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S-NSSAI location validity information supported |
| A2X over Uu capability (A2X-Uu) (octet 10, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability for A2X over Uu, as specified in 3GPP TS 24.577 [60]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over Uu not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A2X over Uu supported |
| MCS indicator update (MCSIU) (octet 11, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support MCS indicator update via the UE configuration update procedure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MCS indicator update not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MCS indicator update supported |
| Network verified UE location over satellite NG-RAN (NVL-SATNR) (octet 11, bit 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support network verified UE location over satellite NG-RAN as specified in 3GPP TS 23.501 [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network verified UE location over satellite NG-RAN not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network verified UE location over satellite NG-RAN supported |
| Ranging and sidelink positioning over PC5 for located UE support (RSLPL) (octet 11, bit 3) This bit indicates the capability to support ranging and sidelink positioning over PC5 for located UE support. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning for located UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning for located UE supported |
| Network slice usage control (NSUC) (octet 11, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support network slice usage control. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice usage control not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Network slice usage control supported |
| Ranging and sidelink positioning with V2X capable UE (RSLPVU) (octet 11, bit 5) (see NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning with V2X capable UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning with V2X capable UE supported |
| Ranging and sidelink positioning with 5G ProSe capable UE support (RSLPPU) (octet 11, bit 6) (see NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning with 5G ProSe capable UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ranging and sidelink positioning with 5G ProSe capable UE supported |
| Access technology utilization control (ATUC) (octet 11, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of access technology utilization control. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | access technology utilization control not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | access technology utilization control supported |
| LP-WUSPS assistance information support indication (LP-WUSPSAI-SI) (octet 11, bit 8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of LP-WUSPS assistance information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LP-WUSPS assistance information not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LP-WUSPS assistance information supported |
| Operator policy for high priority access exemptions for service area restrictions (OPHPAE) (octet 12, bit 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support operator policy for high priority access exemptions for service area restrictions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Operator policy for high priority access exemption for service area restrictions is not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Operator policy for high priority access exemption for service area restrictions is supported |
| 5G ProSe Multi-hop relay communication indication (5G ProSe-MCI) (octet 12, bit 2) (see NOTE 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the support of multi-hop relay communication capability. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multi-hop relay communication not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multi-hop relay communication supported |
| Multiple LCS secured user plane connections (MLCSUP) (octet 12, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to support multiple LCS secured user plane connections (see 3GPP TS 23.273 [6B]). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multiple LCS secured user plane connections not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Multiple LCS secured user plane connections supported |
| 5G ProSe layer-3 intermediate UE-to-network relay UE (5G ProSe-L3IMrelay) (octet 12, bit 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to act as a 5G ProSe layer-3 intermediate UE-to-network relay UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 intermediate UE-to-network relay UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-3 intermediate UE-to-network relay UE supported |
| 5G ProSe layer-2 intermediate UE-to-network relay UE (5G ProSe-L2IMrelay) (octet 12, bit 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This bit indicates the capability to act as a 5G ProSe layer-2 intermediate UE-to-network relay UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 intermediate UE-to-network relay UE not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Acting as a 5G ProSe layer-2 intermediate UE-to-network relay UE supported |
| Minimization of service interruption support in EPS (MINT-EPS) (octet 12, bit 6) This bit indicates the capability to support MINT-EPS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MINT-EPS not supported |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MINT-EPS supported |
| Bits 7 to 8 in octet 12 and bits in octets 13 to 15 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: If both the RSLPVU bit is set to "Ranging and sidelink positioning with V2X capable UE not supported" and the RSLPPU bit is set to "Ranging and sidelink positioning with 5G ProSe capable UE not supported" in the 5GMM capability IE, then the receiving entity shall ignore the RSLPVU bit and the RSLPPU bit. NOTE 2: If the 5G ProSe-MCI bit is set to "Muti-hop relay communication supported", then at least one of the following bits should be included in the 5GMM capability IE and set to 1: "5G ProSe-l2relay", "5G ProSe-l3relay", "5G ProSe-l2rmt", "5G ProSe-l3rmt", "5G ProSe-l3U2U relay", "5G ProSe-l3end", "5G ProSe-L2IMrelay" or "5G ProSe-L3IMrelay". The UE supporting only multi-hop UE-to-network relay or multi-hop UE-to-UE relay as specified in 3GPP TS 24.554 [19E] shall set the 5G ProSe-MCI bit to "Multi-hop relay communication supported" along with the corresponding 5GMM capability IE(s). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *