3GPP TSG CT WG3 Meeting #143	C3-254150

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0433 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete the definition of the OpenAPI description of the CAPIF_Open_Discover_Service_API |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | It is needed to apply some corrections and enhancements (e.g., query parameters descriptions) to the OpenAPI description of the new CAPIF_Open_Discover_Service_API to align with the related main body provisions. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed issues. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed issues are not addressed and the definition of this new CAPIF_Open_Discover_Service_API is not completed. |  |  |  |  |  |  |
| Clauses affected: |  | A.13 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the OpenAPI descriptions of the following APIs: TS29222_CAPIF_Open_Discover_Service_API.yaml |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

# A.13	CAPIF_Open_Discover_Service_API

openapi: 3.0.0

info:

title: CAPIF_Open_Discover_Service_API

version: 1.0.0-alpha.1

description: |

API for Routing information.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.222 V19.4.0; Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/open-api-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/service-apis:

get:

description: >

Request Open discovery of the

Service API(s) according to a set of filtering criteria.

parameters:

- name: api-names

in: query

description: >

Contains the name(s) of the target Service API(s).

Each Service API name shall be set to the value of the <apiName> placeholder of the

target Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14].

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: api-versions

in: query

description: >

Contains the major version(s) (e.g., v1) of the target Service API(s).

Each Service API version shall be set to the value of the <apiVersion> placeholder of

the target Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14].

The key of the map shall be set to the value of the Service API name (i.e., the value of

the <apiName> placeholder of the Service API URI structure as defined in clause 5.2.4 of

3GPP TS 29.122 [14]) of the Service API to which the provided list of Service API

version(s) provided within the map value applies.

schema:

type: object

additionalProperties:

type: array

items:

type: string

minItems: 1

minProperties: 1

- name: comm-type

in: query

description: >

Contains the communication type supported by the target Service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/CommunicationType'

- name: protocols

in: query

description: >

Contains the protocol(s) supported by the target Service API(s).

style: form

explode: false

schema:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

minItems: 1

- name: data-format

in: query

description: >

Contains data format supported by the target Service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

- name: api-cats

in: query

description: >

Contains the category(ies) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: preferred-aef-loc

in: query

description: >

Contains the preferred location information for AEF(s) exposing the target

Service API(s).

This query parameter is ignored by the CCF if there are no matching records at the CCF.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefLocation'

- name: api-prov-names

in: query

description: >

Contains the name(s) of the provider(s) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: api-supported-features

in: query

description: >

Contains the list of the features supported by the target Service API(s) identified by

the api-name query parameter.

This query parameter may be present only if the api-name query parameter is also

present

.

The key of the map shall be set to the value of the Service API name (among the ones

provided within the api-names query parameter) of the Service API to which the provided

list of supported feature(s) provided within the map value applies.

schema:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

minProperties: 1

- name: service-kpis

in: query

description: >

Contains iInformation about service characteristics provided by the target

Service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceKpis'

- name: api-ids

in: query

description: >

Contains the identifier(s) of the targeted Service APIs.

required: false

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: res-ops

in: query

description: >

Contains the list of supported Service API resource(s) and service operation(s).

This query parameter may be present only if the api-names query parameter is present.

style: form

explode: false

schema:

type: array

items:

$ref: 'TS29222_CAPIF_Discover_Service_API.yaml#/components/schemas/ResOperInfo'

minItems: 1

- name: supported-features

in: query

description: >

Contains the list of supported features among the ones defined in clause 8.11.6.

This attributed shall be present only when feature negotiation is required.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The result of the requested Open Service APIs discovery is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/OpenDiscoveryResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

OpenDiscoveryResp:

type: object

description: >

Represents the Open Service API Discovery response.

properties:

discApis:

type: array

items:

$ref: '#/components/schemas/OpenAPIDetails'

minItems: 0

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- discApis

OpenAPIDetails:

type: object

description: >

Represents the Service API details provided within an Open Service API Discovery

response.

properties:

apiName:

type: string

apiId:

type: string

apiStatus:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ApiStatus'

description:

type: string

serviceAPICategory:

type: string

apiSuppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

apiProvName:

type: string

aefProfiles:

type: array

items:

$ref: '#/components/schemas/OpenAefProfile'

minItems: 1

required:

- apiName

OpenAefProfile:

type: object

description: >

Represents the AEF Profile details provided within an Open Service API Discovery response.

properties:

aefId:

type: string

versions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Version'

minItems: 1

protocol:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

dataFormat:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

aefLocation:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefLocation'

serviceKpis:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceKpis'

* * * * End of changes * * * *