3GPP TSG-CT WG3 Meeting #144	C3-255103

# Dallas, United States, 17 - 21 November 2025

Source:	Ericsson

Title:	Pseudo-CR on corrections of AIMLES_MLModelTraining API

Spec:	3GPP TS 29.482 V1.2.0

Agenda item:	19.41

Document for:	Agreement

1. Introduction

TS 29.482 AIMLES_MLModelTraining has the following problems:

-	change data type of clientId in FeatureList data type to AimleClientId.

-	Editorial corrections spaces, and text style.

-	change "members" attribute name to "clients" in Trainrequest, to align with the rest of the document

-	correct PerfParam name to PerfParams

-	Correct reference 3GPP°TS°23.482°[9] to 3GPP TS 23.482 [9].

-	Correct ClientCapability	reference in reused data table from 29.560 to TS 24.560

-	change VAL server to VAL Server

-	correct typo traning to training.

-	add minimum and maximum for modelAccuracy in OpenAPI

2. Reason for Change

Correct the problems identified in the introduction.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 V1.2.0.

# *** First Change ***

##### 5.2.14.2.1	Introduction

The service operations defined for the AIMLES_MLModelTraining API are shown in the Table 5.2.14.2.1-1.

Table 5.2.14.2.1-1: Service operations of the AIMLES_MLModelTraining API

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_MLModelTraining_Request | This service operation enables a service consumer to request AIMLE server for ML model training. | e.g., VAL Server |
| AIMLES_MLModelTraining_Notify | This service operation enables a service consumer to receive ML model training notifications. | e.g., VAL Server |

# *** Next Change ***

5.2.14.2.2.2	AIMLE ML Model Training Request

Figure 5.2.14.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request AIMLE ML Model Training (see also clause 8.3 of 3GPP TS 23.482 [9]).

Figure 5.2.14.2.2.2-1: Procedure for AIMLE ML Model Training Request

1.	In order to request to AIMLE ML model training, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the custom operation "RequestTrain" in "/request-train".

2a.	Upon success, the AIMLE Server shall respond with an HTTP "200 OK" status code with the response body containing the "MlModelTrainResp" data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.1.8.7.

# *** Next Change ***

5.2.14.2.3.2	AIMLE ML Model Training Notification

Figure 5.2.14.2.3.2-1 depicts a scenario where the AIMLE Server sends a request to notify a previously subscribed service consumer on AIMLE ML Model Training report(s) (see also clause 8.15 of 3GPP TS 23.482 [9]).

Figure 5.2.14.2.3.2-1: Procedure for AIMLE ML Model Training Notification

1.	In order to notify a previously subscribed service consumer on AIMLE ML Model Training report(s), the AIMLE Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation of the corresponding AIMLE ML Model Training Request using the procedures defined in clauses 5.2.14.2.3.2, and the request body including the MlModelTrainNotif data structure.

2a.	Upon success, the service consumer shall respond to the AIMLE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.8.7.

# *** Next Change ***

##### 6.1.8.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.8.6.1-1 specifies the data types defined for the AIMLES_MLModelTraining API.

Table 6.1.8.6.1-1: AIMLES_MLModelTraining API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| CommonId | 6.1.8.6.2.12 | Represents the common features information. |  |
| EarlyStopCri | 6.1.8.6.2.10 | Represents the early stopping criteria. |  |
| FeatureList | 6.1.8.6.2.13 | Represents the feature list of an AIMLE client. |  |
| MemberInfo | 6.1.8.6.2.6 | Represents the list of AIMLE clients selected or de-selected for the ML model training. |  |
| MemberSelCriteria | 6.1.8.6.2.4 | Represents the criteria to be continuously monitored for selecting the member clients. |  |
| MlModelInfo | 6.1.8.6.2.5 | Represents the ML model information. |  |
| MlModelTrainNotif | 6.1.8.6.2.3 | Represents the ML Model training notification |  |
| MlModelTrainResp | 6.1.8.6.2.8 | Represents the ML model training response. |  |
| PerfParams | 6.1.8.6.2.7 | Represents the output of training, e.g., ML model performance parameters for the training. |  |
| TrainingErr | 6.1.8.6.3.4 | Represents the error encountered during training. |  |
| TrainRequest | 6.1.8.6.2.2 | Represents the ML Model training request |  |
| TrainingObj | 6.1.8.6.2.9 | Represents the termination condition for ML model training. |  |
| TrainingType | 6.1.8.6.3.3 | Represents the training type. |  |
| VFLParam | 6.1.8.6.2.11 | Represents the VFL paramerters. |  |

Table 6.1.8.6.1-2 specifies data types re-used by the AIMLES_MLModelTraining API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the AIMLES_MLModelTraining API.

Table 6.1.8.6.1-2: AIMLES_MLModelTraining API re-used Data Types

| Data type |  |  | Reference | Comments |  |  | Applicability |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AimleClientId |  |  | 6.1.1.6.3.2 | Represents unique identifier of a AIMLE client. |  |  |  |
| ClientCapability |  |  | 3GPP TS 24.560 [12] | Represents the client capability information. |  |  |  |
| EndPoint |  |  | 3GPP TS 29.558 [15] | Represents the endpoint information. |  |  |  |
| Float |  |  | 3GPP TS 29.571 [11] | Used to represent the fractional part of the proximity range in the reference UE details. |  |  |  |
| LocationArea5G |  | 3GPP TS 29.122 [2] |  |  |  | Used to indicate a location area represented as list of geographic areas, civic addresses and network area. |  |
| MLModel |  |  | 6.2.1.6.2.4 | Represents the ML model that has to be distributed to the selected member clients for training. |  |  |  |
| PerformanceMetric |  |  | 6.1.11.6.3.3 | Represents the performance metric for training the ML model. |  |  |  |
| TimeWindow |  |  | 3GPP TS 29.122 [2] | Indicates the time window. |  |  |  |
| Uinteger | 3GPP TS 29.571 [11] | Represents an unsigned Integer. |  |
| Uri |  |  | 3GPP TS 29.122 [2] | Represent an URI, used to indicate the notification URI. |  |  |  |

# *** Next Change ***

6.1.8.6.2.2	Type: TrainRequest

Table 6.1.8.6.2.2-1: Definition of type TrainRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trnType | TrainingType | M | 1 | Contains the type of training to be performed. |  |
| members | array(AimleClientId) | C | 1..N | Contains the list of member clients to be utilized for training the ML model. (NOTE 1) |  |
| clientSetId | string | C | 0..1 | Contains the AIMLE client set identifier. (NOTE 1) |  |
| memSelCrit | MemberSelCriteria | C | 0..1 | Contains the criteria that needs to be continuously monitored for selecting the member clients. (NOTE 1) |  |
| numReqClients | Uinteger | O | 0..1 | Contains the requested number of AIML clients to be selected based on member selection criteria. |  |
| modelInf | MlModelInfo | C | 0..1 | Contains the ML model that has to be distributed to the selected member clients for training. (NOTE 2) |  |
| modelReq | MLModel | C | 0..1 | Contains the requirement for selecting a model to be trained and the filtering criteria for selecting the model. (NOTE 2) |  |
| trainObj | TrainingObj | O | 0..1 | Contains the termination condition for the ML model training. |  |
| datasetId | array(string) | M | 1..N | Contains the identifiers for dataset used for HFL or VFL training. For VFL, multiple dataset identifiers can be specified. |  |
| dataSamples | Uinteger | O | 0..1 | Contains the number of data samples required for one round of HFL or VFL training. |  |
| opSchedule | string | O | 0..1 | Contains the schedule for when training is to occur. |  |
| vflParam | VFLParam | C | 0..1 | Contains the parameters specific to VFL training. This IE shall be present for VFL training. |  |
| memUpdNotif | boolean | O | 0..1 | Indicates whether the requestor needs to be notified whenever there is update related to new member clients selected or de-selected. Set to "true" to indicate that requestor needs to be notified whenever there is update related to new member clients selected or de-selected. Default value is "false" if omitted. |  |
| notifUri | Uri | C | 0..1 | Contains the notification URI where notifications should be sent. This attribute shall be present when the attribute "memUpdNotif" is set to "true". |  |
| NOTE 1: At least one of these attributes shall be present. NOTE 2: At least one of these attributes shall be present. |  |  |  |  |  |

# *** Next Change ***

6.1.8.6.2.5	Type: MlModelInfo

Table 6.1.8.6.2.5-1: Definition of type MlModelInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModelId | string | C | 0..1 | Contains the identifier for ML model. |  |
| mlModelLoc | EndPoint | C | 0..1 | Contains the URI, fqdn or address that maps to the resource where ML model is stored. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** Next Change ***

6.1.8.6.2.7	Type: PerfParams

Table 6.1.8.6.2.7-1: Definition of type PerfParams

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| modelAccuracy | integer | C | 0..1 | Indicates the achieved ML model accuracy, expressed as a percentage. Minimum: 0, maximum: 100. |  |
| modelPrecision | Float | C | 0..1 | Represents the accuracy for the positive predictions made by the model. Minimum: 0, maximum: 1. |  |
| modelRecall | Float | C | 0..1 | Represents model's ability to identify all the actual positive instances within a dataset. Minimum: 0, maximum: 1. |  |
| modelF1Score | Float | C | 0..1 | Represents the combined metric for precision and recall. Minimum: 0, maximum: 1. |  |
| errorMeanSquare | Float | C | 0..1 | Represents the mean squared error between predicted values and actual values. |  |
| errorMeanAbs | Float | C | 0..1 | Represents the mean absolute error between predicted values and actual values. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# *** Next Change ***

###### 6.1.8.6.2.8	Type: MlModelTrainResp

Table 6.1.8.6.2.8-1: Definition of type MlModelTrainResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trainingId | string | M | 1 | Contains an identifier for ML model training request. |  |
| modelId | string | O | 0..1 | Contains the identifier for the ML model selected by AIMLE server. |  |

# *** Next Change ***

###### 6.1.8.6.2.9	Type: TrainingObj

Table 6.1.8.6.2.9-1: Definition of type TrainingObj

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| objType | PerformanceMetric | M | 1 | Contains the metric to be optimized by the ML model (e.g., accuracy). |  |
| targetValue | Float | O | 0..1 | Contains the threshold to be reached for the objective type. |  |
| earlyStopCri | EarlyStopCri | O | 0..1 | Contains the metric to be used for early stopping. |  |
| maxEpochs | Uinteger | O | 0..1 | Contains the maximum number of training epochs. |  |
| accTrainingErr | Float | O | 0..1 | Contains the maximum acceptable training error. |  |
| inferenceLatency | Float | O | 0..1 | Contains the inference latency requirements for the trained model. |  |

# *** Next Change ***

###### 6.1.8.6.2.10	Type: EarlyStopCri

Table 6.1.8.6.2.10-1: Definition of type EarlyStopCri

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| minDelta | Float | O | 0..1 | Contains the threshold to consider as improvement. |  |
| patience | Uinteger | O | 0..1 | Contains the number of epochs without improvement to stop the training. |  |

# *** Next Change ***

###### 6.1.8.6.2.11	Type: VFLParam

Table 6.1.8.6.2.11-1: Definition of type VFLParam

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| datasetComm | array(CommonId) | O | 1..N | Contains the list of one or more common features required for VFL training. (E.g., UE ID, AIMLE client ID, Group ID, VAL service ID, etc) |  |
| featureList | array(FeatureList) | O | 1..N | Contains the list of features for each data domain of the dataset at the client. |  |
| featureAlign | array(FeatureList) | O | 1..N | Contains the information to align features from dataset of different domains for VFL training. |  |
| dataLabels | array(string) | O | 1..N | Contains the ground truth data for VFL training. |  |

# *** Next Change ***

###### 6.1.8.6.2.12	Type: CommonId

Table 6.1.8.6.2.12-1: Definition of type CommonId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| featureType | string | O | 0..1 | Contains the type of common features that can be UE identifiers, AIMLE client identifiers, group identifier, VAL service identifier, area of interest, and VAL service area. |  |
| featureId | string | O | 0..1 | Contains the common identifier for the given feature type. |  |

# *** Next Change ***

###### 6.1.8.6.2.13	Type: FeatureList

Table 6.1.8.6.2.13-1: Definition of type FeatureList

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientId | AimleClientId | O | 0..1 | Contains the identifier for the AIMLE clients participating in the VFL training. |  |
| list | array(string) | O | 1..N | Contains the list of all the features for dataset at the AIMLE clients. |  |

# *** Next Change ***

# A.19	AIMLES_MLModelTraining API

openapi: 3.0.0

info:

title: AIMLE ML Model Training API

version: 1.0.0-alpha.1

description: |

API for AIMLE ML Model Training Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 v1.2.0; Service Enabler Architecture Layer for Verticals (SEAL); Artificial

Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/aimles-trn/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request-train:

post:

summary: Request the AIMLE server for AIMLE ML Model Training.

operationId: RequestTrain

tags:

- AIMLE ML Model Training service (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrainRequest'

responses:

'200':

description: >

OK. The requested ML model training response containing ML model ID and ML model

training ID shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MlModelTrainResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

summary: Notify on the requested data.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MlModelTrainNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

TrainRequest:

description: >

Represents the AIMLE ML Model Training service.

type: object

properties:

trnType:

$ref: '#/components/schemas/TrainingType'

members:

type: array

items:

$ref: 'TS29482_AIMLES_ContextTransfer.yaml#/components/schemas/AimleClientId'

minItems: 1

clientSetId:

type: string

memSelCrit:

$ref: '#/components/schemas/MemberSelCriteria'

numReqClients:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

modelInf:

$ref: '#/components/schemas/MlModelInfo'

modelReq:

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModel'

trainObj:

$ref: '#/components/schemas/TrainingObj'

datasetId:

type: array

items:

type: string

minItems: 1

dataSamples:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

opSchedule:

type: string

vflParam:

$ref: '#/components/schemas/VFLParam'

memUpdNotif:

type: boolean

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

required:

- trnType

- datasetId

MlModelTrainNotif:

description: >

Represents the AIMLE ML Model Training Notification.

type: object

properties:

trainingID:

type: string

members:

type: array

items:

$ref: '#/components/schemas/MemberInfo'

minItems: 1

trainOut:

$ref: '#/components/schemas/PerfParams'

percentageComp:

type: integer

minimum: 0

maximum: 100

trainErr:

$ref: '#/components/schemas/TrainingErr'

MlModelTrainResp:

description: >

Represents the AIMLE service status information.

type: object

properties:

trainingId:

type: string

modelId:

type: string

required:

- trainingId

MemberSelCriteria:

description: >

Represents the criteria that needs to be continuously monitored for selecting

the member clients.

type: object

properties:

clientLoc:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

clientAvailability:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

clientCapability:

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/ClientCapability'

MlModelInfo:

description: >

Represents the ML model that has to be distributed to the selected member clients

for training.

type: object

properties:

mlModelId:

type: string

mlModelLoc:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

MemberInfo:

description: >

Represents the list of AIMLE clients selected/de-selected for ML model training.

type: object

properties:

clientSel:

type: boolean

clientUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

PerfParams:

description: >

Represents the output of training, e.g., ML model parameters for the training.

type: object

properties:

modelAccuracy:

type: integer

minimum: 0

maximum: 100

modelPrecision:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

modelRecall:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

modelF1score:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

errorMeanSquare:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

errorMeanAbs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

TrainingObj:

description: >

Represents the termination condition for the ML model training.

type: object

properties:

objType:

$ref: 'TS29482_AIMLES_AssistedMLModelSelection.yaml#/components/schemas/PerformanceMetric'

targetValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

earlyStopCri:

$ref: '#/components/schemas/EarlyStopCri'

maxEpochs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

accTrainingErr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

inferenceLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

required:

- objType

EarlyStopCri:

description: >

Represents the AIMLE service status information.

type: object

properties:

minDelta:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

patience:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

VFLParam:

description: >

Represents the parameters specific to VFL training.

type: object

properties:

datasetComm:

type: array

items:

$ref: '#/components/schemas/CommonId'

minItems: 1

featureList:

type: array

items:

$ref: '#/components/schemas/FeatureList'

minItems: 1

featureAlign:

type: array

items:

$ref: '#/components/schemas/FeatureList'

minItems: 1

dataLabels:

type: array

items:

type: string

minItems: 1

CommonId:

description: >

Represents the list of one or more common features required for VFL training.

type: object

properties:

featureType:

type: string

featureId:

type: string

FeatureList:

description: >

Represents the list of features for each data domain of the dataset at the

client.

type: object

properties:

clientId:

$ref: 'TS29482_AIMLES_ContextTransfer.yaml#/components/schemas/AimleClientId'

list:

type: array

items:

type: string

minItems: 1

# SIMPLE DATA TYPES

#

# ENUMERATIONS

#

TrainingType:

anyOf:

- type: string

enum:

- TRAIN_VFL

- TRAIN_HFL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE service operation status.

Possible values are:

- TRAIN_VFL: Indicates that the training type is horizontal federated learning

- TRAIN_HFL: Indicates that the training type is vertical federated learning

TrainingErr:

anyOf:

- type: string

enum:

- UNDERFITTING

- OVERFITTING

- PERFORMANCE_ERRORS

- DATA_LEAKAGE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the training error encountered during ML model training.

Possible values are:

- UNDERFITTING: Indicates that the trained model is underfitting the

training data.

- OVERFITTING: Indicates that the trained mode is overfitting the training

data.

- PERFORMANCE_ERRORS: Indicates that the trained model is unable to meet the

desired performance.

- DATA_LEAKAGE: Indicates that there is data leakage from evaluation data set

to the training data.

# *** End of Changes ***