3GPP TSG-CT WG3 Meeting #144	C3-255145

Dallas, United States, 17 - 21 November, 2025	(Revision of C3-25xxxx))

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.552 | CR | 0167 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to events in procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNetAE19 |  |  |  |  |  | Date: |  |  | 2025-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the procedures of NF load Analytics, User Data Congestion Analytics, QoS Sustainability Analytics, Dispersion Analytics, WLAN Performance Analytics, Session Management Congestion Control Experience Analytics and Redundant Transmission Experience Analytics, the event naming and supported feature has been described in the original steps, while the other procedures still missing event naming and supported feature description in the initial step which is not clear and not match with the description of In order to obtain the specific event analytics, hence need to add them in the affected clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the missing event naming and supported feature in the original steps of the affected clauses. Removed the if the event related descriptions in step 2a-2b in clauses 5.7.20, 5.7.21 and 5.7.22 which is not aligned with stage 2 procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not matched and not aligned procedure steps description for specific event with supported feature in the initial steps of procedures of analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2, 5.7.3, 5.7.5, 5.7.6, 5.7.7, 5.7.8, 5.7.9, 5.7.16, 5.7.17, 5.7.18, 5.7.19, 5.7.20, 5.7.21, 5.7.22, 5.7.23, 5.7.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The removal of the events in step 2a-2b in clauses 5.7.2, 5.7.3, 5.7.5 and 5.7.9 is included in another TS 29.552 CR Corrections to authorization in procedures, for effective implementation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

### 5.7.2	Network Slice (Instance) load level Analytics

This procedure is used by the NF to obtain the network slice (instance) load level analytics which are calculated by the NWDAF based on the information collected from the NSACF, NRF, AMF, SMF and/or OAM. If the NF is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.2-1: Procedure for Network Slice (Instance) load level Analytics

1a.	In order to obtain the network slice (instance) load level analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "LOAD_LEVEL_INFORMATION", or "NSI_LOAD_LEVEL" with the supported feature "NsiLoad".

1b-1c.	In order to obtain the network slice (instance) load level analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "SLICE_LOAD_LEVEL", or "NSI_LOAD_LEVEL" with the supported feature "NsiLoad".

2a-2b.	If the event is set to "LOAD_LEVEL_INFORMATION"/"SLICE_LOAD_LEVEL" or "NSI_LOAD_LEVEL", the NWDAF invokes Nnrf_NFDiscovery_Request service operation as described in clause 5.3.2.2 of 3GPP TS 29.510 [23] to discover the AMF, SMF and NSSF instance(s) relevant to the analytics filters provided in the subscription request. The NRF responds to the NWDAF an HTTP "200 OK" response.

3a-3b.	(Only for "NSI_LOAD_LEVEL")The NWDAF may invoke Nnssf_NSSelection_Get service operation as described in clause 5.2.2.2 of 3GPP TS 29.531 [21] to obtain the NSI ID(s) corresponding to the S-NSSAI in the subscription request. The NSSF responds to the NWDAF an HTTP "200 OK" response.

4a-4b.	(Only for "NSI_LOAD_LEVEL")The NWDAF may invoke Nnrf_NFManagement_NFStatusSubscribe service operation as described in clause 5.2.2.5 of 3GPP TS 29.510 [23] to subscribe to the resource usage information of a network slice instance obtained from its constituent NF instances. The NRF responds to the NWDAF an HTTP "201 Created" response.

5a-5b. (Only for "NSI_LOAD_LEVEL")	If step 4a and step 4b are performed, the NRF may invoke Nnrf_NFManagement_NFStatusNotify service operation as described in clause 5.2.2.6 of 3GPP TS 29.510 [23]. The NWDAF responds to the NRF an HTTP "204 No Content" response.

6a-6b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to subscribe to the notification of individual UE registration/deregistration registered to an S-NSSAI or to an S-NSSAI and NSI ID, or request the total number of UEs served by the AMF per S-NSSAI or per S-NSSAI and NSI ID. The AMF responds to the NWDAF an HTTP "201 Created" response.

7a-7b.	If step 6a and step 6b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

8a-8b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" to subscribe to the notification of individual PDU session established or PDU session released in an S-NSSAI or request the total number of PDU Sessions established in an S-NSSAI. The SMF responds to the NWDAF an HTTP "201 Created" response.

9a-9b.	If step 8a and step 8b are performed, the SMF may invoke Nsmf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 8a. The NWDAF responds to the SMF an HTTP "204 No Content" response.

10.	The NWDAF may collect "Performance measurement" to the OAM to get the mean number of UEs registered as described in clause 5.2.1 of TS 28.552 [27], mean number of PDU sessions established as described in clause 5.3.1 of TS 28.552 [27] and/or the resource usage information of a network slice instance obtained from its constituent NF instances as described in clause 6.2 of TS 28.552 [27]. (Obtaining the resource usage information of a network slice instance is only applicable for "NSI_LOAD_LEVEL" event).

11a-11b.	The NWDAF may invoke Nnsacf_SliceEventExposure_Subscribe service operation as described in clause 5.3.2.2 of 3GPP TS 29.536 [20] to request the number of UEs registered to the network slice and/or the number of PDU sessions established to the network slice.

12a-12b.	The NSACF may invoke Nnsacf_SliceEventExposure_Notify service operation as described in clause 5.3.2.4 of 3GPP TS 29.536 [20]. The NWDAF responds to the NSACF an HTTP "204 No Content" response.

13.	The NWDAF calculates the network slice (instance) load level analytics based on the data collected from AMF, SMF, NRF, NSACF and/or OAM.

14a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

14b-14c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

15a-15b.	The same as step 5a and step 5b.

16a-16b.	The same as step 7a and step 7b.

17a-17b.	The same as step 9a and step 9b.

18.	The same as step 10.

19a-19b.	The same as step 12a and step 12b.

20.	The same as step 13.

21a-21b.	The same as step 14b and step 14c.

NOTE 1:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 2:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

# *** 2nd Change ***

### 5.7.3	Observed Service Experience Analytics

This procedure is used by the NF to obtain the Service Experience analytics which are calculated by the NWDAF based on the information collected from the AMF, SMF, UPF, GMLC, AF, OAM and/or MDAF. If the NF is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.3-1: Procedure for Service Experience Analytics

1a.	In order to obtain the Service Experience analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "SERVICE_EXPERIENCE" with the supported feature "ServiceExperience".

1b-1c.	In order to obtain the Service Experience analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "SERVICE_EXPERIENCE" with the supported feature "ServiceExperience".

2a-2b.	If the event is set to "SERVICE_EXPERIENCE", the NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to subscribe to the notification of UE ID and UE location. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" to subscribe to QFI allocation event. The SMF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	If step 4a and step 4b are performed, the SMF may invoke Nsmf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the SMF an HTTP "204 No Content" response.

6a-6b.	If the AF is trusted, the NWDAF may invoke Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions" to request the service data and performance data from AF directly. The AF responds to the NWDAF an HTTP "201 Created" response.

7a-7b.	If step 6a and step 6b are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 6a. The NWDAF responds to the AF an HTTP "204 No Content" response.

8a-8d.	If the AF is untrusted, the NWDAF may invoke Nnef_EventExposure_Subscribe service operation to the NEF by sending an HTTP POST request targeting the resource "Network Exposure Event Subscriptions" and then the NEF invokes Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions". The AF responds to the NEF an HTTP "201 Created" response and then the NEF responds to the NWDAF an HTTP "201 Created" response.

9a-9d.	If step 8a to step 8d are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NEF identified by the notification URI received in step 8b and the NEF invokes Nnef_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 8a. The NWDAF responds to the NEF an HTTP "204 No Content" response and then the NEF responds to the AF an HTTP "204 No Content" response.

10a-10b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" as described in 3GPP TS 29.508 [6] to subscribe via the SMF to UPF information for a specific UE. The SMF subscribes to the UPF on behalf of the NWDAF via N4 Session Reporting Rule as described in clause 5.2.8 of 3GPP TS 29.244 [45] and, after having received the successful response from the UPF, it responds to the NWDAF with an HTTP "201 Created" response.

11a-11b.	If step 10a and step 10b are performed, the UPF may invoke Nupf_EventExposure_Notify service operation as described in 3GPP TS 29.564 [40] by sending an HTTP POST request to the NWDAF identified by the notification URI provided in step 10a. The NWDAF responds to the UPF an HTTP "204 No Content" response.

12a-12b.	If the NF requestes fine granularity location analytics information, the NWDAF may invoke Ngmlc_Location_ProvideLocation service operation to retrieve UE Location and UE Location Accuracy by sending an HTTP POST request to the URI associated with the "provide-location" custom operation as described in 3GPP TS 29.515 [41] clause 5.2.2.2. The GMLC responds to the NWDAF an HTTP "200 OK" response.

13a-13b.	If step 12a and step 12b are performed, the GMLC may invoke Ngmlc_Location_EventNotify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 12a. The NWDAF responds to the GMLC an HTTP "204 No Content" response.

14a.	The NWDAF may collect Reference Signal Received Power and Reference Signal Received Quality as specified in clause 5.5 of TS 38.331 [33] and clause 5.5.5 of TS 36.331 [34], Signal-to-noise and interference ratio as specified in clause 5.1 of TS 38.215 [35], the mapping information between cell ID and frequency and/or Cell Energy Saving State data as specified in clauses 3.1 and 6.2 of TS 28.310 [36] from OAM. If the NF is AF or NEF requesting per application analytics with UE granularity, the NWDAF may collect the average UL/DL RAN throughput, the UL/DL RAN packet delay and the UL/DL RAN Packet loss rate as described in clause 5.2.1.1 and 5.4.1.1 of TS 37.320 [29]. If the NWDAF maps the provided Application ID as DASH, MTSI or VR in step 1a or step 1b, the NWDAF may act as an MnS Consumer to collect the QoE measurements data for all the UEs in the interested area from OAM as defined in TS 28.404 [48], TS 28.405 [49] and TS 28.406 [50].

NOTE 1:	Care needs to be taken to avoid end to end excessive signalling for collecting the QoE measurements from the applications in the UE.

14b.	The NWDAF may collect service experience and energy saving state analysis as specified in clause 8.4.2.1.3 and clause 8.4.4.3 of TS 28.104 [38] from MDAF. The information element of the analysis from MDAF include ServiceExperienceIssueType, AffectedObjects, ServiceExperienceStatistics, ServiceExperiencePredictions, and StatisticsOfCellsEsState.

15.	The NWDAF calculates the Service Experience analytics based on the data collected from AMF, SMF, UPF, GMLC, AF, OAM and/or MDAF.

16a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

16b-16c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

17a-17b.	The same as step 3a and step 3b.

18a-18b.	The same as step 5a and step 5b.

19a-19d.	The same as step 7a and step 7b.

20a-20d.	The same as step 9a and step 9b.

21a-21b.	The same as step 11a and step 11b.

22a-22b.	The same as step 13a and step 13b.

23.	The same as step 15.

24a-24b.	The same as step 16b and step 16c.

NOTE 2:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 3:	For details of Nnef_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.591 [11].

NOTE 4:	For details of Naf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.517 [12].

# *** 3rd Change ***

### 5.7.5	Network Performance Analytics

This procedure is used by the NF to obtain the network performance analytics which are calculated by the NWDAF based on the information collected from the AMF, NRF and/or OAM. If the NF is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.5-1: Procedure for Network Performance Analytics

1a.	In order to obtain the network performance analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "NETWORK_PERFORMANCE" with the supported feature "NetworkPerformance".

1b-1c.	In order to obtain the network performance analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "NETWORK_PERFORMANCE" with the supported feature "NetworkPerformance".

2a-2b.	If the subscription/request is authorized, the NWDAF may invoke Nnrf_NFManagement_NFStatusSubscribe service operation as described in clause 5.2.2.5 of 3GPP TS 29.510 [23] to subscribe to NF (e.g. SMF, UPF) load and status information. The NRF responds to the NWDAF an HTTP "200 OK" response.

3a-3b.	If step 2a and step 2b are performed, the NRF may invoke Nnrf_NFManagement_NFStatusNotify service operation as described in clause 5.2.2.6 of 3GPP TS 29.510 [23]. The NWDAF responds to the NRF an HTTP "204 No Content" response.

4a-4b.	If the event is set to "NETWORK_PERFORMANCE" and the type of network performance is set to "NUM_OF_UE", the NWDAF invokes Nnrf_NFDiscovery_Request service operation as described in clause 5.3.2.2 of 3GPP TS 29.510 [23] to discover the AMF(s) belonging to the AMF Region(s) that include(s) the Area of Interest. The NRF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to collect the number of UEs located in the Area of Interest from AMF. The AMF responds to the NWDAF an HTTP "201 Created" response.

6a-6b.	If step 5a and step 5b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

7.	The NWDAF may collect "Performance measurement" to the OAM to get the status and load information and the performance per Cell Id in the Area of Interest as described in clause 5.1 of TS 28.552 [27].

8.	The NWDAF calculates the network performance analytics based on the data collected from AMF, NRF and/or OAM.

9a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

9b-9c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

10a-10b.	The same as step 3a and step 3b.

11a-11b.	The same as step 6a and step 6b.

12.	The same as step 7.

13.	The same as step 8.

14a-14b.	The same as step 9b and step 9c.

NOTE:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

# *** 4th Change ***

### 5.7.6	UE Mobility Analytics

This procedure is used by the NF to obtain UE mobility analytics, which is calculated by the NWDAF based on the information collected from the AMF, AF and/or OAM. If the NF is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.6-1: Procedure for UE Mobility analytics

1a.	In order to obtain the UE mobility analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "UE_MOBILITY" with the supported feature "UeMobility".

1b-1c.	In order to obtain the UE mobility analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "UE_MOBILITY" with the supported feature "UeMobility".

2a-2b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18]. This step may be skipped when e.g. the UE mobility information is available. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in clause 5.3.2.4 of 3GPP TS 29.518 [18]. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	If the AF is trusted, the NWDAF may invoke Naf_EventExposure_Subscribe service operation to the AF directly by sending an HTTP POST request targeting the resource "Application Event Subscriptions". The AF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	If step 4a and step 4b are performed, the AF invokes Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the AF an HTTP "204 No Content" response.

6a-6d.	If the AF is untrusted, the NWDAF may invoke Nnef_EventExposure_Subscribe service operation to the NEF by sending an HTTP POST request targeting the resource "Network Exposure Event Subscriptions" and then the NEF invokes Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions". The AF responds to the NEF an HTTP "201 Created" response and then the NEF responds to the NWDAF an HTTP "201 Created" response.

7a-7d.	If step 6a to step 6d are performed, the AF invokes Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NEF identified by the notification URI received in step 6b and the NEF invokes Nnef_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 6a. The NWDAF responds to the NEF an HTTP "204 No Content" response and then the NEF responds to the AF an HTTP "204 No Content" response.

8.	The NWDAF may invoke "streaming data reporting service reportStreamData" service operation to the OAM as described in clause 12.5.1.1.5 of 3GPP TS 28.532 [19].

9.	The NWDAF may invoke the "File data reporting service subscribe" service operation to the OAM as described in clause 12.6.1.1.3 of 3GPP TS 28.532 [19] or invoke "File data reporting service listAvailableFiles" service operation to the OAM as described in clause 12.6.1.1.2 of 3GPP TS 28.532 [19].

10. The NWDAF calculates the requested UE mobility analytics based on the data collected from AMF, AF and/or OAM.

11a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

11b-11c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

12a-12b.	The same as step 3a and step 3b.

13a-13b.	The same as step 5a and step 5b.

14a-14d.	The same as step 7a and step 7d.

15.	The same as step 9.

16.	The same as step 10.

17a-17b.	The same as step 11b and step 11c.

NOTE 1:	For details of Naf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.517 [12].

NOTE 2:	For details of Nnef_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.591 [11].

NOTE 3:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

# *** 5th Change ***

### 5.7.7	UE Communication Analytics

This procedure is used by the NF to obtain UE communication analytics, which is calculated by the NWDAF based on the information collected from the AMF, SMF, UPF, and/or AF. If the NF is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.7-1: Procedure for UE Communication analytics

1a.	In order to obtain the UE communication analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "UE_COMMUNICATION" with the supported feature "UeCommunication".

1b-1c.	In order to obtain the UE communication analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "UE_COMMUNICATION" with the supported feature "UeCommunication".

2a-2b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to retrieve one or more Type Allocation codes, UE connection management state, UE access behaviour trends and UE location trends from AMF. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF may invoke Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" as described in 3GPP TS 29.508 [6] to request the information of the UE to calculate the analytics. The SMF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	If step 4a and step 4b are performed, the SMF may invoke Nsmf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the SMF an HTTP "204 No Content" response.

6a-6b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" as described in 3GPP TS 29.508 [6] to subscribe via the SMF to UPF information for a specific UE. The SMF subscribes to the UPF on behalf of the NWDAF using the Nupf_EventExposure_Subscribe service operation as described in 3GPP TS 29.564 [40] and, after having received the successful response from the UPF, it responds to the NWDAF with an HTTP "201 Created" response.

7a-7b.	If step 6a and step 6b are performed, the UPF may invoke Nupf_EventExposure_Notify service operation as described in 3GPP TS 29.564 [40] by sending an HTTP POST request to the NWDAF identified by the notification URI provided in step 6a. The NWDAF responds to the UPF an HTTP "204 No Content" response.

8a-8b.	If the AF is trusted, the NWDAF may invoke Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions" to request the service data from AF directly. The AF responds to the NWDAF an HTTP "201 Created" response.

9a-9b.	If step 8a and step 8b are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 8a. The NWDAF responds to the AF an HTTP "204 No Content" response.

10a-10d.	If the AF is untrusted, the NWDAF may invoke Nnef_EventExposure_Subscribe service operation to the NEF by sending an HTTP POST request targeting the resource "Network Exposure Event Subscriptions" and then the NEF invokes Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions". The AF responds to the NEF an HTTP "201 Created" response and then the NEF responds to the NWDAF an HTTP "201 Created" response.

11a-11d.	If step 10a to step 10d are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NEF identified by the notification URI received in step 8b and the NEF invokes Nnef_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 8a. The NWDAF responds to the NEF an HTTP "204 No Content" response and then the NEF responds to the AF an HTTP "204 No Content" response.

12. The NWDAF calculates the requested UE communication analytics based on the data collected from AMF, SMF, UPF and/or AF.

13a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

13b-13c.	If step 1b and step 1c are performed, the NWDAF may invoke Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

14a-14b.	The same as step 3a and step 3b.

15a-15b.	The same as step 5a and step 5b.

16a-16b.	The same as step 7a and step 7b.

17a-17b.	The same as step 9a and step 9b.

18a-18d.	The same as step 11a and step 11d.

19.	The same as step 12.

20a-20b.	The same as step 13b and step 13c.

NOTE 1:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 2:	For details of Naf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.517 [12].

NOTE 3:	For details of Nnef_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.591 [11].

NOTE 4:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

# *** 6th Change ***

### 5.7.8	Expected UE behavioural Analytics

This procedure is used by the NF to obtain the expected UE behavioural parameters, which are calculated by the NWDAF based on the information collected from the AMF, SMF, AF and/or OAM. If the NF is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.8-1: Procedure for Expected behavioural Analytics

1a.	In order to obtain the expected UE behavioural parameters, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "UE_MOBILITY" with the supported feature "UeMobility", and/or "UE_COMMUNICATION" with the supported feature "UeCommunication".

1b-1c.	In order to obtain expected UE behavioural parameters, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "UE_MOBILITY" with the supported feature "UeMobility", and/or "UE_COMMUNICATION" with the supported feature "UeCommunication".

2a.	If the event is set to "UE_MOBILITY", the NWDAF collects data from AMF, AF and/or OAM as described in clause 5.7.6 from step 2a to step 9.

2b.	If the event is set to "UE_COMM", the NWDAF collects data from AMF, SMF and/or AF as described in clause 5.7.7 from step 2a to step 9d.

3. The NWDAF calculates the expected UE behavioural parameters based on the collected data from AMF, SMF, AF and/or OAM.

4a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

4b-4c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

5a-5b.	The AMF, SMF, AF and/or OAM send the notifications to the NWDAF if it has subscribed to the related events in step 2a or step 2b.

6.	The same as step 3.

7a-7b.	The same as step 4b and step 4c.

NOTE:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

# *** 7th Change ***

### 5.7.9	Abnormal UE behavioural Analytics

This procedure is used by the NF to obtain the abnormal UE behavioural analytics which are calculated by the NWDAF based on the information collected from the AMF, SMF, AF and/or OAM. If the NF is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.9-1: Procedure for Abnormal UE behavioural Analytics

1a.	In order to obtain the abnormal UE behavioural parameters, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "ABNORMAL_BEHAVIOUR" with the supported feature "AbnormalBehaviour".

1b-1c.	In order to obtain abnormal UE behavioural parameters, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "ABNORMAL_BEHAVIOUR" with the supported feature "AbnormalBehaviour".

2a.	If the event is set to "ABNORMAL_BEHAVIOUR" and the expected UE analytics type is set to "MOBILITY" or "MOBILITY_AND_COMMUN", or the list of exception IDs includes "UNEXPECTED_UE_LOCATION", "PING_PONG_ACROSS_CELLS", "UNEXPECTED_RADIO_LINK_FAILURES", or "UNEXPECTED_WAKEUP", the NWDAF collects data from AMF, AF and/or OAM as described in clause 5.7.6 from step 2a to step 9.

2b.	If the event is set to "ABNORMAL_BEHAVIOUR" and the expected UE analytics type is set to "COMMUN" or "MOBILITY_AND_COMMUN", or the list of exception IDs includes "SUSPICION_OF_DDOS_ATTACK", "UNEXPECTED_LONG_LIVE_FLOW", "UNEXPECTED_LARGE_RATE_FLOW", "WRONG_DESTINATION_ADDRESS", "TOO_FREQUENT_SERVICE_ACCESS" or "UNEXPECTED_WAKEUP", the NWDAF collects data from AMF, SMF and/or AF as described in clause 5.7.7 from step 2a to step 9d.

3.	The NWDAF calculates the abnormal UE behavioural parameters based on the collected data from AMF, SMF, AF and/or OAM.

4a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

4b-4c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

5a-5b.	The AMF, SMF, AF and/or OAM send the notifications to the NWDAF if it has subscribed to the related events in step 2a or step 2b.

6.	The same as step 3.

7a-7b.	The same as step 4b and step 4c.

NOTE:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

# *** 8th Change ***

### 5.7.16	DN Performance Analytics

This procedure is used by the NF to obtain DN performance analytics, which is calculated by the NWDAF based on the information collected from the AMF, SMF, AF, UPF and/or OAM. If the NF is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.16-1: Procedure for DN Performance Analytics

1a.	In order to obtain the DN performance analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "DN_PERFORMANCE" with the supported feature "DnPerformance".

1b-1c.	In order to obtain the DN performance analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "DN_PERFORMANCE" with the supported feature "DnPerformance".

2a-2b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to retrieve the UE location information and SUPI(s) from AMF. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF may invoke Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" to subscribe to QFI allocation event. The SMF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	If step 4a and step 4b are performed, the SMF may invoke Nsmf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the SMF an HTTP "204 No Content" response.

6a-6b.	To collect QoS flow bit rate, QoS flow packet delay, packet transmission and packet retransmission information from UPF, after step 5 was performed, the SMF may subscribe to the UPF on behalf of the NWDAF via N4 Session Reporting Rule as described in clause 5.2.8 of 3GPP TS 29.244 [45].

7a-7b.	The UPF invokes Nupf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 6a. The NWDAF responds to the UPF an HTTP "204 No Content" response.

7c.	The NWDAF may collect performance data from AF as described in clause 5.7.7 from step 6a to step 9d.

8.	The NWDAF may collect Reference Signal Received Power and Reference Signal Received Quality as specified in clause 5.5 of TS 38.331 [33] and clause 5.5.5 of TS 36.331 [34], Signal-to-noise and interference ratio as specified in clause 5.1 of TS 38.215 [35], the mapping information between cell ID and frequency and Cell Energy Saving State data as specified in clauses 3.1 and 6.2 of TS 28.310 [36] from OAM.

9.	The NWDAF calculates the DN performance analytics based on the data collected from AMF, SMF, AF, UPF and/or OAM.

10a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

10b-10c.	If step 1b and step 1c are performed, the NWDAF may invoke Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

11a-11b.	The same as step 3a and step 3b.

12a-12b.	The same as step 5a and step 5b.

13.	The same as step 6.

14.	The same as step 9.

15a-15b.	The same as step 10b and step 10c.

NOTE 1:	For details of N4 Session Reporting Report refer to 3GPP TS 29.244 [45].

NOTE 2:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 3:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

# *** 9th Change ***

### 5.7.17	PFD Determination Analytics

This procedure is used by the NWDAF service consumer e.g. NEF(PFDF) to obtain PFD determination analytics for the know application(s), which is calculated by the NWDAF based on the information collected from the UPF and the NEF(PFDF).

Figure 5.7.17-1: Procedure for PFD Determination Analytics

1a-1b.	In order to obtain the PFD determination analytics, the consumer NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "PFD_DETERMINATION" with the supported feature "PfdDetermination".

2a-2b.	The NWDAF may invoke Nnef_PFDmanagement_Subscribe service operation as described in clause 4.2.3 of 3GPP TS 29.551 [39] by sending an HTTP POST request targeting the resource "PFD subscriptions" to subscribe to retrieve PFDs for an Application Identifier(s) from the NEF (PFDF). The NEF (PFDF) responds to the NWDAF an HTTP "201 Created" response.

2c-2d.	The NEF (PFDF) may invoke Nnef_PFDmanagement_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 2a. The NWDAF responds to the NEF (PFDF) an HTTP "204 No Content" response.

3.	The NWDAF determines whether it needs to collect data.

3a-3b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" as described in 3GPP TS 29.508 [6] to subscribe via the SMF to UPF to retrieve the application traffic flow related information for the application. The SMF responds to the NWDAF an HTTP "201 Created" response after having received HTTP "201 Created" response from the UPF.

4a-4b.	The SMF subscribes to the UPF on behalf of the NWDAF by invoking Nupf_EventExposure_Subscribe service operation as described in clause 5.2.2.2 of 3GPP TS 29.564 [40] to retrieve the application traffic flow related information from the UPF. The UPF responds to the SMF an HTTP "201 Created" response.

5a-5b.	The UPF invokes Nupf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the UPF an HTTP "204 No Content" response.

6.	The NWDAF calculates the PFD determination analytics based on the data collected from UPF and NEF (PFDF).

7a-7b.	The NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1 containing the PFD determination analytics information in the case that the NWDAF decides the PFD information for the existing Application ID is new or to be updated.

NOTE 1:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 2:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify service operations refer to 3GPP TS 29.520 [5].

NOTE 3:	For details of Nnef_PFDManagement_Subscribe/Unsubscribe/Notify service operations refer to 3GPP TS 29.551 [39].

NOTE 4:	For details of Nupf_EventExposure_Subscribe/Unsubscribe/Notify service operations refer to 3GPP TS 29.564 [40].

# *** 10th Change ***

### 5.7.18	E2E data volume transfer time analytics

This procedure is used by the NWDAF service consumer (may be an NF e.g. AF or NEF) to obtain E2E (end-to-end) data volume transfer time analytics, which is calculated by the NWDAF based on the information collected from the AMF, SMF, UPF, AF, LCS via GMLC and/or OAM. If the NF is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.18-1: Procedure for E2E data volume transfer time analytics

1a.	In order to obtain the E2E data volume transfer time analytics, the NWDAF service consumer may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "E2E_DATA_VOL_TRANS_TIME" with the supported feature "E2eDataVolTransTime".

1b-1c.	In order to obtain the E2E data volume transfer time analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "E2E_DATA_VOL_TRANS_TIME" with the supported feature "E2eDataVolTransTime".

2a-2b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to retrieve the UE location information and SUPI(s) from AMF. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF may invoke Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4d.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" as described in 3GPP TS 29.508 [6], and the SMF may subscribe to the UPF on behalf of the NWDAF via N4 Session Reporting Rule as described in clause 5.2.8 of 3GPP TS 29.244 [45]. After having received the successful response from the UPF, the SMF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	If step 4a and step 4d are performed, the SMF may invoke Nsmf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the SMF an HTTP "204 No Content" response.

5c-5d.	If step 4b and step 4c are performed, the UPF may invoke Nupf_EventExposure_Notify service operation as described in 3GPP TS 29.564 [40] by sending an HTTP POST request to the NWDAF identified by the notification URI provided in step 4b. The NWDAF responds to the UPF an HTTP "204 No Content" response.

6a-6b.	If the AF is trusted, the NWDAF may invoke Naf_EventExposure_Subscribe service operation to the AF directly by sending an HTTP POST request targeting the resource "Application Event Subscriptions" to collect the End-to-end data volume transfer time information. The AF responds to the NWDAF an HTTP "201 Created" response.

7a-7b.	If step 6a and step 6b are performed, the AF invokes Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 7a. The NWDAF responds to the AF an HTTP "204 No Content" response.

8a-8d.	If the AF is untrusted, the NWDAF may invoke Nnef_EventExposure_Subscribe service operation to the NEF by sending an HTTP POST request targeting the resource "Network Exposure Event Subscriptions" and then the NEF invokes Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions" to collect the E2E data volume transfer time information. The AF responds to the NEF an HTTP "201 Created" response and then the NEF responds to the NWDAF an HTTP "201 Created" response.

9a-9d.	If step 8a to step 8d are performed, the AF invokes Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NEF identified by the notification URI received in step 9b and the NEF invokes Nnef_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 9a. The NWDAF responds to the NEF an HTTP "204 No Content" response and then the NEF responds to the AF an HTTP "204 No Content" response.

10a-10b.	The NWDAF may invoke Ngmlc_Location_ProvideLocation service operation to retrieve UE Location and UE Location Accuracy by sending an HTTP POST request to the URI associated with the "provide-location" custom operation as described in 3GPP TS 29.515 [41] clause 5.2.2.2. The GMLC responds to the NWDAF an HTTP "200 OK" response.

10c-10d.	If step 10a and step 10b are performed, the GMLC may invoke Ngmlc_Location_EventNotify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the GMLC an HTTP "204 No Content" response.

11.	The NWDAF may collect RAN part delay for DL and UL per 5QI and S-NSSAI as specified in clauses 6.3.1.2 and 6.3.1.7 of 3GPP TS 28.554 [30], RAN Throughput for DL and UL as specified in clauses 5.2.1.1 and 5.4.1.1 of 3GPP TS 37.320 [29], RAN Packet delay / Packet loss rate for DL and UL per UE including per DRB per UE packet delay / Packet loss rate as specified in clauses 5.4.1.1 of 3GPP TS 37.320 [29], the average of UL/DL packet delay between PSA UPF and UE per S-NSSAI as specified in clauses 5.4.9.1.1 and 5.4.9.2.1 of 3GPP TS 28.552 [27], the average DL/UL UE throughput in the gNB per QoS level (mapped 5QI) and per S-NSSAI as specified in clauses 5.1.1.3.1 and 5.1.1.3.3 of 3GPP TS 28.552 [27], per UE per supported S-NSSAI and per QoS level of RAN part delay, RAN Packet loss rate and average DL/UL UE throughput in gNB as specified in clause 6.3.1 of 3GPP TS 28.558 [45]; per UE per supported S-NSSAI and per QoS level of average UL/DL packet delay between PSA UPF and UE, between PSA UPF and RAN as specified in clause 6.2.2.1 of 3GPP TS 28.558 [46] from OAM.

12. The NWDAF calculates the End-to-end data volume transfer time analytics based on the data collected from AMF, SMF, UPF, AF, LCS via GMLC and/or OAM.

13a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

13b-13c.	If step 1b and step 1c are performed, the NWDAF may invoke Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

14a-14b.	Follow the same as step 3a and step 3b.

15a-15d.	Follow the same as step 5a to step 5d.

16a-16b.	Follow the same as step 7a and step 7b.

17a-17d.	Follow the same as step 9a to  9d.

18a-18b.	Follow the same as step 10c and step 10d.

19.	Follow the same as step 11.

20.	Follow the same as step 12.

21a-21b.	Follow the same as step 13b and step 13c.

NOTE 1:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 2:	For details of Naf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.517 [12].

NOTE 3:	For details of Nnef_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.591 [11].

NOTE 4:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

# *** 11th Change ***

### 5.7.19	PDU Session Traffic Analytics

This procedure is used by the NWDAF service consumer (e.g. PCF) to subscribe or request PDU Session Traffic analytics on whether traffic of UEs via one or multiple PDU Sessions is according to the information provided by the service consumer or not.

Figure 5.7.19-1: Procedure for PDU Session Traffic Analytics

1a.	In order to obtain the PDU Session Traffic analytics, the NWDAF service consumer (e.g. PCF) may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "PDU_SESSION_TRAFFIC" with the supported feature "PduSesTraffic".

1b-1c.	In order to obtain the PDU Session Traffic analytics, the NWDAF service consumer (e.g. PCF) may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "PDU_SESSION_TRAFFIC" with the supported feature "PduSesTraffic".

2.	The NWDAF determines whether it needs to collect the data and, if needed, it collects the data either directly from the UPF or indirectly via the SMF and identifies the SMF(s) and/or UPF(s) to retrieve the data.

3a-3b.	[Conditional Option 1] The NWDAF shall invoke the Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" as described in 3GPP TS 29.508 [6] to subscribe via the SMF to UPF to retrieve "UserDataUsageMeasures" UPF event. The SMF responds to the NWDAF an HTTP "201 Created" response after having received HTTP "201 Created" response from the UPF.

4a-4b.	The SMF subscribes to the UPF on behalf of the NWDAF by invoking Nupf_EventExposure_Subscribe service operation as described in clause 5.2.2.2 of 3GPP TS 29.564 [40] to retrieve "USER_DATA_USAGE_MEASURES" UPF event from the UPF. The UPF responds to the SMF an HTTP "201 Created" response.

5a-5b.	[Conditional Option 2] The NWDAF may directly invoke the Nupf_EventExposure_Subscribe service operation as described in clause 5.2.2.2 of 3GPP TS 29.564 [40] to retrieve "USER_DATA_USAGE_MEASURES" UPF event from the UPF. The UPF responds to the NWDAF an HTTP "201 Created" response.

NOTE 1:	The conditions for subscribing to UPF events directly at the UPF or via the SMF, i.e., the conditions for invoking step 3 or step 5 are described in 3GPP TS 29.564 [40].

6a-6b.	The UPF invokes Nupf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a or step 5a. The NWDAF responds to the UPF an HTTP "204 No Content" response.

7.	The NWDAF derives analytics indicating a list of UEs and their detected traffic that is matched according to the information provided by the consumer (i.e. Traffic Descriptor, S-NSSAI and DNN), including its volume and a list of UEs and their detected traffic that it is not matched according to the information provided by the consumer (i.e. Traffic Descriptor, S-NSSAI and DNN) including its volume.

8a.	If step 1a is performed, the NWDAF sends the Nnwdaf_AnalyticsInfo_Request response containing the PDU Session Traffic analytics as described in clause 5.2.3.1.

8b-8c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation containing the PDU Session Traffic analytics as described in clause 5.2.2.1.

NOTE 2:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

NOTE 3:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

# *** 12th Change ***

### 5.7.20	Relative Proximity Analytics

This procedure is used by the NWDAF service consumer e.g. NEF or AF to obtain Relative Proximity Analytics among UEs provided by NWDAF to assist more accurately localize a cluster (or a set) of UEs via provisioning statistics and/or prediction information related to their relative proximity. If the NWDAF service consumer is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.20-1: Procedure for Relative Proximity Analytics

1a.	In order to obtain the Relative Proximity Analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "RELATIVE_PROXIMITY" with the supported feature "RelativeProximity".

1b-1c.	In order to subscribe to the Relative Proximity Analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "RELATIVE_PROXIMITY" with the supported feature "RelativeProximity".

2a-2b.	If the subscription/request is authorized, the NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to subscribe to the notification of UE ID and UE location. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	The NWDAF may invoke Ngmlc_Location_ProvideLocation service operation to retrieve UE Location and UE Location Accuracy by sending an HTTP POST request to the URI associated with the "provide-location" custom operation as described in 3GPP TS 29.515 [41] clause 5.2.2.2. The GMLC responds to the NWDAF an HTTP "200 OK" response.

5a-5b.	If step 4a and step 4b are performed, the GMLC may invoke Ngmlc_Location_EventNotify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the GMLC an HTTP "204 No Content" response.

6a-6b.	If the AF is trusted, the NWDAF may invoke Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions" to subscribe the Proximity attributes and/or Proximity related input data of UE(s) from AF directly. The AF responds to the NWDAF an HTTP "201 Created" response.

7a-7b.	If step 6a and step 6b are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 6a. The NWDAF responds to the AF an HTTP "204 No Content" response.

8a-8d.	If the AF is untrusted, the NWDAF may invoke Nnef_EventExposure_Subscribe service operation to the NEF by sending an HTTP POST request targeting the resource "Network Exposure Event Subscriptions" and then the NEF invokes Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions". The AF responds to the NEF an HTTP "201 Created" response and then the NEF responds to the NWDAF an HTTP "201 Created" response.

9a-9d.	If step 8a to step 8d are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NEF identified by the notification URI received in step 8b and the NEF invokes Nnef_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 8a. The NWDAF responds to the NEF an HTTP "204 No Content" response and then the NEF responds to the AF an HTTP "204 No Content" response.

10.	The NWDAF may collect Per UE information of Speed and Orientation related input data as specified in clause 4.3.30 of TS 28.622 [42], clause 11.1 of TS 28.532 [19] and clause 5.10.29 of TS 32.422 [43].

11.	The NWDAF derives the Relative Proximity Analytics based on the data collected from AMF, GMLC, (DC)AF, and/or OAM.

12a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

12b-12c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

13a-13b.	The same as step 3a and step 3b.

14a-14b.	The same as step 5a and step 5b.

15a-15b.	The same as step 7a and step 7b.

16a-16d.	The same as step 9a and step 9b.

17.	The same as step 10.

18.	The same as step 11.

19a-19b.	The same as step 12b and step 12c.

NOTE 1:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

NOTE 2:	For details of Nnef_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.591 [11].

NOTE 3:	For details of Naf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.517 [12].

# *** 13th Change ***

### 5.7.21	Movement Behaviour Analytics

This procedure is used by the NWDAF service consumer e.g. NEF or AF to obtain Movement Behaviour Analytics provided by NWDAF regarding the location, direction and velocity of UEs during an analytics target period in a target area. If the NWDAF service consumer is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.21-1: Procedure for Movement Behaviour Analytics

1a.	In order to obtain the Movement Behaviour Analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "MOVEMENT_BEHAVIOUR" with the supported feature "MovementBehaviour".

1b-1c.	In order to subscribe to the Movement Behaviour Analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "MOVEMENT_BEHAVIOUR" with the supported feature "MovementBehaviour".

2a-2b.	If the subscription/request is authorized, the NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to subscribe to the notification of UE ID and UE positions. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	The NWDAF may invoke Ngmlc_Location_ProvideLocation service operation to retrieve UE Fine granularity locations by sending an HTTP POST request to the URI associated with the "provide-location" custom operation as described in 3GPP TS 29.515 [41] clause 5.2.2.2. The GMLC responds to the NWDAF an HTTP "200 OK" response.

5a-5b.	If step 4a and step 4b are performed, the GMLC may invoke Ngmlc_Location_EventNotify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the GMLC an HTTP "204 No Content" response.

6.	The NWDAF derives the Movement Behaviour Analytics based on the data collected from AMF and/or GMLC.

7a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

7b-7c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

NOTE:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

# *** 14th Change ***

### 5.7.22	Location Accuracy Analytics

This procedure is used by the NWDAF service consumer e.g. LMF to obtain Location Accuracy Analytics provided by NWDAF based on the information collected from the AMF, LCS and/or AF. If the service consumer NF is an untrusted AF, it will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.22-1: Procedure for Location Accuracy Analytics

Pre-condition: The NWDAF has a trained supervised ML Model for predicting the location accuracy. In the training phase, the NWDAF consumes input data as define in 3GPP TS 29.520 [5]. To pre-train the ML Model, the NWDAF may collect input data based on area of interest (AoI) which is determined by the NWDAF.

1a.	In order to obtain the Location Accuracy Analytics, the NF service consumer may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "LOC_ACCURACY" with the supported feature "LocAccuracy".

1b-1c.	In order to subscribe to the Location Accuracy Analytics, the NF service consumer may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "LOC_ACCURACY" with the supported feature "LocAccuracy".

2a-2b.	If the subscription/request is authorized, the NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to subscribe to the notification of UE IDs with AoI as filtering criteria. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	The NWDAF may invoke Ngmlc_Location_ProvideLocation service operation to retrieve UE location information by sending an HTTP POST request to the URI associated with the "provide-location" custom operation as described in 3GPP TS 29.515 [41] clause 5.2.2.2. The GMLC responds to the NWDAF an HTTP "200 OK" response.

5a-5b.	If step 4a and step 4b are performed, the GMLC may invoke Ngmlc_Location_EventNotify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the GMLC an HTTP "204 No Content" response.

6a-6b.	If the AF is trusted, the NWDAF may invoke Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions" to request the ground truth UE location information from AF directly. The AF responds to the NWDAF an HTTP "201 Created" response.

7a-7b.	If step 6a and step 6b are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 6a. The NWDAF responds to the AF an HTTP "204 No Content" response.

8a-8d.	If the AF is untrusted, the NWDAF may invoke Nnef_EventExposure_Subscribe service operation to the NEF by sending an HTTP POST request targeting the resource "Network Exposure Event Subscriptions" and then the NEF invokes Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions". The AF responds to the NEF an HTTP "201 Created" response and then the NEF responds to the NWDAF an HTTP "201 Created" response.

9a-9d.	If step 8a to step 8d are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NEF identified by the notification URI received in step 8b and the NEF invokes Nnef_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 8a. The NWDAF responds to the NEF an HTTP "204 No Content" response and then the NEF responds to the AF an HTTP "204 No Content" response.

10.	The NWDAF derives the Location Accuracy Analytics based on the data collected from AMF and/or GMLC.

11.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

12.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

NOTE 1:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

NOTE 2:	For details of Nnef_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.591 [11].

NOTE 3:	For details of Naf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.517 [12].

# *** 15th Change ***

### 5.7.23	Signalling Storm Analytics

This procedure is used by the NWDAF service consumer to obtain Signalling Storm Analytics provided by NWDAF based on the information collected from AMF, SMF, PCF, NRF, AF, OAM, SCP and/or MDAF. If the NWDAF service consumer is an AF which is untrusted, the AF will obtain analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

Figure 5.7.23-1: Procedure for Signalling Storm Analytics

1a.	In order to obtain the Signalling Storm analytics, the NF may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "SIGNALLING_STORM" with the supported feature "SignallingStorm".

1b-1c.	In order to obtain the Signalling Storm analytics, the NF may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "SIGNALLING_STORM" with the supported feature "SignallingStorm".

2a-2b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to subscribe to the notification of signalling information from the AMF. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF invokes Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4b.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation to subscribe to the notification of signalling information from the SMF. The SMF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	If step 4a and step 4b are performed, the SMF may invoke Nsmf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the SMF an HTTP "204 No Content" response.

6a-6b.	The NWDAF may invoke Npcf_EventExposure_Subscribe service operation to subscribe to the notification of signalling information from the PCF. The PCF responds to the NWDAF an HTTP "201 Created" response.

7a-7b.	If step 6a and step 6b are performed, the PCF may invoke Npcf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 6a. The NWDAF responds to the PCF an HTTP "204 No Content" response.

8a-8b.	If the AF is trusted, the NWDAF may invoke Naf_EventExposure_Subscribe service operation to subscribe to the notification of signalling information from the AF directly. The AF responds to the NWDAF an HTTP "201 Created" response.

9a-9b.	If step 8a and step 8b are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 6a. The NWDAF responds to the AF an HTTP "204 No Content" response.

10a-10d.	If the AF is untrusted, the NWDAF may invoke Nnef_EventExposure_Subscribe service operation to the NEF and then the NEF invokes Naf_EventExposure_Subscribe service operation to subscribe to the notification of signalling information from the AF via the NEF. The AF responds to the NEF an HTTP "201 Created" response and then the NEF responds to the NWDAF an HTTP "201 Created" response.

11a-11d.	If step 10a to step 10d are performed, the AF may invoke Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NEF identified by the notification URI received in step 10b and the NEF invokes Nnef_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 10a. The NWDAF responds to the NEF an HTTP "204 No Content" response and then the NEF responds to the AF an HTTP "204 No Content" response.

12a-12b.	The NWDAF may invoke Nnrf_NFManagement_NFStatusSubscribe service operation as described in clause 5.2.2.5 of 3GPP TS 29.510 [26] to subscribe to the notification of signalling information from the NRF. The NRF responds to the NWDAF an HTTP "201 Created" response.

13a-13b.	If step 12a and step 12b are performed, the NRF may invoke Nnrf_NFManagement_NFStatusNotify service operation as described in clause 5.2.2.6 of 3GPP TS 29.510 [26]. The NWDAF responds to the NRF an HTTP "204 No Content" response.

14a.	The NWDAF may collect Number of NF service registration requests, Number of NF service update requests and Number of NF discovery requests including successful and/or failed requests as specified in clause 5.10 of TS 28.552 [27] from OAM. The NWDAF may also collect NF load status information, capacity and priority information of NFs and NF Services from OAM if not available in NRF, and/or unexpected operational status occurs indication based on thresholds or rules configured by operator at the OAM.

14b.	The NWDAF may collect control plane congestion analysis as specified in clause 8.4.7.1.3.3 of TS 28.104 [38] from MDAF. The information elements of the control plane congestion analysis include affectedObject and cPCongestionIssueID. The NWDAF may also collect failure prediction analysis as specified in clause 8.4.3.1.3 of TS 28.104 [38] from MDAF. The information elements of the failure prediction analysis include failurePrecictionObject, potentialFailureType, eventTime, issueID and perceivedSeverity.

15a-15b.	The NWDAF may invoke Nscp_EventExposure_Subscribe service operation as described in clause 5.2.2.2 of 3GPP TS 29.570 [52] to subscribe to the notification of SCP Signalling Information from the SCP. The SCP responds to the NWDAF an HTTP "201 Created" response.

16a-16b.	If step 15a and step 15b are performed, the SCP invokes Nscp_EventExposure_Notify service operation as described in clause 5.2.2.4 of 3GPP TS 29.570 [52]. The NWDAF responds to the SCP an HTTP "204 No Content" response.

17.	The NWDAF calculates the Signalling Storm analytics based on the data collected from AMF, SMF, PCF, NRF, AF, OAM, SCP and/or MDAF.

18a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

18b-18c.	If step 1b and step 1c are performed, the NWDAF invokes Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

NOTE 1:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

NOTE 2:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 3:	For details of Naf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.517 [12].

NOTE 4:	For details of Nnef_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.591 [11].

NOTE 5:	For details of Npcf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.523 [51].

# *** 16th Change ***

### 5.7.24	QoS and Policy Assistance Analytics

This procedure is used by the NWDAF service consumer to obtain QoS and Policy Assistance Analytics provided by NWDAF based on the information collected from AMF, SMF, UPF, AF, GMLC and/or OAM. The known service consumer is PCF. If the NWDAF service consumer is an AF which is untrusted, the AF will request analytics via the NEF as described in clause 5.2.2.2 and clause 5.2.3.2.

-	The input data defined for Observed Service Experience analytics as defined in clause 5.7.3 might be reused by QoS and Policy Assistance Analytics.

-	The input data collected from the OAM for Network Performance analytics as defined in clause 5.7.5, e.g. the RAN status, load and performance per Cell Id for the traffic type of interest and in the Area of Interest.

Figure 5.7.24-1: Procedure for QoS and Policy Assistance Analytics

1a.	In order to obtain the QoS and Policy Assistance analytics, the NF service consumer may invoke Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1, the requested event is set to "QOS_POLICY_ASSISTANCE" with the supported feature "QoSPolicyAssist".

1b-1c.	In order to obtain the QoS and Policy Assistance analytics, the NF service consumer may invoke Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 5.2.2.1, the subscribed event is set to "QOS_POLICY_ASSISTANCE" with the supported feature "QoSPolicyAssist".

2a-2b.	The NWDAF may invoke Namf_EventExposure_Subscribe service operation as described in clause 5.3.2.2.2 of 3GPP TS 29.518 [18] to retrieve the UE location information and SUPI(s) from AMF. The AMF responds to the NWDAF an HTTP "201 Created" response.

3a-3b.	If step 2a and step 2b are performed, the AMF may invoke Namf_EventExposure_Notify service operation as described in 3GPP TS 29.518 [18] clause 5.3.2.4. The NWDAF responds to the AMF an HTTP "204 No Content" response.

4a-4d.	The NWDAF may invoke Nsmf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "SMF Notification Subscriptions" as described in 3GPP TS 29.508 [6], and the SMF may subscribe to the UPF on behalf of the NWDAF via N4 Session Reporting Rule as described in clause 5.2.8 of 3GPP TS 29.244 [45]. After having received the successful response from the UPF, the SMF responds to the NWDAF an HTTP "201 Created" response.

5a-5b.	If step 4a and step 4d are performed, the SMF may invoke Nsmf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the SMF an HTTP "204 No Content" response.

5c-5d.	If step 4b and step 4c are performed, the UPF may invoke Nupf_EventExposure_Notify service operation as described in 3GPP TS 29.564 [40] by sending an HTTP POST request to the NWDAF identified by the notification URI provided in step 4b. The NWDAF responds to the UPF an HTTP "204 No Content" response.

6a-6b.	If the AF is trusted, the NWDAF may invoke Naf_EventExposure_Subscribe service operation to the AF directly by sending an HTTP POST request targeting the resource "Application Event Subscriptions" to collect the End-to-end data volume transfer time information. The AF responds to the NWDAF an HTTP "201 Created" response.

7a-7b.	If step 6a and step 6b are performed, the AF invokes Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 7a. The NWDAF responds to the AF an HTTP "204 No Content" response.

8a-8d.	If the AF is untrusted, the NWDAF may invoke Nnef_EventExposure_Subscribe service operation to the NEF by sending an HTTP POST request targeting the resource "Network Exposure Event Subscriptions" and then the NEF invokes Naf_EventExposure_Subscribe service operation by sending an HTTP POST request targeting the resource "Application Event Subscriptions" to collect the E2E data volume transfer time information. The AF responds to the NEF an HTTP "201 Created" response and then the NEF responds to the NWDAF an HTTP "201 Created" response.

9a-9d.	If step 8a to step 8d are performed, the AF invokes Naf_EventExposure_Notify service operation by sending an HTTP POST request to the NEF identified by the notification URI received in step 9b and the NEF invokes Nnef_EventExposure_Notify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 9a. The NWDAF responds to the NEF an HTTP "204 No Content" response and then the NEF responds to the AF an HTTP "204 No Content" response.

10a-10b.	The NWDAF may invoke Ngmlc_Location_ProvideLocation service operation to retrieve UE Location and UE Location Accuracy by sending an HTTP POST request to the URI associated with the "provide-location" custom operation as described in 3GPP TS 29.515 [41] clause 5.2.2.2. The GMLC responds to the NWDAF an HTTP "200 OK" response.

10c-10d.	If step 10a and step 10b are performed, the GMLC may invoke Ngmlc_Location_EventNotify service operation by sending an HTTP POST request to the NWDAF identified by the notification URI received in step 4a. The NWDAF responds to the GMLC an HTTP "204 No Content" response.

11.	The NWDAF may collect the UL/DL GTP packet delay as defined in clause 5.4.7 of TS 28.552 [27] and Delay in RAN as defined in clause 5.1.1.1 of TS 28.552 [27] from OAM.

12.	The NWDAF calculates the QoS and Policy Assistance Analytics based on the data collected from AMF, SMF, UPF, GMLC, AF and/or OAM.

13a.	If step 1a is performed, the NWDAF responds to the Nnwdaf_AnalyticsInfo_Request service operation as described in clause 5.2.3.1.

13b-13c.	If step 1b and step 1c are performed, the NWDAF may invoke Nnwdaf_EventsSusbcription_Notify service operation as described in clause 5.2.2.1.

14a-14b.	The same as step 3a and step 3b.

15a-15d.	The same as step 5a to step 5d.

16a-16b.	The same as step 7a and step 7b.

17a-17d.	The same as step 9a to  9d.

18a-18b.	The same as step 10c and step 10d.

19.	The same as step 11.

20.	The same as step 12.

21a-21b.	The same as step 13b and step 13c.

NOTE 1:	For details of Nsmf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.508 [6].

NOTE 2:	For details of Naf_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.517 [12].

NOTE 3:	For details of Nnef_EventExposure_Subscribe/Notify service operations refer to 3GPP TS 29.591 [11].

NOTE 4:	For details of Nnwdaf_EventsSubscription_Subscribe/Unsubscribe/Notify or Nnwdaf_AnalyticsInfo_Request service operations refer to 3GPP TS 29.520 [5].

*** End of Changes ***