3GPP TSG-CT3 Meeting #144	C3-255204

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.512 | CR | 1433 | rev | - | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Wrong attribute name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The "maxDataBurstVol" attribute is misspelled three times as "maxDataBurst". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the wrong attribute name. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between OpenAPI and data model Tables leading to possible interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.8, 5.6.2.16, 5.6.2.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR does not impact any OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 5.6.2.8	Type QosData

Table 5.6.2.8-1: Definition of type QosData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qosId | string | M | 1 | Univocally identifies the QoS control policy data within a PDU session. |  |
| 5qi | 5Qi | C | 0..1 | Identifier for the authorized QoS parameters for the service data flow. It shall be included when the QoS data decision is initially provisioned and "defQosFlowIndication" is not included or is included and set to false. |  |
| maxbrUl | BitRateRm | O | 0..1 | Indicates the maximum bandwidth in uplink. |  |
| maxbrDl | BitRateRm | O | 0..1 | Indicates the maximum bandwidth in downlink. |  |
| gbrUl | BitRateRm | O | 0..1 | Indicates the guaranteed bandwidth in uplink. (NOTE 3) |  |
| gbrDl | BitRateRm | O | 0..1 | Indicates the guaranteed bandwidth in downlink. (NOTE 3) |  |
| arp | Arp | C | 0..1 | Indicates the allocation and retention priority. It shall be included when the QoS data decision is initially provisioned and "defQosFlowIndication" is not included or is included and set to false. |  |
| qnc | boolean | O | 0..1 | Indicates whether notifications are requested from 3GPP NG-RAN when the GFBR is no longer (or again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. - Set to "true": the notifications are requested. - Set to "false": the notifications are not requested. - The default value "false" is used if this attribute is not present and has not been supplied previously. (NOTE 3) |  |
| reflectiveQos | boolean | O | 0..1 | Indicates whether the QoS information is reflective for the corresponding non-GBR service data flow. - Set to "true": the QoS information is reflective. - Set to "false": the QoS information is not reflective. - The default value "false" is used if this attribute is not present and has not been supplied previously. |  |
| sharingKeyDl | string | O | 0..1 | Indicates, by containing the same value, what PCC rules may share resources in the downlink direction. | ResShare |
| sharingKeyUl | string | O | 0..1 | Indicates, by containing the same value, what PCC rules may share resources in the uplink direction. | ResShare |
| priorityLevel | 5QiPriorityLevelRm | O | 0..1 | Indicates a priority in scheduling resources among QoS Flows. (NOTE 1) |  |
| averWindow | AverWindowRm | O | 0..1 | Represents the duration over which the guaranteed and maximum bitrates shall be calculated. (NOTE 1) (NOTE 3) |  |
| maxDataBurstVol | MaxDataBurstVolRm | O | 0..1 | Denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB. (NOTE 1, NOTE 2) |  |
| maxPacketLossRateDl | PacketLossRateRm | O | 0..1 | Indicates the maximum downlink packet loss rate for that can be tolerated for the service data flow. | RAN-Support-Info |
| maxPacketLossRateUl | PacketLossRateRm | O | 0..1 | Indicates the maximum uplink packet loss rate that can be tolerated for the service data flow. | RAN-Support-Info |
| defQosFlowIndication | boolean | O | 0..1 | Indicates that the dynamic PCC rule shall always have its binding with the QoS Flow associated with the default QoS rule. - Set to "true": binding with the QoS Flow associated with the default QoS rule applies. - Set to "false": binding with the QoS Flow associated with the default QoS rule does not apply. - The default value "false" is used if this attribute is not present and has not been supplied previously. |  |
| extMaxDataBurstVol | ExtMaxDataBurstVolRm | O | 0..1 | Denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB. (NOTE 1, NOTE 2) | EMDBV |
| packetDelayBudget | PacketDelBudget | O | 0..1 | Unsigned integer. It indicates the Packet Delay Budget expressed in milliseconds. | AuthorizationWithRequiredQoS |
| packetErrorRate | PacketErrRate | O | 0..1 | String indicating the packet error rate. Examples: Packet Error Rate 4x10-6 shall be encoded as "4E-6". Packet Error Rate 10-2 shall be encoded as"1E-2". | AuthorizationWithRequiredQoS |
| pduSetQosDl | PduSetQosParaRm | O | 0..1 | Contains the PDU Set QoS Parameters which are used to support PDU Set based QoS handling in the downlink direction. | PDUSetHandling |
| pduSetQosUl | PduSetQosParaRm | O | 0..1 | Contains the PDU Set QoS Parameters which are used to support PDU Set based QoS handling in the uplink direction. | PDUSetHandling |
| NOTE 1: Applicable only when a value different from the standardized value for this 5QI, provided in table 5.7.4-1 3GPP TS 23.501 [2], is required. NOTE 2: Either the maxDataBurstVol attribute or the extMaxDataBurstVol attribute may be present for a Delay Critical GBR QoS flow. If the maximum data burst volume value to be transmitted is lower than or equal to 4095 Bytes, the maxDataBurstVol attribute is used. If the EMDBV feature is supported by both the PCF and the SMF, the extMaxDataBurstVol attribute is used to transmit the maximum data burst volume values higher than 4095 Bytes (see clause 4.2.2.1). NOTE 3: This attribute is only applicable to GBR type or delay critical GBR type 5QIs. |  |  |  |  |  |

# *** Next Change ***

#### 5.6.2.16	Type QosCharacteristics

Table 5.6.2.16-1: Definition of type QosCharacteristics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| 5qi | 5Qi | M | 1 | Identifier for the authorized QoS parameters for the service data flow. Applies to PCC rule and PDU session level. |  |
| resourceType | QosResourceType | M | 1 | Indicates whether the resource type is GBR, delay critical GBR, or non-GBR. |  |
| priorityLevel | 5QiPriorityLevel | M | 1 | Unsigned integer indicating the 5QI Priority Level, within a range of 1 to 127. |  |
| packetDelayBudget | PacketDelBudget | M | 1 | Unsigned integer indicates the packet delay budget. Packet Delay Budget expressed in milliseconds. |  |
| packetErrorRate | PacketErrRate | M | 1 | String indicating the packet error rate. Examples: Packer Error Rate 4x10-6 shall be encoded as "4E-6". Packer Error Rate 10-2 shall be encoded as"1E-2". |  |
| averagingWindow | AverWindow | C | 0..1 | Indicates the averaging window. This IE shall be present only for a GBR QoS flow or a Delay Critical GBR QoS flow. |  |
| maxDataBurstVol | MaxDataBurstVol | C | 0..1 | Unsigned Integer. Indicates the maximum data burst volume. (NOTE) |  |
| extMaxDataBurstVol | ExtMaxDataBurstVol | C | 0..1 | Unsigned Integer. Indicates the maximum data burst volume. (NOTE) | EMDBV |
| NOTE: Either the maxDataBurstVol IE or the extMaxDataBurstVol IE may be present for a Delay Critical GBR QoS flow. If the maximum data burst volume value to be transmitted is lower than or equal to 4095 Bytes, the maxDataBurstVol IE is used. If the EMDBV feature is supported by both the PCF and the SMF, the extMaxDataBurstVol IE is used to transmit maximum data burst volume values higher than 4095 Bytes (see clause 4.2.2.1). |  |  |  |  |  |

# *** Next Change ***

#### 5.6.2.34	Type AuthorizedDefaultQos

Table 5.6.2.34-1: Definition of type AuthorizedDefaultQos

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| 5qi | 5Qi | C | 0..1 | 5G QoS Identifier. It shall be included when the Authorized Default QoS is initially provisioned. |  |
| arp | Arp | C | 0..1 | Indicates the allocation and retention priority. It shall be included when the Authorized Default QoS is initially provisioned. |  |
| priorityLevel | 5QiPriorityLevelRm | O | 0..1 | Unsigned integer indicating the 5QI Priority Level, within a range of 1 to 127. |  |
| averWindow | AverWindowRm | O | 0..1 | Indicates the averaging window. (NOTE 1) |  |
| maxDataBurstVol | MaxDataBurstVolRm | O | 0..1 | Unsigned integer indicating the maximum data burst volume. (NOTE 2) |  |
| gbrUl | BitRateRm | O | 0..1 | Indicates the guaranteed bandwidth in uplink. (NOTE 1) |  |
| gbrDl | BitRateRm | O | 0..1 | Indicates the guaranteed bandwidth in downlink. (NOTE 1) |  |
| maxbrUl | BitRateRm | O | 0..1 | Indicates the max bandwidth in uplink. (NOTE 1) |  |
| maxbrDl | BitRateRm | O | 0..1 | Indicates the max bandwidth in downlink. (NOTE 1) |  |
| extMaxDataBurstVol | ExtMaxDataBurstVolRm | O | 0..1 | Unsigned integer indicating the maximum data burst volume. (NOTE 2) | EMDBV |
| NOTE 1: This attribute is only applicable to GBR type or delay critical GBR type 5QI. NOTE 2: Either the maxDataBurstVol IE or the extMaxDataBurstVol IE may be present for a Delay Critical GBR QoS flow. If the maximum data burst volume value to be transmitted is lower than or equal to 4095 Bytes, the maxDataBurstVol IE is used. If the EMDBV feature is supported by both the PCF and the SMF, the extMaxDataBurstVol IE is used to transmit maximum data burst volume values higher than 4095 Bytes (see clause 4.2.2.1). |  |  |  |  |  |

# *** End of Changes ***