3GPP TSG-CT WG3 Meeting #145	C3-260042

# Goa, India, 09 - 13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.561 | CR | 0209 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Media over QUIC transport |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2026-01-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to draft-ietf-moq-transport-15 the length of Key-Value-Pair exists if the Type is odd, see below from the draft: Length: Only present when Type is odd. Specifies the length of the Value field in bytes. The maximum length of a value is 2^16-1 bytes. If an endpoint receives a length larger than the maximum, it MUST close the session with a Protocol Violation. Since the length must exist for the value which contains media related information, therefore the Type must be chosen as odd value. Draft-ietf-moq-transport-15 reserves extension types 0-63 and the next odd value is 65. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Current type value is changed to 65. Interoperability analysis The change is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect value for Type, which results in not having Length for Key-Value-Pair |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************************** First Change ***************************************

### 22.3.5	Sending Media Related Information using Media over QUIC transport

If the media traffic is transmitted over Media over QUIC Transport between the AS and the UE, the MoQ relay in the UPF and the AS may exchange the MRI using MoQ Object Extension header(s) as defined in draft-ietf-moq-transport [69].

MoQ Extension header shall be added as the Key-Value-Pair as defined in draft-ietf-moq-transport [69], where:

a)	Type shall be set to the value "65";

b)	Length shall be set to the length of the MRI, in bytes; and

c)	Value shall be set to Media Related Information as defined in defined in clause 22.2.

*************************************** End of Change ***************************************