3GPP TSG CT WG3 Meeting #145	C3-260048

Goa, India, 9 - 13 February, 2026	(revision of xx-yyxxxx)

# Source:	Xiaomi

# Title:	New WID on CT aspects of Integrated Sensing and Communication

{"Revised" to be used only for WID already approved at plenary. For Revised WIDs, two versions have to be provided in a zip file: a clean one and one with revision marks showing the differences with the previously plenary-approved WID.}

# Document for:	Endorsement

# Agenda Item:	20.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	CT aspects of Integrated Sensing and Communication

Acronym:	Sensing_ARC

Unique identifier:	TBD

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| Sensing_ARC | SA2 | TBD | Stage 2 of Integrated Sensing and Communication |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 950003 | Study on Integrated Sensing and Communication | Rel-19 SA1 study item, which provides potential scenarios and service requirements of Integrated Sensing and Communication services. |
| 1000026 | Stage 1 of Integrated Sensing and Communication | Rel-19 SA1 work item, which provides normative service requirements of Integrated Sensing and Communication services. |
| 1070012 | Study on Stage 2 for Integrated Sensing and Communication | Rel-20 SA2 study item, which specifies architecture aspects of Integrated Sensing and Communication services. |
| 1090020 | Study on Security and Privacy Aspects of Integrated Sensing and Communication | Rel-20 SA3 study item, which specifies security and privacy aspects of Integrated Sensing and Communication services. |
| 1020086 | Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR | Rel-19 RAN study item, which provides channel modelling for Integrated Sensing and Communication services. |
| 1080070 | Study on Integrated Sensing And Communication (ISAC) for NR | Rel-20 RAN study item, which provides RAN aspects of Integrated Sensing and Communication services. |

Dependency on non-3GPP (draft) specification: N/A

# 3	Justification

SA2 has studies the Stage 2 of Integrated Sensing and Communication under the study item FS_Sensing_ARC. The study work has been concluded. The conclusions are captured in 3GPP TR 23.700-14 and provide a good overview of what is to be continued into normative phase and impacts to other working groups. Furthermore, the corresponding SA2 work item "Stage 2 of Integrated Sensing and Communication" (Sensing_ARC) for SA2 normative work is planned for approval in TSG SA#111. The normative work will have impact on stage 3 specifications. Therefore, it is necessary for a new CT work item to implement the stage-2 requirements for Integrated Sensing and Communication.

# 4	Objective

The objective of this work is to specify the CT aspects of Integrated Sensing and Communication in order to enhance the CT WGs specifications based on the stage-2 requirements. Normative work to be developed by SA WGs and RAN WGs which impact CT WGs will be considered as soon as those are available.

The following area of work are expected to be covered:

CT3:

- defines new or updates to the NEF northbound service to support the provisioning of sensing service parameters by an AF;

- defines new or updates to the NEF northbound service to support the exposure of sensing results to an untrusted AF;

NOTE:	Based on SA WG2 discussions, potential updates to the UDR may be necessary to support sensing service parameters repository and exposure.

CT4:

- defines the new SF network function to support authorization and revocation of sensing service request;

- potential updates to AMF to support control plane sensing signalling;

NOTE:	Sensing control signalling can be transmitted between the SF and the Sensing Entity (i.e. gNB) via direct connection or via AMF indirectly. Protocol (e.g. SCTP, GTP-U) related to direct transmission will be defined within RAN WG3 without CT impact. Protocol related to indirect transmission via AMF will have CT4 impact.

- updates to the NRF to support SF registration and discovery;

- updates to common data type to support sensing service.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TS | 29.XXX | 5G System; Integrated Sensing and Communication Services; Stage 3 | TSG CT#115 (March 2027) | TSG CT#116 (June 2027) | Ruby Gong gongyubing@xiaomi.com (CT4 responsibility) |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.522 | 1. Defines new or update to the NEF northbound service to support the exposure of sensing results to an AF. 2. Defines new or updates to the NEF northbound service to support the provisioning of sensing service parameters by an AF. | TSG CT#116 (June 2027) | CT3 Responsibility |
| 29.510 | Updates to the NRF to support SF registration and discovery | TSG CT#116 (June 2027) | CT4 Responsibility |
| 29.518 | Potential updates to AMF to support control plane sensing signalling | TSG CT#116 (June 2027) | CT4 Responsibility |
| 29.571 | Updates to common data type to support sensing service. | TSG CT#116 (June 2027) | CT4 Responsibility |

# 6	Work item Rapporteur(s)

Ruby GONG, Xiaomi, gongyubing@xiaomi.com

# 7	Work item leadership

CT4

# 8	Aspects that involve other WGs

SA3 for the security aspects.

# 9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

Xiaomi

Apple?

AT&T?

CATT?

China Mobile?

China Telecom?

China Unicom?

CEWiT?

FirstNet?

FutureWei?

Google?

Interdigital?

Lenovo?

LG Electronics?

NEC?

NTT DOCOMO?

Ofinno?

OPPO?

Spreadtrum?

Vivo?

ZTE?