3GPP TSG CT WG3 Meeting #145	C3-260053

# Goa, India, 9 – 13 February 2026

3GPP TSG CT WG1 Meeting #159	C1-260155

# Goa, India, 9 – 13 February 2026

Source:	Ericsson

Title:	Discussion on Status of Application Enablement to Support Energy Saving study item in SA6

Document for:	Information

# Agenda Item:	20.1

Abstract of the contribution:

# 1	Action requested

Discuss the status of SA6 study item on FS_EnergySys_Ph2_APP (Rel-20 5GA) in order to identify the potential impact in CT1 and CT3.

# 2	References

[1]	3GPP TR 23.700-44: "Study on Application Enablement to Support Energy Saving; Phase 2”.

# 3	Rationale

SA6 develops mechanisms in application enablement layer to support energy saving in Release-20 under FS_EnergySys_Ph2_APP study item. The study item is reported as 60% complete, related solutions evaluations and key issue conclusions are in progress. This discussion paper increases Stage 3 awareness of changes in Stage 2 related to FS_EnergySys_Ph2_APP and facilitates understanding the expected impact in stage 3 Release-20 work. Thus, this discussion paper aims to update Stage 3 Working Groups (CT1 and CT3) on potential impact (i.e., no solution evaluations and key issue conclusions are available).

Ericsson is the rapporteur of FS_EnergySys_Ph2_APP in SA6.

# 4	Discussion

Energy Saving is ubiquitous to all services, ending up impacting many Application Enablement Services in this study. Therefore, it is important in stage 3 to have a discussion document that helps with understanding possibly new solutions and that verifies which solution have impact on CT3 and CT1.

The results of FS_EnergySys_Ph2_APP study may potentially impact the following services:

1.	Potential new SEAL Energy Saving Service.

2.	Edge Enabler Layer.

3.	AIML Enablement Service.

4.	ADAE Service.

5.	Network Slice Capability Enablement Service.

6.	Location Management Service.

SA6 WG is currently studying whether and what new services are needed, and how to improve the existing standardized services, to include energy aware mechanisms enabling energy efficient/saving according to user requirements. The current SA6 study is still in progress being early to start work in CT1 and CT3, so this discussion paper focus on summarizing proposed solutions and highlighting which solutions have impact on CT3 and CT1. This discussion increases awareness in stage 3 about solutions being studied in stage 2, which may later be included in the technical specifications.

| Solution # | Summary | CT1 impact | CT3 impact |
| --- | --- | --- | --- |
| 1 | This solution proposes SEAL server to expose an energy saving assistance, enabling consumer (e.g. VAL server, SEAL server/client) to track the energy consumption or the most/least energy efficient entity (e.g. VAL UE, SEAL server). | Y | Y |
| 2 | This solution enables EES to subscribe to EAS about whether it support usage of renewable energy or not, which then can be leverage on the EAS discovery procedures. | N | Y |
| 3 | This solution proposes to enhance EAS discovery, by enabling EEC sending in the discovery request to EES the required energy type as a filter (renewable/non-renewable energy) and to include the energy type supported by the EAS in the response from EES to EEC. | Y | Y |
| 4 | This solution proposes enhancements to the ML model information management and AIMLE client selection procedures, to enable the storage and use of energy consumption information for AIMLE client selection. | Y | Y |
| 5 | This solution proposes to enhance the ML repository service on FL member registration and FL-related event subscription, which enabling the registering and selection of FL members that support renewable energy. | N | Y |
| 6 | This solution proposes a new ADAE analytics on AI/ML energy consumption, which enables consumer (e.g. VAL server, AIMLE server) to subscribe for AI/ML energy consumption analytics from ADAES. | Y | Y |
| 7 | This solution proposes a new ADAE analytics on AIMLE client energy sustainability, which enables consumers (e.g. AIMLE server, VAL server) to request AIMLE client energy analytics from ADAES. | Y | Y |
| 8 | This solution proposes a new VAL server policy, where VAL server can request NSCE server to perform slice modifications based on monitoring of energy-related information, such as energy consumption and the renewable energy ratio, at the network slice level. | N | Y |
| 9 | This solution enhances ADAE DN energy efficiency analytics by adding new analytics events, i.e., "Energy analytics per application service” and “Ratio of renewable energy analytics”, which enables VAL server to subscribe and/or request energy consumption/efficiency analytics from ADAES with finer granularity. | N | Y |
| 10 | This solution proposes LMS consumes ADAE analytics on energy information and based on the analytics to adjust location reporting configuration to reduce energy consumption at the target VAL UE. | N | Y |
| 11 | This solution introduces the functional architecture to support energy saving. | Y | Y |
| 12 | This solution proposes new SEAL service on energy data collection and processing, which enables consumer (e.g. VAL server, SEAL server/client) to subscribe to the SEAL server (Energy Saving)for collection and processing of energy data. | Y | Y |
| 13 | This solution proposes new SEAL service on energy information monitoring, which enables consumer (e.g. VAL server, SEAL client/server) to request and receive notifications on energy consumption of target VAL UEs. | Y | Y |
| 14 | This solution proposes that EAS updates QoS request to EES based on the energy consumption information of the target VAL UE or QoS Flow, and whether renewable energy is used or not. | N | Y |
| 15 | This solution proposes a new procedure, where AIMLE server selects target AI/ML members for AI/ML task transfer based on energy consumption information. The AI/ML task transfer assist request is enhanced with a new parameter where consumers provide the energy criterion for the target AI/ML member. | Y | Y |
| 16 | This solution proposes to enhance AI/ML split operation by considering the energy credit or budget of the target VAL UE or application service, where AIMLE client can trigger AIMLE server to execute an AI/ML split operation based on the energy consumption, credit, or budget of the VAL UE or application. | Y | Y |
| 17 | This solution proposes to enhance FL member availability notifications from ML repository/AIMLE server with information about energy credit/budget. | N | Y |
| 18 | This solution proposes to enhance VAL-server triggered and network-based network slice adaptation service exposed by NSCE server, to enable slice adaptation based on energy information, for improving energy efficiency or increasing renewable energy usage of VAL application slice. | N | Y |
| 19 | This solution proposes to enhance multiple slices coordinated resource optimization service from NSCE server, enabling VAL server to request multiple slice optimization to optimize energy efficiency or increase renewable energy usage. | N | Y |
| 20 | This solution proposes to improve location reporting from location management client to LMS with low battery indication and improve LMS client-triggered or VAL-triggered location reporting with three different energy saving modes as energy saving requirements. | Y | Y |
| 21 | This solution proposes to optimize location reporting when multiple devices of the same user share the same location, enabling LMS to select one device to report location information based on energy information of these devices, while the others have location reporting disabled to save energy. | Y | Y |

Observation 1: There are 6 key issues. KI#1 with five solutions. KI#2 with one solution, KI#3 with three solutions, KI#4 with seven solutions, KI#5 with three solutions, and KI#6 with six solutions. Evaluations and conclusions are not available for any of the KIs.

Observation 2: All solutions have CT3 impact; meanwhile, some solutions have CT1 impact.

# 5	Conclusion

This discussion paper summarizes and identifies solutions that affect CT1 and CT3 proposed in SA6 FS_EnergySys_Ph2_APP study. Ericsson plans to submit the Stage 3 Work Item Description (WID) based on upcoming EnergySys_Ph2_APP WID (SA6 WID) in the upcoming meetings.