3GPP TSG-CT WG3 Meeting # 145	C3-260080

# Goa, India, 09 – 13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.437 | CR | 0005 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SS_SmDiscovery API correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following issues related to SS_SmDiscovery API are identified: - In 6.2.2.3.2.3, SpatialMapDiscResp has the spelling mistake with “s” missing between Map & Disc. Also, The cardinality for SpatialMapsDiscResp is mandatory which is unaligned with the OpenAPI. - In the OpenAPI, SpatialMapsDiscResp datatype condition is not aligned with the datatype. The API can return only spatial maps, or only SMAS information, or both, depending on query scope. Therefore, the schema must allow either mapsDisc or smasList. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The above issues are fixed in this CR with - In 6.2.2.3.2.3, attribute is updated as SpatialMapsDiscResp and its cardinality is updated as “O”. - In A.6, AnyOf condition is added for mapsDisc, smasList. This update ensures the API documentation and OpenAPI schema remain harmonized and consistent for implementers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The quality of the SS_SmDiscovery API is not up to the mark. Also, it creates the confusion for the implementors due to the inconsistency between the data model and the OpenAPI. Implementations may misinterpret mandatory parameters or schema structure, causing interoperability issues during REST API integration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.3.2.3, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible correction to the OpenAPI descriptions of the following APIs: TS29437_SS_SmDiscovery.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

6.2.2.3.2.3	Resource Standard Methods

The HTTP GET method allows a service consumer to retrieve a spatial map information from the SM Server.

This method shall support the URI query parameters specified in table 6.2.2.3.2.3.1-1.

Table 6.2.2.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| app-svc-id | string | O | 0..1 | Contains the identifier of VAL application service. |  |
| loc-info | LocationInfo | O | 0..1 | Contains the three-dimensional area information, location information and direction. |  |
| map-disc-fltr | SpatialMapsDiscFilter | O | 0..1 | Contains the set of characteristics related to spatial maps. |  |
| pose | Pose | O | 0..1 | Contains the current pose information. |  |
| supp-feats | SupportedFeatures | O | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 6.2.2.8. This query parameter shall be present only when feature negotiation is required. |  |

This method shall support the request data structures specified in table 6.2.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.2.3.2.3.1-3.

Table 6.2.2.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.2.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SpatialMapsDiscResp | O | 1 | 200 OK | Successful case. The requested spatial map information shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.2.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative SM Server. |

Table 6.2.2.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative SM Server. |

# * * * *2nd change * * * *

# A.6	SS_SmDiscovery API

openapi: 3.0.0

info:

title: SM Server Spatial Maps and SMAS Discovery Service

version: 1.0.0

description: |

Spatial Maps and SMAS Discovery Service.

©2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.437 V19.0.0; Service Enabler Architecture Layer for Verticals (SEAL);

Metaverse Enablement Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.437/

servers:

- url: "{apiRoot}/ssm-disc/v1"

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/spatial-maps:

get:

summary: Discover spatial maps or SMAS information from the SM Server.

operationId: DiscSpatialMapsSmas

tags:

- Spatial Maps (Collection)

parameters:

- name: app-svc-id

in: query

description: >

Contains the identifier of VAL application service.

schema:

type: string

- name: map-disc-fltr

in: query

description: >

Contains the set of characteristics related to the spatial maps.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapsDiscFilter'

- name: loc-info

in: query

description: >

Contains the three-dimensional area information, location information and direction.

content:

application/json:

schema:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

- name: pose

in: query

description: >

Contains the pose information.

content:

application/json:

schema:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/Pose'

- name: supp-feats

in: query

description: >

Contains the list of supported feature(s) among the ones defined in clause 6.2.2.8.

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The requested spatial map information is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapsDiscResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: "{tokenUrl}"

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SpatialMapsDiscFilter:

description: >

Represents the discovery filter information for spatial maps.

type: object

properties:

mapLyrList:

$ref: 'TS29437_SS_SmManagement.yaml#/components/schemas/SpatialMapLayers'

SpatialMapsDiscResp:

description: >

Represents the spatial maps discovery response.

type: object

properties:

svcId:

type: string

mapsDisc:

type: array

items:

$ref: '#/components/schemas/SpatialMapDiscovered'

minItems: 1

smasList:

type: array

items:

$ref: 'TS29437_SS_SmSmasRegistration.yaml#/components/schemas/SMASProfile'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

anyOf:

- required: [mapsDisc]

- required: [smasList]

SpatialMapDiscovered:

description: >

Represents the discovered spatial maps.

type: object

properties:

mapId:

$ref: 'TS29437_SS_SmManagement.yaml#/components/schemas/SpatialMapId'

required:

- mapId

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# Data types describing alternative data types or combinations of data types:

#

# *** End of Changes ***