3GPP TSG CT WG3 Meeting #145	C3-260221

# Goa, India, 9 - 13 February, 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.392 | CR | 0003 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the API definition clauses of the MMTel_DCAppCall API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMTel_App |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The following issues have been identified in the definition of the MMTel_DCAppCall API: - In several occurrences across the API definition clauses, the notification URI variable name is incorrect, "notificationURI" instead of "notificationInfo", which creates a misalignment with the OpenAPI description. - Various issues and misalignments (e.g., cardinality, presence consitions, incorrect data type or attribute name) with the OpenAPI description in the data model definition under clause 6.2.6. - Various additional corrections and alignments with the NBI TS Skeleton and the other Rel-19 NBI TSs are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR proposes to: - Correct the above-detailed issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The above-detailed issues remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2, 6.2.4.1, 6.2.4.2, 6.2.4.2.1, 6.2.4.2.2, 6.2.4.3, 6.2.4.3.1, 6.2.4.3.2, 6.2.5, 6.2.5.1.1, 6.2.5.1.2, 6.2.5.1.3.1, 6.2.6.1, 6.2.6.2.2, 6.2.6.2.3, 6.2.6.2.4, 6.2.6.2.5, 6.2.6.2.6, 6.2.6.2.7, 6.2.6.3.2, 6.2.6.3.3, 6.2.6.3.4, 6.2.6.4, 6.2.6.5, 6.2.6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions of the APIs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 6.2.1	Introduction

The MMTel_DCAppCall shall use the MMTel_DCAppCall API.

The API URI of the MMTel_DCAppCall shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "mmtel-dcappcall".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.2, the MMTel Enabler Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

==============Next change==============

### 6.2.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2.2 of 3GPP TS 29.122 [2] shall apply for the MMTel_DCAppCall API.

==============Next change==============

#### 6.2.4.1	Overview

The structure of the custom operation URIs of the MMTel_DCAppCall API is shown in Figure 6.2.4.1-1.

Figure 6.2.4.1-1: Custom operation URI structure of the MMTel_DCAppCall API

Table 6.2.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the MMTel_DCAppCall API.

Table 6.2.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| DcCallRequest | /dccall | POST | Request to establish a DC capability enabled call to a given MMTel Enabler Server. |
| DcMediaUpdateRequest | /update-dc-media | POST | Request to update DC media of an application call to a given MMTel Enabler Server. |

==============Next change==============

#### 6.2.4.2	Operation: DcMediaUpdateRequest

==============Next change==============

##### 6.2.4.2.1	Description

The custom operation is used by the service consumer to request DC media update to a given MMTel Enabler Server.

==============Next change==============

##### 6.2.4.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 6.2.4.2.2-1 and Table 6.2.4.2.2-2.

Table 6.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DCMediaUpdateReq | M | 1 | Represents the requested DC media update information. |

Table 6.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DCMediaUpdateResp | M | 1 | 200 OK | Successful case. The requested update information of DC media is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.4.2.2-: Headers supported by the 307 method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative MMTel Enabler Server. |

Table 6.2.4.2.2-5: Headers supported by the 308 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative MMTel Enabler Server. |

==============Next change==============

#### 6.2.4.3	Operation: DcCallRequest

==============Next change==============

##### 6.2.4.3.1	Description

The custom operation is used by the service consumer to request a call enabled by DC capabilities to a given MMTel Enabler Server.

==============Next change==============

##### 6.2.4.3.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 6.2.4.3.2-1 and Table 6.2.4.3.2-2.

Table 6.2.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcCallReq | M | 1 | Represents the requested DC Call. |

Table 6.2.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcCallResp | M | 1 | 200 OK | Successful case. The requested DC Call information is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.4.3.2-3: Headers supported by the 307 method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative MMTel Enabler Server. |

Table 6.2.4.3.2-5: Headers supported by the 308 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative MMTel Enabler Server. |

==============Next change==============

### 6.2.5	Notifications

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [2].

Table 6.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| DC Media Notification | {notificationInfo} | POST | Notify about DC Media changes from MMTel Enabler Server. |

==============Next change==============

##### 6.2.5.1.1	Description

The DC Media Notification is used by the MMTel Enabler Server to notify a previously subscribed service consumer on DC Media changes.

==============Next change==============

##### 6.2.5.1.2	Target URI

The Callback URI "{notificationInfo}" shall be used with the callback URI variables defined in table 6.2.5.1.2-1.

Table 6.2.5.1.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationInfo | Represents the callback URI encoded as a string formatted as a URI. |

==============Next change==============

###### 6.2.5.1.3.1	POST

This method shall support the request data structures specified in table 6.2.5.1.3.1-1 and the response data structures and response codes specified in table 6.2.5.1.3.1-2.

Table 6.2.5.1.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcMediaNotifyReq | M | 1 | Represents the DC Media Notification. |

Table 6.2.5.1.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcMediaNotifyResp | M | 1 | 200 OK | Successful case. The Notification is successfully received and DC Media notification related information is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 6.2.5.1.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 6.2.5.1.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

==============Next change==============

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined specifically for the MMTel_DCAppCall API service.

Table 6.2.6.1-1: MMTel_DCAppCall API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AdcType | 6.2.6.3.3 | Represents the type of DC communication. |  |
| CallType | 6.2.6.3.4 | Represents the call is established as A2P or P2P call. |  |
| DcCallReq | 6.2.6.2.2 | Represents the DC Call establishment request. |  |
| DcCallResp | 6.2.6.2.3 | Represents the DC Call establishment response. |  |
| DcMediaUpdateReq | 6.2.6.2.4 | Represents the DC media update request. |  |
| DcMediaUpdateResp | 6.2.6.2.5 | Represents the DC media update response. |  |
| DcMediaNotifyReq | 6.2.6.2.6 | Represents the DC media update notification request. |  |
| DcMediaNotifyResp | 6.2.6.2.6 | Represents the DC media update notification response. |  |

Table 6.2.6.1-2 specifies data types re-used by the MMTel_DCAppCall API service.

Table 6.2.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DcAppUpdateParameters | 6.1.6.2.7 | Represents parameters of DC application profile. |  |
| Uri | 3GPP TS 29.571 [7] | Represents a URI. |  |

==============Next change==============

##### 6.2.6.2.2	Type: DcCallReq

Table 6.2.6.2.2: Definition of type DcCallReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| originatingId | Uri | C | 0..1 | The identifier of the caller, corresponding to participant address in OMA Third Party Call API [15], which could be ‘sip’ URI, ‘tel’ URI or ‘acr’ URI. (NOTE) |  |
| terminatingId | Uri | M | 1 | The identifier of the callee, corresponding to participant address in OMA Third Party Call API [15], which could be ‘sip’ URI, ‘tel’ URI or ‘acr’ URI. |  |
| mediaInfo | array(string) | O | 1..N | Identifier of one or more media type(s), corresponding to the mediaInfo of Call |  |
| dcMediaInfo | boolean | O | 0..1 | Identifier of whether DC media is expected to be used. |  |
| appProfileRequested | DcAppUpdateParameters | O | 0..1 | The DC application profile expected to be used. |  |
| notificationInfo | Uri | M | 1 | The address where call related notification is sent, corresponding to callbackReference in OMA Third Party Call API. |  |
| callType | CallType | M | 1 | Indicate if the call is established as A2P or P2P call. (NOTE) |  |
| NOTE: The "originatingId" attribute shall be present if the "callType" attribute is set to "P2P". |  |  |  |  |  |

==============Next change==============

##### 6.2.6.2.3	Type: DcCallResp

Table 6.2.6.2.3-1: Definition of type DcCallResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| callResult | string | M | 1 | Indication if the Call establishment is success or failure. |  |
| sessionId | string | O | 0..1 | The identifier of the call session, corresponding to callSessionId in OMA Third Party Call API |  |
| failureCause | string | O | 0..1 | The reason for failure, corresponding to callSessionId in OMA Third Party Call API |  |
| originatingId | Uri | C | 0..1 | The identifier of the caller, corresponding to participant address in OMA Third Party Call API [15], which could be ‘sip’ URI, ‘tel’ URI or ‘acr’ URI. (NOTE) |  |
| terminatingId | Uri | M | 1 | The identifier of the callee, corresponding to participant address in OMA Third Party Call API [15], which could be ‘sip’ URI, ‘tel’ URI or ‘acr’ URI. |  |
| mediaInfo | array(string) | O | 1..N | Identifier of one or more media type(s) |  |
| dcMediaInfo | string | O | 0..1 | Identifier of whether DC media is expected to be used |  |
| appProfileRequested | DcAppUpdateParameters | O | 0..1 | The DC application profile expected to be used |  |
| callType | CallType | M | 1 | Indicate if the call is established as A2P or P2P call. (NOTE) |  |
| NOTE: The "originatingId" attribute shall be present if the "callType" attribute is set to "P2P" |  |  |  |  |  |

==============Next change==============

##### 6.2.6.2.4	Type: DcMediaUpdateReq

Table 6.2.6.2.4-1: Definition of type DcMediaUpdateReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dcAppId | string | M | 1 | The identifier of the DC application |  |
| sessionId | string | M | 1 | The session identifier for media update |  |
| mediaResourceInfo | object | O | 0..1 | The media information to be passed via the Data Channel |  |
| appProfileRequested | DcAppUpdateParameters | O | 0..1 | The DC application profile expected to be used |  |
| notificationInfo | Uri | M | 1 | The address where call related notification is sent to |  |
| mediaDirection | AdcType | O | 0..1 | Type of DC communication, e.g. A2P or P2A. |  |

==============Next change==============

##### 6.2.6.2.5	Type: DcMediaUpdateResp

Table 6.2.6.2.5-1: Definition of type DcMediaUpdateResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionId | string | M | 1 | The session identifier of the session |  |
| result | string | M | 1 | Indicates if the request was successful or failed |  |
| cause | string | O | 0..1 | Indicates the cause of request failure |  |

==============Next change==============

##### 6.2.6.2.6	Type: DcMediaNotifyReq

Table 6.2.6.2.6-1: Definition of type DcMediaNotifyReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionId | string | M | 1 | The session identifier of the session |  |
| mediaResourceInfo | object | M | 1 | The media information to be passed via the Data Channel |  |
| mediaDirection | AdcType | O | 0..1 | Type of DC communication, e.g. A2P or P2A. |  |

==============Next change==============

##### 6.2.6.2.7	Type: DcMediaNotifyResp

Table 6.2.6.2.7-1: Definition of type DcMediaNotifyResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionId | string | M | 1 | The session identifier of the session |  |
| result | string | M | 1 | Indicates if the request was successful or failed |  |
| cause | string | O | 0..1 | Indicates the cause of request failure |  |

==============Next change==============

##### 6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

==============Next change==============

##### 6.2.6.3.3	Enumeration: AdcType

The enumeration AdcType represents the type of the data channel media direction. It shall comply with the provisions defined in table 6.2.6.3.3-1.

Table 6.2.6.3.3-1: Enumeration AdcType

| Enumeration value | Description |
| --- | --- |
| A2P | Indicates that the type of the data channel media direction is a Data Channel initiated by the Application. |
| P2A | Indicates that the type of the data channel media direction is a Data Channel initiated by the UE. |

==============Next change==============

##### 6.2.6.3.4	Enumeration: CallType

The enumeration CallType represents the established call type. It shall comply with the provisions defined in table 6.2.6.3.4-1.

Table 6.2.6.3.4-1: Enumeration CallType

| Enumeration value | Description |
| --- | --- |
| A2P | Indicates that the established call type is an Application call initiated by the Application towards the UE. |
| P2P | Indicates that the established call type is a Third Party call established between two UEs. |

==============Next change==============

#### 6.2.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

==============Next change==============

#### 6.2.6.5	Binary data

##### 6.2.6.5.1	Binary Data Types

Table 6.2.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

==============End of change==============