# 3GPP TSG CT WG3 Meeting #143	C3-254501

# Sophia-Antipolis, France, 13 – 17 October, 2025					 (Revision of C3-254246)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.521 | CR | 0227 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the PCF binding API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-9-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When the Npcf_PolicyAuthorization is supported, at least one of the FQDN and the IP end point(s) of the PCF handling the MBS session shall be provided for a new MBS session binding. However, the current procedure specifies that both of them shall be provided and not aligned with the data type, OpenAPI. In NOTE 9, "pcfDiamHost" and "pcfDiamRealm" correspond to the Rx interface, "pcfFqdn" or "pcfEndPoints" conrespond to the Npcf_PolicyAuthorization service, the descriptions are not in a proper order for understanding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the at least one of the FQDN and the IP end point(s) shall be provided. Clarify NOTE 9 for attributes "pcfDiamHost" and "pcfDiamRealm". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Notification mechanism is not implemented correctly and requirements for parameters are not self-contained within the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.4, 5.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has no impact on OpenAPI |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 4.2.2.4	Register a new PCF for an MBS Session binding information

Figure 4.2.2.4-1: PCF for an MBS Session Binding information Registration procedure

1.	The NF service consumer (e.g. PCF for an MBS Session) shall invoke the Nbsf_Management_Register service operation to register a new PCF for an MBS Session binding at the BSF. The NF service consumer shall send for this purpose an HTTP POST request targeting the "PCF for an MBS Session Bindings" resource URI, i.e. "{apiRoot}/nbsf-management/<apiVersion>/pcf-mbs-bindings", with the request body containing the PcfMbsBinding data structure that shall include:

-	the identifier of the MBS Session to which the MBS Session binding is related, within the "mbsSessionId" attribute; and

-	at least one of the following:

-	the FQDN of the PCF handling the MBS Session, if available, within the "pcfFqdn" attribute;

-	the IP end point(s) of the PCF handling the MBS Session, if available, within the "pcfIpEndPoints" attribute;

and may include:

-	the identifier of the PCF instance handling the concerned MBS Session, within the "pcfId" attribute;

-	the identifier of the PCF set to which the PCF instance handling the concerned MBS Session belongs, within the "pcfSetId" attribute;

-	the level of binding of the PCF handling the concerned MBS Session, within the "bindLevel" attribute; and

-	the recovery timestamp of the NF service consumer (e.g. PCF for an MBS Session), within the "recoveryTime" attribute.

If the NF service consumer (e.g. PCF for an MBS Session) provides the PCF instance ID within the "pcfId" attribute, and optionally the recovery timestamp within "recoveryTime" attribute, the BSF may use this information to carry out the clean-up procedures defined in subclause 6.4 of 3GPP TS 23.527 [17], if necessary.

2.	Upon successful processing of the received HTTP POST request, the BSF shall check if there is an existing MBS Session Binding information with the same "mbsSessionId" attribute value. If it is the case, the the BSF shall reject the request with an HTTP "403 Forbidden" status code with the response body containing the MbsExtProblemDetails data structure that shall include the FQDN of the existing PCF within the "pcfFqdn" attribute or the description of the IP endpoints of the existing PCF within the "pcfIpEndPoints" attribute of the MbsBindingResp data structure, and the "cause" attribute of the ProblemDetails data structure set to "EXISTING_BINDING_INFO_FOUND".

If there is not existing MBS Session Binding information for the provided "mbsSessionId" attribute, the BSF shall create a new "Individual PCF for an MBS Session Binding" resource to store the requested PCF for an MBS Session binding. The BSF shall then respond to the NF service consumer with an HTTP "201 Created" status code including an HTTP Location header field containing the URI of the created "Individual PCF for an MBS Session Binding" resource, and the response body containing a representation of the created resource within the PcfMbsBinding data structure.

If errors occur when processing the HTTP POST request, the BSF shall apply the error handling procedures, as specified in subclause 5.7.

# *** Next Changes ***

#### 5.6.2.2	Type PcfBinding

Table 5.6.2.2-1: Definition of type PcfBinding

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |
| gpsi | Gpsi | O | 0..1 | Generic Public Subscription Identifier |  |
| ipv4Addr | Ipv4Addr | C | 0..1 | The IPv4 Address of the served UE. (NOTE 4) (NOTE 8) |  |
| ipv6Prefix | Ipv6Prefix | C | 0..1 | The IPv6 Address Prefix of the served UE. (NOTE 4) (NOTE 5) (NOTE 8) |  |
| addIpv6Prefixes | array(Ipv6Prefix) | O | 1..N | The additional IPv6 Address Prefixes of the served UE. (NOTE 4) (NOTE 5) (NOTE 8) (NOTE 10) | MultiUeAddr |
| ipDomain | string | O | 0..1 | IPv4 address domain identifier. (NOTE 1) (NOTE 8) |  |
| macAddr48 | MacAddr48 | C | 0..1 | The MAC Address of the served UE. (NOTE 8) |  |
| addMacAddrs | array(MacAddr48) | O | 1..N | The additional MAC Addresses of the served UE. (NOTE 8) (NOTE 10) | MultiUeAddr |
| dnn | Dnn | M | 1 | DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. (NOTE 11) |  |
| pcfFqdn | Fqdn | C | 0..1 | FQDN of the PCF hosting the Npcf_PolicyAuthorization service. (NOTE 2) (NOTE 9) |  |
| pcfIpEndPoints | array(IpEndPoint) | C | 1..N | IP end points of the PCF hosting the Npcf_PolicyAuthorization service. (NOTE 2) (NOTE 9) |  |
| pcfDiamHost | DiameterIdentity | C | 0..1 | The diameter host for an individual PCF. (NOTE 3) (NOTE 9) |  |
| pcfDiamRealm | DiameterIdentity | C | 0..1 | The diameter realm for an individual PCF. (NOTE 3) (NOTE 9) |  |
| pcfSmFqdn | Fqdn | O | 0..1 | FQDN of the PCF hosting the Npcf_SMPolicyControl service. (NOTE 7) (NOTE 10) | SamePcf |
| pcfSmIpEndPoints | array(IpEndPoint) | O | 1..N | IP end points of the PCF hosting the Npcf_SMPolicyControl service. (NOTE 7) (NOTE 10) | SamePcf |
| snssai | Snssai | M | 1 | The identification of slice. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features as described in clause 5.8. Shall be present in the HTTP POST request/response; or in the HTTP GET response if the "supp-feat" attribute query parameter is included in the HTTP GET request. |  |
| pcfId | NfInstanceId | O | 0..1 | PCF instance identifier |  |
| pcfSetId | NfSetId | O | 0..1 | The PCF set Id |  |
| recoveryTime | DateTime | O | 0..1 | Recovery time of the PCF |  |
| paraCom | ParameterCombination | O | 0..1 | If it is included, the BSF shall check whether there is an existing PCF binding information for the indicated combination. (NOTE 6) (NOTE 10) | SamePcf |
| bindLevel | BindingLevel | O | 0..1 | Contains the level of binding. |  |
| ipv4FrameRouteList | array(Ipv4AddrMask) | O | 1..N | List of Framed Route information of IPv4. |  |
| ipv6FrameRouteList | array(Ipv6Prefix) | O | 1..N | List of Framed Route information of IPv6. |  |
| NOTE 1: The ipDomain attribute may only be provided if the ipv4Addr attribute is present. NOTE 2: When the "ExtendedSamePcf" feature is not supported, at least one of "pcfFqdn" or "pcfIpEndPoints" shall be included if the PCF supports the Npcf_PolicyAuthorization service. When the "ExtendedSamePcf" feature is supported these attributes may be provided if available. NOTE 3: When the "ExtendedSamePcf" feature is not supported, both pcfDiamHost and pcfDiamRealm are provided if the PCF supports Rx interface. When the "ExtendedSamePcf" feature is supported these attributes may provided if available. NOTE 4: 5G-RG and FN-RG replaces UE for wireline access support. See 3GPP TS 23.316 [19]. NOTE 5: IPv6 prefix(es) shorter than /64, according to 3GPP TS 23.501 [2], clause 5.8.2.2 and 3GPP TS 23.316 [19], clause 8.3.1, or full IPv6 address(es) with a /128 prefix, according to 3GPP TS 23.316 [19], clause 8.3.1, may be encoded as the "ipv6Prefix" and "addIpv6Prefixes" attributes. NOTE 6: If the BSF finds that there is an existing Individual PCF for a PDU Session Binding resource for the indicated combination containing Npcf_SMPolicyControl service addressing information, the BSF shall not check other Individual PCF for a PDU Session Binding resources and shall reject the ongoing registration, and return the FQDN or IP endpoints of the Npcf_SMPolicyControl service of the matching Individual PCF for a PDU Session Binding resource to the requesting PCF. NOTE 7: At least one of the "pcfSmFqdn" attribute or the "pcfSmIpEndPoints" attribute shall be included in the binding information, if the binding refers to an SM Policy association and if the "SamePcf" feature is supported and the PCF determines that the same PCF shall be selected for the SM Policy associations with the same SUPI/DNN/S-NSSAI parameter combination in the non-roaming or home-routed scenario based on operator's policies and configuration. NOTE 8: When the "ExtendedSamePcf" feature is not supported the address information of the served UE shall be provided, i.e., either the "ipv4Addr", the "ipv6Prefix" and/or "addIpv6Prefixes" attributes or the "macAddr48" and/or "addMacAddrs" attributes shall be provided as specified in clause 4.2.2.2. NOTE 9: When the "ExtendedSamePcf" feature is not supported the address information of the Npcf_PolicyAuthorization service and/or Rx interface shall be provided, i.e., at least one of the "pcfFqdn" or "pcfEndPoints" and/or both "pcfDiamHost" and "pcfDiamRealm" shall be provided as specified in clause 4.2.2.2. NOTE 10: This attribute does not apply when the NF service consumer is an AF/NEF. NOTE 11: The BSF uses the DNN as received from the NF service consumer without applying any transformation. To successfully perform DNN matching, in a specific deployment a DNN shall always be encoded either with the full DNN (e.g., because there are multiple Operator Identifiers for a Network Identifier) or the DNN Network Identifier only. The NF service consumer may include the DNN Operator Identifier based on local configuration. |  |  |  |  |  |

# *** End of Changes ***