GPP TSG CT WG3 Meeting #144	C3-255194

# Dallas, USA, 17th – 21st November, 2025

Source:	Nokia, Ericsson

Title:	Updates for Naf_Training API

Spec:	3GPP TS 29.530

Agenda item:	19.39 (AIML_CN)

Document for:	Agreement

1. Introduction

The Naf_Training data model contains various mistakes, such as wrong clause numbers and ambiguous presence condition NOTE.

Moreover, the agreed TS 23.288 CR 1508 (S2-2509564) in clauses 6.2H.4 and 7.5.2 required that the AF provides to the consumer optionally as output information the estimated VFL training completion time and the VFL correlation ID.

2. Reason for Change

Correct and complete Naf_Training and correct Naf_Inference data models in TS 29.530.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.530 V 1.1.0.

* * * * First change * * * *

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the data types defined for the Naf_Training service-based interface protocol.

Table 6.3.6.1-1: Naf_Training API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| EventNotif | 6.3.6.2.7 | Represents notification on event training. |  |
| EventSubsc | 6.3.6.2.4 | Represents an event training subscription. |  |
| MlModelMonitorInfo | 6.3.6.2.5 | Represents the ML model monitoring information. |  |
| TrainEventsNotif | 6.3.6.2.6 | Represents notification on training event(s) that occurred. |  |
| TrainEventsSubsc | 6.3.6.2.2 | Represents event(s) training subscription. |  |
| TrainEventsSubscPatch | 6.3.6.2.3 | Represents the requested modifications to an event training subscription. |  |

Table 6.3.6.1-2 specifies data types re-used by the Naf_Training service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naf_Training service-based interface.

Table 6.3.6.1-2: Naf_Training API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Accuracy | 3GPP TS 29.520 [18] | Represents accuracy levels of interest for ML models. |  |
| DateTime | 3GPP TS 29.571 [16] | Identifies the time. |  |
| DelayEventNotif | 3GPP TS 29.520 [18] | Indicates and provides information about that the AF is not able to complete the training procedure within the maximum response time a. |  |
| EventFilter | 3GPP TS 29.520 [18] | Represents the event filters used to identify the subscribed analytics. |  |
| InferenceDataForModelTrain | 3GPP TS 29.520 [18] | Represents the inference data for model training. |  |
| InputDataInfo | 3GPP TS 29.520 [18] | Represents the metrics of the input data. |  |
| MLModelMetric | 3GPP TS 29.520 [18] | Represents the ML Model Metric. |  |
| NetworkAreaInfo | 3GPP TS 29.554 [23] | Identifies the network area. |  |
| NwdafEvent | 3GPP TS 29.520 [18] | Represents the NWDAF event. |  |
| ReportingInformation | 3GPP TS 29.523 [17] | Represents the event reporting requirements. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| TargetUeInformation | 3GPP TS 29.520 [18] | Identifies the target UE information. |  |
| TimeWindow | 3GPP TS 29.122 [22] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [16] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |
| VflTermCause | 3GPP TS 29.520 [18] | Identifies the VFL termination cause. |  |

* * * * Next change * * * *

##### 6.3.6.2.7	Type: EventNotif

Table 6.3.6.2.7-1: Definition of type EventNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NwdafEvent | M | 1 | Contains the identifier of the analytics to which the training notification is related. |  |
| delayEventNotif | DelayEventNotif | O | 0..1 | Contains information indicating that the AF is not able to provide the training results within the maximum response time. |  |
| trainingInd | boolean | O | 0..1 | Set to "true" to indicate that the ML model training is ongoing. Set to "false" to indicate that the ML model training is done. Default value is "false" if omitted. (NOTE) |  |
| validityPeriod | TimeWindow | O | 0..1 | Indicates the time period when the provided ML model applies. (NOTE) |  |
| spatialValidity | NetworkAreaInfo | O | 0..1 | Indicates the area where the provided ML model applies. (NOTE) |  |
| repRatio | Uinteger | O | 0..1 | Minimum percentage of UEs whose data is used for training an ML model when the target of ML model reporting is a group of UEs. Minimum = 0. Maximum = 100. (NOTE) |  |
| accMLModel | Uinteger | O | 0..1 | Indicates the accuracy value of the ML model. Minimum = 0. Maximum = 100. (NOTE) |  |
| termCause | VflTermCause | O | 0..1 | Contains the termination cause. (NOTE) |  |
| vflCorrId | string | O | 0..1 | Contains the VFL Correlation ID. |  |
| NOTE: If the "termCause" attribute is present, the rest of these attributes shall not be present. |  |  |  |  |  |

* * * * Next change * * * *

##### 6.3.6.5.1	Binary Data Types

Table 6.3.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

* * * * Next change * * * *

#### 6.3.7.3	Application Errors

The application errors defined for the Naf_Training service are listed in Table 6.3.7.3-1.

Table 6.3.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| OVERLOAD | 403 Forbidden | Indicates the AF is overloaded. |  |
| NOTE: Including a "ProblemDetails" data structure with the "cause" attribute in the HTTP response is optional unless explicitly mandated in the service operation clauses. |  |  |  |

* * * * Next change * * * *

### 6.3.8	Feature negotiation

The optional features in table 6.3.8-1 are defined for the Naf_Training API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.3.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

* * * * Next change * * * *

# A.4	Naf_Training API

openapi: 3.0.0

info:

title: Naf_Training Service API

version: 1.0.0-alpha.1

description: |

AF Training Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.530 V1.0.0; 5G System; Application Function Artificial Intelligence/Machine Learning

(AI/ML) Services.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.530/

servers:

- url: '{apiRoot}/naf-train/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- naf-train

paths:

/subscriptions:

post:

summary: Request the creation of a Training Subscription.

operationId: CreateTrainSubsc

tags:

- Training Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

responses:

'201':

description: >

Created. The Training Subscription is successfully created and a representation of

the created Individual Training Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual Training Subscription resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

TrainEventsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsNotif'

responses:

'204':

description: >

No Content. The Training Notification is successfully received

and acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the Individual Training Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Training Subscription resource.

operationId: GetIndTrainSubsc

tags:

- Individual Training Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Training Subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Training Subscription resource.

operationId: UpdateIndTrainSubsc

tags:

- Individual Training Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

responses:

'200':

description: >

OK. The Individual Training Subscription resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

'204':

description: >

No Content. The Individual Training Subscription resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Training Subscription resource.

operationId: ModifyIndTrainSubsc

tags:

- Individual Training Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/TrainEventsSubscPatch'

responses:

'200':

description: >

OK. The Individual Training Subscription resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/TrainEventsSubsc'

'204':

description: >

No Content. The Individual Training Subscription resource is successfully modified

and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Training Subscription resource.

operationId: DeleteIndTrainSubsc

tags:

- Individual Training Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Training Subscription resource is successfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

naf-train: >

Enables to access all the resources and custom operations of the Naf_Training API.

schemas:

#

# STRUCTURED DATA TYPES

#

TrainEventsSubsc:

description: Represents a Training Subscription.

type: object

properties:

trainEventSubs:

type: object

additionalProperties:

$ref: '#/components/schemas/EventSubsc'

minProperties: 1

description: >

Contains the subscribed training events. The key of the map shall be set to the

value of the "event" attribute in the EventSubsc data structure.

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

notifCorreId:

type: string

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

eventNotifs:

$ref: '#/components/schemas/EventNotif'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- trainEventSubs

- notifUri

- notifCorreId

TrainEventsSubscPatch:

description: Represents the requested modifications to a Training Subscription.

type: object

properties:

trainEventSubs:

type: object

additionalProperties:

$ref: '#/components/schemas/EventSubsc'

minProperties: 1

description: >

Contains the updated subscribed training events. The key of the map shall be set to the

value of the "event" attribute in the EventSubsc data structure.

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

notifCorreId:

type: string

reportingReqs:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

EventSubsc:

description: Represents events training subscription.

type: object

properties:

event:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

useCaseCxt:

type: string

trainFilter:

$ref: 'TS29520_Nnwdaf_AnalyticsInfo.yaml#/components/schemas/EventFilter'

tgtUe:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/TargetUeInformation'

repRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

targetPeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

inferInputData:

$ref: 'TS29520_Nnwdaf_MLModelProvision.yaml#/components/schemas/InputDataInfo'

timeModelNeeded:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

modelMonInfo:

$ref: '#/components/schemas/MlModelMonitorInfo'

accuLevel:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/Accuracy'

required:

- event

MlModelMonitorInfo:

description: Represents an event training subscription.

type: object

properties:

modelMetric:

$ref: 'TS29520_Nnwdaf_MLModelProvision.yaml#/components/schemas/MLModelMetric'

accuThreshold:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

storedData:

$ref: 'TS29520_Nnwdaf_MLModelProvision.yaml#/components/schemas/InferenceDataForModelTrain'

TrainEventsNotif:

description: Represents a events training notification.

type: object

properties:

notifCorreId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/EventNotif'

minItems: 1

required:

- notifCorreId

- eventNotifs

EventNotif:

description: Represents an event training report.

type: object

properties:

event:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

expCompTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

trainingInd:

type: boolean

description: >

Set to "true" to indicate that the ML model training is ongoing. Set to "false" to

indicate that the ML model training is done. Default value is "false" if omitted.

validityPeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

spatialValidity:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

repRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

accMLModel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

termCause:

$ref: 'TS29520_Nnwdaf_VFLInference.yaml#/components/schemas/VflTermCause'

vflCorrId:

type: string

required:

- event

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# Data types describing alternative data types or combinations of data types:

#

* * * * End of changes * * * *