3GPP TSG CT WG3 Meeting #144	C3-255214

# Dallas, US, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1750 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Defining the AIoT message size related restrictions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | As clarified by RAN2 in their LS in R2-2507932, the maximum size of an AIoT NAS message (and consequently the maximum size of AIoT application data) that can be transported by AS is 119 octets in the R2D direction and 125 octets in the D2R direction. This result can be obtained/concluded from the corresponding message specification in 3GPP TS 38.391. C1-257026 to TS 24.369 is submitted to the parallel CT1#158 meeting to align the maximum AIoT NAS message size, with the maximum allowed AIoT application data length set to 99 for both read and write commands. The necessary alignments to the application data length that can be set by the AF/NEF should hence be specified by CT3. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed issues. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed issues are not addressed and the definition of the related Ambient IoT requirements/provisions is not completed. |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.49.3, 5.45.5.2.4, 5.45.7.3 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions of the APIs. |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G".

[4]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[5]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[6]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[7]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 29.521: "5G System; Binding Support Management Service; Stage 3".

[10]	Void.

[11]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[12]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[13]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[14]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[15]	Void.

[16]	Void

[17]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[18]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[19]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[20]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[21]	3GPP TR 21.900: "Technical Specification Group working methods".

[22]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[23]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Control Data, Application Data and Structured Data for Exposure; Stage 3".

[24]	3GPP TS 29.541: "5G System; Network Exposure (NE) function services for Non-IP Data Delivery (NIDD) and Short Message Services (SMS); Stage 3".

[25]	3GPP TS 29.542: "5G System, Session management services for Non-IP Data Delivery (NIDD); Stage 3".

[26]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[27]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[28]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G system (5GS)".

[29]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[30]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[31]	Void

[32]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[33]	3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[34]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[35]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services; Stage 3".

[36]	3GPP TS 23.273: "5G System Location Services (LCS)".

[37]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[38]	3GPP TS 29.535: "5G System; AKMA Anchor Services; Stage 3".

[39]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[40]	IETF RFC 7542: "The Network Access Identifier".

[41]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[42]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[43]	3GPP TS 29.534: "5G System; Access and Mobility Policy Authorization Service; Stage 3".

[44]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[45]	IEEE Std 1588-2019: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control".

[46]	IEEE Std 802.1AS-2020: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

[47]	3GPP TS 29.536: "5G System; Network Slice Admission Control Services; Stage 3".

[48]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[49]	3GPP TS 24.555: "Proximity based services (ProSe) in 5G system (5GS); User Equipment (UE) policies; Stage 3".

[50]	3GPP TS 29.565: "5G System; Time Sensitive Communication and Time Synchronization Function Services; Stage 3".

[51]	IEEE 802.1Q: "Virtual Bridged Local Area Networks".

[52]	3GPP TS 29.532: "5G System; 5G Multicast-Broadcast Session Management Services; Stage 3".

[53]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[54]	IETF RFC 6733: "Diameter Base Protocol".

[55]	3GPP TS 23.003: "Numbering, addressing and identification".

[56]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications; Stage 2".

[57]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[58]	3GPP TS 29.517: "5G System; Application Function (AF) event exposure service".

[59]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[60]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[61]	3GPP TS 29.564: "5G System; User Plane Function Services; Stage 3".

[62]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[63]	3GPP TS 29.537: "Multicast/Broadcast Policy Control Services; Stage 3".

[64]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[65]	3GPP TS 26.502: "5G multicast–broadcast services; User Service architecture".

[66]	3GPP TS 29.580: "Multicast/Broadcast Service Function Services; Stage 3".

[67]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[68]	3GPP TS 29.543: "5G System; Data Transfer Policy Control Services; Stage 3".

[69]	3GPP TS 24.578: "Aircraft-to-Everything (A2X) services in 5G System (5GS); UE policies".

[70]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[71]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[72]	3GPP TS 24.514: "Ranging based services and sidelink positioning in 5G system(5GS); Stage 3".

[73]	3GPP TS 29.591: "5G System; Network Exposure Function Southbound Services; Stage 3".

[74]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[75]	GSMA PRD NG.135, version 3.0: "E2E Network Slicing Requirements".

[76]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[77]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[78]	3GPP TS 29.175: "IP Multimedia Subsystem (IMS) Application Server (AS) Services Stage 3".

[79]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[80]	3GPP TS 29.562: "5G System; Home Subscriber Server (HSS) services; Stage 3".

[81]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[82]	3GPP TS 29.566: "5G System; Energy Information Function Services; Stage 3".

[83]	3GPP TS 29.569: "5G System; Ambient IoT Function (AIOTF) Services; Stage 3".

[84]	3GPP TS 24.369: "Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS); Stage 3".

* * * * Next changes * * * *

#### 4.4.49.3	Procedures for AIoT Command Management

This procedure is used by an AF to request to perform an AIoT Command operation to the NEF (see also clause 6.2.3 of 3GPP TS 23.369 [81]).

In order to perform an AIoT Command operation, the AF shall send an HTTP POST request to the NEF targeting the URI of the corresponsing custom operation (i.e., "CommandRequest"), with the request body including the CommandReq data structure.

Upon reception of this HTTP POST request message, the NEF shall:

-	check whether the AF is authorized perform this operation based on operator policies, local configuration information (e.g., SLA with the AF) and/or by interacting with the ADM as defined in clause 5.6 of 3GPP TS 23.369 [81];

-	if the AF is not authorized, reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "AF_NOT_AUTHORIZED" application error; and

-	if the AF is authorized:

-	determine the Target AIoT Area information based on the received External Target AIoT Area information;

-	follow the procedures defined in clause 6.2.3 of 3GPP TS 23.369 [81] by interacting with the selected AIOTF(s);

-	if the provided length of application data is too long (e.g., above the allowed maximum value), the NEF shall reject the request with an HTTP "403 Forbidden" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "APP_DATA_TOO_LONG" application error; and

-	if the NEF fails to process the AIoT Command request, reject the request with an HTTP "500 Internal Server Error" status code with the response body including the ProblemDetails data structure containing the "cause" attribute set to the "UNSPECIFIED_FAILURE" application error.

Upon successful response from the AIOT and successful processing of the request, the NEF shall return an HTTP "200 OK" status code to the AF, with the response body including AIoT Command related information within the CommandResp data structure.

On failure or if the NEF receives an error code from the ADM and/or AIOTF(s), the NEF shall take proper error handling actions, as specified in clause 5.45.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

* * * * Next changes * * * *

##### 5.45.5.2.4	Type: CommandReq

Table 5.45.5.2.4-1: Definition of type CommandReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| commandType | CommandType | M | 1 | Contains the type of the requested command. |  |
| targetArea | ExtAIoTArea | C | 0..1 | Contains the target AIoT Service Area within which the requested command operation shall apply. (NOTE 1) |  |
| targetDevices | AIoTDevices | C | 0..1 | Contains the target AIoT device(s) related information. (NOTE 1) |  |
| numDevices | Uinteger | O | 0..1 | Contains the approximative number of the targeted AIoT device(s). |  |
| msgSize | Uinteger | O | 0..1 | Contains the approximative message size in units of Bytes. This attribute may be present only if the "commandType" attribute is set to "READ". |  |
| offset | Uinteger | C | 0..1 | Contains the offset, expressed in units of bytes. This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the offset from which to read the application data. - If the "commandType" attribute is set to "WRITE", this attribute contains the offset from which to write the application data. |  |
| length | Uinteger | C | 0..1 | Contains the length of application data, expressed in units of bytes (i.e., byte length). This attribute shall be present only if the "commandType" attribute is set to "READ" or "WRITE": - If the "commandType" attribute is set to "READ", this attribute contains the length of application data to read. - If the "commandType" attribute is set to "WRITE", this attribute contains the length of application data to write. (NOTE 2) |  |
| data | Bytes | C | 0..1 | Contains the application data to write. This attribute shall be present only if the "commandType" attribute is set to "WRITE". |  |
| notifUri | Uri | M | 1 | Contains the URI via which the AIoT Command operation related notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.45.6. This attribute shall be present only when feature negotiation is required. |  |
| NOTE 1: At least one of these attributes shall be present. NOTE 2: The maximum value that this attribute can contain shall be as specified in clause 7.2.4 of 3GPP TS 24.369 [84]. |  |  |  |  |  |

* * * * Next changes * * * *

#### 5.45.7.3	Application Errors

The application errors defined for the AIoT API are listed in table 5.45.7.3-1.

Table 5.45.7.3-1: Application errors

| Application Error | HTTP status code |  | Description | Applicability |  |
| --- | --- | --- | --- | --- | --- |
| AF_NOT_AUTHORIZED | 403 Forbidden |  | The request for AIoT services is rejected because the AF is not authorized for the requested AIoT Services. |  |  |
| APP_DATA_TOO_LONG | 403 Forbidden |  | The AIoT Command request is rejected because the provided length of application data is too long (e.g., above the allowed maximum value). |  |  |
| UNSPECIFIED_FAILURE | 500 Internal Server Error |  | The request for AIoT services is rejected because the NEF failed to process it. |  |  |

* * * * End of changes * * * *