3GPP TSG-CT WG4 Meeting #131	C4-254078

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.674 | CR | 0149 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of QoS Analysis event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UPF is the only node in the core network to handle the packets related to the user’s service, and UPF can evalute the quality of the service based on some analysis methods. If the UPF can report the information to the other NFs, like NWDAF, it can help the NWDAF to evalute the end-to-end service experience and trigger the optimization on the service quality. Currently, UPF can only report limited information to the consumer, it is proposed to define a new event QoS Analysis, and report more information from the UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support the new event QoS Analysis in the UPF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service will be impacted as network does not have enough information to optimise the QoS parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.3.X(new), 6.1.6.1, 6.1.6.2.3, 6.1.6.2.A(new), 6.1.6.2.B(new), 6.1.6.2.C(new), 6.1.6.2.D(new), 6.1.6.3.3, 6.1.6.3.A(new), 6.1.6.3.B(new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution introduces backward compatibility new features to the OpenAPI files of: TS29520_Nnwdaf_AnalyticsInfo.yaml TS29520_Nnwdaf_DataManagement.yaml TS29520_Nnwdaf_RoamingData.yaml TS29564_Nupf_EventExposure.yaml TS29574_Ndccf_DataManagement.yaml TS29575_Nadrf_DataManagement.yaml TS29576_Nmfaf_3caDataManagement.yaml TS29576_Nmfaf_ContextManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.2.1.3.X	QoS Analysis

Table 5.2.1.3.X-1: QoS Analysis event

| Description | This event provides QoS Analysis information, i.e. QoS experience related to the sub application and service flows. |
| --- | --- |
| Subscription type | Subscription via SMF using Nupf_EventExposure Subscribe |
| Subscription inputs to UPF | Required: - UE IP address (for an IP PDU session type), SUPI (for a non-IP PDU session type) - UPF event consumer notification URI - Notification correlation ID Optional: - DNN - S-NSSAI - Port number of the remote end - IP domain - either Application ID(s) or Traffic filters |
| Report type | One-Time Report Periodic Report |

* * * * Next Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nupf_EventExposure service.

Table 6.1.6.1-1: Nupf_EventExposure specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| NotificationData | 6.1.6.2.2 | The list of NotificationItems |  |
| NotificationItem | 6.1.6.2.3 | Represents a report on one subscribed event |  |
| QosMonitoringMeasurement | 6.1.6.2.4 | QoS Monitoring Measurement information |  |
| UserDataUsageMeasurements | 6.1.6.2.5 | User Data Usage Measurements |  |
| VolumeMeasurement | 6.1.6.2.6 | Volume Measurement |  |
| ThroughputMeasurement | 6.1.6.2.7 | Throughput Measurement |  |
| ApplicationRelatedInformation | 6.1.6.2.8 | Application Related Information |  |
| ThroughputStatisticsMeasurement | 6.1.6.2.9 | Throughput Statistics Measurement |  |
| DomainInformation | 6.1.6.2.10 | Domain Name and Domain Name Protocol |  |
| UpfEventSubscription | 6.1.6.2.11 | Represents an individual event subscription resource on UPF |  |
| UpfEventMode | 6.1.6.2.12 | Describes how the reports shall be generated for a subscribed event |  |
| UpfEvent | 6.1.6.2.13 | Describes an event to be subscribed |  |
| CreateEventSubscription | 6.1.6.2.14 | Data within a create UPF event subscription request |  |
| CreatedEventSubscription | 6.1.6.2.15 | Data within a create UPF event subscription response |  |
| ReportingSuggestionInformation | 6.1.6.2.16 | Reporting Suggestion Information |  |
| TscManagementInfo | 6.1.6.2.17 | TSC Management Information |  |
| UeNatMappingInfo | 6.1.6.2.18 | UE NAT Mapping Information |  |
| NatMapping | 6.1.6.2.19 | NAT mapping |  |
| HandlingOfPayloadHeader | 6.1.6.2.20 | Handling of Payload Header |  |
| SkipReportingInstruction | 6.1.6.2.21 | Skip Reporting Instruction |  |
| ThresholdCond | 6.1.6.2.22 | Threshold Conditions |  |
| QosAnalysisInfo | 6.1.6.2.A | Report on the QoS Analysis |  |
| QosMonitoringReport | 6.1.6.2.B | Report on the QoS monitoring |  |
| SubAppQkkReport | 6.1.6.2.C | Report on the QoS exerience on sub application |  |
| SubAppMonitoringReport | 6.1.6.2.D | Report on additional QoS monitoring on sub application |  |
| EventType | 6.1.6.3.3 | Event Type |  |
| UpfEventTrigger | 6.1.6.3.4 | Describes how the UPF generates the report for the event |  |
| MeasurementType | 6.1.6.3.5 | Type of Measurement |  |
| GranularityOfMeasurement | 6.1.6.3.6 | Granularity Of Measurement |  |
| DnProtocol | 6.1.6.3.7 | Domain Name Protocol |  |
| ReportingUrgency | 6.1.6.3.8 | Reporting Urgency |  |
| TerminationCause | 6.1.6.3.9 | The reason to terminate the subscription |  |
| RemainingDataReports | 6.1.6.3.10 | Indication on how to handle the data collected by the source UPF and not yet reported to the NF service consumer, at the termination of the subscription |  |
| SkipReportingCondition | 6.1.6.3.11 | Skip Reporting Condition |  |
| QualityStatus | 6.1.6.3.A | Quality level of the service |  |
| RunningStatus | 6.1.6.3.B | Running Status of the service |  |

Table 6.1.6.1-2 specifies data types re-used by the Nupf_EventExposure service from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nupf_EventExposure service.

Table 6.1.6.1-2: Nupf_EventExposure re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.571 [16] | Date time |  |
| Dnn | 3GPP TS 29.571 [16] | DNN |  |
| Gpsi | 3GPP TS 29.571 [16] | GPSI |  |
| Snssai | 3GPP TS 29.571 [16] | S-NSSAI |  |
| Uint32 | 3GPP TS 29.571 [16] | Uint32 |  |
| MacAddr48 | 3GPP TS 29.571 [16] | MAC Address |  |
| Ipv4Addr | 3GPP TS 29.571 [16] | IPv4 address |  |
| Ipv6Prefix | 3GPP TS 29.571 [16] | IPv6 address prefix |  |
| Uint64 | 3GPP TS 29.571 [16] | Unsigned 64-bit integer |  |
| BitRate | 3GPP TS 29.571 [16] | Bit rate |  |
| PacketRate | 3GPP TS 29.571 [16] | Packet rate |  |
| TrafficVolume | 3GPP TS 29.571 [16] | Traffic Volume |  |
| ApplicationId | 3GPP TS 29.571 [16] | The application identifier. |  |
| DurationSec | 3GPP TS 29.571 [16] |  |  |
| NotificationFlag | 3GPP TS 29.571 [16] | Notification flag. |  |
| PartitioningCriteria | 3GPP TS 29.571 [16] | Used to partition UEs before applying sampling. |  |
| ProblemDetails | 3GPP TS 29.571 [16] |  |  |
| SamplingRatio | 3GPP TS 29.571 [16] | Sampling Ratio. |  |
| Uri | 3GPP TS 29.571 [16] |  |  |
| IpAddr | 3GPP TS 29.571 [16] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [16] |  |  |
| Supi | 3GPP TS 29.571 [16] |  |  |
| Pei | 3GPP TS 29.571 [16] |  |  |
| Uinteger | 3GPP TS 29.571 [16] | Unsigned Integer |  |
| PortManagementContainer | 3GPP TS 29.512 [19] | PMIC |  |
| BridgeManagementContainer | 3GPP TS 29.512 [19] | UMIC |  |
| FlowInformation | 3GPP TS 29.512 [19] | IP or Ethernet Flow Information |  |
| PatchItem | 3GPP TS 29.571 [16] | Patch item of JSON PATCH |  |
| PatchResult | 3GPP TS 29.571 [16] | Patch result of JSON PATCH |  |
| MutingExceptionInstructions | 3GPP TS 29.571 [16] | Muting exception instructions. |  |
| MutingNotificationsSettings | 3GPP TS 29.571 [16] | Muting notifications settings. |  |
| TransportProtocol | 3GPP TS 29.571 [16] | Transport Protocol |  |
| HeaderHandlingAction | 3GPP TS 29.514 [21] | Header Handling Action |  |
| RecurTime | 3GPP TS 29.503 [22] | Recurring time |  |
| RatType | 3GPP TS 29.571 [16] | RAT Type |  |

* * * * Next Change * * * *

6.1.6.2.3	Type: NotificationItem

Table 6.1.6.2.3-1: Definition of type NotificationItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventType | EventType | M | 1 | The event type of the event for which the notification is generated. |  |
| ueIpv4Addr | Ipv4Addr | C | 0..1 | IPv4 address of the UE (NOTE 1, NOTE 3, NOTE 5) |  |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | IPv6 address prefix of the UE (NOTE 1, NOTE 3, NOTE 5) |  |
| ueMacAddr | MacAddr48 | O | 0..1 | MAC address of the UE. (NOTE 2, NOTE 3) |  |
| dnn | Dnn | O | 0..1 | When present, this attribute indicates the DNN of the PDU session for which the notification is generated. |  |
| snssai | Snssai | O | 0..1 | When present, this attribute indicates the S-NSSAI of the PDU session for which the notification is generated. |  |
| gpsi | Gpsi | O | 0..1 | When present, this attribute indicates the GPSI of the UE for which the notification is generated. |  |
| supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |
| timeStamp | DateTime | M | 1 | The value represents the UTC time when the information in this report was generated. |  |
| startTime | DateTime | O | 0..1 | When present, this attribute shall provide the timestamp when the information measured for generating this report was started. |  |
| ratType | RatType | C | 0..1 | This IE shall be present if the NF service consumer requested the UPF to include the RAT Type in the event reports and if the RAT type is available in the UPF for the PFCP session. When present, it shall contain the RAT Type on which the UE is camping when the measurement is performed. |  |
| qosMonitoringMeasurement | QosMonitoringMeasurement | C | 0..1 | This attribute shall be present if eventType is set to "QOS_MONITORING". |  |
| userDataUsageMeasurements | array(UserDataUsageMeasurements) | C | 1..N | This IE shall be present if eventType is set to "USER_DATA_USAGE_MEASURES" or "USER_DATA_USAGE_TRENDS". |  |
| tscMngtInfo | TscManagementInfo | C | 0..1 | This attribute shall be present if eventType is set to "TSC_MNGT_INFO". |  |
| UeNatMappingInfo | UeNatMappingInfo | C | 0..1 | This IE shall be present if eventType is set to "UE_NAT_MAPPING_INFO" (NOTE 4) |  |
| handlingOfPayloadHeaderInfo | array(HandlingOfPayloadHeader) | C | 1..N | This attribute shall be present if eventType is set to "HANDLING_OF_PAYLOAD_HEADERS_INFO". When present, it shall contain a list of applied header handling actions. |  |
| terminationCause | TerminationCause | O | 0..1 | This IE may be present if the eventType is set to "SUBSCRIPTION_TERMINATION". When present, this IE shall indicate the reason for the subscription termination. |  |
| skippedReportInfo | array(SkipReportingCondition) | O | 1..N | This IE may be present if prior to the event report, there were some event reports for this event were skipped due to one or more of the following reasons: - "SKIP_NULL_REPORT" - "SKIP_OUTSIDE_VALIDITY_TIME" - "SKIP_BELOW_THRESHOLD_REPORT". | SEER |
| qosAnaInfo | QosAnalysisInfo | C | 0..1 | This IE shall be present if eventType is set to "QOS_ANA". |  |
| NOTE 1: At least one of ueIpv4Addr and ueIpv6Prefix shall be present if the subscription applies to an IP PDU session. NOTE 2: An NF service consumer subscribing to receive QoS Monitoring Measurement report for an ethernet PDU session shall accept the NotificationItem having neither ueIpv4Addr nor ueIpv6Prefix. NOTE 3: At least one of ueIpv4Addr, ueIpv6Prefix and ueMacAddr shall be present. NOTE 4: The UPF may report a list of NatMapping for the same private UE IP address and the same remote IP address, for different destination port numbers, if no remote port number was received in the subscription. NOTE 5: For a UPF deployed with NAT functionality, this IE shall contain the UE (private) IP address allocated by the 5GC. |  |  |  |  |  |

* * * * Next Change * * * *

##### 6.1.6.2.A	Type: QosAnalysisInfo

Table 6.1.6.2.A-1: Definition of type QosAnalysisInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| appId | ApplicationId | C | 0..1 | When present, this IE shall contain the application identifier. |
| subAppId | ApplicationId | C | 0..1 | When present, this IE shall contain the sub application identifier. |
| subAppQosMonReport | QosMonitoringReport | C | 0..1 | When present, this IE shall contain the QoS Monitoring reporting information of sub application. |
| subAppQosQuality | QualityStatus | C | 0..1 | When present, this IE shall indicate the quality of the sub application |
| subAppStatus | RunningStatus | C | 0..1 | When present, this IE shall indicate the running status of the sub application. |
| flowStatus | array(RunningStatus) | C | 1..N | When present, this IE shall indicate the running status of the flows reated to the sub application. |
| 5qi | arry(5Qi) | C | 1..N | When present, this IE shall contain the Identifier for the authorized QoS parameters for the service data flow. |
| flowQosQualities | array(QualityStatus) | C | 1..N | When present, this IE shall contain the quality of the service data flow. |
| flowQosDescs | array(FlowDescription) | C | 1..N | When present, this IE shall contain the packet filters of the IP flow(s). Each entry of the array describes a single IP flow. |
| flowQosMonReports | array(QosMonitoringReport) | C | 1..N | When present, this IE shall contain the QoS Monitoring reporting information of each flow. |
| userLocationInfo | UserLocation | C | 0..1 | When present, this IE shall contain the location(s) where the served UE is camping. |
| subAppQkkRpt | SubAppQkkReport | C | 0..1 | When present, this IE shall contain the QoS Analysis reporting information of the sub application. |
| subAppMonRpt | SubAppMonitoringReport | C | 0..1 | When present, this IE shall contain the QoS Monitoring reporting information of the sub application. |

* * * * Next Change * * * *

##### 6.1.6.2.B	Type: QosMonitoringReport

Table 6.1.6.2.B-1: Definition of type QosMonitoringReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| delayAn | integer | C | 0..1 | When present, this IE shall contain the round-trip packet delay in millisecond (ms) between the UE and the UPF. |
| delayDn | integer | C | 0..1 | When present, this IE shall contain the round-trip packet delay in millisecond (ms) between the UPF and the DN. |
| bandwidthUl | BitRateRm | C | 0..1 | When present, this IE shall contain the uplink bandwidth. |
| bandwidthDl | BitRateRm | C | 0..1 | When present, this IE shall contain the downlink bandwidth. |
| lostPacketsUl | integer | C | 0..1 | When present, this IE shall contain the lost uplink packets |
| totalPacketsUl | integer | C | 0..1 | When present, this IE shall contain the total uplink packets |
| lostPacketsDl | integer | C | 0..1 | When present, this IE shall contain the lost downlink packets |
| totalPacketsDl | integer | C | 0..1 | When present, this IE shall contain the total downlink packets. |

* * * * Next Change * * * *

##### 6.1.6.2.C	Type: SubAppQkkReport

Table 6.1.6.2.C-1: Definition of type SubAppQkkReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| avgQoe | Uinteger | C | 0..1 | When present, this IE shall contain the average score of the QoS analysis. |
| maxResolution | string | C | 0..1 | When present, this IE shall contain the maximum resolution. |
| mostResolution | string | C | 0..1 | When present, this IE shall contain the longest duration resolution. |
| maxBitRateUl | BitRate | C | 0..1 | When present, this IE shall contain the maximum uplink bit rate in kbps. |
| avgBitRateUl | BitRate | C | 0..1 | When present, this IE shall contain the average uplink bit rate in kbps. |
| maxBitRateDl | BitRate | C | 0..1 | When present, this IE shall contain the maximum downlink bit rate in kbps. |
| avgBitRateDl | BitRate | C | 0..1 | When present, this IE shall contain the average downlink bit rate in kbps. |
| stallSec | DurationSec | C | 0..1 | When present, this IE shall contain the duration in second if the service is lagging. |
| stallNum | integer | C | 0..1 | When present, this IE shall contain the number if the service is lagging. |
| serviceDelay | integer | C | 0..1 | When present, this IE shall contain the delay of the service in millisecond(s). |
| initialDur | integer | C | 0..1 | When present, this IE shall contain the Initial buffering duration of service in millisecond(s). |

* * * * Next Change * * * *

##### 6.1.6.2.D	Type: SubAppMonitoringReport

Table 6.1.6.2.D-1: Definition of type SubAppMonitoringReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| subAppStartTime | DateTime | C | 0..1 | When present, this IE shall contain the start time of the sub application. |
| subAppEffDur | DurationSec | C | 0..1 | When present, this IE shall contain the duration of the sub application in second(s). |
| avgBwUl | BitRateRm | C | 0..1 | When present, this IE shall contain the uplink average bandwidth of the sub application. |
| avgBwDl | BitRateRm | C | 0..1 | When present, this IE shall contain the downlink average bandwidth of the sub application. |
| subAppVol | VolumeMeasurement | C | 0..1 | When present, this IE shall contain the volume of the sub application. |
| maxBwUl | BitRateRm | C | 0..1 | When present, this IE shall contain the uplink maximum bandwidth of the sub application. |
| maxBwDl | BitRateRm | C | 0..1 | When present, this IE shall contain the downlink maximum bandwidth of the sub application. |
| maxDelayAn | integer | C | 0..1 | When present, this IE shall contain the round-trip packet delay in millisecond (ms) between the UE and the UPF for the sub application. |
| maxDelayDn | integer | C | 0..1 | When present, this IE shall contain the round-trip packet delay in millisecond (ms) between the UPF and the DN for the sub application. |

* * * * Next Change * * * *

6.1.6.3.3	Enumeration: EventType

The enumeration EventType represents the type of event to which the NF service consumer may subscribe to and for which the notification is generated. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration EventType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "QOS_MONITORING" | QoS Monitoring Measurement (see clause 5.2.1.3.2) |  |
| "USER_DATA_USAGE_MEASURES" | User Data Usage Measures (see clause 5.2.1.3.3) |  |
| "USER_DATA_USAGE_TRENDS" | User Data Usage Trends (see clause 5.2.1.3.4) |  |
| "TSC_MNGT_INFO" | TSC Management Information |  |
| "UE_NAT_MAPPING_INFO" | NATed UE Public IP Address and Port number Information (see clause 5.2.1.3.6) |  |
| "HANDLING_OF_PAYLOAD_HEADERS_INFO" | Handling of Payload Headers Information (see clause 5.2.1.3.7) |  |
| "SUBSCRIPTION_TERMINATION" | This event type is used by the UPF to inform the NF service consumer that the subscription is terminated at the UPF. (NOTE) |  |
| "QOS_ANA" | QoS Analysis (see clause 5.2.1.3.X) |  |
| NOTE: This event type shall be used in event report from UPF to the NF service consumer to report the subscription is terminated when the subTerminationReportInd is included in the subscription, it shall not be used in event subscription. |  |  |

* * * * Next Change * * * *

##### 6.1.6.3.A	Enumeration: QualityStatus

Table 6.1.6.3.A-1: Enumeration QualityStatus

| Enumeration value | Description |
| --- | --- |
| "QOE_QUALITY1" | Indicates the quality level of service is one, which means the User’s experience is too bad to tolerate. |
| "QOE_QUALITY2" | Indicates the quality level of service is two, which means the User’s experience is not good but can be kept on. |
| "QOE_QUALITY3" | Indicates the quality level of service is three, which means the User’s experience is average. |
| "QOE_QUALITY4" | Indicates the quality level of service is four, which means the User’s experience is good. |
| "QOE_QUALITY5" | Indicates the quality level of service is five, which means the User’s experience is excellent. |
| NOTE: How to distinguish the level of QoS experience is left to implementation. |  |

* * * * Next Change * * * *

##### 6.1.6.3.B	Enumeration: RunningStatus

Table 6.1.6.3.B-1: Enumeration RunningStatus

| Enumeration value | Description |
| --- | --- |
| "RUNNING" | Indicates the status of the application is running. |
| "TERMINATED" | Indicates the status of the application is terminated. |

* * * * Next Change * * * *

A.2	Nupf_EventExposure API

openapi: 3.0.0

info:

title: 'UPF Event Exposure Service'

version: 1.2.0-alpha.4

description: |

UPF Event Exposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

NotificationItem:

description: represents a report on one subscribed event

type: object

required:

- eventType

- timeStamp

anyOf:

- required: [ ueIpv4Addr ]

- required: [ ueIpv6Prefix ]

- required: [ ueMacAddr ]

properties:

eventType:

$ref: '#/components/schemas/EventType'

ueIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

ueMacAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

qosMonitoringMeasurement:

$ref: '#/components/schemas/QosMonitoringMeasurement'

tscMngtInfo:

$ref: '#/components/schemas/TscManagementInfo'

userDataUsageMeasurements:

type: array

items:

$ref: '#/components/schemas/UserDataUsageMeasurements'

minItems: 1

ueNatMappingInfo:

$ref: '#/components/schemas/UeNatMappingInfo'

handlingOfPayloadHeadersInfo:

type: array

items:

$ref: '#/components/schemas/HandlingOfPayloadHeader'

minItems: 1

terminationCause:

$ref: '#/components/schemas/TerminationCause'

skippedReportInfo:

type: array

items:

$ref: '#/components/schemas/SkipReportingCondition'

minItems: 1

qosAnaInfo:

$ref: '#/components/schemas/QosAnalysisInfo'

[…]

ExtNotificationData:

oneOf:

- $ref: '#/components/schemas/NotificationData'

- type: array

items:

$ref: '#/components/schemas/NotificationData'

minItems: 2

QosAnalysisInfo:

description: represents a report on the QoS Analysis

type: object

properties:

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

subAppId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

subAppQosMonReport:

$ref: '#/components/schemas/QosMonitoringReport'

subAppQosQuality:

$ref: '#/components/schemas/QualityStatus'

subAppStatus:

$ref: '#/components/schemas/RunningStatus'

flowStatus:

type: array

items:

$ref: '#/components/schemas/RunningStatus'

minItems: 1

5qi:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

minItems: 1

flowQosQualities:

type: array

items:

$ref: '#/components/schemas/QualityStatus'

minItems: 1

flowQosDescs:

type: array

items:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/FlowDescription'

minItems: 1

flowQosMonReports:

type: array

items:

$ref: '#/components/schemas/QosMonitoringReport'

minItems: 1

userLocationInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/userLocation'

subAppQkkRpt:

$ref: '#/components/schemas/SubAppQkkReport'

subAppMonRpt:

$ref: '#/components/schemas/SubAppMonitoringReport'

QosMonitoringReport:

description: represents a report on the QoS monitoring

type: object

properties:

delayAn:

type: integer

delayDn:

type: integer

bandwidthUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRateRm'

bandwidthDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRateRm'

lostPacketsUl:

type: integer

totalPacketsUl:

type: integer

lostPacketsDl:

type: integer

totalPacketsDl:

type: integer

SubAppQkkReport:

description: represents a report on the QoS exerience on sub application

type: object

properties:

avgQoe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxResolution:

type: string

mostResolution:

type: string

maxBitRateUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

avgBitRateUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxBitRateDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

avgBitRateDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

stallSec:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

stallNum:

type: integer

serviceDelay:

type: integer

initialDur:

type: integer

SubAppMonitoringReport:

description: represents a report on additional QoS monitoring on sub application

type: object

properties:

subAppStartTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

subAppEffDur:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

avgBwUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRateRm'

avgBwDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRateRm'

subAppVol:

$ref: '#/components/schemas/VolumeMeasurement'

maxBwUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRateRm'

maxBwDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRateRm'

maxDelayAn:

type: integer

maxDelayDn:

type: integer

[…]

EventType:

description: Event Type

anyOf:

- type: string

enum:

- QOS_MONITORING

- USER_DATA_USAGE_MEASURES

- USER_DATA_USAGE_TRENDS

- TSC_MNGT_INFO

- UE_NAT_MAPPING_INFO

- HANDLING_OF_PAYLOAD_HEADERS_INFO

- SUBSCRIPTION_TERMINATION

- QOS_ANA

- type: string

[…]

SkipReportingCondition:

description: Conditions for which the UPF can skip sending the event report

anyOf:

- type: string

enum:

- SKIP_NULL_REPORT

- SKIP_OUTSIDE_VALIDITY_TIME

- SKIP_BELOW_THRESHOLDS_REPORT

- type: string

QualityStatus:

description: Quality level of the service

anyOf:

- type: string

enum:

- QOE_QUALITY1

- QOE_QUALITY2

- QOE_QUALITY3

- QOE_QUALITY4

- QOE_QUALITY5

- type: string

RunningStatus:

description: Running Status of the service

anyOf:

- type: string

enum:

- RUNNING

- TERMINATED

[…]

* * * * End of Changes * * * *