3GPP TSG-CT WG4 Meeting #131	C4-254099

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.176 | CR | 0044 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to AvatarMedia data type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As agreed in CR(#1602) of TS23.228,the Resource URL and Media Processing Specification are only used in network centric rendering mode. As agreed CR(#1663) of TS23.228, the resource UE ID and requester UE ID are included in Avatar media for UE centric rendering mode. So, this CR propose to - change the presence condition of Resource URL and Media Processing Specification to C and upate the descpriton accordingly. - Add the resource UE ID and requester UE ID to Avatar media data type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the presence condition and description of mediaProcessSpec and resourceUrl; - Add the resourceUeId and requesterUeId to Avatar media description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.10, 6.1.6.3.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29175_Nimsas_MediaControl.yaml TS29176_Nmf_MRM.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

6.1.6.2.10	Type: AvatarMedia

Table 6.1.6.2.10-1: Definition of type AvatarMedia

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceUrl | Uri | C | 0..1 | Represents the URL that MF to retrieve the avatar representation to perform network rendering. This IE shall be included if renderingMode set to "NET_CENTRIC_MF" or "NET_CENTRIC_DCAS". |  |
| mediaProcessSpec | string | C | 0..1 | Represents how the media stream associated with the resource URL should be processed. This IE shall be included if renderingMode set to "NET_CENTRIC_MF" or "NET_CENTRIC_DCAS". |  |
| renderingMode | RenderingMode | M | 1 | Represents the rendering mode of avatar media. |  |
| resourceUeId | UeId | C | 0..1 | Represents the UE that owns the avatar representation to be retrieved by the MF. It shall be included if renderingMode set to "UE_CENTRIC". |  |
| requesterUeId | UeId | C | 0..1 | Represents the UE that is requesting the avatar representation to be retrieved by the MF. It shall be included if renderingMode set to "UE_CENTRIC". |  |

* * * 2nd Change * * * *

6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| SdpString | string | Represents the content after "m=" of the "m=" line and the content after "a=" of the asocciated "a=" lines. |  |
| UeId | string | Represents the IMS Public Identity (IMPU). Pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15})$" |  |

* * * 3rd Change * * * *

A.2	Nmf_MRM API

openapi: 3.0.0

info:

title: 'MF Media Resource Management (MRM) Service'

version: 1.1.0-alpha.3

description: |

MF Media Resource Management (MRM) Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.176 V19.3.0; IP Multimedia Subsystem (IMS); Media Function (MF) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.176/

servers:

- url: '{apiRoot}/nmf-mrm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

[…]

AvatarMedia:

description: Represents the Avatar media descriptor.

type: object

required:

- renderingMode

properties:

resourceUrl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

mediaProcessSpec:

type: string

renderingMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RenderingMode'

resourceUeId:

$ref: '#/components/schemas/UeId'

requesterUeId:

$ref: '#/components/schemas/UeId'

UeId:

description: The IMS Public Identity (IMPU)

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15}|.+)$'

[…]

* * * End of Changes * * * *