3GPP TSG-CT WG4 Meeting #131	C4-254184

Sophia-Antipolis; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0380 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE ID with LCS User Plane Connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eCLS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For LMF and UE supporting LCS user plane connections, the LMF and UE can establish an LCS-UPP connection for exchanging N1 LCS related messages during UE positioning. The LMF use UE ID to associate the subsequent location requests and the corresponding LCS-UPP connections. This requires the AMF to always provide consistent UE ID to the LMF, i.e. either SUPI or GPSI to the LMF in location request when LCS User plane connection is supported. However, there are still issues in above requirements: 1/ one UE may have multiple GPSIs. If the LCS-UPP connection is associated with GPSI-1, there is no way to handle location requests with GPSI-2. 2/ After UE mobility with AMF change, the new AMF does not know the exact UE ID associated with the LCS-UPP connection, thus cannot provide the consistent UE ID in subsequent location request. To resolve this, this CR propose to mandate the SUPI for location request and UPP-CM handling, when LCS-UPP connection is supported. See Discussion paper (C4-254169) for more details. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the SUPI shall be provided in location requests and UPP-CM related operations when LCS User Plane Connection is supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When LCS user plane connection is supported, the location request and LCS-UPP connection may fail when UE has multiple GPSIs or after the UE moves to a new AMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2, 6.1.6.2.52 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any yaml files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.2	Type: InputData

Table 6.1.6.2.2-1: Definition of type InputData

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| externalClientType |  | ExternalClientType |  | O |  | 0..1 |  | When present, this IE shall carry the external client type of the requester. |  |  |  |
| correlationID |  | CorrelationID |  | O |  | 0..1 |  | When present, this IE shall carry the correlation ID of the request. |  |  |  |
| amfId |  | NfInstanceId |  | O |  | 0..1 |  | Indicates the AMF Instance serving the UE. LMF shall use the AMF Instance to forward LCS related N1/N2 messages to the UE/RAN. |  |  |  |
| locationQoS |  | LocationQoS |  | O |  | 0..1 |  | When present, this IE shall carry the QoS of the location request. |  |  |  |
| supportedGADShapes |  | array(SupportedGADShapes) |  | O |  | 1..N |  | When present, this IE shall carry the GAD shapes supported by the requester. |  |  |  |
| supi |  | Supi |  | O |  | 0..1 |  | Indicates the SUPI of the target UE. (NOTE 6) |  |  |  |
| pei |  | Pei |  | O |  | 0..1 |  | Indicates the PEI of the target UE. |  |  |  |
| gpsi |  | Gpsi |  | O |  | 0..1 |  | Indicates the GPSI of the target UE. (NOTE 6) |  |  |  |
| ecgi |  | Ecgi |  | O |  | 0..1 |  | When present, this IE shall indicate the identifier of the E-UTRAN cell serving the UE or the serving cell identifier of the Primary Cell in the Master RAN Node that is an E-UTRAN node on Dual Connectivity scenarios. (NOTE 2) |  |  |  |
| ecgiOnSecondNode |  | Ecgi |  | O |  | 0..1 |  | When present, the serving cell identifier of the Primary Cell in the Secondary RAN Node that is an E-UTRAN node when available on Dual Connectivity scenarios. (NOTE 3) (NOTE 4) |  |  |  |
| ncgi |  | Ncgi |  | O |  | 0..1 |  | When present, this IE shall indicate the identifier of the NR cell serving the UE or the serving cell identifier of the Primary Cell in the Master RAN Node that is a NR node on Dual Connectivity scenarios. (NOTE 2) |  |  |  |
| ncgiOnSecondNode |  | Ncgi |  | O |  | 0..1 |  | When present, the serving cell identifier of the Primary Cell in the Secondary RAN Node that is a NR node when available on Dual Connectivity scenarios. (NOTE 3) (NOTE 4) |  |  |  |
| priority |  | LcsPriority |  | O |  | 0..1 |  | When present, this IE shall indicate the priority of the location request. |  |  |  |
| velocityRequested |  | VelocityRequested |  | O |  | 0..1 |  | When present, this IE shall indicate whether velocity is requested or not. |  |  |  |
| ueLcsCap |  | UeLcsCapability |  | O |  | 0..1 |  | When present, this IE shall indicate the LCS capability supported by the UE. |  |  |  |
| lcsServiceType |  | LcsServiceType |  | O |  | 0..1 |  | The LCS service type |  |  |  |
| ldrType |  | LdrType |  | O |  | 0..1 |  | The type of LDR |  |  |  |
| hgmlcCallBackURI |  | Uri |  | C |  | 0..1 |  | Callback URI of the H-GMLC It shall be present, if attribute LdrType is present. This IE shall also be present for location service in PNI-NPN with signalling optimisation, as specified in 3GPP TS 23.273 [19] clause 6.1.2. |  |  |  |
| lirGmlcCallBackUri |  | Uri |  | C |  | 0..1 |  | This IE shall be present when the intermediateLocationInd IE is present with the value "true". When present, this IE shall contain callback URI of the GMLC to receive the intermediate location reports. |  |  |  |
| vgmlcAddress |  | Uri |  | C |  | 0..1 |  | V-GMLC address that corresponds to the V-GMLC that receives Location Request It shall be present, if attribute LdrType is present and the target UE is in roaming case. |  |  |  |
| ldrReference |  | LdrReference |  | C |  | 0..1 |  | LDR Reference Number It shall be present, if attribute LdrType is present. This IE shall be present for location service in PNI-NPN with signalling optimisation, as specified in 3GPP TS 23.273 [19] clause 6.1.2. |  |  |  |
| lirReference |  | LirReference |  | C |  | 0..1 |  | This IE shall be present when the intermediateLocationInd IE is present with the value "true". When present, this IE shall contain the LIR Reference Number for a multiple location request |  |  |  |
| periodicEventInfo |  | PeriodicEventInfo |  | C |  | 0..1 |  | Information for periodic event reporting. This IE shall be present when ldrType is set to "PERIODIC". |  |  |  |
| areaEventInfo |  | AreaEventInfo |  | C |  | 0..1 |  | Information for area event reporting. This IE shall be present when ldrType is set to "ENTERING_INTO_AREA", "LEAVING_FROM_AREA" or "BEING_INSIDE_AREA". |  |  |  |
| motionEventInfo |  | MotionEventInfo |  | C |  | 0..1 |  | Information for motion event reporting. This IE shall be present when ldrType is set to "MOTION". |  |  |  |
| reportingAccessTypes |  | array(ReportingAccessType) |  | O |  | 1..N |  | Allowed access types for event reporting |  |  |  |
| ueConnectivityStates |  | array(UeConnectivityState) |  | O |  | 1..N |  | When present, this IE shall indicate the UE connectivity state per access type |  |  |  |
| ueLocationServiceInd |  | UeLocationServiceInd |  | C |  | 0..1 |  | If UE sends an MO-LR Request message, this IE shall be present and indicate the request type for a 5GC-MO-LR. |  |  |  |
| moAssistanceDataTypes |  | LcsBroadcastAssistanceTypesData |  | O |  | 0..1 |  | When present, this IE shall indicate a list of one or more types of location assistance data that UE subscribed. |  |  |  |
| lppMessage |  | RefToBinaryData |  | C |  | 0..1 |  | If UE includes the first LPP message in MO-LR Request, this IE shall be present and Indicate the binary data of LPP message. (NOTE 5) |  |  |  |
| lppMessageExt |  | array(RefToBinaryData) |  | C |  | 1..N |  | If UE includes the additional LPP messages (maximum 3) in MO-LR Request, this IE shall be present and Indicates the binary data of LPP message. (NOTE 5) |  |  |  |
| supportedFeatures |  | SupportedFeatures |  | C |  | 0..1 |  | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |  |  |  |
| uePositioningCap |  | UePositioningCapabilities |  | O |  | 0..1 |  | When present, this IE shall indicate the positioning capabilities supported by the UE. |  |  |  |
| tnapId |  | TnapId |  | O |  | 0..1 |  | When present, this IE shall contain the TNAP Identifier. This IE may be present for non-3GPP access. |  |  |  |
| twapId |  | TwapId |  | O |  | 0..1 |  | When present, This IE shall contain the TWAP Identifier. This IE may be present for non-3GPP access. |  |  |  |
| ueCountryDetInd |  | boolean |  | O |  | 0..1 |  | When present, This IE shall contain an indication of determining the UE geographical area identified by the country, area within a country or international area indication where UE is located for PLMN selection verification. |  | SAT |  |
| scheduledLocTime |  | DateTime |  | O |  | 0..1 |  | When present, this IE shall contain the scheduled time (in UTC) that the UE needs to be located. |  |  |  |
| reliableLocReq |  | boolean |  | C |  | 0..1 |  | This IE shall be included with the value true to indicate that reliable UE location information is required, as specified in 3GPP TS 33.256 [26] clause 5.3.2. When present, this IE shall be set as following: - true: the reliable UE location information is required - false (default): the reliable UE location information is not required |  |  |  |
| evtRptAllowedAreas |  | array(ReportingArea) |  | O |  | 1..250 |  | When present, this IE shall contain a list of event report allowed areas, where UE is allowed to generate and send the event report to network during the deferred 5GC-MT-LR procedure for UE power saving purpose. |  |  |  |
| ueUnawareInd |  | boolean |  | C |  | 0..1 |  | UE Unaware Positioning indication. If the UE Unaware Positioning is required, as specified in 3GPP TS 23.273 [19] clause 5.12, this IE shall be included and set to true; otherwise, the IE shall be absent. |  |  |  |
| intermediateLocationInd |  | boolean |  | C |  | 0..1 |  | This IE shall be included by the AMF if received from the GMLC, during a 5GC-MT-LR multiple location procedure for the regulatory location service (see clause 6.1.3 and clause 6.10.4 of 3GPP TS 23.273 [19]). When present, this IE shall indicate the acceptance of intermediate location response at the GMLC: - true: intermediate location response acceptable - false (default): intermediate location response not acceptable |  |  |  |
| maxRespTime |  | DurationSec |  | C |  | 0..1 |  | This IE shall be included by the AMF if received from the GMLC. When present, this IE shall contain the maximum response time for the GMLC to receive the final location response. The AMF may overwrite the received maximum response time when passing it to the LMF, e.g., to avoid timeout of the HTTP service request. |  |  |  |
| lpHapType |  | LpHapType |  | C |  | 0..1 |  | This IE shall be included and set to "LOW_POW_HIGH_ACCU_POS" to request low power and high accuracy positioning, as specified in clause 6.1.2 of 3GPP TS 23.273 [19]. |  |  |  |
| ueUpPosCaps |  | array(UeUpPositioningCapabilities) |  | O |  | 1..N |  | When present, this IE shall indicate the user plane positioning capabilities supported by the UE. |  |  |  |
| reportingInd |  | ReportingInd |  | O |  | 0..1 |  | This IE may be present if the evtRptAllowedAreas IE is present. When present, this IE shall indicate whether the UE is allowed to generate and send the reports inside or outside the event report allowed areas: - Inside reporting (default) - Outside reporting (see 3GPP TS 23.273 [19] clause 5.14 and 6.3.1) |  |  |  |
| mbsrInfo |  | MbsrInfo |  | O |  | 0..1 |  | Indicates that serving cell of the UE belongs to a MBSR |  | MBSR |  |
| additionalUeInfo |  | AdditionalUeInfo |  | O |  | 0..1 |  | This IE may be present if the mbsrInfo IE is present. It shall indicate the ULI of the MBSR UE (i.e. ULI of IAB UE). See clause 5.35A.6 of 3GPP TS 23.501 [2]. |  | MBSR |  |
| integrityRequirements |  | IntegrityRequirements |  | O |  | 0..1 |  | When present, this IE shall indicate the integrity requirements. |  |  |  |
| requestedRangingSlResult |  | array(RangingSlResult) |  | O |  | 1..N |  | This IE shall contain the type of result requested for ranging and sidelink positioning, such as absolute locations, relative locations or distances and directions related to the UEs, etc. |  |  |  |
| relatedUes |  | array(RelatedUe) |  | O |  | 1..N |  | This IE contains a list of the information for the related UEs for the ranging and sidelink positioning. |  |  |  |
| upLocRepAddrAf |  | UpLocRepAddrAfRm |  | O |  | 0..1 |  | This IE shall be present if the request is for the location reporting over user plane. |  |  |  |
| upCumEvtRptCriteria |  | UpCumEvtRptCriteria |  | O |  | 0..1 |  | This IE may be present if the upLocRepAddrAf is present. |  |  |  |
| mappedQoSEps |  | MappedLocationQoSEps |  | C |  | 0..1 |  | This IE shall be present if the Multiple QoS Class is indicated in the locationQoS IE. When present, this IE shall indicate the mapped Location QoS applicable to EPS ("BEST_EFFORT" or "ASSURED") based on the Multiple Location QoS (see clause 6.19 of 3GPP TS 23.273 [19]). |  |  |  |
| coordinateID |  | integer |  | O |  | 0..1 |  | This IE may be present when requestedRangingSlResult indicates "ABSOLUTE_LOCATION". When present, this IE represents a local coordinate (see clause 6.20.3 of 3GPP TS 23.273 [19]). |  |  | Ranging_SL |  |
| rangingSlCapability |  | RangingSlCapability |  | O |  | 0..1 |  | When present, this IE shall indicate that the UE supports Ranging/Sidelink Positioning Capability. |  |  | Ranging_SL |  |
| NOTE 1: At least one of the attributes defined in this table shall be present in the InputData structure. NOTE 2: Attribute "ecgi" and "ncgi" shall not be present at the same time. NOTE 3: Attribute "ecgiOnSecondNode" and "ncgiOnSecondNode" shall not be present at the same time. NOTE 4: Attribute "ecgiOnSecondNode" or "ncgiOnSecondNode" shall not be present if neither attribute "ecgi" nor "ncgi" is present. NOTE 5: If 3 LPP messages are received, then first LPP message shall be encoded in lppMessage IE and additional 2 LPP messages shall be encoded in lppMessageExt IE. NOTE 6: Based on UE user plane positioning capabilities, if the target UE supports the UE user plane positioning capability for LCS-UPP, the AMF shall provide the SUPI and optionally the GPSI of the UE, as defined in 3GPP TS 23.273 [19]. |  |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.52	Type: UpConfig

Table 6.1.6.2.52-1: Definition of type UpConfig

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upNotifyCallBackUri | Uri | M | 1 | Callback URI of the NF Service Consumer |
| notifCorrelationId | CorrelationID | M | 1 | Notification correlation ID. |
| supi | Supi | C | 0..1 | SUPI (NOTE) |
| gpsi | Gpsi | C | 0..1 | GPSI (NOTE) |
| amfReallocationInd | boolean | O | 0..1 | Indicates AMF reallocation indication. When present, it shall be set as follows: - true: AMF reallocated. - false (default): AMF not reallocated. |
| lcsUpConnectionInd | LcsUpConnectionInd | O | 0..1 | LCS-UP connection indication |
| targetLMFId | LMFIdentification | O | 0..1 | Target LMF identifier. |
| NOTE: The supi IE shall be provided and the gpsi IE may be provided. |  |  |  |  |

* * * End of Changes * * * *