3GPP TSG-CT WG4 Meeting #131	C4-254260

# Sophia Antipolis, France; 13th – 17th October 2025									   was C4-254087

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.003 | CR | 0726 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Filtering Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Lenovo, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Errors and misleading text exist in the definition of Filtering Information. It requires correction for the following: a) a maximum length of 256 bits is indicating the Filtering Information, while it seems to indicate the length of Filtering Element; b) the Filtering Information contains a field of Counting of Filtering Element and at least one Filtering Element. At most one Filtering Element shall be present for the ID Type, PLMN ID, NID and Third Party Identification; c) the Identification Information is one of the filtering elements, while it is missing from the description text. And whether it allows multiple instances of filtering element for Identification Information should also be addressed; d) the Count of Filtering Element is a component of the Filtering Information, while it seems to be a component of the Filtering Element from the description; e) it is quite confusing of "there should be one Identification Information Filter within a Filtering Element" in bullet 6), and should be clarified; f) terminology is not aligned, such as: "filtering element" should be "Filtering Element", "Filtering information for PLMN ID" should be "Filtering Element for PLMN", etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the errors and misleading text in the definition of Filtering Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors and misleading text exist in the definition of Filtering Information which results in difficulty for readers to understand. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 31.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1: - Further refine the description for: a) inclusion restrictions of Filtering Element for each component, b) description of Filtering Element for each component. - Merge the changes from Lenovo C4-254220 on clairfying the multiple instances of Filtering Element for Identification Information. - Refine the description and correct typos. - Add Lenovo, Huawei as co-source/supporting company. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 31.3	Structure of Filtering Information

The Filtering Information is used to filter multiple AIoT Devices, and shall have a maximum length of 256 bits. It includes a list of filtering elements as specified in clause 5.8 of 3GPP TS 23.369 [152], with the structure shown in Figure 31.3.1.

Figure 31.3.1: Structure of Filtering Information

The Filtering Information contains a field of Count of Filtering Element and includes at least one Filtering Element. The Filtering Information:

- shall include at most one Filtering Element for each of the following component of AIoT Device Permanent Identifier: ID Type, PLMN ID, NID and Third Party Identifier.

- may include multiple Filtering Elements for the following component of AIoT Device Permanent Identifier: Identification Information.

A Filtering Element within a Filtering Information shall be coded as the binary representation of its value which should be matched when performing filtering. If more than one Filtering Element is included within a Filtering Information, all Filtering Elements shall be matched.

The Filtering Information is composed of the following fields:

1)	The Count of Filtering Element field shall have a fixed length of 4 bits to indicate the number of Filtering Element which included in a Filtering Information

2)	The Filtering Element field is composed of the following parts, with structure shown in Figure 31.3.2:

Figure 31.3.2: Structure of Filtering Element

1)	Filtering Type indicates which component of the AIoT Device Permanent Identifier is used to match the bitstring value of the Filtering Element, with the following mapping:

-	000	indicate the Filtering Type for the Filtering Element is ID Type;

-	001	indicate the Filtering Type for the Filtering Element is PLMN ID;

-	010	indicate the Filtering Type for the Filtering Element is NID;

-	011	indicate the Filtering Type for the Filtering Element is Third Party Identifier;

-	100	indicate the Filtering Type for the Filtering Element is Identification Information;

-	101	reserved;

-	110	reserved;

-	111	reserved.

2)	Filtering Element for ID Type: This field has a variant length of 7 to 14 bits, and is present only if the Filtering Type is set to 000. The structure is show in Figure 31.3.3.

Figure 31.3.3: Structure of Filtering Element for ID Type

-	Offset consisting of 3 bits. It represents the position of the ID Type's bitstring to be compared. Value 0 means the starting position is 1, and value 7 means the starting position is 8.

-	Length consisting of 3 bits. It represents the length of bitstring value of the component to be compared.

-	Component Value consisting of 1 - 8 bits. It represents the bitstring value of the component to be compared.

NOTE:	It is suggested that a Filtering Information does not only include ID Type Filter Element as Filter Information.

3)	Filtering Element for PLMN ID: This field shall have a fixed length of 24 bits, and is present only if the Filtering Type is set to 001.

4)	Filtering Element for NID: This filed shall have a fixed length of 44 bits, and is present only if the Filtering Type is set to 010.

5)	Filtering Element for Third Party Identifier: This field shall have a fixed length of 24 bits, and is present only if the Filtering Type is set to 011.

6)	Filtering Element for Identification Information: This field has a variant length of 15 to 142 bits, and is present only if the Filtering Type is set to 100.

It is used to filter the target devices where the specified portion of Identification Information matches the given bitstring. The structure is show in Figure 31.3.4.

Figure 31.3.4: Structure of Filtering Element for Identification Information

-	Offset consisting of 7 bits. It represents the position of the Identification Information's bitstring to be compared. Value 0 means the starting position is 1, and value 127 means the starting position is 128.

-	Length consisting of 7 bits. It represents the length of the bitstring value of the component to be compared.

-	Component Value consisting of 1 - 128 bits. It represents the bitstring value of the component to be compared.

* * * End of Change * * * *