3GPP TSG-CT WG4 Meeting #131	C4-254303

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.564 | CR | 0151 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the notification with the EventType of SUBSCRIPTION_TERMINATION |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The EventType of SUBSCRIPTION_TERMINATION is only included in the notification request rather than in a subscription request. The presence condition is specified in 6.1.6.3.3: NOTE: This event type shall be used in event report from UPF to the NF service consumer to report the subscription is terminated when the subTerminationReportInd is included in the subscription, it shall not be used in event subscription. In last CT4 meeting, a note was added to clarify the notification with this event in clause 5.2.3.3.1. NOTE 1: The Notification request message does not indicate that an event report is sent due to the remainingDataReports set in the subscription. It means when the EventType included in one NotificationItem is set to SUBSCRIPTION_TERMINATION, the NotificationData may include one or more NotificationItems indicating the related subscribed event(s) that are requested to be terminated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note to clarify the case of the EventType setting to SUBSCRIPTION_TERMINATION included in notification request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EventType setting to SUBSCRIPTION_TERMINATION may be used incorrectly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2, 6.1.6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not introduce any change to the OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: 1. Update the added note in 1st change. 2. Corrections in the coverpage |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.1.6.2.2	Type: NotificationData

Table 6.1.6.2.2-1: Definition of type NotificationData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notificationItems | array(NotificationItem) | M | 1..N | List of NotificationItem, whereby each entry shall correspond to a report for one subscribed event per PDU session. (NOTE X) |  |
| correlationId | string | C | 0..1 | The UPF shall include this attribute in the notification if the "Notification Correlation ID" IE was received via N4 interface (see clause 7.5.2.9 of 3GPP TS 29.244 [15]) or if the notifyCorrelationId IE was received in the Nupf_EventExposure Subscribe request. When present, it shall be set to the notification correlation ID received via the N4 or Nupf interface. |  |
| achievedSampRatio | SamplingRatio | O | 0..1 | This IE may be included for an event subscription for any UE to indicate the ratio of the random subset of target PDU sessions achieved by the UPF. |  |
| eventNotifyUri | Uri | C | 0..1 | This IE shall be present if the Nupf_EventExposure_Notify request message is sent to the URI received in the bundledEventNotifyUri IE. When present, it shall be set to the URI received in the eventNotifyUri IE of the subscription for which the notification data is sent. (NOTE 1) | BERMS |
| NOTE 1: This IE may be used by the receiver of the notification message to correlate the NotificationData with the related events subscription, e.g. if the NF service consumer provides distinct eventNotifyUri for each subscription and if it does not use the correlationId for correlating the notification data with the subscription. NOTE X: When this IE includes a NotificationItem with the eventType setting to "SUBSCRIPTION_TERMINATION", one or more NotificationItems with the eventType related subscribed event(s) if remaing data related with the subscribed event(s) has been collected but not yet sent to the consumer for subscribed events requesting to send the remaining data for the related events to the NF service consumer and/or based on local configuration if subscribed. |  |  |  |  |  |

* * * Next Change * * * *

6.1.6.3.3	Enumeration: EventType

The enumeration EventType represents the type of event to which the NF service consumer may subscribe to and for which the notification is generated. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration EventType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "QOS_MONITORING" | QoS Monitoring Measurement (see clause 5.2.1.3.2) |  |
| "USER_DATA_USAGE_MEASURES" | User Data Usage Measures (see clause 5.2.1.3.3) |  |
| "USER_DATA_USAGE_TRENDS" | User Data Usage Trends (see clause 5.2.1.3.4) |  |
| "TSC_MNGT_INFO" | TSC Management Information |  |
| "UE_NAT_MAPPING_INFO" | NATed UE Public IP Address and Port number Information (see clause 5.2.1.3.6) |  |
| "HANDLING_OF_PAYLOAD_HEADERS_INFO" | Handling of Payload Headers Information (see clause 5.2.1.3.7) |  |
| "SUBSCRIPTION_TERMINATION" | This event type is used by the UPF to inform the NF service consumer that the subscription is terminated at the UPF. (NOTE) |  |
| NOTE: This event type shall be used in event report from UPF to the NF service consumer to report that the subscription is terminated when the subTerminationReportInd is included in the subscription. It shall not be used in event subscription. |  |  |

* * * End of Change * * * *