3GPP TSG RAN meeting #109									           RP-251934

Beijing, China, September 15-18th, 2025							rev of RP-xxxx

## Status Report to TSG

Title:			Status report for SID: Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation; rapporteur: Thales

Agenda item:			9.2.2

| WI / SI Name | Rel-20 Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_NTN_GNSS_resilient |  |  |  |  |
| Unique ID | 1080071 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-251863 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2026 | Core part: N/A | Performance part: N/A | Testing part: |  |
| Overall Completion level | Study Item: Overall: 10% RAN1: 15% RAN4: 0% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Mohamed El Jaafari |
|  | Company | Thales |
|  | Email | Mohamed.el-jaafari@thalesaleniaspace.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

In excel sheet attached, TU changes as follow:

- RAN2 TUs are removed (No RAN2 activities planned in the SID)

- RAN4 TUs start at RAN4#118 (in line with RAN4 chair guidance)

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Note that the agreed work plan can be found in R1-2505361 Work plan for NR NTN Phase 4  THALES

#### 2.1.1.1 Decisions during RAN1#122

Agreement

For the study on GNSS resilient operation, scenarios should be considered where:

- Scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link.

- Scenario 2: There is a previously acquired GNSS based position

  - UE has not received GNSS information for time duration T from the last acquired GNSS position and hence, the GNSS accuracy is degraded

    - FFS: value of T

    - FFS: GNSS based UE location validity duration

    - FFS: GNSS accuracy

- Note: Two scenarios above belong to description as in SID.

Agreement

For the study on GNSS resilient operation, the following scenarios are considered for evaluation purposes:

| Satellite orbit | GSO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude | 35786 km | 1200 km | 600 km |
| Satellite scenario parameters | At least Table 6.1.1.1-1 in TR 38.821, Parameters in Table 6.1.1.1-2 in TR 38.821could be considered. |  |  |
| Beam size (note 3) | At least values captured in Table 6.1.1.1-1/2 are considered, |  |  |
| Minimum elevation angle | 30° (LEO), 12.5° (GSO) |  |  |
| Frequency ranges/bands | FR1 NTN (2GHz/Ku: 14 GHz) and FR2 NTN (30 GHz) |  |  |
| UE type | Handheld (L/S band), VSAT (Ku/Ka) |  |  |
| UE speed (note 1) | L/S band: 3 km/h, 120km/h, 1500 km/h, Ku/Ka: 3 km/h , 120 km/h, 1500 km/h, (note 2) |  |  |
| Beam/cell type (note 4) | (Quasi)-Earth-fixed beams/cells and Earth-moving |  |  |
| Note 1: UE altitude is considered Note 2: 10 km altitude in case of aircraft scenario. 1500 km/h is for aircraft scenario. Note 3: Same values can be reused for other elevation angle than nadir. Beam size of the edge beam can be reported by companies. Note 4: Footprint of the beam could be: case 1: within the orbital plane, case 2: at 90° with respect to orbital plane. Companies to report the elevation angle of the beam. Note 5: Other parameters can be reported by companies. |  |  |  |

Note: The cyan highlighted parameters are prioritized for evaluation purposes.

Agreement:

For PRACH performance evaluation for existing PRACH formats, adopt the following methods:

- Baseline: Analytical characterization of performance based on e.g. properties of ZC sequences and values of differential Delay / Doppler.

- Optional: Link level evaluations

  - The following table is proposed for LLS parameters

| Configurations | S/L-band | Ka/Ku-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | For Ka: UL 30GHz, for Ku: UL 14 GHz |
| Channel Model | Baseline NTN TDL-C rural model in TR38.811 |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx | 1 Rx |
| Antenna Configuration at the UE | A single omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| PRACH format | To be reported by companies. | To be reported by companies |
| PRACH configuration | To be reported by companies. |  |
| Metric | PRACH detection rate, FAR (Based on the preamble pool size is not less than 64), CDF of estimation error for frequency/timing |  |
| Receiver | Companies to report the receiver for PRACH detection. |  |

Agreement:

For evaluation regarding GNSS-resilient operation, based on GNSS information or the lack of it, it is assumed that the UE has a horizontal location uncertainty within an area

- Case a: the location uncertainty area is the area served by the cell or beam.

- Case b: the location uncertainty area is a circle of radius X km

  - X to be reported

UE altitude error should be taken into account.

- UE altitude error to be reported.

For both Case a and Case b above, RAN1 to evaluate at least:

- Differential delay values

- Differential frequency offset/Doppler values

#### 2.1.2	Remaining Open issues

Assess impact on initial access and connected mode procedures for NR-NTN.

## 2.2	RAN4

#### 2.2.1	Agreements

None

#### 2.2.1.1 Decisions during RAN4#116

None

#### 2.2.2	Remaining Open issues

Assess potential impact on RRM specifications related to initial access and connected mode procedures for NR-NTN.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention. 
-

## 4.	References

## 4.1	RAN1

RAN1#122 meeting, Bengaluru, India, August 25-29th, 2025:

- R1-2505124 Discussion on NR-NTN GNSS resilience	SageRAN

- R1-2505142 On GNSS-resilient operation for NR-NTN	Ericsson

- R1-2505153 Discussion on Rel-20 GNSS resilient NR NTN operation	FUTUREWEI

- R1-2505169 Discussion on NR NTN GNSS resilience	Spreadtrum, UNISOC

- R1-2505223 GNSS resilience for NR-NTN	Huawei, HiSilicon

- R1-2505307 Discussion on NR-NTN GNSS resilience	CATT

- R1-2505359 Considerations on NR-NTN Resilience to GNSS Unavailability and Degradation	THALES

- R1-2505413 Discussion on NR-NTN GNSS resilience	vivo

- R1-2505460 Discussion on NR-NTN GNSS resilience	Xiaomi

- R1-2505508 Discussion on NR-NTN GNSS resilience	ZTE Corporation, Sanechips

- R1-2505581 Discussion on NR-NTN GNSS resilience	Samsung

- R1-2505590 Discussion on NR-NTN GNSS resilience	Fraunhofer IIS, Fraunhofer HHI

- R1-2505632 GNSS Resilient NR-NTN Operation in 5G Adv	Tejas Network Limited

- R1-2505672 Discussion on NR-NTN GNSS resilience	Ofinno

- R1-2505695 Random access for NR NTN GNSS resilient operation	Sharp

- R1-2505754 Discussion on NR-NTN GNSS resilience	OPPO

- R1-2505779 Discussion on NR-NTN GNSS resilience	China Telecom

- R1-2505859 Considerations for the study of NR-NTN GNSS resilience	Eutelsat Group

- R1-2505910 Considerations of NR-NTN GNSS Resilient Operations	Apple

- R1-2505937 Discussion on NR-NTN GNSS resilience	NEC

- R1-2505940 Discussion on the GNSS resilient NR-NTN operation	TCL

- R1-2505949 Discussion on GNSS resilient NR-NTN operation	LG Electronics

- R1-2505977 Discussion on GNSS resilient NR-NTN	Panasonic

- R1-2506001 Discussion on NR-NTN GNSS resilience	HONOR

- R1-2506040 GNSS resilient operations in NR NTN	MediaTek Inc.

- R1-2506048 Initial views on GNSS resilient operation for NR over NTN	Nokia

- R1-2506062 Discussion on NR-NTN GNSS resilient operations	ETRI

- R1-2506094 Discussion on NR-NTN GNSS resilience	CMCC

- R1-2506115 Discussion on GNSS resilient NR-NTN operation	Sony

- R1-2506132 Discussion on NR-NTN GNSS Resilient Operations	Lenovo

- R1-2506154 Considerations for GNSS resilient NR-NTN operation	ST Engineering iDirect

- R1-2506155 NR-NTN GNSS resilience	InterDigital, Inc.

- R1-2506165 Discussion on GNSS resilient NR-NTN operation	TOYOTA Info Technology Center

- R1-2506215 NR-NTN GNSS resilience	Qualcomm Incorporated

- R1-2506264 Discussion of NR-NTN GNSS Resilience	Johns Hopkins University APL

- R1-2506302 Study on NR-NTN GNSS resilience	NTT DOCOMO, INC.

- R1-2506334 Discussion on GNSS resilience for NR-NTN	CSCN

- R1-2506343 Discussion on NR-NTN GNSS resilience	Google

- R1-2506357 Discussion on GNSS Resilient Operation for NR NTN	CEWiT

- R1-2506385 Discussion on NR-NTN GNSS resilience	ViaSat Satellite Holdings Ltd, Inmarsat, Terrestar, Ligado, Space42, Thuraya

## 4.2	RAN4

RAN4#116 meeting, Bengaluru, India, August 25-29th, 2025: None

END