3GPP TSG RAN meeting #109		      RP-252004

Beijing, China, 15th Sep – 18th Sep 2025

Source:	vivo

Title:	Discussion on 6G requirements for migration

Agenda Item:	8.2.1.3

Document for:	Discussion and Decision

- Introduction

In RAN#108, Migration from 5G NR to 6GR as well as interworking and mobility has been agreed as one objective in 6G radio study [1]. And, it was noted that some of the migration solutions for study should be triggered by RAN plenary. In this contribution, we will discuss the necessity of studying migration objective (b).

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4]
NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study.

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

- Discussion

  - General Issues in 6GR migration from 5G NR

5G-6G Multi-RAT Spectrum Sharing (MRSS) is considered as effective way to smoothly upgrade 5G RAN to 6G and provide better 6G coverage with low band. It is foreseen that 6GR rollout will include lower band with MRSS as well as new mid-band spectrum in order to provide better peak data rate and support 6G specific services. However, even MRSS rollout cannot be done in one day. As a result, due to limited 6G coverage, service interruption and degradation in user experience is inevitable in early 6GR deployment.

Inter-RAT handover or cell re-selection will be used as the baseline migration solution. In RAN#108, the issue was raised that if any additional migration mechanism needed to be studied, especially for services with low interruption requirement, e.g., voice or immersive service.

In our view, support of voice service is critical for 6GR. It requires minimum interruption in mobility case, which requires fast switch between 5G and 6G networks. Besides, effective fallback solution to provide sufficient coverage and stable user experience may also be needed since it is not clear so far whether 6G native voice would be lunched commercially in day-one. Furthermore, in SA1 requirement TR22.870 [2], some new 6G data services also require low interruption time, e.g., advanced immersive service with computing, and cloud gaming etc.

In the following section, we will analyse potential performance gap in different scenarios.

  - Voice Service Support in 6G

There are two potential ways to deploy voice service in 6G day-one.

- Case 1: 6G native voice deployed for day-one over 6G radio, and if UE moves out of 6G coverage, inter-RAT voice handover is required for service continuity.

- Case 2: No native 6G voice deployed for day-one, voice fallback to 5G is enabled.

For the first case, UE will initiate voice service directly in 6G when UE camps or connects with 6GS. When UE moves out of the 6G coverage due to its mobility, an inter-RAT handover with voice transmission would be triggered. Such interworking solution may require N26-like interface deployed, otherwise, the interruption time will be unacceptable. In 6G early deployment, due to limited 6G coverage, frequent inter-CN node handover would be involved, which would cause long interruption time and lots of inter-CN node signaling compared to intra-CN node handover.

Figure 1: an example procedure of IRAT handover from 6G to 5G

Another issue to be considered is the call setup rate. For voice support in 5G deployment, according to our experience, it could take long time, even more than one year, for VoNR to mature. According to our survey, some statistics on successful call setup rate with NR SA early deployment is illustrated in figure 2. Even in the areas where NR SA had relatively good coverage, we can see the successful call setup rate is still lower than LTE for most of the tested UE models.

Figure 2: successful call setup rate for LTE and NR SA for Mobile-Originated and Mobile-Terminated cases

For the second case, similar to 5G voice rollout, there might be no native 6G voice deployed from day-one, voice fallback to 5G is enabled through IRAT handover or redirection.

When a UE camps on the 6G cell that need to initiate or terminate a voice call, it needs to setup the RRC connection in 6G and perform a voice fallback procedure. Figure 3 illustrates the delay in voice fallback procedure from 5G to 4G [3]. NG-RAN decides to trigger EPS Fallback or RAT Fallback taking account of the indication from AMF (i.e. Redirection for EPS fallback), gNB’s capability (e.g. NG-RAN supports handover or redirection to E-UTRAN connected to 5GC, or NG-RAN supports EPS fallback for IMS voice) and UE’s capability (e.g., UE supports IMS voice over EPS Fallback, or E-UTRAN via 5GC, or NR and so on) when it receives the request to set up QoS flow for IMS voice. The latency of the IMS voice setup from the calling perspective based on the EPS Fallback is around 2 to 4 seconds considering measurement configuration and report, handover or redirection, and whether the MO and MT call procedure can be carried out in parallel.

Figure3: illustration of IMS voice based on EPS Fallback procedure and latency

In 6G, similar procedure might be applied for 6G to 5G voice fallback if we consider two independent cores.

Furthermore, some operators may have large number of EPS deployed than 5GC in 6G initial deployment, thus the VoIMS over EPS handover or fallback to 6GS may also need to be considered. As a result, more interruption time would be involved.

When we consider on going some advanced data services (e.g. immersive or computing services), voice fallback and inter-RAT handover may further impact performance for data transmission. For example, when UE has 6G specific service on-going while need to receive a voice call, if 6G native voice is not ready and voice fallback to 5G is required, then on-going 6G specific services may be interrupted. Another example is that when UE with on-going voice services in 5G moves to 6G coverage and wants to initiate a 6G specific service, inter-RAT handover to 6G maybe triggered if 6G native voice is supported, such voice interruption also need to be considered.

From above analysis, the following observation can be drawn:

Observation 1：In 6G day-one deployment, user experience of 6G voice service may be degraded due to inter-RAT handover.

  - Further Study on 6G migration

As discussed above, if a UE can directly initialize voice service in 5G, it may solve service interruption and network signalling overhead issues, and alleviate the impacts to 6G specific services. In last RAN plenary meeting, additional migration solutions like inter-RAT dual connectivity, inter-RAT dual-stack or dual-registration were discussed.

It can be noticed that in 5G migration from 4G, inter-RAT dual connectivity is widely deployed in early rollout. The non-standalone deployment enabled operator’s faster implementation of 5G radio with legacy 4G EPC. However, from user experience perspective, it delayed UEs enjoying performance boost with more efficient real SA 5G deployment. Additionally, the upgrade of NSA 5G to SA 5G also incurs extra OPEX and complexity. As a result, the non-standalone architecture caused substantial investment to the operators, yet the features had a short lifecycle and lower cost-effectiveness. From the perspective of the UE, the UE needs to connect to both NG RAN and E-UTRAN simultaneously, which leads to higher power consumption and short standby time, thus adversely affecting the user experience. It is foreseen that dual connectivity brings huge RAN complexity and specification effort with less chance to be deployed in real network.

In conclusion, we propose the additional 6G migration study can focus on easy-deployment solution and rule out IRAT dual connectivity

For dual-stack/dual-switching solutions, several options can be considered. In order to facilitate further discussion and smooth progress of the study, we attempt to provide the following classifications, which follows the same principle as defined for ATSSS in NR [4]:

Dual-Splitting: for a single data flow, two RATs can be used to provide data transmission simultaneously, i.e. data flow splitting in NR and 6GR.

Dual-Steering: select a preferred RAT for a data flow, UE may have simultaneous data transmission in two RATs for two different services, e.g. immersive service in 6G while voice in 5G.

Dual-Switching: seamless changing the RAT for a given data flow, i.e., no simultaneous data transmission in NR and 6GR

We provide some initial analysis on the three categories in the following table.

| Traffic Handling Category | Pros | Cons |
| --- | --- | --- |
| Dual-Splitting | high peak data rate for certain traffic with full utilization of both NR and 6GR radio resources; | Deployment complexity requires CN and RAN negotiation; Specification complexity requires new UP protocol to handle data splitting and re-ordering; High UE capability requirement |
| Dual-Steering | Minimized voice interruption performance together with high User experience for 6G new data services; Limited RAN impact and easy to deploy; | High UE capability requirement; |
| Dual-Switching | Alleviate voice interruption performance and provide relative higher User experience for 6G new data services; Limited RAN impact; Limited UE capability requirement; | CN procedure impacts; |

As analysed above, user experience for voice service can be improved by simultaneously supporting voice over 5G and data service over 6G, i.e., dual steering and improving inter-RAT handover i.e. dual switching. For dual-splitting, although peak data rate can be boosted, it will cause additional complexity for the UE and the network. As a result, we propose RAN to further study Dual-Steering and Dual-Switching considering with different UE capabilities, RAN impacts and potential gain, in coordination with SA2.

Proposal 1: Only standalone architecture should be considered for 6G.

Proposal 2: Additional solutions need to be further studied to meet the interruption requirement for voice and new 6G real-time services. Solutions to be studied should be easy deployed with limited RAN complexity.

Proposal 2a: Further study Dual-Steering and Dual-Switching solution on benefits and UE/RAN impacts.

- TP for 38.914

----------------------------------------------------start of TP--------------------------------------------------------------

5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The following requirements shall be considered:

–	6G standalone architecture should be supported from day one.

–	support voice service with no interruption from 6G data service and vice versa. (e.g. 6G native voice with fast switching and simultaneously support voice over 5G and data service over 6G)

- Conclusion

In this paper, we discuss the necessity of studying migration objective (b). we have the following observations and proposals:

Observation 1：In 6G day one deployment, MRSS together with inter-RAT handover may not meet the 6G service requirement for voice service scenario.

Proposal 1: Only standalone architecture should be considered for 6G.

Proposal 2: Additional solutions need to be further studied to meet the interruption requirement for voice and new 6G real-time services. Solutions to be studied should be easy deployed with limited RAN complexity.

Proposal 2a: Further study Dual-Steering and Dual-Switching solution on benefits and UE/RAN impacts.

Proposal 3: Agree and Capture the above TP into TR 38.914.

- Reference

- RP-251881	New SID: Study on 6G Radio

- S1-252934	draft TR on Study on 6G Use Cases and Service Requirements

- R2-2205054 Early measurement for EPS Fallback	vivo, China Telecom, CMCC, SoftBank, China Unicom, Vodafone, Ericsson

- TS 23.501

- RP-251873 draft TR38.914