3GPP TSG RAN#109 Meeting 	RP-252159

Beijing, China, September 15-18th, 2025					rev from RP-250879

Title:	6G study around 7 GHz

Source: 	Thales, Airbus, SES, Hispasat

Type:	discussion

Document for:	discussion

Agenda Item:	8.2.1	Study on 6G Scenarios and Requirements [RAN led SI: FS_6G_RAN_Scen_Req]

# Introduction

The RAN led study on 6G Scenarios and requirements has been approved at RAN#108 [1]. Among its objectives, it includes

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

  - …

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - …

This document proposes some requirements on the study of the Radio access technology for 6G deployments around 7 GHz.

# Discussion

A significant amount of additional spectrum has already been identified for mobile services (in particular IMT) below 3.8 GHz and in the 6 425 – 7 125 MHz range by ITU at WRC-23.

Among the bands being considered under WRC-27 Agenda Item 1.7 for possible IMT identification, the band 7125-8400 MHz was opposed by many countries and regional groups, including CEPT, due to strategic usages. These strategic usages are, and will remain, critical in all of this frequency band.

This frequency range is intensively used by a large number of government satellite systems for down-links (i.e. 7 250 – 7 750 MHz) and up-links (i.e. 7 900 – 8400 MHz). The upper part of the band (i.e. 8 025-8 400 MHz) is also used for the downlink of Earth observation data by a large majority of Earth Observation satellites.

Observation 1: Proposals in ITU-R to introduce an IMT identification in the frequency range 7250-7750 MHz and 7900-8400 MHz have been opposed by many administrations and regional groups.

There are often no alternative bands to support the above communication and earth observation satellite systems which would be drastically impacted by the reduction of available frequency bands.

As part of the activities carried out under WRC-27 Agenda Item 1.7, submitted studies [2] show that frequency sharing between these strategic systems, and any prospective roll out of mobile networks would be not viable in practice.

Observation 2: Submitted studies conclude that frequency sharing between these strategic space assets and IMT systems will not be possible, while there will be continued need to operate existing strategic space systems in the frequency range 7250-7750 MHz and 7900-8400 MHz.

3GPP has provided the following contribution to ITU for Agenda Item 1.7 for WRC27 [3]

Observation 3: 3GPP RAN has already provided preliminary parameters to ITU for the Agenda Item 1.7 WRC 27 study.

Therefore, we suggest that the frequency range [7250 - 7750 MHz] and [7900 - 8400 MHz] be excluded from the 6G study and normative work scope, at least until WRC27 final decision.

Proposal 1: RAN shall postpone any 6G study and normative work, with regard to IMT identification, on the frequency range [7250 - 7750 MHz] and [7900 - 8400 MHz], at least until WRC27 final decision.

Proposal 2: Subject to WRC27 decision, RAN shall ensure consistency between the parameters provided by 3GPP to ITU and future work carried out in 3GPP in the frequency range [7250-7750 MHz] and [7900-8400 MHz].

# Conclusion

Observation 1: Proposals in ITU-R to introduce an IMT identification in the bands [7250-7750 MHz] and [7900-8400 MHz] have been opposed by many administrations and regional groups.

Observation 2: Submitted studies conclude that frequency sharing between these strategic space assets and IMT systems will not be possible, while there will be continued need to operate existing strategic space systems in the frequency range [7250-7750 MHz] and [7900-8400 MHz].

Observation 3: 3GPP RAN has already provided preliminary parameters to ITU for the Agenda Item 1.7 WRC 27 study.

Proposal 1: RAN shall postpone any 6G study and normative work, with regard to IMT identification, on the frequency range [7250 - 7750 MHz] and [7900 - 8400 MHz], at least until WRC27 final decision.

Proposal 2: Subject to WRC27 decision, RAN shall ensure consistency between the parameters provided by 3GPP to ITU and future work carried out in 3GPP in the frequency range [7250-7750 MHz] and [7900-8400 MHz].

# References

[1]	3GPP RP-250810 “Revised SID: Study on 6G Scenarios and requirements”, Source:	CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

[2]	Contribution 5D/548 to ITU-R WP5D “Sharing studies between FSS, MSS and MMSS (Earth-to-space), and IMT systems in the frequency band 7 900-8 400 MHz under WRC-27 agenda item 1.7”

[3]	3GPP R4-2414449 / ITU Document 5D/269-E “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”

END