3GPP TSG RAN Meeting #109			      RP-252640

Beijing, China, September 15-18, 2025

Agenda item:	9.2.2

Source:	Xiaomi

Title:	Consideration for study on GNSS resilient NR-NTN operation

Document for:	Discussion and Decision

- Introduction

In RAN1#122 meeting, RAN WG-level SI on GNSS resilient NR-NTN operation was kicked off. The following agreements were made on the study scenario and evaluation setting.

Agreement For the study on GNSS resilient operation, scenarios should be considered where: - Scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link. - Scenario 2: There is a previously acquired GNSS based position - UE has not received GNSS information for time duration T from the last acquired GNSS position and hence, the GNSS accuracy is degraded - FFS: value of T - FFS: GNSS based UE location validity duration - FFS: GNSS accuracy - Note: Two scenarios above belong to description as in SID. Agreement: For evaluation regarding GNSS-resilient operation, based on GNSS information or the lack of it, it is assumed that the UE has a horizontal location uncertainty within an area - Case a: the location uncertainty area is the area served by the cell or beam. - Case b: the location uncertainty area is a circle of radius X km - X to be reported UE altitude error should be taken into account. - UE altitude error to be reported. For both Case a and Case b above, RAN1 to evaluate at least: - Differential delay values - Differential frequency offset/Doppler values

In this paper, we elaborate some considerations on RANP guidelines deemed necessary to the ongoing RAN1 study on GNSS resilient operation.

- Discussion

During RAN1#122 meeting discussion, there are divergent interpretations for the agreement made:

Interpretation 1: There will be four study cases according to the full combination of the study scenarios and evaluation cases.

- (Combination 1) Scenario 1 – case a: The UE doesn’t rely on GNSS information at all for service link timing and frequency compensation. It suffers a maximum error of differential delay/frequency offset corresponding to cell or beam size.

- (Combination 2) Scenario 1 – case b:  The UE doesn’t rely on GNSS information at all for service link timing and frequency compensation. It suffers a maximum error of X corresponding to the positioning accuracy

- (Combination 3) Scenario 2 – case a:  The GNSS information is unavailable to UE within a time duration T. The GNSS accuracy is degraded due to either power saving purpose or other degradation causes. It suffers a maximum error of differential delay/frequency offset corresponding to cell or beam size.

- (Combination 4) Scenario 2 – case b: The GNSS information is unavailable to UE within a time duration T. the GNSS accuracy is degraded due to either power saving purpose or other degradation causes. It suffers a maximum error of differential delay/frequency offset corresponding to positioning error X.

Interpretation 2: There will be two study cases according to the most relevant combination of the study scenarios and evaluation cases.

- (Combination 1) Scenario 1 – case a: The UE doesn’t rely on GNSS information at all for service link timing and frequency compensation. It suffers a maximum error of differential delay/frequency offset corresponding to cell or beam size.

- (Combination 2) Scenario 2 – case b: The GNSS information is unavailable to UE within a time duration T. the GNSS accuracy is degraded due to either power saving purpose or other degradation causes. It suffers a maximum error of differential delay/frequency offset corresponding to positioning error X.

In the SID objective [1] part, the following highlighted sentence explicitly stated the initial focus should be on 1/(GNSS information is temporarily unavailable). Since the study is supposed to have some initial focus, and RAN1 interpretations actually deviates from the focus by involving an interpretation with more combinations than the other, it is suggested RANP provides guidance on which interpretation RAN1 needs to follow. Moreover, under an interpretation aligned with RANP guidance, RAN1 needs to focus on the actual combination(s) that should be focused on initially as per SID guidelines.

The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement periodicity for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario

Observation 1: Guidance is needed from RANP to RAN1 regarding which interpretation to take and which combination(s) to follow.

For scenario 2 under interpretation 1, our understanding is that there is no functional difference between combination 3 and combination 4. The reason is the mechanism or degradation cause for GNSS accuracy is same. The only thing combination 4 needs to consider to be inclusive of combination 3 is setting the X value as the same value of the cell/beam size. From our perspective, there is no need to further diverge the study which is supposed to focus on 1/ (where GNSS information is temporarily unavailable) initially.

Meanwhile, for combination 1 under interpretation 1, the maximum error of differential delay/frequency offset is inherently determined by cell or beam size. This is also aligned with the evaluation setting from TR 38.821 [2] assuming UE does not rely on GNSS for time and frequency pre-compensation.

For combination 2 under interpretation 1, the degradation value X can be smaller than the cell/beam size which is determined by positioning accuracy. During RAN1 discussion, a proposed viable solution to have this accuracy smaller than cell/beam size seems to be via DL-TDOA positioning relying on idle state PRS configuration. This is hardly feasible in the NTN multi-satellite setting given TS 38.215 already specifies the DL RSTD from multiple transmission points is constrained by 1 ms as extracted below. In SID, the note already specified “Note: This study does not include any enhancement on positioning for the above scenario”. To comply with SID guidance, proponents need to first account for the feasibility of fulfilling this measurement under NTN before RAN1 further discusses this topic.

### 5.1.29 DL reference signal time difference (DL RSTD) Definition; DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE.. Applicable for; RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE.

Hence, we think interpretation 2 should be the one that RAN1 shall take.

Under interpretation 2, the follow up issue is which combination corresponds to the actual use case that RAN1 needs to focus on initially. From our perspective, although the SID already clearly states that the degradation cases are not the items the initial study needs to focus on, it’s also OK to include both cases since both cases can be deemed as GNSS unavailable.

Summing up, we propose the following proposal.

Proposal 1: RANP tasks RAN1 to strive to study and evaluate Differential delay/Doppler offset towards both combination 1 (Scenario 1 – case a) and combination 2 (Scenario 2- case b) under interpretation 2.

- Conclusions

In this contribution, we provide the following observations and proposals.

Observation 1: Guidance is needed from RANP to RAN1 regarding which interpretation to take and which combination(s) to follow given a RANP confirmed interpretation.

Proposal 1: RANP tasks RAN1 to strive to study and evaluate Differential delay/Doppler offset towards both combination 1 (Scenario 1 – case a) and combination 2 (Scenario 2- case b) under interpretation 2.

- References

[1] RP-251863, Study on GNSS resilient NR-NTN operation, Thales, RAN#108

[2] TR 38.821 v16.2.0, Solutions for NR to support non-terrestrial networks (NTN), Rel-16