3GPP TSG RAN Meeting #110	                                                            RP-253179

Baltimore, US, December 08-11, 2025

Agenda Item:	8.2.1.3.2

Source:	Spreadtrum, UNISOC

Title:	Views on 6G diverse device types

Document for:	Discussion and decision

Introduction

6G RAN study item was approved in RAN#107 [1].

In this contribution, we provide our views on 6G diverse device types based on “Target scalable and forward compatible design for diverse device types”.

Discussion

Lesson/Experience from 5G NR

Firstly, in 5G NR day 1, only one UE type/category was introduced, and other new UE types (e.g. RedCap, 2RX XR, etc.) with urgent market demand were introduced at later releases. From the perspective of network deployment and complexity reduction, 5G NR cannot provide a good eco-environment for those UE types introduced at later releases.

Secondly, flexible UE capability report mechanism was adopted in 5G NR to support various features/use cases. However, it would not only cause some hinder to implement new feature at earlier stage but also complex signaling and IODT test to UE side.

In short, 5G NR intends to avoid market fragmentation issue caused by LTE UE category approach and achieve flexible function configurability, unfortunately, it is overcorrection.

Views on 6G diverse device types

In RAN#109, the following proposals was endorsed only for RANP.

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement.

Proposal 4: In terms of diverse device types, at least form factor limitations is considered. Sstudy further: - The parameters related to form factor limitations, e.g., the number of Tx antennas/chains, Rx antennas/chains, power classes, etc. - Other Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI - Note: some of the above parameters/factors may be related with form factor - Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

In RAN1#123, the following agreement on scalable 6GR design had been achieved.

Agreement - For scalable 6GR design for diverse device types, RAN1 can at least the following, targeting applicable to all 6G device types, - Basic initial access procedures from RAN1 perspective - Other PHY features after initial access procedure, e.g., Other DL/UL control, scheduling/HARQ - Coverage features to meet the identified coverage target - Energy saving both at BS and UE sides - MRSS - Note: Whether these features are supported, mandatory or optional is separate discussion

In our view, the basic design principle for 6GR device type should be as follows:

- A limited set of device types to avoid market fragmentation in day 1

- Device type should be categorized based on main communication usage scenarios (e.g eMBB, Massive IoT) together with some device hardware limitation (e.g. form factor)

- Scalable and forward-compatible design

- A common basic mandatory function set is defined for all device types.

- On top of it, a different additional mandatory capability set is defined for each device type.

- Further, each device type may additionally indicate optional capability/feature sets for different new usage scenarios or new services.

Regarding the communication capabilities, at least BW, peak data rate, and number of antenna should be considered. It is worth noting that, the number of communication capabilities should be limited to avoid introducing too many device types.

Based on the basic design principle for 6GR device type we proposed above, we have the following two proposals.

Proposal 1: The following Table 1could be considered as the starting point to achieve scalable and forward-compatible design.

Table 1: 6GR device types example

| Usage scenarios | Example device type | Scalable and forward-compatible design avoid market fragmentation |  |  |  |  | Example devices/use cases |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A common basic mandatory function set for all device types | A different additional mandatory capability set for each device type | Optional capability/feature sets for different new usage scenarios or new services. |  |  |  |
| eMBB | Type A | - Basic initial access procedures - Other PHY features after initial access procedure - Coverage features to meet the identified coverage target - Energy saving both at BS and UE sides - MRSS - etc. | - 200M - 2T/4R - Modulation: 256 QAM DL/UL - Power class: 3 | Optional set A1 - #MIMO layers - Processing time - Spectrum aggregation capabilities - Max output power, etc. | Set A2 … | … | Smart Phone, CPE/FWA, VUE, VSAT, Robots, … |
| Massive IoT | Type B |  | - 20M - 1T1R - Modulation: 64 QAM DL/UL - Power class: 3 or 5 | Optional set B1 - Processing time - Max output power - Access control - Scaling factor, etc. | Set B2 … | … | Wearables, Industrial sensors, video surveillance … |

Proposal 2: To avoid market fragmentation and to achieve economic scale, only two device types should be defined for 6GR: eMBB device, IoT device.

- A common basic mandatory function set is defined for both 6GR eMBB device and 6GR IoT device.

- Additional mandatory capability set should be defined respectively for 6GR eMBB device and 6GR IoT device based on their most potentially popular in implementation and marketing.

- Different optional capability/feature sets should be defined to meet requirement of different new usage scenarios or new services.

Conclusion

In this contribution, we provide our views on 6G diverse device types with the following proposals:

Proposal 1: The following Table 1could be considered as the starting point to achieve scalable and forward-compatible design.

Table 1: 6GR device types example

| Usage scenarios | Example device type | Scalable and forward-compatible design avoid market fragmentation |  |  |  |  | Example devices/use cases |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A common basic mandatory function set for all device types | A different additional mandatory capability set for each device type | Optional capability/feature sets for different new usage scenarios or new services. |  |  |  |
| eMBB | Type A | - Basic initial access procedures - Other PHY features after initial access procedure - Coverage features to meet the identified coverage target - Energy saving both at BS and UE sides - MRSS - etc. | - 200M - 2T/4R - Modulation: 256 QAM DL/UL - Power class: 3 | Optional set A1 - #MIMO layers - Processing time - Spectrum aggregation capabilities - Max output power, etc. | Set A2 … | … | Smart Phone, CPE/FWA, VUE, VSAT, Robots, … |
| Massive IoT | Type B |  | - 20M - 1T1R - Modulation: 64 QAM DL/UL - Power class: 3 or 5 | Optional set B1 - Processing time - Max output power - Access control - Scaling factor, etc. | Set B2 … | … | Wearables, Industrial sensors, video surveillance … |

Proposal 2: To avoid market fragmentation and to achieve economic scale, only two device types should be defined for 6GR: eMBB device, IoT device.

- A common basic mandatory function set is defined for both 6GR eMBB device and 6GR IoT device.

- Additional mandatory capability set should be defined respectively for 6GR eMBB device and 6GR IoT device based on their most potentially popular in implementation and marketing.

- Different optional capability/feature sets should be defined to meet requirement of different new usage scenarios or new services.

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.