# 3GPP TSG-RAN Meeting #110	RP-253236

# Baltimore, US, 8-11 Dec, 2025

Agenda Item:	8.2.1.3.1

Source: 	Xiaomi

Title: 	View on 6GR spectrum

Document for:	Discussion

- Introduction

According to the 6G SID, 6GR target frequency includes FR1, FR2-1 and frequency range between 7GHz ~ 24GHz:

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

Compared to 5G system, the brand new clean spectrum for 6GR day 1 deployment is rare. Following potentail spectrum sharing/co-existence scenarios are observed:

- Spectrum sharing/co-existence with legacy system (4G and 5G) on existing reframing frequency in FR1 and FR2-1.

In this contribution, our preliminary view on 6G spectrum is provided on following areas:

- Spectrum related regulation

- Band and CA simplification

- Flexcible spectrum intergration

- Discussion

  - Spectrum Regulation

Agenda Item 1.7 for the 2027 World Radiocommunication Conference (WRC-27) focuses on studying the potential identification of additional frequency bands for International Mobile Telecommunications (IMT). This includes current and future generations of mobile networks such as 5G, 6G, and IMT-2030.

Frequency Bands Under Study:

- 4 400–4 800 MHz

- 7 125–8 400 MHz

- 14.8–15.35 GHz

These bands are being considered to support the growing demand for mobile broadband, especially for high-capacity, low-latency applications expected in 6G and beyond.

Key Technical and Regulatory Considerations:

- Spectrum requirements for future IMT systems

- Compatibility and sharing studies with existing services (e.g., satellite, fixed, and scientific services)

- Feasibility of regulatory measures to ensure coexistence

- Global or regional harmonization to support economies of scale and cross-border coordination

As mobile data traffic continues to surge, new mid-band spectrum is essential to balance coverage and capacity. AI 1.7 is critical for preparing the regulatory framework for 6G, ensuring that sufficient spectrum is available to meet future connectivity demands.

Observation 1: The ITU-R is working on searching and identification new spectrum for IMT 6G, the potential frequency bands are 4 400–4 800 MHz, 7 125–8 400 MHz and 14.8–15.35 GHz.

In WRC-27 agenda item 1.13,  study possible primary allocations to the Mobile-Satellite Service (MSS) in the 694 MHz – 2.7 GHz range so that MSS systems—especially Direct-Connectivity MSS (DC-MSS) that link ordinary handsets to NGSO or GSO satellites—can operate in the same bands already harmonized for terrestrial IMT.

Key tasks & expected outcome of WRC-27 Agenda Item 1.13 can be summarized as: set interference limits that let MSS share IMT bands without harming 5G, and deliver a sharing report plus draft treaty text for WRC-27.

Scope of studies:

- Bands already identified for IMT in ITU-R M.1036: 694-960 MHz, 1 710-1 880 MHz, 1 885-2 025 MHz, 2 110-2 200 MHz, 2 300-2 400 MHz, 2 496-2 690 MHz.

- Both GSO and NGSO MSS constellations; uplink and downlink directions; voice, data, IoT and broadcast-style services.

- Worst-case terrestrial densities (urban macro, indoor hotspot, high-speed rail) and MSS beam-edge geometries to be modelled in the compatibility analysis.

Observation 2: The ITU-R is working on the in-band sharing and compatibility study for searching and identification new spectrum for DC-MSS-IMT service, the potential frequency bands including the IMT bands: 694-960 MHz, 1 710-1 880 MHz, 1 885-2 025 MHz, 2 110-2 200 MHz, 2 300-2 400 MHz, 2 496-2 690 MHz.

  - Band and band combination

Pain points in 5G for band/band combination introduction:

In 5G beginning, NR introduced both 4G migrated bands and new bands for above 3GHz and FR2. These cumulative bands brought lots of work a in initial stage and many of them was not deployed/commercialized in 5G day1.

And in 5G, NTN and TN bands were specified separately with different band numbers even the spectrum is partially overlapped or fully overlapped.

NR also developed band combinations under CA structure including intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA. And 3GPP developed a procedure with basket WI approach per operators’ request for band combination introduction.  Accumulative band combinations increase specification work, product development complexity and conformance test complexity and less than 10% combos are really deployed.

Observation 3:  Following pain points observed for NR band and band combination introduction

- In NR beginning, 3GPP introduced both 4G migrated bands and new bands for above 3GHz and FR2. These cumulative bands brought lots of work in initial stage and many of them not deployed/commercialized in 5G day1.

- Accumulative band combinations increase specification work, product development complexity and conformance test complexity (less than <10% combos really deployed).

Proposal 1: 6GR aims to efficient use existing refarming spectrum and potential new spectrum by introducing band and band combination ensuring the alignment with the commercial deployment.

  - Spectrum intergration

On 6G spectrum usage, 3GPP needs to investigate potentail solutions on fully usage of operators’ spectrum with affordable cost from UE and gNB. Several directions can be considered in 6GR intial stage on flexcible spectrum usage:

- DL/UL spectrum pairing

- Fragmented spectrum

- CA via swicthing and uplink Tx swicthing

6GR will further extend spectrum to 7GHz or even 10GHz, how to efficient use spectrum to guranteen both coverage and high data rate request. By natually UE can utlize low bands spectrum for coverage (UL) and  high bands spectrum for DL data tranmission.  Decouple DL and UL spectrum is one potential research area for flexicble spectrum integration.3GPP needs to evaluate the operation restrcition and potentail pros and cons for flexcible spectrum paring across low frequency to high frequency.

Fragemented spectrum

Operators’ spectrum maybe fragemented over same band or across different adjacent bands.

In Rel-19 RAN4 estiablished a study item to study using shared RF chain to receive multi non-contiguous carriers within same band.

The benefits of shared chain to receiver multi fragmented carriers as following:

- Save cost to receive multi-carriers with reduced RF/BB chain

- Support high order band combinations with more carriers with limited Tx/Rx chains

- Support high order of MIMO layers for same band combination

In Rel-19 fragmented carrier study, the restriction and performance impact were also anayzed with shared RF chain with two restrction realized:

- With shared RF chain, UE performance in in-gap interference rejection degraded as no sub-block analogue filter

- REFSENS degradation due Tx leakage with reduced attenuation on UL carrier frequency with wideband LPF under “single Rx chain mode”.

Based on RAN4 analyzed in Rel-19, some restriction maybe applied for single Rx mode:

- Power imbalance between wanted carriers

- In gap interference (power imbalance between wanted carrier and in-gap interference)

- Maximum frequency separation between carriers

For 6G, the fragmented carrier scenario can be further extended to inter-band cases as following:

Obseravtion 4: on fragmented spectrum:

- Operators’ spectrum maybe fragemented over same band or across adjacent bands which bring implemenation difficulty and contraints

- It’s worth to study enhanced solutions on supported fragemented carriers with affordale cost and improved spectrum usage efficiency.

UL Tx switching/ CA via switching

NR introduced UL Tx switching and CA via switching to dynamic/semi-static switching Tx/Rx chains over different bands/carriers to efficient use the available Tx/Rx chain and flexible spectrum.

For UL Tx switching, different cases and scenarios were introduced under different release including CA UL switching between 1Tx-2Tx across 2 bands, 2Tx-2Tx UL switching between 2 bands, 1/2Tx-1/2Tx switching between 3/4 bands including SUL bands. In Rel-19 RAN4 maintenance, RAN4 also discussed 3Tx switching and NUL-SUL switching with non-zero switching time.

Under Rel-19, RAN4 also discussed and introduced LB CA via switching for SDL and FDD carrier due to RF front-end limitation and the candidate bands as show in below table.

Observation 5: on UL Tx switching:

- NR developed UL Tx chain switching in several release to accommodate different cases/scenarios which is lack of harmonized scheme and solutions and make the feature fragmented over several release and difficulty for implementation.

- It’s worth to study unified solutions/schemes in 6G day1 to accommodate different scenarios/cases to simplify implementation and improve the performance

Under Rel-19, 3GPP also discussed and introduced LB CA via switching for SDL and FDD carrier due to RF front-end limitation and the candidate bands as show in below table.

Rel-19 LB ca via switching only focus on FDD-SDL carrier with switching 2 carriers and Rel-20 will support LB CA via switching for FDD-SDL carrier with dual DL.

Observation 6: on LB CA band combinations

- Due to RF front-end design challenge (antenna, filter etc.) , it’s difficult to support simultaneously Tx/Rx across closed/overlapped bands.

- It’s worth to study the restriction and optimized solution to support difficult band combinations in 6GR day e.g. LB and LB band combination.

There is commonality between UL tx switching and carrier switching in terms of switching time, switching mechanism and potential RF/RRM impact basically the ideal was to share Tx/RF chain across closed operating frequency ranges within band group to achieve MIMO or frequency diversity gain.  It’s worth to study unified framework and switching mechanism for all above mention cases and new cases which on discussion in Rel-20/Rel-19 maintenance.

Proposal 2: 6GR aims to enable flexible spectrum usage matching with realistic deployment scenarios by considering UE implementation constraints with following potential area:

- Efficient Fragmented spectrum usage

- Spectrum aggregation via switching and Uplink Tx switching with unified framework and mechanism

Proposal 3: RAN4 shall be deeply involved in day 1 for flexible spectrum usage/spectrum aggregation study with tight cooperation with RAN1/RAN2 e.g., DL/UL spectrum pairing, Fragmented carrier (single cell multi carriers), UL Tx switching/CA via switching

- Conclusion

In this contribution, our preliminary view on 6G spectrum is provided on following areas:

Spectrum related regulation

Observation 1: The ITU-R is working on searching and identification new spectrum for IMT 6G, the potential frequency bands are 4 400–4 800 MHz, 7 125–8 400 MHz and 14.8–15.35 GHz.

Observation 2: The ITU-R is working on the in-band sharing and compatibility study for searching and identification new spectrum for DC-MSS-IMT service, the potential frequency bands including the IMT bands: 694-960 MHz, 1 710-1 880 MHz, 1 885-2 025 MHz, 2 110-2 200 MHz, 2 300-2 400 MHz, 2 496-2 690 MHz.

Band and CA simplification

Observation 3:  Following pain points observed for NR band and band combination introduction

- In NR beginning, 3GPP introduced both 4G migrated bands in below 3GHz and new bands for above 3GHz and FR2. These cumulative bands brought lots of work in initial stage and many of them not deployed/commercialized in 5G day1.

- Accumulative band combinations increase specification work, product development complexity and conformance test complexity (less than <10% combos really deployed).

Proposal 1: 6GR aims to efficient use existing refarming spectrum and potential new spectrum by introducing band and band combination ensuring the alignment with commercial deployment.

Flexcible spectrum intergration

Obseravtion 4: on fragmented spectrum:

- Operators’ spectrum maybe fragemented over same band or across adjacent bands which bring implemenation difficulty and contraints

- It’s worth to study enhanced solutions on supported fragemented carriers with affordale cost and improved spectrum usage efficiency.

Observation 5: on UL Tx switching:

- NR developed UL Tx chain switching in several release to accommodate different cases/scenarios which is lack of harmonized scheme and solutions and make the feature fragmented over several release and difficulty for implementation.

- It’s worth to study unified solutions/schemes in 6G day1 to accommodate different scenarios/cases to simplify implementation and improve the performance

Observation 6: on LB CA band combinations

- Due to RF front-end design challenge (antenna, filter etc.) , it’s difficult to support simultaneously Tx/Rx across closed/overlapped bands.

- It’s worth to study the restriction and optimized solution to support difficult band combinations in 6GR day e.g. LB and LB band combination.

Proposal 2: 6GR aims to enable flexible spectrum usage matching with realistic deployment scenarios by considering UE implementation constraints with following potential area:

- Efficient Fragmented spectrum usage

- Spectrum aggregation via switching and Uplink Tx switching with unified framework and mechanism

Proposal 3: RAN4 shall be deeply involved in day 1 for flexible spectrum usage/spectrum aggregation study with tight cooperation with RAN1/RAN2 e.g., DL/UL spectrum pairing, Fragmented carrier (single cell multi carriers), UL Tx switching/CA via switching

- Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] TR 38.755, Study on NR FR1 DL fragmented carriers

[3] TR 38.768, Low NR band carrier aggregation via switching