3GPP TSG RAN Meeting #110	RP-253277

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.4

Source: 	CATT

Title: 	TP on requirements of PNT and related regulatory services

Document for:	Discussion and Decision

1. Introduction

This contribution discusses the requirements of the regulatory service and location/positioning service, and provides the corresponding TP on Requirements of these services.

2. Discussion

### 2.1 Regulatory service

According to the discussion in RAN#109, the following proposal [1] was endorsed.

Moderator proposal 1: It is agreed to capture following into TR 38.914 section 5.4: - Mobile broadband - Immersive Communication - Massive Communication (IoT) - Sensing - AI - Voice - Regulatory services

It is clear that regulatory service would be supported in 6G. In SA1 6G TR 22.870 [2], it mentions:

The 6G system shall be able to support the following services: - Mission Critical Services, i.e. MCPTT, MCData, MCVideo, ref TS 22.179 [53], TS 22.280 [54], TS 22.281 [55]. and TS 22.282 [56], - Message service, ref TS 22.262 [57], - SMS, ref TS 22.101 [58], - Multimedia communication services, ref TS 22.101 [58], TS 22.261 [14], TS 22.173 [59], - IMS Multimedia Telephony Service, ref TS 22.261 [14], TS 22.173 [59], - Roaming services, ref TS 22.011 [60], TS 22.101 [58], TS 22.261 [14], - Location and positioning services, ref TS 22.261 [14], TS 22.071 [61], - Broadcast and Multicast Services, ref TS 22.261 [14], - Emergency Services, ref TS 22.101 [58], - PWS, ref TS 22.268 [62], - MPS, ref TS 22.153 [63], - Lawful Interception, ref TS 22.261 [14] and TS 33.126 [312], and - Other regulatory services, based on regional/national regulatory requirements. The following services do not need to be supported by the 6G system: - CS related telephony services, e.g. CS Fallback, CS based voice call,

So the requirements of regulatory service should at least include these legacy services. According to TR 38.913 [3], requirements of emergency communications should be supported in 6G. Please see TP for section 5.4.7 below.

### 2.2 Location/positioning/navigation/time services

According SA1 6G requirements for services (see above highlighted in blue) [2], location and positioning service shall be supported in 6G. Similar to 5G, both RAT-dependent and RAT-independent methods should be supported.

In SA1 6G TR 22.870[2], navigation is mentioned many times. We can understand that 6G should support navigation services.

In current wireless system, time service is a critical service since both UE and network needs the time to complete transmission and receiver. BDS/GNSS can be used for 6G to get the time. But it should enable the network and UE to get the time from terrestrial time service centers when GNSS is unavailable.

It is proposed to capture requirements of location/positioning, navigation and time services in TR 38.914. Corresponding TP can be found below.

3. TP for TR38.914

<<<<<<<<<<<<<<<<<<<< Start of change >>>>>>>>>>>>>>>>>>>>

### 5.4.7	Regulatory Services

### 5.4.7.x	Emergency communications

The 6GR and 6G RAN architecture shall provide mechanisms to enable emergency calls including positioning/location for emergency calls (such as is found in 3GPP TS 22.101 [x1] section 10 for emergency calls and 3GPP TS 22.071 [x2] for position/location) when appropriate.

The 6GR and 6G RAN architecture shall provide mechanisms to enable Multimedia Priority Services (such as is found in 3GPP TS 22.153 [x3]).

<<<<<<<<<<<<<<<<<<<< The next change >>>>>>>>>>>>>>>>>>>>

### 5.4.x	Location/positioning/Navigation/Timing Services

The 6GR and 6G RAN architecture shall support the following requirements for location/positioning and navigation services:

- It should enable, and improve if suitable, state-of-art positioning techniques, such as RAT-dependent methods (Cell-ID, E-CID, UL-TDOA, UL-AoA, DL-TDOA, DL-AOD, Multi-RTT, etc.) and RAT-independent methods ((A-)GNSS, Bluetooth, WLAN, Terrestrial Beacon Systems (TBS), sensors, etc.).

- 6GR positioning shall exploit high bandwidth, massive antenna systems, network architecture/ functionalities (e.g. heterogeneous networks, broadcast) and deployment of massive number of devices. 6GR positioning shall support indoors and outdoors use cases.

The 6GR shall support regulatory positioning requirements.

The 6GR shall support the following requirements for time service:

- It should support time service and maintain time via GNSS.

- It should support provide time to the UE by network.

- It should enable the synchronization of network and UE via terrestrial time service centres when GNSS is unavailable.

<<<<<<<<<<<<<<<<<<<< End of change >>>>>>>>>>>>>>>>>>>>

4. TP for TR38.914

[1] RP-252943	Moderator's summary for RAN led 6G SI: Requirements of new and existing services	CMCC

[2] 3GPP TR 22.870 Study on 6G Use Cases and Service Requirements; Stage 1

[3] 3GPP TR 38.913 Study on Scenarios and Requirements for Next Generation Access Technologies