3GPP TSG RAN#110 Meeting 	RP-253302

Baltimore, USA, December 15-18th, 2025					rev from RP-252163

Title:	Introduction of a TDD mode for NR-NTN

Source: 	Thales, Iridium, SES

Type:	discussion

Document for:	discussion

Agenda Item:	9.1.1 – Proposals led by RAN1 (Rel-20 5G advanced)

# Introduction

This document proposes to undertake a RAN1 led study followed by a possible normative activity on the introduction of a NR-NTN TDD mode as part of the Rel-20 5G Advanced work plan.

# Discussion

As stated in section 7. 3.6 of [38.811] in case the regulations allow it, TDD mode can be considered for both HAPS and LEO satellite based access network with potential NR impacts if required.

In the context of Rel-20, we propose to study the introduction of a TDD mode for NR-NTN radio interface especially for NGSO satellite networks and HAPS in FR1-NTN range.

This new feature should seek commonalities with the NR-NTN FDD operation, and potentially, if technically meaningful. Furthermore, it should be designed to allows the operator to use the radio resources in configurable TDD-DL-UL-Pattern. Including a configurable DownlinkToUplinkGuardPeriod which in Non-Terrestrial networks depends - as outlined in clause 7.3.6 of [38.811] - on the propagation delay between UE and Base Station. Thereby, it should commensurate at least the round trip delay (RTD). The RTD could take different values depending on satellite orbit (LEO600km, LEO1200km or GEO) and the position of the reference point for uplink synchronization. For example:

-	Minimum Downlink to Uplink Guard time for TDD operation for LEO at 600 km would range from 12.88 ms to 25.77 ms

-	Minimum Downlink to Uplink Guard time for TDD operation for LEO at 1200 km would range from 20.88 ms to 41.77 ms.

The targeted frequency bands should include, but not limited to unpaired spectrum allocated to satellite services (1616 MHz – 1626.5 MHz frequency band).

The study should consider as baseline: NGSO (e.g. 600 km altitude) and PC3 UE type.

The following aspects should be addressed during the study at RAN1, RAN2 and/or RAN4 levels:

- Impact on UE synchronization [RAN1, RAN4]

- Impact on frame structure and procedures [RAN1, RAN2]

- Coexistence study with TN or NTN operating in adjacent bands [RAN1, RAN4]

# Conclusion and Way forward

Proposal 1: RAN to study as part of the Rel-20 5G Advanced work plan, the introduction of a TDD mode for NR-NTN, starting in June 2026.

Proposal 2: RAN may consider for the study

1616 MHz – 1626.5 MHz frequency band as exemplary

PC3 UE type

NGSO @ 600 km altitude.

END