3GPP TSG RAN Meeting #110		RP-253611
Baltimore, USA, Dec 8-11, 2025

Title:	Switching period for 1T-1T band combination switching in Rel-16

Source:	OPPO, Xiaomi, vivo

Agenda item:	9.7

Document for:	Approval

Introduction

In RAN4 Nov meeting, LS [1] was sent to RAN2 and cc RAN1 to introduce the switching period for SUL band combinations and inter-band UL CA band combinations with 1T-1T switching and make it implement from Rel-16. This paper shares some background information and initial analysis of how this feature can be supported from early release.

1 Overall description When SUL was introduced, it was assumed that SUL+NUL band combinations are supported by UE with 2Tx. However, there are many UEs on the market that only support 1Tx chain. RAN4 has therefore discussed the RF requirements to allow 1Tx UE to support 1Tx-1Tx switching for NR inter-band UL CA and SUL band combinations from the releases earlier than release 18. RAN4 would like to enable this optional feature from Rel-16. - RAN4 agrees that non-zero 1Tx-1Tx switching is supported for NR inter-band UL CA and SUL band combinations. - The switching period values are: {35us, 140us, 210us}. RAN4 is kindly asking RAN2 to introduce this feature as early implementation from Rel-16 if possible by reusing current signalling as preferred, otherwise, it depends on RAN2 to decide how this feature can be supported in early release from signalling perspective. 2 Actions To RAN2: RAN4 respectfully asks RAN2 to introduce this feature as early implementation from Rel-16 if possible

Discussion

Why switching period is needed

Using low band for UL transmission has been considered as an important approach to increase UL coverage. The low band can be further combined with some mid/high bands to achieve higher throughputs by Tx switching in inter-band CA or SUL+NUL.

In current specs, the UL Tx switching was introduced in Rel-16 with some switching periods defined for 1T-2T switching. Afterwards, 2T-2T switching was introduced in Rel-17, and 1T-1T switching with non-zero switching period was introduced in Rel-18.

Currently, there are large amount of UEs (>80%) on the market can only transmit with 1Tx chain. For these UEs, in order to achieve good coverage with UL Tx switching, some switching period is definitely needed but unfortunately the switching period for 1T-1T switching was introduce only from Rel-18 [2] and these 1Tx UEs on the market are expected to only support up to Rel-16 3GPP specifications. The specification and market status/demands are not matched.

Large amount of UEs on the market only have 1Tx chain. For these UEs, switching period is needed to support the 1T-1T switching between inter-band carriers like SUL+NUL and CA.

The 1T-1T switching periods are only defined in Rel-18 and afterward, however, these 1Tx UEs on the market can only support up to Rel-16 specifications. There is mismatch between the market status/demands and specification.

This issue has been discussed in RAN4, and the market demands of introducing non-zero switching period for such 1Tx capable UEs were well recognized. In November meeting, RAN4 has sent an LS to RAN2 and cc RAN1 to introduce the non-zero switching period for 1T-1T switching from Rel-16.

As shown in the LS, RAN4 agreed to introduce this feature from Rel-16. Regarding how this feature to be introduced, e.g., by reusing current signaling or other approaches is up to RAN2 design.

From RAN4 point of view, the switching period for 1T-1T is desired to be supported from Rel-16 to meet market demands. Meanwhile, RAN4 agreed that how the signaling design is up to RAN2 and reusing current signaling is preferred.

Potential solutions in RAN2

As afore mentioned, currently, in R18 Tx Switching, the 1Tx-1Tx switching has been supported according to RAN1 LS [3] before. In order to support the 1Tx-1Tx switching from Rel-16 as required by R4, several approaches can be considered.

- One approach is to consider to make Rel-18 Tx switching signalling to be early implemented from R16. It can be considered as a basis of early implementation towards Rel-16.

- While another approach to consider to introduce Rel-16 late non-critical extensions. This can be considered as a way to introduce capability for Rel-16 and Rel-17 specifically.

- Other approaches can also be considered.

Multiple ways can be considered in RAN2 to support this feature from Rel-16, while detailed discussion can be handled in WG level.

Potential impacts in RAN1

Uplink switching has been introduced in R16 and covers 3 use cases: CA, EN-DC and SUL. To support R16 switching, switching gap NTx1-Tx2 , physical layer procedures related with switching gap like interruption during switching gap and processing timeline modification were introduced in 38.214.

In current R16 RAN1 specification, the description on Tx switching, i.e. “If the UE is to transmit any uplink channel or signal on a different uplink from the preceding transmission”, is very general without restriction on Tx number combinations like 1Tx-2Tx or 1Tx-1Tx. In other words, the RAN1 R16 specification can accommodate 1Tx-1Tx switching case already. If switching gap NTx1-Tx2 values are same for 1Tx-2Tx and 1Tx-1Tx switching case, then we do not see RAN1 spec impact.

No RAN1 impact is observed to support the non-zero switching period for 1T-1T switching from Rel-16. And RAN plenary is not included in the LS.

WGs continue discussing how to enable the switch period for 1T-1T SUL+NUL and inter-band CA switching per RAN4 LS request, and no RAN action is needed.

Conclusion

This paper shares some background information about the RAN4 LS on introducing non-zero switching period for 1T-1T switching under UL CA or SUL+NUL. Meanwhile, this paper also shares some analysis on the impacts/solutions to RAN1/2 in supporting this non-zero switching period from Rel-16. Based on these analyses, in our view, RAN1/2 can start working on the signaling and technical discussion based on RAN4 request in WG level and no RAN action is needed.

Observation 1: 	Large amount of UEs on the market only have 1Tx chain. For these UEs, switching period is needed to support the 1T-1T switching between inter-band carriers like SUL+NUL and CA.

Observation 2: 	The 1T-1T switching periods are only defined in Rel-18 and afterward, however, these 1Tx UEs on the market can only support up to Rel-16 specifications. There is mismatch between the market status/demands and specification.

Observation 3: 	From RAN4 point of view, the switching period for 1T-1T is desired to be supported from Rel-16 to meet market demands. Meanwhile, RAN4 agreed that how the signaling design is up to RAN2 and reusing current signaling is preferred.

Observation 4: 	Multiple ways can be considered in RAN2 to support this feature from Rel-16, while detailed discussion can be handled in WG level.

Observation 5: 	No RAN1 impact is observed to support the non-zero switching period for 1T-1T switching from Rel-16. And RAN plenary is not included in the LS.

Proposal 1: 	WGs continue discussing how to enable the switch period for 1T-1T SUL+NUL and inter-band CA switching per RAN4 LS request, and no RAN action is needed.

Reference

R4-2522413 LS on UE NUL+SUL 1Tx-1Tx switching period capability from Release 16, R4->R2/cc R1, RAN4#117 2025-11

R4-2506926 R18 correction of switching time between NUL and SUL carriers, OPPO, RAN4#115 2025-05

R1-2407505 Reply LS on UL Tx switching, R1->R2/4, RAN1#118 2024-08