3GPP TSG-RAN4 Meeting #117	R4-2521634

# Dallas, USA, 17 – 21 Novermber, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6195 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on measurement delay requirements with SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR is based on draftCR R4-2515123 endorsed in RAN4#116-bis. Same change as in R4-2515123 is extended to other applicable requirements: L1-SRS-RSRP, L1-CLI-RSSI, RLM, BFD, CBD and L1-RSRP. There are some issues with counting of unavialable occasions in CSI-RS based L1-SINR requirements with SBFD: 1) the delay would extend infinitely in DRX when the CSI-RS resource has occasions on both SBFD symbols and non-SBFD symbols, and 2) the meaning of L=0 is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the counting of unavailable occasions in L1-SINR requirements with SBFD to resolve the 2 issues above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS based L1-SINR requirements with SBFD are not fully correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.2, 8.5.3.2, 8.5.6.2, 9.5.4.2, 9.8.4.1, 9.8.4.2, 9.8.4.3, 9.18.2.5, 9.18.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 8.1.3.2	Minimum requirement

<Texts without change are omitted>

For the value of L1Out and L1In,

1>	If the UE does not support supportSBFD or SBFD is not configured by the network,

2>	 L1Out is not applicable;

2>  L1In is not applicable;

1>	Else if the UE supports supportSBFD and SBFD is configured by the network,

2>	 If DRX is not configured,

3> L1Out is the number of occasions of the CSI-RS resource for RLM that are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_Out_CSI-RS;

3> L1In is the number of occasions of the CSI-RS resource for RLM that are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_In_CSI-RS, when DRX is not configured

2>	 If DRX is configured,

3>	 L1Out is the number of DRX cycles in which at least one occasion of the CSI-RS resource for RLM is overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_Out_CSI-RS;

3>	 L1In is the number of DRX cycles in which at least one occasion of the CSI-RS resource for RLM is overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_In_CSI-RS;

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that all the occasions of the CSI-RS resource are in the same type (i.e., SBFD or non-SBFD) of symbols.

<Texts without change are omitted>

<End of Change 1>

<Start of Change 2>

#### 8.5.3.2	Minimum requirement

<Texts without change are omitted>

For the value of L1,

1>	If the UE does not support supportSBFD or SBFD is not configured by the network

2>	L1 is not applicable;

1>	Else if the UE supports supportSBFD and SBFD is configured by the network

2>	If DRX is not configured,

3>	L1 is the number of occasions of the configured BFD-RS resource that are overlapping with the dynamic UL transmission on SBFD symbols during TEvaluate_BFD_CSI-RS;

2>	Else if DRX is configured,

3> L1 is the number of DRX cycles in which at least one occasion of the BFD-RS resource is overlapping with the dynamic UL transmission on SBFD symbols during TEvaluate_BFD_CSI-RS.

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that all occasions of the BFD-RS resource are in the same type (i.e., SBFD or non-SBFD) of symbols.

<Texts without change are omitted>

<End of Change 2>

<Start of Change 3>

8.5.6.2	Minimum requirement

<Texts without change are omitted>

For the value of L1,

1>	If the UE does not support supportSBFD or SBFD is not configured by the network,

2>	L1 is not applicable;

1>	Else if the UE supports supportSBFD and SBFD is configured by the network,

2>	If DRX is not configured,

3> L1 is the number of occasions of the CSI-RS resource for candidate beam detection that are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_CBD_CSI-RS;

2>	If DRX is configured,

3> L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource for candidate beam detection is overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_CBD_CSI-RS.

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that all the occasions of the CSI-RS resource are in the same type (i.e., SBFD or non-SBFD) of symbols.

<Texts without change are omitted>

<End of Change 3>

<Start of Change 4>

#### 9.5.4.2	CSI-RS based L1-RSRP Reporting

<Texts without change are omitted>

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1 is not applicable

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if UE is configured to report L1-RSRP for SBFD symbols

3>	When DRX is not configured, L1 is the number of occasions of the CSI-RS resource that are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS

3>	When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource is overlapping with dynamic UL transmission or in which all occasions of the CSI-RS resource are overalapping with non-SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS

2>	if UE is configured to report L1-RSRP for non-SBFD symbols

3>		When DRX is not configured, L1 is the number of occasions of the CSI-RS resource that are overlapping with SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS, when DRX is not configured

3>		When DRX is configured, L1 is the number of DRX cycles in which all occasions of the CSI-RS resource are overlapping with SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS

<Texts without change are omitted>

<End of Change 4>

<Start of Change 5>

#### 9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

<Texts without change are omitted>

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1 is not applicable

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if UE is configured to report L1-SINR for SBFD symbols

3>	When DRX is not configured, L1 is the number of occasions of the CSI-RS resource as CMR that are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

3>	When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource as CMR is overlapping with dynamic UL transmission or in which all occasions of the CSI-RS resource as CMR are overalapping with non-SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

2>	if UE is configured to report L1-SINR for non-SBFD symbols

3>		When DRX is not configured, L1 is the number of occasions of the CSI-RS resource as CMR that are overlapping with SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

3>		When DRX is configured, L1 is the number of DRX cycles in which all occasions of the CSI-RS resource as CMR are overlapping with SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

For a UE supporting LB CA via switching, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent GAP(s) with NCSG(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent measurement gap and musim-GapPreference-r17 and when concurrent GAPs or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

<Texts without change are omitted>

#### 9.8.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured

<Texts without change are omitted>

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1 is not applicable

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if UE is configured to report L1-SINR for SBFD symbols

3>		When DRX is not configured, L1 is the number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_SSB_CMR_IMR

3>		When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS/CSI-IM resource as IMR is overlapping with dynamic UL transmission or in which all occasions of the CSI-RS/CSI-IM resource as IMR are overalapping with non-SBFD symbols during TL1-SINR_Measurement_Period_SSB_CMR_IMR

2>	if UE is configured to report L1-SINR for non-SBFD symbols

3>	When DRX is not configured, L1 is the number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_SSB_CMR_IMR

3>	When DRX is configured, L1 is the number of DRX cycles in which all occasions of the CSI-RS/CSI-IM resource as IMR are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_SSB_CMR_IMR

<Texts without change are omitted>

#### 9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

<Texts without change are omitted>

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1 is not applicable

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if UE is configured to report L1-SINR for SBFD symbols

3>	When DRX is not configured, L1 is the maximum between number of occasions of the CSI-RS resource as CMR that are overlapping with dynamic UL transmission or with non-SBFD symbols and number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with dynamic UL transmission or with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

3>	When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource as CMR or at least one occasion of the CSI-RS/CSI-IM resource as IMR is overlapping with dynamic UL transmission or in which all occasions of the CSI-RS resource as CMR or all occasions of the CSI-RS/CSI-IM resource as IMR are overalapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

2>	if UE is configured to report L1-SINR for non-SBFD symbols

3>	When DRX is not configured, L1 is the maximum between number of occasions of the CSI-RS resource as CMR that are overlapping with non-SBFD symbols and number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

3>	When DRX is configured, L1 is the number of DRX cycles in which all occasions of the CSI-RS resource as CMR or at least one occasion of the CSI-RS/CSI-IM resource as IMR are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

<Texts without change are omitted>

<End of Change 5>

<Start of Change 6>

#### 9.18.2.5	L1-SRS-RSRP measurement period

The UE shall be capable of performing L1-SRS-RSRP measurement based on the configured SRS resource. For periodic and semi-persistent SRS resource, the UE shall be capable of reporting L1-SRS-RSRP measured over measurement period of TL1_SRS_RSRP_measurement_period for FR1 and FR2.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1 is not applicable

1>	else if UE supports supportSBFD and SBFD is configured by the network

2>	if UE is configured to report L1-SRS-RSRP for SBFD symbols

3>		When DRX is not configured, L1 is the number of occasions of the L1-SRS-RSRP resource that are overlapping with non-SBFD symbols during TL1-SRS-RSRP_measurement_period

3>		When DRX is configured, L1 is the number of DRX cycles in which all occasions of the L1-SRS-RSRP resource are overlapping with non-SBFD symbols during TL1-SRS-RSRP_measurement_period

2>	if UE is configured to report L1-SRS-RSRP for non-SBFD symbols

3>	When DRX is not configured, L1 is the number of occasions of the L1-SRS-RSRP resource that are overlapping with SBFD symbols, during TL1-SRS-RSRP_measurement_period

3>	When DRX is configured, L1 is the number of DRX cycles in which all occasions of the L1-SRS-RSRP resource are overlapping with SBFD symbols, during TL1-SRS-RSRP_measurement_period

Table 9.18.2.5-1 Measurement period TL1_SRS_RSRP_measurement_period

| Configuration | TL1_SRS_measurement_period (ms) |
| --- | --- |
| No DRX | Max(TReport, (M +L1) X TSRS) |
| DRX cycle ≤ 320 ms | Max(TReport, Ceil(1.5 X (M +L1)) X max(TSRS, TDRX)) |
| DRX cycle > 320 ms | (M +L1) X TDRX |
| NOTE 1: TSRS is SRS measurement periodicity configured SRS-PeriodicityAndOffset, and TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. |  |

If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TL1_SRS_RSRP_measurement_period.

<End of Change 6>

<Start of Change 7>

#### 9.18.3.5	L1-CLI-RSSI measurement period

The UE capable of supportL1-CLI-RSSI-Measurement shall perform L1-CLI-RSSI measurement based on the configured L1-CLI-RSSI measurement resource. For periodic and semi-persistent L1-CLI-RSSI resource, the UE shall be capable of reporting L1-CLI-RSSI measured over measurement period of TL1-CLI_RSSI_measurement_period for FR1 and FR2.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1 is not applicable

1>	else if UE supports supportSBFD and SBFD is configured by the network

2>	if UE is configured to report L1- CLI-RSSI for SBFD symbols

3>		When DRX is not configured, L1 is the number of occasions of the L1- CLI-RSSI resource that are overlapping with non-SBFD symbols during TL1-CLI-RSSI_measurement_period

3>		When DRX is configured, L1 is the number of DRX cycles in which all occasions of the L1- CLI-RSSI resource are overlapping with non-SBFD symbols during TL1-CLI-RSSI_measurement_period

2>	if UE is configured to report L1- CLI-RSSI for non-SBFD symbols

3>	When DRX is not configured, L1 is the number of occasions of the L1- CLI-RSSI resource that are overlapping with SBFD symbols, during TL1-CLI-RSSI_measurement_period

3>	When DRX is configured, L1 is the number of DRX cycles in which all occasions of the L1- CLI-RSSI resource are overlapping with SBFD symbols, during TL1-CLI-RSSI_measurement_period

Table 9.18.3.5-1 Measurement period TL1-CLI_RSSI_measurement_period

| Configuration | TL1-CLI_RSSI measurement_period (ms) |
| --- | --- |
| No DRX | Max(TReport, (M+L1) X TCLI-RSSI) |
| DRX cycle ≤ 320ms | Max(TReport, Ceil(1.5 X (M+L1)) X max(TCLI-RSSI, TDRX)) |
| DRX cycle > 320ms | (M+L1) X TDRX |
| NOTE 1: TCLI-RSSI is L1-CLI-RSSI measurement periodicity configured RSSI-PeriodicityAndOffset, and TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. |  |

If the configured L1-CLI-RSSI measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TL1-CLI_RSSI_measurement_period.

<End of Change 7>