3GPP TSG-RAN WG4 Meeting #117	R4-2522837

# Dallas, USA, 17th – 25th

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6234 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_duplex_evo-Core)CR on Scheduling restriction of L1 CLI for SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Change 1: The scheduling restriction due to L1 CLI measurements was discussed in core part and they were captured into the endorsed big CR. Considering CA case, only intra-band CA is touched, absent for the case of inter-band CA. Based on the following RAN1 progress, no restriction on the applicability of SBFD in intra-band or inter-band CA. Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case - Agreement In order to support inter-band CA with SBFD operation for a UE supporting simultaneousRxTxInterBandCA, the scheduling restriction of L1 CLI measurement is to be captured in the specification reusing the legacy principle in L1 measurement.. For inter-band CA, similar as all other L1 measurement, no scheduling restriction on the serving cells of another band. These should be captured in the spec. |
| Summary of change: |  | Capture the scheduling restriction for inter-band CA case under SBFD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scheduling restriction of inter-band CA case for SBFD is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x.2.6.1, 9.x.2.6.2, 9.x.3.6.1, 9.x.3.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============Start of change #1==============

## 9.x	L1 Cross Link Interference measurements

#### 9.x.2.6	Scheduling availability of UE during L1-CLI measurements

Scheduling availability when the UE is performing L1-CLI measurements which are L1-SRS-RSRP and L1-CLI-RSSI are described in the following clause.

##### 9.x.2.6.1	Scheduling availability of UE performing L1-SRS-RSRP measurement on FR1

The following scheduling restriction applies due to L1-SRS-RSRP measurements.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 1 data symbol before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 15 kHz and 30 kHz subcarrier spacing.

-	For the UE which does not support L1-SRS-RSRP-FDM_DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbol(s) on which the UE performs L1-SRS-RSRP measurements, and on 1 data symbol before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 15 kHz and 30 kHz subcarrier spacing.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-SRS-RSRP measurement, and on 2 data symbols before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 60 kHz subcarrier spacing.

-	For the UE which does not support L1-SRS-RSRP-FDM_DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs L1-SRS-RSRP measurement, and on 2 data symbols before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 60 kHz subcarrier spacing.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where CLI measurements are performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on serving cell where CLI measurements are performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which the serving cell where L1-SRS-RSRP measurement is performed is configured.

##### 9.x.2.6.2	Scheduling availability of UE performing L1-SRS-RSRP measurement on FR2

The following scheduling restriction applies due to L1-SRS-RSRP measurements.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 1 data symbol before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 60 kHz subcarrier spacing.

-	For the UE which does not support L1-SRS-RSRP-FDM_DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 1 data symbol before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 60 kHz subcarrier spacing.

-	For the UE which supports L1-SRS-RSRP-FDM_DL but the SRS resources are not TypeD QCL-ed with PDCCH/PDSCH, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 1 data symbol before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 60 kHz subcarrier spacing.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 2 data symbols before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 120 kHz subcarrier spacing.

-	For the UE which does not support L1-SRS-RSRP-FDM_DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the OFDM symbol(s) on which the UE performs L1-SRS-RSRP measurements, and on 2 data symbols before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 120 kHz subcarrier spacing.

-	For the UE which supports L1-SRS-RSRP-FDM_DL but the SRS resources are not TypeD QCL-ed with PDCCH/PDSCH, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 2 data symbols before the OFDM symbol(s) used for SRS-RSRP measurements for 120 kHz subcarrier spacing.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where L1-SRS-RSRP measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 serving cells in the bands due to L1-SRS-RSRP measurement performed on FR2 serving cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair.

==============End of change #1==============

==============Start of change #2==============

9.X.3.6.1	Scheduling availability of UE performing L1-CLI-RSSI measurement on FR1

The following scheduling restriction applies due to the L1-CLI-RSSI measurement:

- The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-CLI-RSSI measurements.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where CLI measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on serving cell where CLI measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which the serving cell where L1-CLI-RSSI measurement is performed is configured.

9.X.3.6.2	Scheduling availability of UE performing L1-CLI-RSSI measurement on FR2

The following scheduling restriction applies due to the L1-CLI-RSSI measurement:

- The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-CLI-RSSI measurements.

- The UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs L1-CLI-RSSI measurements when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH/CSI-RS.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where CLI measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 serving cells in the bands due to L1-CLI-RSSI measurement performed on FR2 serving cell(s) in different band(s).

==============End of change #2==============