UE RF:

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

  - Co-existence study for ACLR, specify ACLR requirements if needed

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 4x4 UL MIMO and TxD

  - Specify the band specific requirements for exemplary bands

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

- PC1.5 1Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 1 x 29 dBm

    - Example bands: n41, n77/n78, n79

  - This work is to start in 2026 Q4

- 2Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 2 x 29 dBm

    - Example bands: n41, n77/n78, n79, [n104]

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

  - This work is to start in 2026 Q4

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 2x2 UL MIMO and TxD

    - Baseline PA configuration: 2 x 26 dBm

  - Specify the band specific requirements for exemplary bands

  - Example bands: n1, n25

6MHz:

- Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4]

  - Only include 15 kHz SCS

  - Specify the spectrum utilization

  - Specify UE RF requirements for UL and DL

  - Specify BS RF requirements (OTA and conductance) for UL and DL

  - Specify the demod requirement

- Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4]

  - BS and UE Channel bandwidth per operating band

  - UE reference sensitivity

  - When needed

    - Additional Maximum Power Reduction (A-MPR)

- Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2]

Note This should be similar to the signalling for 7 MHz channel bandwidths

- Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3]

Note This should be similar to the signalling for 7 MHz channel bandwidths

NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent.

TX EVM Relaxation:

Add the following notes in 5G-A R20 UE RF WID

Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. For 5G-Adv, solution should not be AI based and there should not be any impact to RAN1.

UE OTA:

Objectives for this core part work item are as follows

- Define enhanced test methodologies to cover NTN Ka and Ku bands

  - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing

    - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority

    - FR1 OTA test method extension is not precluded, if also applicable

  - Study whether and how to support circular polarization measurements

    - Linear polarization is the baseline

  - Specify the required quiet zone to cover VSAT device type based on industry needs

    - 60cm QZ as a starting point

  - Specify preliminary MU assessment for test methodologies

During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9, GCF, GSMA TSG-AP, ETSI MSG TFES, and PTCRB shall be maintained to ensure industry coordination on this topic.

Objectives for this performance part work item are as follows

- Specify FR1 TRP and TRS requirements for new bands

  - Handheld UE is the first priority.

  - Bands n20, n38, n79 for 1Tx, n41 and n78 for 2Tx non-coherent UE

    - Maximum 4 bands are targeted for introducing requirements depending on measurement data availability

    - Handheld UEs width >=56mm and <=92mm

Note: Check in RAN #112, whether to include other UE types as second priority if a sufficient number of commercial devices and new phantom (if needed) become available.

- Specify FR1 MIMO OTA requirements for new bands based on static test method

  - Applicable number of layers for targeting bands

    - 2 layers: band n20

    - 4 layers: band n38, n79

  - n20, n38, n79 are first priority

    - Maximum 3 bands are targeted for introducing requirements depending on measurement data availability

  - Handheld UE is the first priority