3GPP TSG RAN Plenary RAN#109                                                     RP-252873

Beijing, September 15th – 19th, 2025

Agenda Item:	8.2.2

Source:	Apple Inc. (Moderator)

Title:	Summary of 6G Diverse Device Types (including Minimum UE Bandwidth)

Document for:	Discussion/Decision

# Introduction

This document provides a summary for 6G device types (including minimum UE bandwidth), as part of RAN level 6G study.

# Input Documents to RAN#109

The following documents on 6G device types were submitted to RAN#109.

| RP-251940 | Text proposal for TR 38.914 for Vehicular Device Type | BMW AG, ROBERT BOSCH GmbH,Volkswagen AG, Toyota ITC, Fraunhofer IIS, Continental Automotive |
| --- | --- | --- |
| RP-251998 | Discussion on UE-type based feature sets | Guangdong OPPO Mobile Telecom. |
| RP-252007 | Discussion on 6G device types | vivo |
| RP-252044 | Discussion on diverse device types | ZTE Corporation, Sanechips |
| RP-252054 | Requirements for possible services and device types (not available) | NEC |
| RP-252136 | Consideration on support of diverse device types for various services | Huawei, HiSilicon |
| RP-252142 | Views on 6G device types | CATT |
| RP-252147 | Architecture/migration requirements for NTN | Thales |
| RP-252154 | Views on 6G diverse device types | Spreadtrum, UNISOC |
| RP-252184 | Views on 6G Device Types and Device Attributes | Qualcomm Incorporated |
| RP-252185 | Views on UE capability framework | Qualcomm Incorporated |
| RP-252198 | On 6GR minimum requirement for UE channel bandwidth | Nokia |
| RP-252199 | 6G device types | Nokia |
| RP-252205 | TP on diverse Device Types for 6G for TR 38.914 | Deutsche Telekom, Vodafone, Orange, Telecom Italia, Turkcell, Spark NZ, Odido, Bouygues Telecom, Telefonica, Telia Company, KPN, CK Hutchison, Telenor, KT Corp. |
| RP-252230 | Requirements for Diverse XR Wearable Device Types for XR Services | Meta USA |
| RP-252231 | Meta's Views on Requirements of XR Services | Meta USA |
| RP-252313 | On 6G Device Types | Apple Inc. |
| RP-252316 | Views on 6G requirements for new and existing services and support of diverse device types | Intel Corporation |
| RP-252320 | Native 3 MHz initial-access profile for low FR1 FDD bands | Rakuten Mobile, Inc |
| RP-252328 | TSG RAN Guidance to RAN1 after RAN1 #122 | AT&T |
| RP-252343 | 6G Device Types | Samsung |
| RP-252469 | Device types for the scalable 6GR design | Panasonic |
| RP-252518 | Scalability for Diverse device types | MediaTek Inc. |
| RP-252519 | Views on 6GR device types | Fujitsu |
| RP-252580 | Support of diverse device types in 6G | Ericsson |
| RP-252654 | 6GR study discussion | Xiaomi |
| RP-252657 | Discussion on SCS and Minimum Bandwidth requirements | Tejas Network Limited |
| RP-252700 | Views and Considerations on Automotive UE Evolution for 6G | ROBERT BOSCH GmbH |
| RP-252657 | Discussion on SCS and Minimum Bandwidth requirements | Tejas Network Limited |
| RP-252729 | Diverse Device Types for 6GR | FUTUREWEI |

Note that RAN1#122 (Augus’2025) made the following agreements:

Agreement

Study a scalable 6GR design for diverse device types, considering aspects:

- What should be commonly applicable to all 6G device types

- FFS: add-on features dedicated to specific device types, if any

Agreement

- Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

# Discussion

## Scalability and Smallest Maximum Bandwidth

As stated in [1], 6G study is required to “target scalable and forward compatible design for diverse device types”.

There are two important aspects to be considered

- Smallest Maximum UE Bandwidth vs. Minimum Spectrum Allocation

- Whether or not to introduce baseline functionality mandatory for all 6G UEs

### Smallest Maximum UE Bandwidth vs. Minimum Spectrum Allocation for 6GR

These two aspects, although highly related, are not necessarily the same, although they can be made the same as a design choice.

Note that RAN1 made the following two agreements:

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

Contributions submitted to RAN#109 are also in general consistent with the above two RAN1 agreements. As a result, it is proposed to endorse the above two RAN1 agreements. At the same time, it is important for companies, especially operators, to provide further inputs regarding the minimum spectral allocation. Since the smallest maximum supported UE bandwidth is strongly related to 6GR design, especially for the initial access and the possible SCS(s), it is proposed to have RAN1 continue the study on this aspect, so that RAN plenary can make a more informed decision later.

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110.

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

### Potential Baseline Functionality Mandatory for all 6G UEs

As 6GR is intended to support a diverse set of device types in the first release, a 6G cell is thus expected to be accessible by all 6G UEs. As a result, it is sensible to mandate a set of mandatory baseline functionalities. At the same time, as in [1], there are many crucial requirements defined for 6G. It is also important to discuss whether some of the requirements and the corresponding design should be mandatory for all UEs as well so as to ensure 6G values. There are also contributions mentioning the possibility of mandating base stations for certain requirements/functionalities.

Based on the above, the following is proposed:

Proposal 2: 6GR aims to specify a set of mandatory baseline functionalities for all UEs, with details to be further studied, by taking into account diverse device types for scalable and forward compatible design, and the requirements, as necessary, as in the 6G SID.

Further study whether there is a need to mandate base stations for certain requirements/functionalities.

## Possible Device Types

The views from the set of contributions are diverging. It was mentioned by many contributions that 6GR should target a limited number of device types, at least to avoid possible market fragmentation. At the same time, there are contributions, or when combining various contributions, listing a large number of device type. There are also contributions questioning the need to define or standardize any device type. Rather, similar but simplified UE capability signaling is proposed to realize diverse device types, built on top of the baseline functionality. There are questions about what terminology should be used for 6G device types. In addition, it was mentioned that it is important to ensure forward compatibility such that potential introduction of additional device types after the first 6G release can be smoothly and readily done.

Among all the proposals, eMBB and 6G IoT (vs. LPWA) seem to be converging, although there are different views regarding whether there should be one or multiple eMBB (or 6G IoT) device types. Other device types mentioned in the contributions include: XR/immersive experiences, FWA, vehicle UEs (VUEs), wearables/RedCap, sensing (as standalone or complementary), NTN (standalone or complementary), AI agents, collaborative robots, etc.

Based on the above, the following is proposed:

Proposal 3: To investigate further:

  - Motivations/justifications behind the proposed diverse device types, which should be a limited set

  - Whether/how to have one or more device types for eMBB or 6G IoT

  - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

  - Whether/how to explicitly standardize device types

  - Ensuring forward compatibility

  - Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

## Defining Device Types

One critical question is how to define/identify different device types. Various parameters have been proposed in the contributions, including whether the parameters should have specific values or a range of values. Even with a device type, it was mentioned that there is some possibility that varying UE capabilities/features may still need to be considered to adapt to different services and scenarios, such as transmission capability, cost/complexity, characteristics of operation environment, etc.

Among the parameters, it seems that form factor limitations is one common ground, as already being done in 5G NR. These limitations may impact parameters such as the number of Tx antennas/chains, Rx antennas/chains, power classes, etc. In addition, the following parameters were mentioned as well.

- Maximum UE bandwidth (DL/UL)

- Peak data rate (DL/UL)

- Maximum MIMO layers (DL/UL)

- Duplex mode

- Max modulation order (DL/UL)

- CA/spectrum aggregation (DL/UL)

- UE processing capabilities

- Coverage

- Energy efficiency

- Mobility/speed

- Sensing

- AI

Based on the above, the following is proposed:

Proposal 4: In terms of diverse device types, at least form factor limitations is considered. Sstudy further:

- The parameters related to form factor limitations, e.g., the number of Tx antennas/chains, Rx antennas/chains, power classes, etc.

- Other Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

Note: some of the above parameters/factors may be related with form factor

Note: aim to have a focused/limited set of parameters/factors for a device type

- The value(s) for the identified parameters for a device type

# Conclusion

The following is proposed for the study of diverse device types for 6GR:

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110.

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

FFS: whether RF and BB UE BW are same or different

Proposal 3: To investigate further:

  - Motivations/justifications behind the proposed diverse device types, which should be a limited set

  - Whether/how to have one or more device types for eMBB or 6G IoT

  - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

  - Whether/how to explicitly standardize device types

  - Ensuring forward compatibility

  - Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

Proposal 4: In terms of diverse device types, study further:

- Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

Note: some of the above parameters/factors may be related with form factor

Note: aim to have a focused/limited set of parameters/factors for a device type

- The value(s) for the identified parameters for a device type

The following proposal was also discussed, but didn’t reach consensus offline:

Proposal 2: 6GR aims to specify a set of mandatory baseline functionalities for all UEs, with details to be further studied, by taking into account diverse device types for scalable and forward compatible design, and the requirements, as necessary, as in the 6G SID.

Further study whether there is a need to mandate base stations for certain requirements/functionalities.

# Reference

- RP-251881	New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025