3GPP TSG RAN Meeting #109								RP-252928

Beijing, China, September 15-18, 2025

Agenda Item:		9.6.1.7

Source:			CMCC, Huawei, T-Mobile USA Inc.

Title:		WI summary for Rel-19 Ambient IoT (Internet of Things) in NR

WI code(s):		Ambient_IoT_Solutions

leading WG:		RAN1

Release:			Rel-19

### 1	Introduction

3GPP is developing new IoT technologies supporting passive devices - battery-less or with limited energy storage - powered by harvesting ambient energy from the environment, with ultra-low complexity and small form factor. The new IoT technology will provide complexity and power consumption orders-of-magnitude lower than the existing 3GPP LPWA technologies (e.g. NB-IoT and eMTC) and will address use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technologies.

In accordance to the approved WID for Ambient IoT (Internet of Things) in NR in Rel-19 [1], the WI objective is to standardize the Ambient IoT (A-IoT) Device 1, which has ~1 µW peak power consumption, energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 105 ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave (CW) provided externally. The overall scope focuses on two representative use cases, i.e. indoor inventory (rUC1) and indoor command (rUC4) defined in TR 38.848 [2]. Deployment scenario 1 with Topology 1 (D1T1) is considered, where D1 refers to device and base station reader (BS) indoors, and T1 refers to device directly and bidirectionally communicates with the BS reader. The targeted spectrum is FR1 licensed FDD spectrum, with R2D in DL spectrum and D2R and carrier-wave in UL spectrum. The targeted traffic types include Device-terminated (DT) and Device-originated Device-terminated triggered (DO-DTT).

### 2	Description

Physical layer design

For R2D, DFT-s-OFDM-based OOK waveform is supported. For in-band operation, the starting position of a A-IoT R2D transmission is aligned in time with the starting position of an NR OFDM symbol. The CP handling method is supported to retain subcarrier orthogonality. When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used. For D2R, OOK and BPSK modulation is supported and chooses by device implementation for a D2R transmission. The D2R transmission is described by a set of chips to which modulated symbols are mapped, and based on backscattering on a carrier wave. The carrier wave is a single-tone sinusoid signal.

For R2D, Manchester encoding is employed to combat large SFO at the device, timing is maintained by detecting transition edges. No FEC and repetition is considered due to limited device complexity. For D2R, TBCC and block-level repetition are supported to improve link performance. CRC are attached to both R2D and D2R links.

A single physical channel PRDCH and PDRCH is supported for R2D and D2R, respectively. An R2D timing acquisition signal is transmitted immediately before a PRDCH, and consists of a start indicator part (SIP) followed by a clock acquisition part (CAP). The device determines that an R2D transmission begins upon determining that a SIP has been received. The CAP indicates the number of chips per OFDM symbol and chip duration for PRDCH. An R2D postamble is transmitted immediately after a PRDCH. A D2R preamble signal for timing acquisition, timing tracking and channel estimation, and zero or more D2R midamble signal(s) for timing tracking and channel estimation, are inserted in a D2R transmission. A D2R preamble signal is transmitted immediately before a PDRCH.

R2D multiplexing is only TDMA. For D2R, FDMA is supported for both A-IoT MSG1 and MSG3 transmissions, based on modulation for small frequency shift, providing higher capacity during an inventory procedure. For A-IoT MSG1 transmission determined by one R2D transmission triggering random access, X = 1 or 2 time domain resource(s) are supported. The time intervals from the end of the R2D transmission and the subsequent corresponding D2R transmission are defined, to determine the starting time of the D2R time domain resource(s) from device perspective.

Protocol stack and signalling procedures

An extremely simplified protocol stack is pursued and the only necessary layers for A-IoT radio interface are A-IoT physical (PHY) layer and MAC layer. The A-IoT MAC layer supports necessary Layer 2 functionalities including paging, random access, data transmission, and packet segmentation.

A-IoT paging allows the reader to trigger one or more A-IoT device(s) to access or re-access. The A-IoT paging may include one or no paging identifier. If one paging identifier is included, it is addressed to a single or a group of A-IoT device(s). If no paging identifier is included, the A-IoT paging message is addressed to all A-IoT devices.

For A-IoT random access procedure, both CBRA and CFRA are supported. For CBRA, the A-IoT device(s) randomly select among A-IoT MSG1 resources provided in A-IoT paging message to send A-IoT MSG1 (i.e., random ID). After A-IoT MSG1 transmission, the device monitors for an A-IoT MSG2 for contention resolution from the reader. Upon successful reception of A-IoT MSG2 which contains the same random ID with MSG1, the A-IoT device considers the A-IoT random access procedure as successfully completed. Otherwise, the device considers the A-IoT random access procedure as failure. After successfully completing CBRA procedure, the device shall report the upper layer device identifier in the next D2R message. For CFRA, the A-IoT device uses the dedicated resource provided in A-IoT paging message to send the first D2R response.

The A-IoT MAC sublayer supports R2D and D2R transmission of upper layer data, including device ID, upper layer command and command response, etc. A D2R upper layer data SDU segmentation is supported in case the size exceeds the scheduled D2R resources.

RAN architecture aspects

A-IoT functions have been realised within the existing 5G system architecture. The AIoT reader function is supported by NG-RAN, more specifically by the gNB, which connects to a new 5GC entity, the “A-IoT Function” (AIOTF) via NG-C, either directly or indirectly via an AMF. A-IoT specific NG-C protocol functions are specified in NGAP. The AIOTF is the termination of the NAS protocol with the device. It manages the services relating to triggering RAN operations, and other aspects relating to transfer of information between an application and the RAN or device.

### 3	References

- RP-250796, “Revised WID: Solutions for Ambient IoT (Internet of Things) in NR,” CMCC, Huawei, T-Mobile USA Inc., 3GPP TSG RAN Meeting #107.

- 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN (Release 18), v18.0.0.