3GPP TSG RAN WG1 #122	R1- 2505128

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:		11.3.2

Source:	Nokia

Title:	Frame Structure and Numerology in 6G Radio Air Interface

WI code:	FS_6G_Radio

Release:	Rel-20

Document for:		Discussion and Decision

# Introduction

Following the approval of the 6G study item in [1], this contribution addresses frame structure and numerology aspects relevant to 6GR.

# Overview

The 6G radio study item [1] includes an explicit objective on frame structure and numerology for 6G, while requiring efficient operation of 5G-6G MRSS by the 6G frame structure and that 6G radio avoids defining multiple numerologies for the same band / sub range:

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] […]

Hence, in the following we discuss relevant aspects for selection of numerology for 6G frame structure, while related 6G waveform considerations are addressed in a companion paper [2].

# Numerology and frame structure in LTE and NR

## Numerology in 4G and 5G

In 4G, it was considered important to ensure that the 4G clock rate is compatible with the 3G chip rate (3.84 MHz). This strategic decision aimed at facilitating network evolution, enhancing interoperability, and optimizing performance across different generations of mobile technology. Based on this, the following numerologies were defined for 4G:

- Sample rate (in specs) is 30.72 MHz (=8*3.84 MHz).

- Subcarrier spacing (SCS): 30.72MHz / 2048 = 15 kHz

- Max channel bandwidth (CBW) = 20 MHz

- Max FFT size: 2048.

In 5G (Release 15), coexistence with previous generations, especially 4G, was considered one of the key requirements. Based on this, the sample rate was defined to be a multiple of the sample rate in 4G. Other changes include doubling the maximum FFT size from 2k to 4k and defining four SCS options for data channels, scaled from the subcarrier spacing defined for LTE by powers of two.

- Sample rate (in specs) is 1.96608 GHz (=64*30.72 MHz).​

- SCS is scaled from LTE SCS: [15 30 60 120] kHz​

- Max CBW (SCS): [50 100 200 400] MHz

- Max FFT size: 4096

NR specifications also support the so-called mixed numerology, which refers to the use of different subcarrier spacings (SCS) and symbol durations within the same cell simultaneously. One goal behind mixed numerology was to optimize subcarrier spacing for different services, such as eMBB, URLLC, and massive MTC, running concurrently in one cell. It should be noted that in practice commercial NR deployments follow a single numerology, as they are optimized for eMBB services.

## Frame structure in 4G and 5G

The frame structure defines the available resource grid in time domain, while the CBW configuration and active bandwidth part define it in the frequency domain.

In 4G, there are separate frame structures for FDD and TDD.

- FDD operation is defined according to Frame Structure 1, with a radio frame of 10 ms, consisting of 10 subframes of 1 ms each, and 2 slots of 0.5 ms.

- TDD operation is defined according to Frame Structure 2, having two 5 ms half-frames within the Radio Frame. Depending on the TDD configuration, a special subframe is located at a predefined position within a 5 ms half-frame, facilitating guard periods and link direction switching.

- 4G supports normal CP length with 14 OFDM symbols per subframe, and extended CP length with 12 OFDM symbols per subframe.

- For normal CP length, the first CP of each slot is extended by 16 samples to accommodate 14 OFDM symbols and their corresponding CPs within a 1ms subframe.

In 5G, frame structure is aligned with FDD and TDD

- FDD and TDD frame structures are aligned in 5G

- Guard Period in NR: Not explicitly defined. Flexible resources enable both link direction switching and adaptable TDD.

- Slot definition change: In NR (New Radio), the concept of a slot differs from 4G. A slot now consists of 14 OFDM symbols (for normal CP length).

- Extended CP length is defined only for 60 kHz SCS (as an optional feature) with 12 OFDM symbols per slot.

- For 15 kHz SCS, the time domain resource grid for NR is fully aligned with LTE. For other subcarrier spacings, the duration of the symbol and CP halves when the SCS doubles.

- The first CP of each 0.5ms period is prolonged by 16 (LTE) samples. Based on that, OFDM symbol boundaries between 4G and 5G are maximally aligned (and fully aligned for 15 kHz SCS).

# Design goals for Frame structure and Numerology in 6G

One of the key requirements for the 6G frame structure and numerology is smooth coexistence with 5G NR, particularly in the MRSS scenario, as explicitly stated in 6G SID [1]. Taking the learnings from DSS for migration from 4G to 5G, it is clear that MRSS has to be more efficient, and it needs to be an integral part of the 6GR design. This includes related MRSS considerations in the studies on frame structure and numerology for 6GR (as noted in the SID directly) to allow support for resource-element level sharing between 5G and 6G to maximize efficiency and avoid additional guard overhead in frequency or time. This, in turn, requires not only that the subcarrier spacing is the same in both 5G and 6G, but also the following: the OFDM symbol timing is aligned, the subcarriers are in frequency sync, and preferably that the RB boundaries are also aligned. Additionally, opportunities for hardware reuse and coexistence should be optimized. Effective interference management and aligned interference characteristics are also crucial. These requirements suggest that it should be possible to configure an overlapping resource grid for 5G and 6G, ensuring that OFDM symbol boundaries and RB boundaries are fully aligned. This means that

- 6G should support subcarrier spacing options being used in the field for 5G.

- 6G should adopt the time domain frame structure (normal CP) being used in the field for 5G.

In addition to subcarrier spacing (SCS), the supported channel bandwidth (CBW) options also need to be defined. Together, SCS and maximum CBW determine the FFT size requirements. We must define not only the minimum and maximum CBW sizes for different scenarios, but also the CBW size options between these minimum and maximum values. The maximum CBW size has increased with each radio generation, typically growing by a factor of approximately 4 to 5 times. In 6G SID [1] it is already defined that channel bandwidth for 6G should be at least 200 MHz, at least above 2 GHz, around 7 GHz. This provides a factor of 2 increase of maximum CBW of 100 MHz in 5G NR, however we believe a factor of 4 increase up to 400 MHz in relevant bands would be beneficial for the 6G system.

Other 6G requirements include simplifying the system and eliminating unnecessary options, as explicitly mentioned in [1]. The 6G SID [1] also requires that the number of subcarrier spacing choices should be minimized for each frequency band/range. Additionally, the support for mixed numerology and various cyclic prefix lengths should be re-evaluated. Based on 5G experiences, latency-critical services and MTC can also be implemented using the same cell-specific numerology. This implies that for a given band, whether FDD or TDD, all numerologies used for shared data channels should employ the same subcarrier spacing. Hence, subcarrier spacing should be a cell-specific parameter rather than a UE-specific parameter.

It should be noted though that for certain channels, such as SSB and PRACH, one may need to deviate from the cell-specific subcarrier spacing in specific scenarios. Additionally, when a UE is configured to operate in multiple bands for uplink and downlink, it will follow cell-specific numerology for each band.

Frame structure and numerology significantly impact hardware design, and are thus reflected in an early milestone for RAN#112, as shown below. Hence, to expedite reaching a 3GPP conclusion, it's crucial to agree on evaluation assumptions promptly, which are addressed in a companion paper [3].

TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026.

Observation 1: Efficient MRSS requiring the 6GR frame structure and numerology to be compatible with 5G NR

Observation 2: MRSS and other coexistence scenarios benefit from resource-element level coexistence between 5G and 6G

Proposal 1: Strive for an early decision on subcarrier spacing(s) and maximum CBW size

Proposal 2: Define subcarrier spacing as a cell and band-specific parameter, with a potential exception for SSB and PRACH

# Time domain frame structure for 6G

3GPP should adopt an evolutionary (scalable) NR numerology for all 6G scenarios: FR1, New Frequency range(s) between 7 and 24 GHz, and FR2-1. This is critical not only from the gNB/UE implementation reasons (coexistence, hardware reuse, etc.) but also for efficient MRSS operation. Based on that, we propose the following time domain frame structure for 6G:

Frame = 10 ms

- Subframe = 1 ms

- Subcarrier spacing (SCS) = 15 kHz ∙ 2m , m = [0,1,2, …]

Slot: each set of 14 consecutive OFDM symbols (including the cyclic prefix), starting from the beginning of a subframe, constitutes one slot.

The radio frame and subframe provide sufficient means to characterize the time-domain frame structure, making the "slot" technically redundant. However, given that subcarrier spacing varies depending on the deployment scenario (e.g., band or cell), the slot serves as a scalable time unit tied to subcarrier spacing, proving useful in many contexts. We propose that the term "slot" be retained in the 6G specification to denote a predefined set of 14 OFDM symbols within a subframe. The definition of a slot need not address time-domain resource allocation, which can be treated as a separate consideration.

Aligned with the principles laid out in the 6G SID [1] about simplification of the system and avoiding multiple options for similar purposes, we propose supporting only the normal cyclic prefix (NCP) length with 14 OFDM symbols per slot. In 5G, the extended cyclic prefix (ECP) is supported exclusively for 60 kHz subcarrier spacing (SCS) and not for other SCSs. The extended CP increases cyclic prefix overhead from 6.7% to 20%, offering no throughput advantage over NCP. During the NR study, it was demonstrated that ECP does not enhance cell coverage in scenarios involving eMBB, 60 kHz SCS, and large delay spreads (TDL-C-1000ns) [4].

Proposal 3: Retain the 5G definition of a frame of 10ms duration and subframes of a 1ms duration, and retain the 5G definition of a slot.

Proposal 4: Support only normal CP length with 14 OFDM symbols per slot.

# SCS, Maximum Channel bandwidth, FFT size

Subcarrier spacing, FFT size and Channel bandwidth (CBW) are connected to each other. Table 1 below shows our proposal for SCS, CBW (Channel Bandwidth) and FFT size for different frequency ranges, and in the following we elaborate on the justifications for each choice of combination of SCS, CBW, and FFT sizes.

Table 1. Subcarrier spacing, FFT size and Maximum CBW Scenarios for 6G

| - FR | - Range | - SCS (kHz) | - Required FFT size | - Max CBW (MHz) |
| --- | --- | --- | --- | --- |
| - FR1 | - 400 MHz – 6.425 GHz | - 15 (FDD) | - 8192 | - 100 |
|  |  | - 30 (TDD) | - 8192 | - 200 |
|  | - 6.425 – 7.125 GHz | - 30* | - 8192 | - 200 |
|  |  | - 60* | - 4096 | - 200 |
| - New FR(s) | - 7.125 – X† GHz | - 30* | - 16384 | - 400 |
|  |  | - 60* | - 8192 | - 400 |
|  | - X† – 24.25 GHz | - 60* | - 8192 | - 400 |
|  |  | - 120* | - 8192 | - 800 |
| - FR2-1 | - 24.25 – 52.6 GHz | - 120 | - 8192 | - 800 |

- * Select one SCS per sub-FR

- † 8.4 GHz ≤ X ≤ 15 GHz

- We propose FR1 [410 MHz-6.425 GHz] to remain aligned with current NR deployments. This is reasonable given that MRSS requires that SCS is aligned with NR. The only difference compared to NR is that we propose to increase the maximum FFT size from 4096 to 8192, which allows the support of 100MHz CBW with 15 kHz SCS and 200MHz CBW with 30 kHz SCS.

- 15 kHz SCS is proposed for all FDD scenarios

- 30 kHz SCS is proposed for all TDD scenarios

- FR1 [6.425-7.125 GHz], a.k.a. Upper 6 GHz (U6GHz) is expected to be a TDD-only scenario. The potential need for MRSS on this band is unclear but from hardware re-use perspective it makes sense to maximize benefit from FR1 similarities. We consider up to 200 MHz CBW, which is supported by the proposed 8k FFT with 30 kHz SCS. In Table 1 we also provide an alternative of 60 kHz SCS for this band, as it is important to ensure continuity to at least lower part of 7.125 – 24.25 GHz band, for which the SCS is less clear at this point of time.

- New frequency range(s) [7.125-24 GHz] is expected to support up to 400 MHz CBW, which requires 16k FFT for 30 kHz SCS and 8k FFT for 60 kHz SCS, respectively. MRSS requirements do not apply here given that this frequency range is not defined for NR. However, again it makes sense to benefit from FR1 similarities due to hardware implementation aspects and to simplify specifications and deployments. Given that 30 kHz is assumed as the default SCS for FR1 TDD (including U6GHz), the same SCS is preferred for the new FR for adjacent carrier deployments (including CA). However, both 30 and 60 kHz SCSs have their benefits for the lower portion of the 7.125 – 24 GHz range:

- 30 kHz: Better coverage, longer CP length, better co-existence between U6GHz and 7.125-8.4 GHz​

- 60 kHz: Improved phase noise robustness, support for higher speeds with given reference signal overhead, lower latency​

- Similarly, applying 120 kHz SCS for the upper portion of the new FR would allow for synergies with FR 2-1 deployments and implementations, but alternatively 60 kHz SCS could be also considered.

- Further discussion and evaluations are needed on the SCS targets, particularly for the 7.125–24 GHz range, including the proposed subdivision into sub-ranges for new FR, before determining the applicable SCS. At present, we prioritize supporting 30 kHz SCS for upper FR1 and the lower portion of the new FR and 120 kHz SCS for the upper portion of the new FR. We also propose that 16k FFT is considered for this new frequency range, in which case both SCSs are available up to 400 MHz BW. This is important as we see 400 MHz CBW as a relevant scenario not only for >10 GHz, but also for <10 GHz.

- The proposed subcarrier spacing for FR2-1 (24.25-52.6 GHz) is aligned with current NR deployments in this range. In our understanding it is enough to support 120 kHz SCS for FR2-1, especially considering the recommendations in 6G SID on simplification and avoiding multiple numerologies for the same band. One difference compared to NR is that we propose to increase the maximum FFT size from 4096 to 8192, which brings the maximum CBW size to the desired level of improvement compared to 5G NR.

- Proposal 5: Consider SCS and FFT size values shown in Table 1 as a starting point for 6G discussions. These can be further downselected during the 6G Study Item with priority on supporting 30 kHz SCS for upper FR1 and lower part of the new FR.

- Proposal 6: Support up to 400 MHz Channel bandwidth for the New Frequency Range(s), and up to 800 MHz Channel bandwidth for FR2-1, respectively.

- Proposal 7: Support at least 8k FFT and further consider support for 16k FFT for the appropriate bands.

# Minimum CBW size for 6G

In 5G NR, the initial access channels in DL (SS block and CORESET#0) define the bottleneck scenario for the minimum channel bandwidth (CBW) size. Until Release 17 the minimum CBW size was 5 MHz, and it turned out to be challenging to build support for the 3 MHz CBW in Release 18.

It is reasonable to assume that the initial access design will be a bottleneck for the minimum CBW​ also in 6G. Hence, we need to ensure availability of the second (narrowband) SSB/CORESET0 design for channel bandwidths below 5MHz such as 3MHz without puncturing for predefined bands in Rel-21. This should be applied for predefined scenarios with specific sync raster points in specific bands and FDD with 15kHz SCS​.

The requirements of 5G NR should be reused for 15 and 30 / 60kHz SCS in FR1 and 30 / 60 kHz in the new FR(s) otherwise, i.e. defined by 25 RBs. For 120 kHz SCS for higher bands, we suggest increasing the minimum CBW to 100MHz to allow for a large number of SSB beams by supporting 240kHz SCS SSB.

Table 2 below shows our proposal for minimum CBW (Channel Bandwidth) for different SCS.

Table 2. Minimum Channel bandwidth size for 6G

| - SCS (kHz) | - Min CBW (MHz) |
| --- | --- |
| - 15 (FDD) | - 3* |
|  | - 5 |
| - 30 (TDD) | - 10 |
| - 60 (TDD) | - 20 |
| - 120 (TDD) | - 100 |

* Supported for certain bands and sync raster points only

Proposal 8: Consider values shown in Table 2 as minimum CBW for different 6G scenarios

Proposal 9: Consider the SSB/CORESET0 design for ~3 MHz CBW with 15 kHz SCS ​for specific bands (and sync raster points).

# Conclusion

In this contribution we have discussed frame structure and numerology aspects relevant to 6GR. Based on the discussion we make the following proposals:

Observation 1: Efficient MRSS requiring the 6GR frame structure and numerology to be compatible with 5G NR

Observation 2: MRSS and other coexistence scenarios benefit from resource-element level coexistence between 5G and 6G

Proposal 1: Strive for an early decision on subcarrier spacing(s) and maximum CBW size

Proposal 2: Define subcarrier spacing as a cell and band-specific parameter, with a potential exception for SSB and PRACH

Proposal 3: Retain the 5G definition of a frame of 10ms duration and subframes of a 1ms duration, and retain the 5G definition of a slot.

Proposal 4: Support only normal CP length with 14 OFDM symbols per slot.

Proposal 5: Consider SCS and FFT size values shown in the following table as a starting point for 6G discussions. These can be further downselected during the 6G Study Item with priority on supporting 30 kHz SCS for upper FR1 and lower part of the new FR.

| - FR | - Range | - SCS (kHz) | - Required FFT size | - Max CBW (MHz) |
| --- | --- | --- | --- | --- |
| - FR1 | - 400 MHz – 6.425 GHz | - 15 (FDD) | - 8192 | - 100 |
|  |  | - 30 (TDD) | - 8192 | - 200 |
|  | - 6.425 – 7.125 GHz | - 30* | - 8192 | - 200 |
|  |  | - 60* | - 4096 | - 200 |
| - New FR(s) | - 7.125 – X† GHz | - 30* | - 16384 | - 400 |
|  |  | - 60* | - 8192 | - 400 |
|  | - X† – 24.25 GHz | - 60* | - 8192 | - 400 |
|  |  | - 120* | - 8192 | - 800 |
| - FR2-1 | - 24.25 – 52.6 GHz | - 120 | - 8192 | - 800 |

- * Select one SCS per sub-FR

- † 8.4 GHz ≤ X ≤ 15 GHz

Proposal 6: Support up to 400 MHz Channel bandwidth for the New Frequency Range(s), and up to 800 MHz Channel bandwidth for FR2-1, respectively.

Proposal 7: Support at least 8k FFT and further consider support for 16k FFT for the appropriate bands.

Proposal 8: Consider values shown in the following table as minimum CBW for different 6G scenarios

| - SCS (kHz) | - Min CBW (MHz) |
| --- | --- |
| - 15 (FDD) | - 3* |
|  | - 5 |
| - 30 (TDD) | - 10 |
| - 60 (TDD) | - 20 |
| - 120 (TDD) | - 100 |

* Supported for certain bands and sync raster points only

Proposal 9: Consider the SSB/CORESET0 design for ~3 MHz CBW with 15 kHz SCS ​for specific bands (and sync raster points).

# References

[1] 	RP-251881, “Study on 6G Radio,” NTT Docomo, June 2025.

[2]		R1-2505127, “Waveform for 6G Radio Air Interface”, Nokia, Aug. 2025.

[3]		R1-2505126, “On Evaluation Assumptions for Study of 6G Radio Air Interface,” Nokia, Aug. 2025.

[4] 		R1-1609661, “Comparison of ECP and NCP link performance with 60 kHz subcarrier spacing,” Nokia, Alcatel-Lucent Shanghai Bell

In 5G NR 200 MHz and 400 MHz are defined only for FR2