3GPP TSG-RAN WG1 Meeting #122	R1-2505218

Bengaluru, India, August 25th-29th, 2025

Agenda Item:	8.11.3

Source:	Huawei, HiSilicon

Title:	Maintenance on uplink capacity/throughput enhancement for FR1-NTN

Document for:	Discussion and Decision

Introduction

In RAN1#121 [1], agreements to support inter-slot OCC in terms of UCI multiplexing, OCC indication/configuration, and signaling were made. In this contribution, we provide our views on the remaining issues to support inter-slot OCC length 2 and 4 for PUSCH in RRC_CONNECTED.

Aspects to support inter-slot OCC with OCC length 2 and 4

  - RV cycling for CG-PUSCH

On RV cycling for inter-slot OCC, RAN1 reached the following agreement in RAN1#120bis [2]:

Agreement For OCC with CG-PUSCH, the RV sequence applied across OCC groups is RRC configured among the RV sequences defined for legacy CG PUSCH, i.e., [0,2,3,1], [0,3,0,3] or [0,0,0,0]. - Note: no new RRC parameter is needed for the above. - FFS: if OCC group dropping is later agreed, how to count RVs may need to be discussed for that case.

In clause 6.1.2.3.1 of TS38.214, the legacy mechanism for RV cycling is defined as the following:

The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant. A UE with OCC operation enabled shall consider the number of PUSCH repetition Type A transmissions, LOCC, within an [OCC group] to be similar to performing K repetitions. The same redundancy version is applied for each repetition within an [OCC group], and the configured RV sequence is applied to each [OCC group]. The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE. If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(((n-mod(n, N))/N)-1,4)+1)th value in the configured RV sequence, where N=1. If a configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at - the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1}, - any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3}, - any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8. ……

It can be found that RV applied for PUSCH transmission is only determined by transmission occasion, and is not related to whether PUSCH is dropped or not. Thus, for inter-slot OCC with CG-PUSCH, legacy mechanism should still be followed, i.e., RV applied to PUSCH should only be determined by the transmission occasions and is still counted even when PUSCH repetitions within the OCC group are dropped.

Proposal 1: When CG-PUSCH repetitions in an OCC group are dropped, RV applied to the dropped PUSCH repetitions should be counted as legacy.

Besides, it is also noted that when a configured grant configuration is configured with startingFromRV0 set to 'on', the initial transmission of a transport block may start at any of the transmission occasion of the K repetitions that are associated with RV = 0. When inter-slot OCC is applied for PUSCH repetition type A, all repetitions of an OCC group should be transmitted to maintain the orthogonality across OCC sequences. If the initial transmission of a transmit block starts at a transmission occasion other than the first one of an OCC group, the first repetition of the OCC group is not transmitted and the orthogonality among different UEs will be compromised. Thus, the initial transmission occasion should always start at the first repetition within an OCC group when inter-slot OCC is applied. A corresponding TP is proposed as the following.

Proposal 2: Adopt the following TP#1 for TS38.214 to support RV cycling and determine the initial transmission for CG-PUSCH when applying inter-slot OCC.

- Reason for change: RV cycling for CG-PUSCH with inter-slot OCC is not defined; How to determine the first transmission occasion of CG-PUSCH is not defined when applying inter-slot OCC.

- Summary of Change: Clarify that RV is still counted when PUSCH repetitions in a OCC group are dropped; The initial transmission occasion of CG-PUSCH should satisfy n mod LOCC = 1 when applying inter slot OCC.

- Consequences if not approved: RV cycling for CG-PUSCH with inter-slot OCC is not complete.

-------------------- Start of TP#1 for 38.214 V19.0.0 -------------------- 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant ## *** Unchanged parts are omitted *** The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE. If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(((n-mod(n, N))/N)-1,4)+1)th value in the configured RV sequence, where N=1. When OCC operation is enabled, if the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, for the transmission occasions in the mth OCC group, m=1,…. KLOCC, it is associated with ((m-1)mod4+1)th value in the configured RV sequence. If a configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at - the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1}, - any of the transmission occasions of the K repetitions that are associated with RV=0, or any of the transmission occasions of the K repetitions that are associated with RV=0 and satisfy n mod LOCC=1 when OCC operation is enabled, if the configured RV sequence is {0,3,0,3}, - any of the transmission occasions of the K repetitions, or any of the transmission occasions of the K repetitions that satisfy n mod LOCC=1 when OCC operation is enabled, if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8. ## *** Unchanged parts are omitted *** -------------------- End of TP#1 for 38.214 V19.0.0 --------------------

In addition, since RV0 and RV3 generally include systematic bits, dropping PUSCH with RV0 and RV3 will greatly compromise the reliability of data transmission. Network could avoid scheduling/configuring PUCCH overlapping with PUSCH repetitions of RV0 or RV3 when applying inter-slot OCC.

Proposal 3: Network could by implementation avoid UE dropping CG-PUSCH repetitions with RV0 or RV3 when applying inter-slot OCC.

  - RV cycling for DG-PUSCH

In clause 6.1.2.1 of TS38.214, the RV to be applied for the OCC group is given as the following:

…… For a PUSCH transmission scheduled by DCI format 0_1, 0_2 or 0_3 when OCC operation is not enabled, or 0_0 with CRC scrambled by TC-RNTI, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to table 6.1.2.1-2. For a PUSCH transmission scheduled by DCI format 0_1 or 0_2 when OCC operation is enabled, the redundancy version to be applied on the mth [OCC group] of the TB, where m = 0, 1, … N∙KLOCC -1, is determined according to table 6.1.2.1-2A. …..

Since there are N∙LOCC slots within each OCC group, the total number of OCC groups should be calculated as N∙KN∙LOCC, i.e. KLOCC, rather than N∙KLOCC. Thus, a TP shown in the following is proposed for TS38.214.

Proposal 4: Adopt the following TP#2 for TS38.214 to determine the total number of OCC groups when applying RV cycling for DG PUSCH with inter-slot OCC.

- Reason for change: The total number of OCC groups in a PUSCH transmission with repetitions is not correct when applying RV cycling for DG PUSCH with inter-slot OCC.

- Summary of Change: The total number of OCC groups is corrected as KLOCC.

- Consequences if not approved: The total number of OCC groups in a PUSCH transmission with repetitions is not correct.

-------------------- Start of TP#2 for 38.214 V19.0.0 -------------------- 6.1.2.1 Resource allocation in time domain ## *** Unchanged parts are omitted *** For a PUSCH transmission scheduled by DCI format 0_1, 0_2 or 0_3 when OCC operation is not enabled, or 0_0 with CRC scrambled by TC-RNTI, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to table 6.1.2.1-2. For a PUSCH transmission scheduled by DCI format 0_1 or 0_2 when OCC operation is enabled, the redundancy version to be applied on the mth [OCC group] of the TB, where m = 0, 1, … N∙KLOCC -1, is determined according to table 6.1.2.1-2A. ## *** Unchanged parts are omitted *** -------------------- End of TP#2 for 38.214 V19.0.0 --------------------

Applicability of inter-slot OCC for TN

In RAN1#121, several companies proposed that inter-slot OCC with PUSCH repetition type A can also be applied for TN. In this section, simulation results of occ-length = 2 are provided to evaluate the performance of inter-slot OCC in TN for PUSCH of VoIP. Link level evaluation assumptions are given in Appendix.

With OCC-length 2, the performance of inter-slot OCC in TN under the channel model of TDL-C (NLoS) for different maximum differential CFO is shown in Figure 1. Under the maximum differential CFO of 200Hz shown in Figure 1(a), it can be observed that inter-slot OCC can hardly work in TN when the number of repetitions is 4. Even when the number of repetitions increases to 8, the gap of required SNR at 2% BLER between inter-slot OCC and single UE w/o OCC is 12.4dB.

- Maximum differential CFO of 200Hz		(b) Maximum differential CFO of 100Hz

Figure 1 Performance of inter-slot OCC2 in TN (TDL-C, NLoS).

Even with a smaller value of the maximum differential CFO, i.e., 100Hz, inter-slot OCC still can hardly work in TN when the number of repetitions is 4, as shown in Figure 1(b). And a degradation loss of 6dB between inter-slot OCC and single UE is observed at 2% BLER when the number of repetitions increases to 8. Thus, it can be noted that inter-slot OCC generally requires a large number of repetitions and a high SNR if applied in TN. In addition, QC also proposed in [3] and [4] that the performance of inter-slot OCC is not well when the number of repetitions is small, whereas it is typical that a small number of repetitions is adopted in TN. Therefore, inter-slot OCC is not suitable to be applied in TN.

Observation 1: inter slot OCC is not suitable in NLOS scenarios in TN.

Conclusion

In this contribution, the remaining aspects to support inter-slot OCC are discussed. The observations and proposals are summarized as the following:

Proposal 1: When CG-PUSCH repetitions in an OCC group are dropped, RV applied to the dropped PUSCH repetitions should be counted as legacy.

Proposal 2: Adopt the following TP#1 for TS38.214 to support RV cycling and determine the initial transmission for CG-PUSCH when applying inter-slot OCC.

- Reason for change: RV cycling for CG-PUSCH with inter-slot OCC is not defined; How to determine the first transmission occasion of CG-PUSCH is not defined when applying inter-slot OCC.

- Summary of Change: Clarify that RV is still counted when PUSCH repetitions in a OCC group are dropped; The initial transmission occasion of CG-PUSCH should satisfy n mod LOCC = 1 when applying inter slot OCC.

- Consequences if not approved: RV cycling for CG-PUSCH with inter-slot OCC is not complete.

Proposal 3: Network could by implementation avoid UE dropping CG-PUSCH repetitions with RV0 or RV3 when applying inter-slot OCC.

Proposal 4: Adopt the following TP#2 for TS38.214 to determine the total number of OCC groups when applying RV cycling for DG PUSCH with inter-slot OCC.

- Reason for change: The total number of OCC groups in a PUSCH transmission with repetitions is not correct when applying RV cycling for DG PUSCH with inter-slot OCC.

- Summary of Change: The total number of OCC groups is corrected as KLOCC.

- Consequences if not approved: The total number of OCC groups in a PUSCH transmission with repetitions is not correct.

Observation 1: inter slot OCC is not suitable in NLOS scenarios in TN.

- References

Chair’s notes for RAN1#121, St Julian’s, Malta, May 19-23, 2025.

Chair’s notes for RAN1#120bis, Wuhan, China, April 7th-11th, 2025.

R1-2408869, NR-NTN uplink capacity/throughput enhancement, Qualcomm.

R1-2410497, NR-NTN uplink capacity/throughput enhancement, Qualcomm.

- Appendix: Link level evaluation assumption

Table 1 Simulation assumption for PUSCH with VoIP

|  | Parameter | Value |
| --- | --- | --- |
| Channel and impairments | carrier frequency | 2GHz |
|  | Channel model | TDL-C The channels from different UE are independent. |
|  | Timing error | Random selection from [-29Ts, +29Ts] for UEs Timing drift is negligible. |
| Transmitter | SCS | 15KHz |
|  | Waveform | DFT-s-OFDM |
|  | Frequency hopping | w/o frequency hopping |
|  | MIMO scheme | 1T1R, |
|  | OCC sequence | Table 6.3.2.6.3-1/2 in TS38.211 for intra symbol OCC |
|  | Velocity of UE | 3km/h |
|  | Number of PRBs | 2 |
|  | DMRS configuration | DMRS for PUSCH with transform precoding DMRS positions for single-symbol DMRS for PUSCH mapping type A defined in Table 6.4.1.1.3 with ld=14, l0=2 and pos1 in [38.211]. |
|  | DMRS port | 1000, 1001 |
|  | PUSCH | PUSCH mapping type A with 12 OS |
|  | TBS | 208bits |
|  | MCS | MCS 11 in MCS Table 6.1.4.1-2 in [TS38.214] |
|  | OCC length | 2 |
|  | Number of UEs | 2 with same transmit power |
| KPI | BLER per UE | SNR @2% BLER |
|  | Aggregated throughput | Total throughout of 2 UEs multiplexed |