3GPP TSG RAN WG1 #122	R1-2505485

Bengaluru, India, August 25th–29th, 2025

Title:                   Discussion on maintenance of AI CSI prediction

Source:              ZTE Corporation, Sanechips

Agenda item:     8.1.3

Document for:   Discussion/Decision

Introduction

In this contribution, we provide our views on the maintenance of AI CSI prediction regarding monitoring report issues. In RAN1#121 meeting, the following conclusions and agreements [1] were achieved for Artificial Intelligence (AI)/Machine Learning (ML) CSI prediction:

Agreement For CSI prediction using UE-side model, for data collection for training, reportQuantity is set to ‘none-CSI-r19’. Agreement For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook, - For PU occupancy, support - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or N is reported by UE - OCPU=0 or M is reported by UE - Note: Detailed values of N and M can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied - For occupancy duration of CPU and APU, - the same occupancy duration is used if both CPU and APU are reported non-zero value - if associated monitoring report is not configured, reuse following legacy occupancy duration - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP is indicated by UE capability. - Note: Detailed values of KP can be further discussed in UE feature. - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. - The total number of dedicated AI/ML PU for AI/ML is reported by UE capability Agreement For CSI prediction using UE-side model, for inference, regarding active resource/port counting, - Reuse legacy active resource/port counting at least if associated monitoring report is not configured Agreement For CSI prediction using UE-side model, for inference, in addition to legacy Z/Z’ for doppler codebook, UE may report the value of t per SCS - Detailed value of t can be discussed in UE feature Agreement For CSI prediction using UE-side model, for UE assisted performance monitoring, - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - For monitoring report type, semi-persistent and aperiodic are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported. Agreement - For CSI prediction using UE-side model, for UE assisted performance monitoring, - If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report; - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report - If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report Agreement For CSI prediction using UE-side model, for data collection for training, - OCPU=1 - the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion. Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. v is the value of the reported RI in the associated inference report.

Although RAN1#121 meeting has handled the majority of the WI for AI CSI prediction, there are important issues remained to be resolved during the maintenance phase in the next few meetings. The issues are identified in two major categories. The first category is the clarification for agreements in the past meetings, such as the definition of SGCS for wideband granularity. The second is to handle the unresolved issues, such as monitoring triggering occasion for semi-persistent monitoring report.

Performance monitoring for AI CSI prediction

Discussions about the wideband SGCS report

In RAN1#120b meeting, companies agreed to use the SGCS as the performance metric for AI CSI prediction as defined in the following agreement [2]. It can be directly observed from the agreement that the SGCS is defined per layer, per subband and per prediction instance.

Agreement For the definition of SGCS, - For a given layer l∈{1,…,v}, subband n3∈{1,…,N3}, and prediction instance n4∈1,…,N4, SGCS is defined as SGCSn4,ln3=wn4,ln3Hwn4,ln3wn4,ln3wn4,ln32 where wn4,in3 is the predicted precoder represented by PMI used for inference for l-th layer, n3-th subband, n4-th prediction instance, and wn4,in3 is the precoder represented by PMI used for ground-truth CSI for l-th layer, n3-th subband, n4-th prediction instance Note: How to handle layer mapping mismatch, if any, is up to UE implementation.

In RAN1#121 meeting, to resolve the reporting content issue for SGCS, companies agreed the following agreement. In this agreement, the SGCS should be reported with wideband granularity.

Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1

After RAN1#121 meeting, in the latest version of TS 38.214 (V19.0.0) [3], the specification defines a new SGCS with wideband granularity, which is not aligned with the agreement for SGCS definition in RAN1#120b meeting.

TS 38.214, Section 5.2.1.4.6 CSI-PAI reporting - for k ∈1,2,…,v, v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, SGCSk1=wkHwkwk.wk2, SGCSk2=wkHwkwk.wk2, where wk is the predicted precoder represented by predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, and for S-th time instance corresponding to the report of the first Reporting Setting, wk is the precoder represented by non-predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, based on the channel measurement corresponding to the second Reporting Setting, and wk is the precoder represented by PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting.

Reason for change:

The ambiguity of SGCS definition is observed between these two agreements regarding the frequency-domain granularity. The agreement in RAN1#120b clearly states that the SGCS is determined on a subband level, while the agreement in RAN1#121 meeting states that the SGCS report is generated on a wideband level. As far as we are concerned, the two agreements have no ambiguity since the first one is related to the definition of a SGCS and the second one is related to the SGCS report. And the specification should capture both agreements with no conflicts. However, as can be concluded from the latest TS 38.214, the definition of SGCSki is not aligned with the agreement made in RAN1#120b meeting for the definition of SGCS because of the ambiguity in defining a wideband SGCS. In such cases, two different interpretations can be obtained regarding how to determine the wideband SGCS, including,

- Option 1: Determine the SGCS firstly as RAN1#120b agreement states, i.e., determining a SGCS per layer, per subband and per prediction instance. Then determine the wideband SGCS report by combining the subband SGCS. The SGCS report is then defined as follows:

SGCSreport=n3pn3wn4,ln3Hwn4,ln3wn4,ln3wn4,ln32,

where the weight vector pn3 can be configured by the gNB or can be fixed, such as pn3=1/N, where N denotes the number of subbands UE decides to generate a wideband SGCS report.

- Option 2: Determine a precoder per layer, per subband and per prediction instance. Then determine the wideband SGCS report by firstly determining a wideband precoder and then the wideband SGCS. The SGCS report is then defined as follows:

SGCSreport=wn4,lHwn4,lwn4,lwn4,l2,

where wn4,l and wn4,l is determined by

wn4,l=n3pn3wn4,ln3, wn4,l=n3pn3wn4,l,

where the weight vector pn3 can be configured by the gNB or can be specified as fixed, such as pn3=1/N, where N denotes the number of subbands UE decides to generate a wideband SGCS report.

It should be further clarified that the SGCS is calculated as option 1 because option 1 eliminates any ambiguity in the specification. Besides, option 2 is not aligned with the agreement from RAN1#120b by defining a new SGCS calculation with no discussions or agreements.

Observation 1: Current definition of wideband SGCS report causes ambiguity without clarifying the definition of a wideband SGCS.

Summary of change: Introduce the explicit definition of wideband SGCS in the specification.

Consequences if not approved: Potential conflict or ambiguity with approved agreement.

Proposal 1: For monitoring report, support option 1 for wideband SGCS definition and report.

Text Proposal 1:

TS 38.214, Section 5.2.1.4.6 CSI-PAI reporting *** Unchanged parts are omitted *** - for k ∈1,2,…,v, v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, SGCSk1=1Nn=0N−1SGCSk,n1, SGCSk2=1Nn=0N−1SGCSk,n2, where SGCSk,n1 and SGCSk,n2 are subband SGCS values, each of which can be calculated as, SGCSk,n1=wk,nHwk,nwk,n.wk,n2, SGCSk,n2=wk,nHwk,nwk,n.wk,n2, where N is the total number of all the subbands configured by csi-ReportingBand. wk,n is the predicted precoder represented by predicted PMI for k-th layer, n-th subband, and for S-th time instance corresponding to the report of the first Reporting Setting, wk,n is the precoder represented by non-predicted PMI for k-th layer, n-th subband, based on the channel measurement corresponding to the second Reporting Setting, and wk is the precoder represented by PMI for k-th layer, n-th subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. SGCSk1=wkHwkwk.wk2, SGCSk2=wkHwkwk.wk2, where wk is the predicted precoder represented by predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, and for S-th time instance corresponding to the report of the first Reporting Setting, wk is the precoder represented by non-predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, based on the channel measurement corresponding to the second Reporting Setting, and wk is the precoder represented by PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting.

Discussions about the triggering occasion for SP monitoring report

In RAN1#121 meeting, companies studied the triggering issue for monitoring report for monitoring report associated with an aperiodic report and came to the following agreement.

Agreement - For CSI prediction using UE-side model, for UE assisted performance monitoring, - If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report; - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report - If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report

Besides, another agreement regarding supported types for UE assisted performance monitoring is also made, such that,

Agreement For CSI prediction using UE-side model, for UE assisted performance monitoring, - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - For monitoring report type, semi-persistent and aperiodic are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported.

It is worth noting that, the two agreements above do not capture the signaling for triggering monitoring report associated with a semi-persistent inference report, which can be further decomposed into the following cases:

- Case 1: AP monitoring report associated with a SP inference report with SP/P inference measurement resources;

- Case 2: SP monitoring report associated with a SP inference report with SP/P inference measurement resources;

Observation 2: The only procedure captured in latest specification is for monitoring report associated with an AP inference report. But monitoring report associated with SP inference report is supported in agreement.

Summary of change: Specification supports concrete signaling for SP monitoring report.

Consequences if not approved: The gNB does not how to trigger a SP monitoring report. And the UE does not know how to act to the potential signaling indicating a SP monitoring report.

Proposal 2: Further study and discuss the specification support for monitoring report associated with SP inference report.

Conclusion

Overall, we have the following proposals for AI CSI prediction maintenance.

For disambiguation for AI CSI prediction, for monitoring report with wideband SGCS:

Observation 1: Current definition of wideband SGCS report causes ambiguity without clarifying the definition of a wideband SGCS.

Proposal 1: For monitoring report, support option 1 for wideband SGCS definition and wideband SGCS report.

Text Proposal 1:

TS 38.214, Section 5.2.1.4.6 CSI-PAI reporting *** Unchanged parts are omitted *** - for k ∈1,2,…,v, v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, SGCSk1=1Nn=0N−1SGCSk,n1, SGCSk2=1Nn=0N−1SGCSk,n2, where SGCSk,n1 and SGCSk,n2 are subband SGCS values, each of which can be calculated as, SGCSk,n1=wk,nHwk,nwk,n.wk,n2, SGCSk,n2=wk,nHwk,nwk,n.wk,n2, where N is the total number of all the subbands configured by csi-ReportingBand. wk,n is the predicted precoder represented by predicted PMI for k-th layer, n-th subband, and for S-th time instance corresponding to the report of the first Reporting Setting, wk,n is the precoder represented by non-predicted PMI for k-th layer, n-th subband, based on the channel measurement corresponding to the second Reporting Setting, and wk is the precoder represented by PMI for k-th layer, n-th subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. SGCSk1=wkHwkwk.wk2, SGCSk2=wkHwkwk.wk2, where wk is the predicted precoder represented by predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, and for S-th time instance corresponding to the report of the first Reporting Setting, wk is the precoder represented by non-predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, based on the channel measurement corresponding to the second Reporting Setting, and wk is the precoder represented by PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting.

For specification support for triggering monitoring report associated with SP inference report:

Observation 2: The only procedure captured in latest specification is for monitoring report associated with an AP inference report. But monitoring report associated with SP inference report is supported in agreement.

Proposal 2: Further study and discuss the specification support for monitoring report associated with SP inference report.

Reference

- R1-2503139, Summary#5 of 9.1.3, RAN1#120b.

- R1-2504939, Summary#6 of 9.1.3, RAN1#121.

- 3GPP TS 38.214 V19.0.0, Physical layer procedures for data (Release 19).