3GPP TSG RAN WG1 #122                         			   		R1-2505829

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.1.2

Source:	InterDigital, Inc.

Title:	Remaining issues on AI/ML based positioning

Document for:	Discussion and Decision

# Introduction

RAN1 completed Release 19 AIML positioning in RAN1#121 [1]. In this contribution, we discuss the remaining issues for AIML based positioning. The discussion in this contribution is based on the moderator’s document [2] which is to be used as the basis for further discussion in RAN1#122.

# Remaining issues for RRC parameters

Explicit or implicit indication of TRP locations

In RAN1#121, the following agreement was made to support both explicit and implicit indication of TRP locations.

Working Assumption For AI/ML based positioning Case 1, regarding info #7 in the assistance information from legacy UE-based DL-TDOA, it can be provided as in legacy UE-based DL-TDOA or implicitly. Agreement Above Working Assumption is confirmed.

According to the agreement, an explicit indication of the TRP locations is supported for Case 1. The UE can obtain explicit or implicit information about TRP location for Case 1. Thus, the following proposal is made.

Proposal 1: Info #1 ~ Info #15 in Table 8.12.2.1.0-1 in 38.305 should be provided in assistance information in LPP for Case 1 AIML based positioning and TRP locations can be indicated explicitly or implicitly.

In RAN1#121, in addition to the confirmed working assumption above, the following agreement was made about the details of an implicit indication of TRP locations.

Agreement For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits)

Therefore, an associated ID, used as an implicit indication of TRP locations, should also be supported in the LPP.

Proposal 2: For Case 1 AIML based positioning, include an associated ID in assistance information in TS 37.355.

It should also be noted that RAN2 is discussing details of the associated ID in POST130][025][AI PHY] [3].

Part B for Case 3b

In RAN1#121, the following agreements were made regarding Case 3b.

Agreement For AI/ML based positioning in Case 3b, if Part B is sent via LPP from UE to LMF, the time stamp of Part B follows the legacy IE measurementReferenceTime-r16, i.e., it is up to UE to make a choice between the following two: - NR-TimeStamp - UTCTime Conclusion For training data collection of AI/ML based positioning Case 3b, from RAN1 perspective, if the label data of location is generated by UE and transferred from UE to LMF, label and quality indicator of label can be provided by reusing existing IEs, legacy procedure, and legacy UE capability. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

In [2], it is indicated that the ground truth and associated timestamp reported by the UE may have an impact on the RRC parameter for Case 3b. As described in TS 37.355 (e.g., NR-DL-TDOA-LocationInformation), the UE can report its location and corresponding timestamp under UE based positioning methods. As stated in the above agreement, the UE can choose between NR-TimeStamp-r16 and UTCTime for reporting.

Therefore, for Case 3b, there is no need to specify, in TS 37.355, the report of the ground truth and corresponding timestamp from the UE. The UE can provide the ground truth to the LMF without any additional specification impacts as the UE can be configured with existing UE-based positioning methods (e.g., UE based DL-TDOA) while the LMF is performing AIML based positioning. The following observation and proposal are made.

Observation 1: There is no need to specify reporting of Part B for Case 3b as reporting of the ground truth and associated timestamp for Case 3b is already supported by the UE based positioning methods in TS 37.355.

Proposal 3: For Case 3b, there is no impact to RRC parameters related to the content of the UE report when the UE reports the ground truth to the LMF.

# Conclusion

In this contribution, the following proposals are made.

Proposal 1: Info #1 ~ Info #15 in Table 8.12.2.1.0-1 in 38.305 should be provided in assitane information in LPP for Case 1 AIML based positioning and TRP locations can be indicated explicitly or implicitly.

Proposal 2: For Case 1 AIML based positioning, include an associated ID in assistance information in TS 37.355.

Observation 1: There is no need to specify reporting of Part B for Case 3b as reporting of the ground truth and associated timestamp for Case 3b is already supported by the UE based positioning methods in TS 37.355.

Proposal 3: For Case 3b, there is no impact to RRC parameters related to the content of the UE report when the UE reports the ground truth to the LMF.

# References

RAN1 chair notes, RAN1#121, St Julian’s, Malta, May, 2025.

R1-2505082, “Collection of Rel-19 AI/ML positioning higher layer parameters list Post RAN1#121,” Moderator (Ericsson), RAN1#121, St Julian’s, Malta, May, 2025.

R2-2505703, “Summary of [POST130][025][AI PHY] 37.355 CR (Qualcomm),” Qualcomm Incorporated (Rapporteur), Bengaluru, India, Aug., 2025.