3GPP TSG RAN WG1 #122									R1-2505928

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.1.1

Source:	NEC

Title: 	Remaining issues on specification support for beam management

Document for:	Discussion and Decision

- Introduction

Rel-19 AI/ML based beam management have been specified [1][2]. In this contribution, we provide our views on the remaining issues. More specifically, Section 2 discusses issues requiring new agreements, and Section 3 provides text proposals for corrections to current TS 38.212 V19.0.0 [1] and TS 38.214 V19.0.0 [2].

- Remaining issues

  - Issue#1: assumed Tx power for predicted RSRP

Predicted RSRP is used in current TS, for the predicted RSRP, the assumed Tx power is still not defined. It could be a problem if the resources in Set B/Set A have different Tx power, because under different Tx power assumptions, predicted RSRP shall be different too. The simplest solution is to mandate the same transmit power for all beams in Set B/Set A, however, the obvious issue of this approach is the loss of flexibility.

In the legacy CSI framework, powerControlOffsetSS is configured per resource and ss-PBCH-BlockPower is provide per cell, while for AI/ML BM cases, the Set A resources need to be configured according to current spec. The Tx power can be assumed based on the configured powerControlOffsetSS of the resource corresponding to the predicted beam in Set A.

For predicted RSRP, the Tx power is assumed based on the configured powerControlOffsetSS of the resource corresponding to the predicted beam in Set A.

  - Issue#2: deactivation of monitoring report

For a monitoring report, if its associated inference report is stopped during performance monitoring, for example, the CSI report configuration of the associated inference report is deactivated or released by the NW, UE will no longer be able to obtain the associated inference results required for calculation of the performance metric. At this time, corresponding performance monitoring behaviors should be defined. From our perspective, given that the associated inference results are not accessible, the monitoring report could also be stopped. On the other hand, considering that UE has already collected some monitoring instances (e.g., within a monitoring window), in order to avoid unnecessary waste, UE could also calculate and report performance metrics based on the monitoring instances already collected.

If inference report associated with monitoring report is stopped during monitoring, the following monitoring behaviors needs to be considered for the monitoring report:

The monitoring report is stopped.

UE calculate (and report) performance metric based on the monitoring instances already collected.

- Text Proposals

  - TS 38.212

    - TP1: mapping order for inference report for NW sided model

Reason for change:

1. The follow agreement has not been reflected in the Table 6.3.1.1.2-8G in TS 38.212 [1], which causes the missing of conditions.

2. nrofReportedRS is not aligned with the parameter name nrofReportedRS-r19 used in TS 38.214 [2]

Agreement @ RAN1#117 For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

Based on the agreement, we have the following text proposal.

Adopt the following TP to TS 38.212 V19.0.0.

<Unchanged part omitted>

Table 6.3.1.1.2-8G: Mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP reporting, if nrofReportedRS-r19 is configured

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1 as in Table 6.3.1.1.2-6 |
|  | CRI or SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | CRI or SSBRI # M as in Table 6.3.1.1.2-6, if reported |
|  | RSRP #1 as in Table 6.3.1.1.2-6 |
|  | Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | Differential RSRP # M as in Table 6.3.1.1.2-6, if reported |
| NOTE: The value of M is configured by the higher layer parameter nrofReportedRS-r19. If M=KsCSI-RS or M=KsSSB, CRI or SSBRI #2, …, CRI or SSBRI #M are not reported. |  |

<Unchanged part omitted>

  - TS 38.214

    - TP2: restriction of the number of resources in a resource set for Set A

Reason for change:

The up to 16 resources restriction ([2], highlighted as below) shall only apply to the resource set for measurement (Set B) but not the resource set for prediction (Set A). Otherwise, the Set A is too small for the AI/ML BM use cases.

5.2.1.4.2 Report quantity configurations <Omitted> If the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-SINR', 'none', 'cri-RSRP- Index', 'cri-SINR- Index, 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19', or ‘rs-pai-r19', and the CSI-ReportConfig is linked to a resource setting configured with the higher layer parameter resourceType set to 'aperiodic', then the UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within the resource setting. <Omitted>

We have the following text proposal.

Adopt the following TP to TS 38.214 V19.0.0.

5.2.1.4.2	Report quantity configurations

<Unchanged part omitted>

If the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-SINR', 'none', 'cri-RSRP- Index', 'cri-SINR- Index, 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19', or ‘rs-pai-r19', and the CSI-ReportConfig is linked to a resource setting configured with the higher layer parameter resourceType set to 'aperiodic', then the UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set except the resource set configured by resourcesForSetA contained within the resource setting.

<Unchanged part omitted>

- Conclusion

In this contribution, we provided our views on remaining issues on specification support for beam management, and we have the following proposals which may require new agreements.

Proposal 1:	For predicted RSRP, the Tx power is assumed based on the configured powerControlOffsetSS of the resource corresponding to the predicted beam in Set A.

Proposal 2:	If inference report associated with monitoring report is stopped during monitoring, the following monitoring behaviors needs to be considered for the monitoring report:

−	The monitoring report is stopped.

−	UE calculate (and report) performance metric based on the monitoring instances already collected.

Meanwhile, we have the following proposals which provides TPs for corrections of current TS.

Proposal 3:	Adopt the following TP to TS 38.212 V19.0.0.

<Unchanged part omitted>

Table 6.3.1.1.2-8G: Mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP reporting, if nrofReportedRS-r19 is configured

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1 as in Table 6.3.1.1.2-6 |
|  | CRI or SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | CRI or SSBRI # M as in Table 6.3.1.1.2-6, if reported |
|  | RSRP #1 as in Table 6.3.1.1.2-6 |
|  | Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | Differential RSRP # M as in Table 6.3.1.1.2-6, if reported |
| NOTE: The value of M is configured by the higher layer parameter nrofReportedRS-r19. If M=KsCSI-RS or M=KsSSB, CRI or SSBRI #2, …, CRI or SSBRI #M are not reported. |  |

<Unchanged part omitted>

Proposal 4:	Adopt the following TP to TS 38.214 V19.0.0.

5.2.1.4.2	Report quantity configurations

<Unchanged part omitted>

If the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-SINR', 'none', 'cri-RSRP- Index', 'cri-SINR- Index, 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19', or ‘rs-pai-r19', and the CSI-ReportConfig is linked to a resource setting configured with the higher layer parameter resourceType set to 'aperiodic', then the UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set except the resource set configured by resourcesForSetA contained within the resource setting.

<Unchanged part omitted>

- References

- TS 38.212 V19.0.0

- TS 38.214 V19.0.0