3GPP TSG RAN WG1 #122	R1-2505968

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	11.3.2

Source: 	Fujitsu

Title:	Discussion on frame structure for 6GR

Document for:		Discussion and decision

Introduction

This contribution provides our views on frame structure of 6GR air interface based on the highlighted objectives of the SID[1].

(2) Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1

Discussion

Numerology

General aspects

According to the SID, a single numerology should be applied within the same band or sub-range. However, it remains unclear whether this principle applies to all signals/channels or only a subset. In our understanding, at least the PRACH should be allowed to use a different SCS from other signals/channels. This is because long PRACH formats are essential for wide coverage and may require SCS values such as 1.25 kHz or 5 kHz, which are not used by other signals/channels.

d) Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

Proposal 1: RAN1 to clarify:

- For 6GR, the signals/channels other than PRACH should use the same numerology for the same band/sub-range.

The 6GR study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). In this case, the numerologies for FR1 and FR2-1 in NR could be the starting point for 6GR. Meanwhile, the aligned numerology between NR and 6GR is also beneficial for efficient MRSS.

Table 1 shows the numerologies supported for FR1 and FR2-1 in NR and their corresponding applicable cases (note that it does not involve PRACH). 60 kHz with extended CP is not supported for the always-on signals/channels like SSB, SIB1. 240 kHz is only supported by SSB.

Table 1: Numerologies for FR1 and FR2-1 in NR

| Cyclic Prefix (CP) | subcarrier spacing (SCS) [kHz] | Applicable frequency range for SSB | Applicable frequency range for SIB1 broadcast | Applicable frequency range for BWP |
| --- | --- | --- | --- | --- |
| normal | 15 | FR1 | FR1 | FR1 |
| normal | 30 | FR1 | FR1 | FR1 |
| normal | 60 | / | FR2-1 | FR1, FR2-1 |
| extended | 60 | / | / | FR1, FR2-1 |
| normal | 120 | FR2-1 | FR2-1 | FR2-1 |
| normal | 240 | FR2-1 | / | / |

Among these numerologies, SCS of {15, 30, 60, 120} kHz are supported by all signals/channels in NR and should also be supported in 6GR. However, the extended CP should not be supported, as it would reduce spectral efficiency and introduce unnecessary design complexity, particularly given the lack of real-world demand. For instance, the extended CP has yet to be commercially adopted in 5G NR.

Proposal 2: For 6GR, support SCS of {15, 30, 60, 120} kHz with the normal CP for physical signals/channels other than PRACH.

- Extended cyclic prefix is not supported.

Numerology in 6G MRSS with 5G NR

The flexibility in 5G NR in terms of SCS configuration introduces significant complexity as the different SCS can be applied in each physical channel. The ability to configure different SCS values can inevitably lead to consider lots of possible scenarios. In 5G discussion, we have experienced such complexity can hinder introducing of new features and the efficient discussion given the limited TU. In the context of MRSS scenarios, the complexity caused by the multiple SCS configurations between RATs in a resource sharing band can become particularly challenges. The need to take into account for all possible cases between different RATs can lead to significant efforts in standardization discussion, and test case development, too.

To mitigate this complexity and ensure to design a simpler 6G radio system, we propose that a single SCS between 5G and 6G should be applied to the frequency band operating in MRSS scenarios. This approach can help to reduce the complexity of 6G radio system design.

Proposal 3: A single SCS between 5G and 6G should be applied to the frequency band operating in MRSS scenarios.

Frame structure

General aspects

In NR Rel-15, a unified frame structure is used for both FDD and TDD operations. In time domain, the frame structure in NR as shown in Figure 1.

Figure 1: Frame structure in NR[2]

In a TDD band, the time resource is split between downlink and uplink. The time duration for the uplink may be limited e.g. subject to regulations, and it would result in reduced coverage, increased latency and reduced capacity.

In NR Rel-19, as an enhancement to resolve that issue, Sub-Band Full-Duplex (SBFD) operation is supported. It allows simultaneous downlink transmission and uplink reception in non-overlapped subbands in a TDD band. However, the design is constrained by backward compatibility. For example, it is complicated to deal with the relation with legacy symbol types, esp. flexible symbols.

For 6GR, the issue still exists. It is preferable to support (sub-band) full-duplex operation in the first release and pursue a common framework which is compatible with FDD, TDD and (sub-band) full-duplex operations. In time domain, the frame structure defined in NR can be reused and shared by FDD, TDD and (sub-band) full-duplex operations.

Proposal 4: For 6GR, support FDD, TDD and (sub-band) full-duplex operations in the first release.

- In time domain, reuse the frame structure defined in NR, including the definition of radio frame (10ms), slots per subframe (1ms), and symbols per slot.

To support FDD, TDD and (sub-band) full-duplex operations, the following three symbol types should be supported in 6GR.

- Downlink symbols: Only downlink signals/channels is allowed.

- Uplink symbols: Only uplink signals/channels are allowed.

- (Sub-band) full-duplex symbols: Simultaneous downlink and uplink signals/channels are allowed.

In addition, for the sake of MRSS and forward compatibility, 6GR should support reserved symbols in which both downlink and uplink signals/channels are prohibited. The reserved symbols can also be utilized to absorb gaps for Tx/Rx switching and in-device coexistence etc. as well.

Proposal 5: 6GR supports the following symbol types in the first release.

- Downlink symbols: Only downlink signals/channels are allowed.

- Uplink symbols: Only uplink signals/channels are allowed.

- (Sub-band) full-duplex symbols: Simultaneous downlink and uplink signals/channels are allowed.

- Reserved symbols: Both downlink and uplink signals/channels are not allowed.

In Rel-15, the framework of NR link direction indication/determination was defined, in which the link direction is indicated/determined per cell. In later releases, more types of half-duplex UEs are considered. For example, in Rel-16, the half-duplex UE which does not support simultaneous transmissions and receptions across intra-band/inter-band cells is defined, and the corresponding enhancement of link direction indication/determination is introduced. In Rel-17, the half-duplex UE not capable of simultaneous transmissions and receptions on a serving cell with paired spectrum (FDD) is defined, and the corresponding enhancement of link direction determination (collision handling rule) is specified.

Observation 1: NR defined additional types of half-duplex UEs after Rel-15. For example,

- Half-duplex UE not capable of simultaneous transmissions and receptions across intra-band/inter-band cells

- Half-duplex UE not capable of simultaneous transmissions and receptions on a serving cell with paired spectrum (FDD)

For the first release of 6GR, at least the UE duplex types defined in NR should be supported. The link direction indication/determination should be designed taking all UE duplex types into account.

Proposal 6: In the first release of 6GR, at least the UE duplex types defined in NR are supported, and the link direction indication/determination should be designed taking all UE duplex types into account.

Slot format considering MRSS

As mentioned in previous section, 5G NR utilizes three symbol types such as downlink and uplink symbols that remain fixed once configured, and flexible symbols that can be dynamically reconfigured via DCI2_0 or statically through RRC signalling. This slot format structure works effectively within single RAT environments but may introduce some challenges when multiple RATs coexist in multi-RAT Spectrum Sharing (MRSS) scenario.

In MRSS, each RAT can maintain independent control over its allocated spectrum resources, including slot format configuration. One concern arises when each RAT configure opposing link directions for the same time domain resources (e.g., symbols/slots). For instance, on a certain time domain resources where the resource is shared between 5G and 6G, when 5G NR configures downlink while 6G simultaneously configures the corresponding symbols for uplink transmission. In this case, the uplink transmission from 6G UE may interfere with 5G UE for the reception of the downlink channel. Such cross-RATs interference significantly degrades the overall network performance.

To address the interference issue, we propose to support the coordinated slot format configuration across RATs sharing spectrum, which is ensuring that all RATs configure identical link directions for time domain resources exploiting spectrum sharing.

Proposal 7: Support the coordinated slot format configuration across RATs sharing spectrum to ensure that all RATs configure same link directions on the time domain resources exploiting spectrum sharing.

Conclusion

According to the discussions above, we have the following proposals.

Proposal 1: RAN1 to clarify:

- For 6GR, the signals/channels other than PRACH should use the same numerology for the same band/sub-range.

Proposal 2: For 6GR, support SCS of {15, 30, 60, 120} kHz with the normal CP for physical signals/channels other than PRACH.

- Extended cyclic prefix is not supported.

Proposal 3: A single SCS between 5G and 6G should be applied to the frequency band operating in MRSS scenarios.

Proposal 4: For 6GR, support FDD, TDD and (sub-band) full-duplex operations in the first release.

- In time domain, reuse the frame structure defined in NR, including the definition of radio frame (10ms), slots per subframe (1ms), and symbols per slot.

Proposal 5: 6GR supports the following symbol types in the first release.

- Downlink symbols: Only downlink signals/channels are allowed.

- Uplink symbols: Only uplink signals/channels are allowed.

- (Sub-band) full-duplex symbols: Simultaneous downlink and uplink signals/channels are allowed.

- Reserved symbols: Both downlink and uplink signals/channels are not allowed.

Observation 1: NR defined additional types of half-duplex UEs after Rel-15. For example,

- Half-duplex UE not capable of simultaneous transmissions and receptions across intra-band/inter-band cells

- Half-duplex UE not capable of simultaneous transmissions and receptions on a serving cell with paired spectrum (FDD)

Proposal 6: In the first release of 6GR, at least the UE duplex types defined in NR are supported, and the link direction indication/determination should be designed taking all UE duplex types into account.

Proposal 7: Support the coordinated slot format configuration across RATs sharing spectrum to ensure that all RATs configure same link directions on the time domain resources exploiting spectrum sharing.

- References

- RP-251881, New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

- 3GPP TR 21.915 V15.0.0 (2019-09).