3GPP TSG RAN WG1 Meeting #122	R1-2506038

Bengaluru, India, 25th-29th August 2025

Agenda Item:	8.11.1

Source:	MediaTek Inc.

Title:	NR-NTN downlink coverage enhancement

Document for:	Discussion and Decision

- 1 Introduction

In RAN#106 [1], a revised WID on NR NTN enhancements was endorsed for Release 19 as copied below with SSB periodicity related objective in “bold text”:

Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

- Link level enhancements are to be specified for the following channels:

  - PDCCH at least for CSS (except for Type-3) via PDCCH repetition

  - PDSCH with Msg4 via PDSCH repetition

  - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

- System-level enhancements are to be specified for the following:

  - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

    - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

This contribution aims to discuss maintenance for DL coverage enhancements.

- 2 Inter-slot PDCCH repetitions for Type0 PDCCH CSS.

RAN#121 made the following working assumption on inter-slot repetition:

Working Assumption

Inter-slot Type-0 CSS PDCCH repetition is only applicable to the SI-RNTI, and the following rule for BD counting is defined:

- 1 BD in first slot.

- In the second slot: 2 BD in RRC connected mode

  - One BD for Type-0 CSS PDCCH repetition with SI-RNTI and one BD for other PDCCH

The Blind Decoding (BD) limit definition should ensure at least the combining of soft-bits for best PDCCH repetition performance when possible. For first slot, BD should be counted as 1 as no soft combining is expected in the early slot. For the second slot, BD could be counted as 1 or 2. There can be 2 BDs for combined PDCCH candidate for SI-RNTI and PDCCH candidate for UE-specific RNTI, as either PDCCH could be scheduled in the later slot and UE has no way of knowing this a priori. The other way would be 1 BD for combined PDCCH candidate for Type-0 CSS PDCCH repetition with SI-RNTI only. For best performance, one BD for Type-0 CSS PDCCH repetition with SI-RNTI and one BD for other PDCCH in connected mode should be assumed.

Proposal 1: Confirm RAN1#121 Working assumption on inter-slot Type-0 CSS PDCCH repetition.

# 7 Conclusion

In this contribution, the following proposal was made as follows:

Proposal 1: Confirm RAN1#121 Working assumption on inter-slot Type-0 CSS PDCCH repetition.

# 8 References

- RP-243300, Moderator (Thales), New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, Madrid, Spain, December 9-12, 2024

- R1-2504953, Moderator (Thales), FL Summary #5: NR-NTN downlink coverage enhancements, RAN1#121, St Julian’s, Malta, May 19th – 23rd, 2025