3GPP TSG RAN WG1 #122	                                              R1-2506100

Bengaluru, India, Aug 25th – 29th, 2025

Source: 	CMCC

Title:	Discussion on modulation schemes for 6GR interface

Agenda item:	11.4.2

Document for:	Discussion & Decision

- Introduction

During the 3GPP RAN#108 meeting, the SID on the 6G SI “Study on 6G Radio” was approved, including the following objectives on physical layer design for 6G radio (6GR) [1]:

… (2) Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1 …

In this meeting, 3GPP RAN1 will start to study the new radio access technology for 6GR, including modulation schemes. In this contribution, we provide some considerations on modulation schemes for 6GR from our side.

- Considerations on modulation schemes for 6GR

In this section, we briefly recall the modulation scheme adopted in 5G NR at first, which can be regarded as a baseline modulation scheme for 6GR. Then we discuss some potential modulation enhancement schemes for study purposes.

- Baseline modulation scheme for 6GR

In the 5G NR system, square 2m-QAM (m=1, 2, 4, 6, 8, 10) and π/2-BPSK modulation schemes are adopted, where the π/2-BPSK modulation is only applicable to DFT-s-OFDM to achieve lower PAPR. The details on the supported modulation schemes for different channels are summarized as follows.

- Modulation schemes for PUSCH (Copy from TS 38.211): - Modulation schemes for PDSCH (Copy from TS 38.211): - Modulation scheme for PUCCH format 0: QPSK - Modulation scheme for PUCCH format 1: BPSK or QPSK - Modulation scheme for PUCCH format 2: QPSK - Modulation scheme for PUCCH format 3 and 4: QPSK or π/2-BPSK - Modulation schemes for PDCCH: QPSK

Considering that the performance of 2m-QAM is fully justified in the 5G NR phase, it seems natural to consider the 2m-QAM as a baseline modulation scheme for the 6GR study when it is designed under OFDM waveforms.

Proposal 1: 2m-QAM defined in 5G NR can be considered as a baseline modulation scheme for the 6GR study when it is designed under OFDM waveforms.

- Potential modulation enhancements for 6GR study

Higher-order modulation is essential to meet the 6G peak data rate targets, e.g., 50Gbps outlined in the ITU recommendation document [2]. However, a performance gap of approximately 1.53dB persists between current high-order QAM modulation and the theoretical bound. Constellation shaping is a promising technique to bridge this gap, primarily through two approaches: geometric shaping [3] and probabilistic shaping [4]. In geometric shaping, a non-uniformly spaced constellation with equiprobable symbols is used, whereas in probabilistic shaping, the constellation is on a uniform grid with differing probabilities per constellation point. In addition, AI-based methods are actively explored to optimize non-uniform constellations to achieve a significant shaping gain.

Before diving into specific enhanced modulation designs for 6G, the following aspects should be carefully considered:

  - Universality over varying channel conditions

Considering the varying channel conditions in mobile communication systems, the modulation constellations should yield robust performances under diverse and dynamic channel conditions. So, the adaptability of non-uniform constellations to varying SNR, fading, and interference requires thorough evaluation.

  - Integration with channel coding

As channel coding and modulation are discussed in two separate agenda items, the potential coupling between them needs careful assessment. We need to identify if and how the modulation design influences the channel coding design.

  - Receiver implementation complexity

While 5G NR’s square/uniform QAM modulation enables low-complexity demodulation by exploiting its lattice structure, non-uniform constellations inherently increase demodulation complexity. To be implementation-friendly, low-complexity receiver algorithms compatible with shaped constellations can be studied. In other words, trade-offs between complexity and performance gains should be carefully considered.

Besides, regarding the 2m-QAM modulation used in 5G NR with uniformly distributed and equiprobable constellation points, the error vector magnitude (EVM) metric is used to validate the modulation performance. For constellation shaping modulation, the current EVM metric cannot be used straightforwardly. To validate the feasibility and performance of constellation shaping, an EVM-like metric is indispensable, which involves RAN4 workloads.

Proposal 2: The following aspects should be carefully considered for the study of modulation schemes for 6GR:

  - Universality over varying channel conditions

  - Integration with channel coding

  - Receiver implementation complexity

  - Feasibility validation via EVM-like metric with RAN4 involved

- Conclusions

In this contribution, we provide some considerations of modulation schemes for 6GR. The following proposals are made.

Proposal 1: 2m-QAM defined in 5G NR can be considered as a baseline modulation scheme for 6GR study when it is designed under OFDM waveform.

Proposal 2: The following aspects should be carefully considered for the study of modulation schemes for 6GR:

  - Universality over varying channel conditions

  - Integration with channel coding

  - Receiver implementation complexity

  - Feasibility validation via EVM-like metric with RAN4 involved

- References

- RP-251881, New SID: Study on 6G Radio, RAN#108, Prague, Czech Republic, June 9-13, 2025.

- ITU-R Recommendation M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

- Feng-Wen Sun and H. C. A. van Tilborg, “Approaching capacity by equiprobable signaling on the Gaussian channel,” in IEEE Transactions on Information Theory, vol. 39, no. 5, pp. 1714-1716, Sept. 1993.

- F. R. Kschischang and S. Pasupathy, “Optimal nonuniform signaling for Gaussian channels,” in IEEE Transactions on Information Theory, vol. 39, no. 3, pp. 913-929, May 1993.