3GPP TSG-RAN WG1 Meeting #122	R1-2506167

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Maintenance on CSI enhancements for large antenna arrays and CJT

Document for:	Discussion, Decision

# 1	Introduction

In the work item description (WID) for NR MIMO Phase 5 we have the following objectives related to CSI enhancement and CJT.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In this paper, we provide Ericsson’s views on maintenance issues related to the above objectives.

# 2	Discussion

## 2.1 Port mapping correction in 38.214 for periodic/semi-persistent NZP CSI-RS resources

In RAN1#121, RAN1 made the following agreement:

Agreement

For the Rel-19 Type-I SP and Type-II codebook refinement for P=48, 64, and 128 CSI-RS ports with K>1 aggregated NZP aperiodic CSI-RS resources for CMR, to implement the previous agreements on the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation

- In TS38.211, extend the enumeration of antenna port p=3000+p’ with p’=0 …P–1

- In TS38.214, remove the term ‘port index for CSI/PMI calculation’

However, the word ‘aperiodic’ in the above agreement was unintentional and a ‘typo’ that was not caught before the agreement was made.  In 38.214 V19.0.0 the agreed port mapping according to the above agreement is only captured for the case when NZP CSI-RS resources are aperiodic, and a different port mapping is captured for periodic and semi-persistent NZP CSI-RS resources.  In our view, port mapping method should be the same regardless of the time domain behaviour of the NZP CSI-RS resources due to following reasons:

- Capturing different port mapping methods for aperiodic NZP CSI-RS and periodic/semi-persistent NZP CSI-RS was not the intention in RAN1 when the above agreement was made, and

- The current port mapping method in 38.214 V19.0.0 for periodic/semi-persistent NZP CSI-RS assumes that CSI-RS port indices are not extended beyond 3031.  However, port mapping captured in 38.211 V19.0.0 assumes that CSI-RS port indices are extended up to 3127 in Rel-19.  Hence, the port mapping method captured in 38.214 V19.0.0 for periodic/semi-persistent NZP CSI-RS contradicts the one in 38.211 and should be revised.

We make the following proposal:

Proposal 1: For the Rel-19 Type-I SP and Type-II codebook refinement for P=48, 64, and 128 CSI-RS ports with K>1 aggregated NZP periodic and semi-persistent CSI-RS resources for CMR, to implement the previous agreements on the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation

- In TS38.211, extend the enumeration of antenna port p=3000+p’ with p’=0 …P–1

- In TS38.214, remove the term ‘port index for CSI/PMI calculation’

- In TS 38.214, align port mapping method for the cases when aggregated NZP CSI-RS resources are aperiodic, periodic, and semi-persistent.

## 2.2 Correction for cjtc-P reporting

In 38.212 V19.0.0, the following table is captured.  In the first row of the table, the field name ‘Reference CSI-RS resource index’ is used while the title refers to ‘Reference CSI-RS resource set index’.

Table 6.3.2.1.2-13: Reference CSI-RS resource set index, and phase offset for reportQuantity=cjtc-P

| Field | Bitwidth |
| --- | --- |
| Reference CSI-RS resource index | log2NTRP, where NTRP is the number of CSI-RS resources configured for the CSI report. |
| Wideband Phase offset | log2MΦ, where MΦ is configured by valueOfMPhi. |
| Subband Phase offset | log2MΦ, where MΦ is configured by valueOfMPhi. |

Since ‘Reference CSI-RS resource index’ is used in the case of ‘cjtc-P’ reporting, we propose to make the following proposal for correction in 38.212 V19.0.0:

Proposal 2: In Table 6.3.2.1.2-13 of 38.212 V19.0.0, make the following correction:

Table 6.3.2.1.2-13: Reference CSI-RS resource set index, and phase offset for reportQuantity=cjtc-P

| Field | Bitwidth |
| --- | --- |
| Reference CSI-RS resource index | log2NTRP, where NTRP is the number of CSI-RS resources configured for the CSI report. |
| Wideband Phase offset | log2MΦ, where MΦ is configured by valueOfMPhi. |
| Subband Phase offset | log2MΦ, where MΦ is configured by valueOfMPhi. |

# 3. Conclusion

- Based on the discussion above, we make the following proposals:

Proposal 1: For the Rel-19 Type-I SP and Type-II codebook refinement for P=48, 64, and 128 CSI-RS ports with K>1 aggregated NZP periodic and semi-persistent CSI-RS resources for CMR, to implement the previous agreements on the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation

- In TS38.211, extend the enumeration of antenna port p=3000+p’ with p’=0 …P–1

- In TS38.214, remove the term ‘port index for CSI/PMI calculation’

- In TS 38.214, align port mapping method for the cases when aggregated NZP CSI-RS resources are aperiodic, periodic, and semi-persistent.

Proposal 2: In Table 6.3.2.1.2-13 of 38.212 V19.0.0, make the following correction:

Table 6.3.2.1.2-13: Reference CSI-RS resource set index, and phase offset for reportQuantity=cjtc-P

| Field | Bitwidth |
| --- | --- |
| Reference CSI-RS resource index | log2NTRP, where NTRP is the number of CSI-RS resources configured for the CSI report. |
| Wideband Phase offset | log2MΦ, where MΦ is configured by valueOfMPhi. |
| Subband Phase offset | log2MΦ, where MΦ is configured by valueOfMPhi. |