3GPP TSG RAN WG1 #122			R1-2506196

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:		9.2

Source: 	Qualcomm Incorporated

Title: 	UE features for NR MIMO phase 5

Document for:	Discussion/Decision

- Introduction

This contribution includes further details for RAN1 UE features list on various Rel-19 MIMO topics.

- Enhancements for UE-initiated/event-driven beam management

For the prerequisites of UE-initiated beam reporting, certain relevant features such as the configuration of measurement resources and unified TCI states, can be inherited from legacy beam reporting (FG2-24, and FG23-1-1 or FG23-10-1).

Based on the discussion, the changes (highlighted in red) in the following UE feature group table are proposed.

Proposal 1-1: Support the following changes of UE feature groups agreed in RAN1 #120bis for UE-initiated/event-driven beam reporting.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management for Event-2 based measurement and report for Mode A | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group | FFS 2-24, and 23-1-1 or 23-10-1 | yes | n/a | UEI/ED beam report is not supported for Event-2 and Mode A | Per band | n/a | n/a | n/a | Component 4 candidate values: {1, 2, …, 64} Note For Component 4 and Component 5, an SSB can be associated with the serving cell PCI or a PCI other than the serving cell PCI | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B | 1. Support of Mode B UE-initiated/event-driven beam report 3. Supported minimum values of X symbols between the last symbol of sending first PUCCH and the first available symbol of transmission occasion of second PUSCH | 59-1-1 FFS for other potential FG(s) | yes | n/a | UEI/ED beam report Mode-B is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: [{0, 1, 2, 4, 8, 16, 32, 64, 128, 256, 512}] | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-3 | Triggering event determination via detecting ≥ M event instances for at least one new beam within a time window. | 1. Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) for at least one new beam within a time window, where M>1 | 59-1-1 FFS for other potential FG(s) | yes | n/a | Triggering event determination via detecting ≥ M event instances within a time window is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report for Mode A | 1. Support of Event-1 based measurement and report for Mode A that L1-RSRP of the current beam becomes worse than a configured threshold based on one event instance | 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-1 based measurement is not supported for Mode A | Per Band | n/a | n/a | n/a | Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report for Mode A | 1. Support of Event-7 based measurement and report for Mode A that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance 32. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively | 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-7 based measurement is not supported for Mode A | Per Band | n/a | n/a | n/a | Component 1 candidate values for Q: bitmap of size 8, the n-th bit signals support for Q=n, n = 1,2,…,8, zero means no support, 1 means support Note: The UE does not expect that the configured Q is greater than the number of the activated DL/joint TCI state(s) | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-6 | First PUCCH and second PUSCH from different PUCCH groups | Support first PUCCH and second PUSCH from different PUCCH groups | FG 59-1-1 | yes | n/a | First PUCCH and second PUSCH from different PUCCH groups is not supported | Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-7 | 1-bit condition met indication in RSRP report format for each report of CRI/SSBRI for Event-2 | Support of 1-bit condition met indication in RSRP report format for each report of CRI/SSBRI for Event-2 | FFS 59-1-1 | yes | n/a | 1-bit condition met indication in RSRP report format for each report of CRI/SSBRI is not supported for Event-2 | Per Band | n/a | n/a | n/a |  | Optional with capability signalling |

- CSI enhancement

##### Up-to-128-port Type-I/II CSI

Proposal 2-1: Support the following changes of RAN1#121-agreed UE feature groups for Up-to-128-port Type-I/II CSI.

- Note that the changes to FG 59-2-1-9 is according to a new agreement made in RAN#121 Malta.

Agreement (RAN1#121, 2025-05)

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, when configured with port subset indication (from the Rel-18 SD NES Type-1), the actual number of ports (the number of 1s in the port-subsetIndicator bitmap) can correspond to a supported value of PCSI-RS for the Rel-19 Type-I SP codebook (i.e. either 48, 64, or 128), or to a supported value of PCSI-RS for the Rel-15 Type-I SP codebook (i.e., either 2, 4, 8, 12, 16, 24, 32), respectively

- The supported number of ports for port subset of the full PCSI-RS ports, is according to UE capability

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-1-6 | CSI-RS resource time domain restriction for Type-I and Type II codebook enhancement for up to 128 ports | Support of the K CSI-RS resources configured within two slots for Type-I and Type II codebook enhancement for up to 128 ports | FFS59-2-1-1, 1a, 1b, 1c, 1d, 1e, 2, 2a, 2b, 3, 3a, 3b, 4, 4a, 5, 5a, or 5b | yes | n/a | 2-slot resource aggregation is not supported | Per-band and per-BC | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1-9 | NES SD Type1 for Rel-19 Type-I single-panel codebook | 1. Support NES SD Type1 for Rel-19 Type-I single-panel codebook 2. Supported NES SD Type1 timeline from two timeline capabilities, for Rel-19 Type-I single-panel codebook 3 Supported number of ports for CSI report subconfig | 59-2-1-1, 1a, 1b, 1c, 1d, or 1e | Yes | n/a | NES SD Type1 for Rel-19 Type-I single-panel codebook is not supported | [Per-band and per-BC] | n/a | n/a | n/a | Component 2 candidate values: - Capability 1: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2) - Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by maxi=1MPi, P/32 where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources Component 3 candidate values: One or more values from {2, 4, 8, 12, 16, 24, 32, 48, 64, 128} | Optional with capability signaling |

##### SRS port grouping

Proposal 2-1a: Support the following changes of RAN1#121-agreed UE feature groups for SRS port grouping.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR MIMO phase 5 | 59-2-1-8 | SRS port grouping | Support of SRS port grouping | FFS2-55 | YES | N/A | SRS port grouping is not supported | [Per-band and per-BC] | n/a | n/a | n/a | Candidate values: {xT8R, xT6R, both} Note: If a UE supports this FG, then the UE supports the corresponding antenna switching configurations and port groupings | Optional with capability signaling |

Proposal 2-1b: Support the following newly-proposed UE feature groups for SRS port grouping.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR MIMO phase 5 | 59-2-1-XX | Both groups of SRS ports for single-CW reception | Support UE antenna ports associated with both groups of SRS ports for single-CW reception | 59-2-1-8 | YES | N/A | UE antenna ports associated with only the first port group of the two SRS port groups are used for single-CW reception | Per-band and per-BC | N/A | N/A | N/A | Applicable to reception of PDSCH with single-CW (i.e. <= 4 layers), and non-PMI report hypotheses of rank1-to-4 | Optional with capability signaling |

##### Multi-CRI

Proposal 2-2a: Support the following changes of RAN1#121-agreed UE feature groups for Multi-CRI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-2-1 | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook | 1. The maximal supported number of CRI report M 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. | FFS2-36 | yes | n/a | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: {1,2,3,4} Component 2 candidate values: a. {2,4,8,12,16, 24, 32} b. {1,2,3,4 … 256} c. {64, …, 256, 1024} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-2-2 | Hybrid BF (CRI-based) with Rel-16 eType-II codebook | 1. The maximal supported number of CRI report M 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. | FFS16-3a | yes | n/a | Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: {1,2} Component 2 candidate values: a. {2,4,8,12,16, 24, 32} b. {1,2,3,4 … 256} c. {64, …, 256, 1024} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-2-3a | Configuration of MR always-reported resources with Rel-15 Type-I SP codebook | 1. Support MR={1,2} for hybrid BF (CRI-based) with Rel-15 Type-I SP codebook] | FFS59-2-2-1 or 59-2-2-2 | yes | n/a | Configuration of MR always-reported resources is not supported, i.e. MR=0 with Rel-15 Type-I SP codebook | Per band and per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-2-3b | Configuration of MR always-reported resources with Rel-16 eType-II codebook with R=1 | Support MR={1} for hybrid BF (CRI-based) with Rel-16 eType-II codebook with R=1 | FFS59-2-2-1 or 59-2-2-2 | yes | n/a | Configuration of MR always-reported resources is not supported, i.e. MR=0 with Rel-16 eType-II codebook with R=1 | Per band and per BC | n/a | n/a | n/a |  | Optional with capability signalling |

Proposal 2-2b: Support the following newly-proposed UE feature groups for Multi-CRI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR MIMO phase 5 | 59-2-2-XX | CSI-RS resource-specific RI restriction | Support of CSI-RS resource-specific RI restriction for Rel-19 CRI-based report | 59-2-2-1 or 59-2-2-2 | YES | N/A | RI restrictions need to be configured same for all CSI-RS resources in the resource set | Per-band and per-BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 59. NR MIMO phase 5 | 59-2-2-XX | CSI-RS resource-specific triggering offset | Support of CSI-RS resource-specific triggering offset for aperiodic CSI-RS | 59-2-2-1 or 59-2-2-2 | YES | N/A | Aperiodic triggering offsets for all CSI-RS resources in the resource set need to be same | Per-band and per-BC | N/A | N/A | N/A |  | Optional with capability signaling |

##### UE-assisted CJTC

Proposal 2-3: Support the following changes of RAN1#121-agreed UE feature groups for UE-assisted CJTC.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-3-3 | CJTC wideband PO report | 1. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC WB PO reporting 2. Supported value of scaling factor X for OCPU calculation 3. Supported slot duration for NTRP P/SP CSI-RS occaions being confined in | 2-35 | yes | n/a | CJTC PO report is not supported | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: {16, 32} Component 2 candidate values: {1, 2} Note：OCPU =X.NTRP Component 3 candidate values: {1, 2,…} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-4 | CJTC subband PO report | 1. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC SB PO reporting 2. Configured minimum subband size in resource blocks for the CJTC subband PO report 3. Supported value of scaling factor X for OCPU calculation 4. Supported slot duration for NTRP P/SP CSI-RS occaions being confined in | 2-35 | yes | n/a | CJTC subband PO report is not supported | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: {16, 32} Component 2 candidate values: {1, 2, 4, 8, 16} Component 3 candidate values: {1, 2} Note：OCPU =X.NTRP Component 4 candidate values: {1, 2,…} | Optional with capability signalling |

- Support for 3-antenna-port codebook-based transmissions

Proposal 3-1: Support the following UE feature groups for 3Tx PUSCH and SRS transmissions.

- Note: the changes with respect to the agreements made in RAN1# 120 are highlighted in red.

| 59. NR_MIMO_Ph5 | 59-3-1 | Non-codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission) 3. Maximum number of simultaneous transmitted SRS resources at one symbol | FFS | yes | n/a | Non-codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 3} Component 2 candidate values: {1,2,3} Component 3 candidate values: {1,2,3} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-3-1a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH transmission for single TRP | 1. Support the association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} simultaneously | 59-3-1 | yes | n/a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH is not supported | Per band | n/a | n/a | n/a | Component 2 candidate value: a. {2, 4, 8, 12, 16, 24, 32} b. {1 to 64} c. {2 to 256} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE | FFS | yes | n/a | Codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} Note: When configured according to Component 4, the number of ports supported by UE for transmission in an SRS resource is 3 | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3 | 3T6R Antenna switching | 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL [4. Support of 3T6R antenna switching configuration(s) as an allowing downgrading configuration of 4T8R] | FFS | yes | n/a | 3TX 3T6R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} [FFS: New component for downgrade antenna switching configurations or a new Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, or supportedSRS-TxPortSwitch to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx.] | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3-1 | Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T6R antenna switching | 59-3-3 | yes | n/a | Only maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS for 3T6R is supported | Per FS | n/a | n/a | n/a | Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3a | 3T3R antenna switching | 1. Support of 3T3R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | FFS | yes | n/a | 3TX 3T3R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, supportedSRS-TxPortSwitch or 59-3-3 to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx Note: ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports. | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3a-1 | Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T3R antenna switching | 59-3-3a | yes | n/a | Only maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS for 3T3R is supported | Per FS | n/a | n/a | n/a | Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-4 | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based | 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based | Per FS | n/a | n/a | n/a | Component 3 candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-4a | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based | Per FS | n/a | n/a | n/a | Component 3 candidate values: {1,2,3} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-5 | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-5a | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2,3} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-6 | PTRS of 3-antenna-port PUSCH transmission | Number of supported PTRS ports for PUSCH transmission | [59-3-1 or] 59-3-2 | yes | n/a | PTRS is not supported for 3TX PUSCH transmission | Per FS | n/a | n/a | n/a | Candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-7 | UL full power transmission mode of fullpower | Support of full-power Mode 0 for codebook-based for 3 Tx operation | 59-3-2 | yes | n/a | UL full power transmission mode of fullpower is not supported | Per FS | n/a | n/a | n/a | Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply | Optional with capability signalling |

- Enhancement for asymmetric DL sTRP/UL mTRP scenarios

In previous RAN1 meetings, the following UE feature groups have been agreed for asymmetric DL sTRP/UL mTRP scenarios:

| 59. NR_MIMO_Ph5 | 59-4-1a | PL offset for PUCCH/PUSCH/SRS power control for joint DL/UL TCI state(s) | Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for joint DL/UL TCI state(s) | 23-1-1 | yes | n/a | PL offset for PUCCH/PUSCH/SRS power control for joint DL/UL TCI state(s) is not supported | FFS Per Band | FFS n/a | FFS Applicable only to FR1 | FFS n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-1b | PL offset for PUCCH/PUSCH/SRS power control for separate DL/UL TCI state(s) | Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for separate DL/UL TCI state(s) | 23-10-1 | yes | n/a | PL offset for PUCCH/PUSCH/SRS power control under separate DL/UL TCI state(s) is not supported | FFS Per Band | FFS n/a | FFS n/a | FFS n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-2a | Path Loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s) | Support of applying path loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s) | 23-1-1 | yes | n/a | Applying path loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s) is not supported | FFS Per Band | FFS n/a | FFS Applicable only to FR1 | FFS n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-2b | Path Loss offset on PDCCH-order PRACH for separate DL/UL TCI state(s) | Support of applying path loss offset on PDCCH-order PRACH for separate DL/UL TCI state(s) | 23-10-1 | yes | n/a | Applying path loss offset on PDCCH-order PRACH under separate DL/UL TCI state(s) is not supported | FFS Per Band | FFS n/a | FFS n/a | FFS n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-3 | Two SRS closed-loop power control adjustment states separate from PUSCH | Support of two separate SRS closed loop indices separate from PUSCH | FFS | yes | n/a | Two separate SRS closed loop indexes is not supported | FFS Per Band | FFS n/a | FFS n/a | FFS n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-4a | Support two TAs enhancement for intra-cell beam management | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management | FFS 23-1-1 | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management is not supported | FFS Per FS | FFS No | FFS No | FFS n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-4b | Support two TAs enhancement for inter-cell beam management operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management | FFS 23-1-1a | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management is not supported | FFS Per FS | FFS No | FFS No | FFS n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-4d | PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA for inter-cell without CORESETPoolIndex | Support of PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA for inter-cell | FFS | yes | n/a | PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA for inter-cell is not supported without CORESETPoolIndex | Per FS | No | No | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-5 | Overlapping UL transmission reduction | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement | FFS | yes | n/a | Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported | Per band | n/a | n/a | n/a | Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap | Optional with capability signalling |
| NR MIMO Phase 5 | 59-4-6 | MAC-CE update of PL offset value(s) | Support of MAC-CE update of the configured PL offset value(s) | FFS | yes | n/a | MAC-CE update of the configured PL offset value(s) is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-7a | Extended value range of starting bit of block in DCI format 2_3 | Support of the extended value range of starting bit of DCI format 2_3 | FFS 8-6 | yes | n/a | The extended value range of starting bit of DCI format 2_3 is not supported | Per band | n/a | n/a | n/a | Note: The starting bit of value range extends to X=45 for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-7b | DCI format 2_3 to indicate TPC for one of two separate SRS closed loop indexes | Support DCI format 2_3 to indicate TPC for one of two separate SRS closed loop indexes. | FFS | yes | n/a | The function of DCI 2_3 indicating TPC command for one of two separate SRS closed loop indexes is not supported. | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 59. NR_MIMO_Ph5 | 59-4-8 | DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) | Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) | FFS | yes | n/a | DCI 1_1 indicating TPC command for separate SRS closed loop index(es) is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indication for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states” | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-9a | DCI format 1_1 to indicate one of two separate SRS closed loop indexes under separate DL/UL TCI state mode | Support of DCI format 1_1 to indicate one of two separate SRS closed loop index(es) under separate DL/UL TCI state mode | FFS | yes | n/a | DCI 1_1 indicating one of two separate SRS closed loop indexes under separate DL/UL TCI state mode is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-9b | DCI format 1_1 to indicate one of two separate SRS closed loop indexes under joint TCI state mode | Support of DCI format 1_1 to indicate one of two separate SRS closed loop index(es) under joint TCI state mode | FFS | yes | n/a | DCI 1_1 indicating one of two separate SRS closed loop indexes under joint TCI state mode is not supported | Per band | n/a | FR1 only | n/a |  | Optional with capability signalling |

In this section, we provide our views on the remaining FFSs for the FGs or asymmetric DL sTRP/UL mTRP and propose new components to some of them.

| 59. NR_MIMO_Ph5 | 59-4-3 | Two SRS closed-loop power control adjustment states separate from PUSCH | Support of two separate SRS closed loop indices separate from PUSCH | FFS 23-1-1 | yes | n/a | Two separate SRS closed loop indexes is not supported | Per Band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-1: The prerequisite FG of FG59-4-3 is 23-1-1.

For the support of two TAs enhancement, separate UE FGs are introduced for intra-cell beam management and inter-cell beam management in RAN1#120 meeting. While for intra-cell beam management, another UE FG is needed for sDCI mTRP since support of two TAs for intra-cell beam management doesn’t mean the UE support two TAs for both sTRP and sDCI mTRP. It is possible that the UE may support two TAs for sDCI mTRP while not support two TAs for sTRP. The UE FG on two TAs for intra-cell beam management cannot achieve this. Therefore, we propose to introduce the following UE FG for two TAs:

Proposal 4-2: Introduce the following FG for two TAs for sDCI mTRP:

| 59. NR_MIMO_Ph5 | 59-4-4c | Support two TAs enhancement for sDCI based intra-cell Multi-TRP operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for sDCI based intra-cell Multi-TRP operation | 40-1-1 | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation sDCI based intra-cell Multi-TRP operation is not supported | Per FS | No | No | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG59-4-4d, to supported PDCCH order sent by one TRP to trigger RACH procedure towards a different TRP based on CFRA for inter-cell scenario, it was agreed to reuse the “PRACH association indicator” field in DCI format 1_0 as shown below. Firstly, FG59-4-4d is needed only when two TAs are supported for inter-cell scenario. Secondly, the presence of “PRACH association indicator” field is conditioned on two TAs. Therefore, to support FG59-4-4d, the prerequisite FG should be FG59-4-4b, i.e., support two TAs enhancement for inter-cell beam management operation.

Agreement

For a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, support to reuse the DCI field ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH:

- The bit field index 0 of this field indicates the DL RS that DMRS of PDCCH order DCI is QCLed with is used as PL RS for PRACH;

- The bit field index 1 of this field is mapped to the additional PCI associated with the active TCI states and indicates the indicated SSB in this DCI is used as PL RS for PRACH:

  - In this case, the PRACH configuration associated with addition PCI is used.

- This DCI field is present when the corresponding RRC parameter is configured and multi-DCI based mTRP is not configured.

| 59. NR_MIMO_Ph5 | 59-4-4d | PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA for inter-cell without CORESETPoolIndex | Support of PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA for inter-cell | FFS 59-4-4b | yes | n/a | PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA for inter-cell is not supported without CORESETPoolIndex | Per FS | No | No | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-3: The prerequisite FG of FG59-4-4d is 59-4-4b.

For FG 59-4-5, the prerequisite is the support of two TAs. Therefore, the prerequisite FG is one of FG59-4-4a, FG59-4-4b or FG59-4-4c.

| 59. NR_MIMO_Ph5 | 59-4-5 | Overlapping UL transmission reduction | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement | FFS 59-4-4a, 59-4-4b or 59-4-4c | yes | n/a | Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported | Per band | n/a | n/a | n/a | Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-4: The prerequisite FG of FG59-4-5 is 59-4-4a or 59-4-4b or 59-4-4c.

| NR MIMO Phase 5 | 59-4-6 | MAC-CE update of PL offset value(s) | Support of MAC-CE update of the configured PL offset value(s) | FFS 59-4-1a or 59-4-1b | yes | n/a | MAC-CE update of the configured PL offset value(s) is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-5: The prerequisite FG of FG 59-4-6 is 59-4-1a or 59-4-1b.

| 59. NR_MIMO_Ph5 | 59-4-7b | DCI format 2_3 to indicate TPC for one of two separate SRS closed loop indexes | Support DCI format 2_3 to indicate TPC for one of two separate SRS closed loop indexes. | FFS 8-6 | yes | n/a | The function of DCI 2_3 indicating TPC command for one of two separate SRS closed loop indexes is not supported. | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4-6: The prerequisite FG of FG59-4-7b is FG8-6 (TPC-SRS-RNTI).

For FG59-4-8, the FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indication for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states is not needed considering FG59-4-9a and FG59-4-9b have been agreed to indicate one of two separate SRS closed loop indexes for the case of two separate SRS CLPC adjustment states.

| 59. NR_MIMO_Ph5 | 59-4-8 | DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) | Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) | FFS | yes | n/a | DCI 1_1 indicating TPC command for separate SRS closed loop index(es) is not supported | FFS Per band | FFS No | FFS No | FFS n/a | FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indication for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states” | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-9a | DCI format 1_1 to indicate one of two separate SRS closed loop indexes under separate DL/UL TCI state mode | Support of DCI format 1_1 to indicate one of two separate SRS closed loop index(es) under separate DL/UL TCI state mode | FFS 59-4-3 | yes | n/a | DCI 1_1 indicating one of two separate SRS closed loop indexes under separate DL/UL TCI state mode is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-9b | DCI format 1_1 to indicate one of two separate SRS closed loop indexes under joint TCI state mode | Support of DCI format 1_1 to indicate one of two separate SRS closed loop index(es) under joint TCI state mode | FFS 59-4-3 | yes | n/a | DCI 1_1 indicating one of two separate SRS closed loop indexes under joint TCI state mode is not supported | Per band | n/a | FR1 only | n/a |  | Optional with capability signalling |

Proposal 4-7: Regarding the FFS in FG59-4-8, FG59-4-9a and FG59-4-9b, our view is provided as follows:

- For FG59-4-8,

  - The prerequisite FG is not needed.

  - The type is per band.

  - FDD/TDD differentiation is not needed.

  - FR1/FR2 differentiation is not needed.

  - Remove the FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indication for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states in FG59-4-8.

- The prerequisite FG of FG 59-4-9a and FG 59-4-9b is FG59-4-3.