3GPP TSG RAN WG1 #122	 R1-2506371

Bengaluru, India, August 25th – August 29th, 2025

Agenda item:       10.2.2

Source:	IIT Kanpur

Title:	Discussion on enhancing DL CSI acquisition

Document for:     Discussion and Decision

# Introduction

The objective for the RAN WG1 #122 related to DL CSI acquisition are as follows:

On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS Res - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

In this document, we provide our views on CSI-RS density reduction.

# Discussion

In Rel-19, higher number of CSI-RS ports (48/64/128) aggregated over multiple CSI-RS resources per legacy specification was introduced to support massive MIMO and higher frequency bands in FR1. In legacy specification CSI-RS resource density 1 and 1/2 are supported. With these values of density, the RS overhead increases significantly. To overcome the increased CSI-RS overhead, it has been agreed in RAN#108 to support additional density values of 1/3, 1/4, 1/6, 1/8.

To support these additional values of density following changes must be made in the table 7.4.1.5.3-1(TS 38 211), the changes are reflected below

TABLE 1

| Row | PortsN | Density ρ | cdm-Type | kq,lq | CDM group index | kq' | lq' |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | noCDM | k0,l0, k0+4,l0, k0+8,l0 | 0,0,0 | 0 | 0 |
| 2 | 1 | 1, 0.5 | noCDM | k0,l0, | 0 | 0 | 0 |
| 3 | 2 | 1, 0.5 | fd-CDM2 | k0,l0, | 0 | 0, 1 | 0 |
| 4 | 4 | 1 | fd-CDM2 | k0,l0, k0+2,l0 | 0,1 | 0, 1 | 0 |
| 5 | 4 | 1 | fd-CDM2 | k0,l0, k0,l0+1 | 0,1 | 0, 1 | 0 |
| 6 | 8 | 1 | fd-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0 | 0,1,2,3 | 0, 1 | 0 |
| 7 | 8 | 1 | fd-CDM2 | k0,l0, k1,l0, k0,l0+1, k1,l0+1 | 0,1,2,3 | 0, 1 | 0 |
| 8 | 8 | 1 | cdm4-FD2-TD2 | k0,l0, k1,l0 | 0,1 | 0, 1 | 0, 1 |
| 9 | 12 | 1 | fd-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0, k4,l0, k5,l0 | 0,1,2,3,4,5 | 0, 1 | 0 |
| 10 | 12 | 1 | cdm4-FD2-TD2 | k0,l0, k1,l0, k2,l0 | 0,1,2 | 0, 1 | 0, 1 |
| 11 | 16 | 1, 0.5,0.33,0.167,0.25,0.125 | fd-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0, k0,l0+1, k1,l0+1, k2,l0+1, k3,l0+1 | 0,1,2,3, 4,5,6,7 | 0, 1 | 0 |
| 12 | 16 | 1, 0.5,0.33,0.167,0.25,0.125 | cdm4-FD2-TD2 | k0,l0, k1,l0, k2,l0, k3,l0 | 0,1,2,3 | 0, 1 | 0, 1 |
| 13 | 24 | 1, 0.5,0.33,0.67 | fd-CDM2 | k0,l0, k1,l0, k2,l0, k0,l0+1, k1,l0+1, k2,l0+1,k0,l1, k1,l1, k2,l1, k0,l1+1, k1,l1+1, k2,l1+1 | 0,1,2,3,4,5, 6,7,8,9,10,11 | 0, 1 | 0 |
| 14 | 24 | 1, 0.5,0.33,0.67 | cdm4-FD2-TD2 | k0,l0, k1,l0, k2,l0, k0,l1, k1,l1, k2,l1 | 0,1,2,3,4,5 | 0, 1 | 0, 1 |
| 15 | 24 | 1, 0.5,0.33,0.67 | cdm8-FD2-TD4 | k0,l0, k1,l0, k2,l0 | 0,1,2 | 0, 1 | 0, 1, 2, 3 |
| 16 | 32 | 1, 0.5,0.25,0.125 | fd-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0,k0,l0+1, k1,l0+1, k2,l0+1, k3,l0+1, k0,l1, k1,l1, k2,l1, k3,l1, k0,l1+1, k1,l1+1, k2,l1+1, k3,l1+1 | 0,1,2,3, 4,5,6,7, 8,9,10,11, 12,13,14,15 | 0, 1 | 0 |
| 17 | 32 | 1, 0.5,0.25,0.125 | cdm4-FD2-TD2 | k0,l0, k1,l0, k2,l0, k3,l0, k0,l1, k1,l1, k2,l1, k3,l1 | 0,1,2,3,4,5,6,7 | 0, 1 | 0, 1 |
| 18 | 32 | 1, 0.5,0.25,0.125 | cdm8-FD2-TD4 | k0,l0, k1,l0, k2,l0, k3,l0 | 0,1,2,3 | 0,1 | 0,1, 2, 3 |

Proposal 1: For supporting the reduce density of CSI-RS resources, incorporate the changes shown below in the table 7.4.1.5.3-1(TS 38 211)

| Row Number | New density values |
| --- | --- |
| 11 | 0.33,0.25,0.167,0.125 |
| 12 | 0.33,0.25,0.167,0.125 |
| 13 | 0.33,0.167 |
| 14 | 0.33,0.167 |
| 15 | 0.33,0.167, |
| 16 | 0.25,0.125 |
| 17 | 0.25,0.125 |
| 18 | 0.25,0.125 |

When the CSI-RS resource density is reduced, the UE no longer has direct channel measurements for every PRB. Instead, it estimates the channel on the PRBs with CSI-RS and interpolates these estimates to the PRBs without CSI-RS. While this works reasonably well for channels that are relatively flat in frequency, its accuracy degrades significantly for highly frequency-selective channels, especially when CSI-RS density is very low, because interpolation cannot capture rapid variations in channel response between measured points.

Observation 1: Channel estimates of the PRB where CSI-RS resource is not available worsen with reduction in density when the channel is highly frequency selective.

Proposal 2: Further study should be done on the overall impact of reduce RS density on the system performance.

- Conclusion

This document provides the following observations and proposals:

Proposal 1: For supporting the reduce density of CSI-RS resources, incorporate the changes shown below in the table 7.4.1.5.3-1(TS 38 211)

| Row Number | New density values |
| --- | --- |
| 11 | 0.33,0.25,0.167,0.125 |
| 12 | 0.33,0.25,0.167,0.125 |
| 13 | 0.33,0.167 |
| 14 | 0.33,0.167 |
| 15 | 0.33,0.167, |
| 16 | 0.25,0.125 |
| 17 | 0.25,0.125 |
| 18 | 0.25,0.125 |

Observation: Channel estimates of the PRB where CSI-RS resource is not available worsen with reduction in density when the channel is highly frequency selective.

Proposal 2: Further study should be done on the overall impact of reduce RS density on the system performance.

# 4. References

[1] 3GPP RAN#108, New WID: NR MIMO Phase 6