3GPP TSG RAN WG1 #122bis	                                                                                R1-2506774

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:	10.1.1.1

Title:	Discussion on inference related aspects of CSI compression

Source:	ZTE Corporation, Sanechips

Document for:	Discussion and Decision

Introduction

AI/ML based CSI feedback has undergone extensive examination in Rel-18 and Rel-19, encompassing an evaluation of its potential benefits and an analysis of potential spec enhancements. Leveraging the insights from Rel-18 and Rel-19, Rel-20 aims to provide normative support for the CSI feedback enhancement (two-sided model) use case [1]. Specifically, the normative work of Rel-20 encompasses specification enhancements for facilitating LCM operations specific to AI/ML CSI feedback without temporal aspects, including model pairing, inference, training data collection, and performance monitoring. In this contribution, we provide our analysis and proposals on inference related aspects of AI/ML CSI feedback enhancement.

General views on AI/ML CSI feedback

According to the work item description for AI/ML for NR air interface enhancements, three inter-vendor training collaboration options are to be specified with a check point set in RAN#110, i.e., fully defined/specified reference model in Direction C, standardized encoder model structure plus parameter exchange ('Direction A option 3a-1' without target CSI sharing), and standardized dataset format/content plus dataset exchange ('Direction A option 4-1'). Notably, the selection of inter-vendor training collaboration options may directly impact the discussions on the inference related aspects. For example, for Direction A sub-option 3a-1, an additional indication on the actual payload size is needed for the inference report assuming that a scalable encoder deployed at the UE side can support multiple payload sizes. However, for Direction A sub-option 4-1, the additional indication on the actual payload size may not be needed if it can be implied by the exchange of dataset ID. Therefore, to ensure the integrity of the standards and the implementation flexibility, spec enhancements of inference related aspects should be compatible to all to be specified inter vendor training collaboration options taking the scalability design into account.

Spec enhancements of inference related aspects should be compatible to all to be specified inter vendor training collaboration options (i.e., Direction A options 3a-1 and 4-1, Direction C) including the associated scalability design.

Inference related aspects for AI/ML CSI feedback enhancement that have been identified by companies includes target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports. Those aspects will be specified when necessary. However, it’s noted that some inference related aspects have already been discussed and specified in AI/ML beam management and CSI prediction use cases in Rel-19 such as CSI processing criteria and timeline. Given that all three AI/ML enabled use cases share the same CSI framework, it's not expected that different methods or signalings are used for the same purpose from the perspective of standard simplicity, unless there is a very strong and urgent motivation. Generally, we suggest to directly reuse the spec enhancements specified for AI/ML beam management and CSI prediction to reduce spec efforts and simplify the spec design, when applicable.

Spec enhancements specified for AI/ML beam management and CSI prediction use cases should be reused for AI/ML CSI feedback to reduce spec efforts and simplify the spec design, when applicable.

Potential specification impacts

Based on the study in Rel-18 and Rel-19 for two-sided model based CSI compression use case, the following specification impacts related to inference aspects has been identified by companies, including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration/codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports. In this section, we will elaborate our views on potential specification impacts of these inference related aspects.

Target CSI type

Regarding the target CSI type for CSI feedback via two-sided model, the following agreement was reached in RAN1#122 meeting. However, whether to support precoding matrix with spatial-frequency domain representation or angular-delay domain representation remains controversial.

Agreement: For CSI feedback via two-sided model, support at least precoding matrix as target CSI type. - FFS whether consider channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc.

The precoding matrix with spatial-frequency domain representation and angular-delay domain representation were identified by companies in R18, and it was thought to be applicable to different number of antenna port configurations. However, we note that the target CSI type would impact many subsequent specification enhancement discussions, such as the CSI payload size and the dataset delivery discussion in RAN1, the standardization of model structures or parameters in RAN4. To simplify the spec design and avoid any unnecessary optimization, we shall only support one type of representation for target CSI in principle.

Regarding the precoding matrix with spatial-frequency domain representation, it has been adopted as the target CSI type by majority companies during the evaluations of Rel-18/19, where minor to significant performance gains over benchmark were observed depending on the payload size as well as model size. In other word, the performance-complexity tradeoff of CSI feedback via two-sided model is justified with the assumption of precoding matrix with spatial-frequency domain representation. Additionally, precoding matrix with spatial-frequency domain representation has been the baseline for studying the standardized model structure in Rel-19, based on which an  exemplary Transformer structure was agreed to be considered for further standardization. Therefore, for the target CSI type, precoding matrix with spatial-frequency domain representation should be supported as a starting point. While for precoding matrix with angular-delay domain representation (i.e., eType II-like reporting), apart from model-related processing, the UE needs to calculate multiple spatial-domain or frequency domain basis vectors based on matrix projection similar as legacy and then indicate the selected basis vectors to the base station, resulting in additionally computation complexity and reporting overhead.

Regarding the FFS point in the above agreement, currently only few companies have submitted the simulation results for explicit channel matrix as target CSI type. However, based on the limited input, as high as 37.8% performance gain over benchmark was observed for layer 1. Considering the promising gain over benchmark, we suggest to further study the explicit channel matrix as target CSI to unleash the power of AI/ML CSI feedback, including the assessment on performance gains and associated workloads.

For the target CSI type for model inference, support precoding matrix with spatial-frequency domain representation as a unified solution for different number of antenna port configurations.

For target CSI type for model inference, further study explicit channel matrix as model input for both FDD and TDD scenarios.

Measurement and report configuration

For the CSI reporting setting for inference, both channel measurement resources and pairing ID are configured, where the channel measurement resources are used for obtaining the target CSI information and the pairing ID is to enable the UE to select an encoder compatible with the decoder used by the gNB. The time domain behaviour of the CSI reporting setting for inference as well as the channel measurement resources can be configured as periodic, semi-persistent, and aperiodic as legacy. The pairing ID may be a specified ID fixed in the specification, or an ID transferred to the UE along with each exchanged dataset or model parameter, depending on the inter vendor training collaboration schemes. Based on measurement on the CSI resources and model inference associated with the pairing ID, the CSI feedback can be derived and then reported to the gNB.

For the CSI reporting setting for inference, pairing ID shall be configured to enable the UE to select an encoder compatible with the decoder used by the gNB.

Due to the limited generalization capability of AI/ML models and the dynamic behavior of UEs, the encoder deployed on the UE side may not always compress the target CSI with sufficient accuracy. For instance, sudden UE movements such as rotation or dropping can cause data distribution mismatches, or the UE's power or battery constraints may prevent it from handling the high computational complexity of AI/ML-based model inference for CSI feedback. In such exceptional cases, where the model is either inapplicable or the UE can detect inference errors without triggering a formal model monitoring procedure, it would be more helpful and efficient for the UE to generate the CSI report using the legacy codebook instead of transmitting stale or outdated CSI in the allocated PUSCH resources or padding the CSI report with dummy values. For example, the UE can indicate in the CSI report whether the CSI content is generated using the legacy codebook or the deployed AI/ML model by additionally reporting a performance indicator, thereby enhancing uplink resource utilization efficiency. This approach can serve as a reliable, always-on fallback mechanism.

| Fallback to legacy codebook based reporting |  |
| --- | --- |
| Performance indicator = 1 | PMI generated by AI/ML model |
| Performance indicator = 0 | PMI generated by legacy codebook |

For the CSI reporting setting for inference, support UE to indicate whether the CSI content is generated using the legacy codebook or the deployed AI/ML model.

CQI RI determination

- CQI determination

The CQI indicates a desired MCS for a hypothetical PDSCH transmission which would result in the hypothetical scheduling being received with a transport block error probability of less than a target BLER. During the study phase in Rel-18, the following agreement was achieved for CQI determination in CSI report.

Agreement: In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

For CQI determination, we analyze the pros and cons of different options in the following table. Generally, if the UE is aware of the recovered precoding matrix, it can calculate and report CQI in a traditional manner, assuming that the recovered precoding matrix will be directly applied for the hypothetical PDSCH precoding. However, in the context of AI/ML-based CSI feedback, the UE lacks knowledge of the recovered precoding matrix. Consequently, a mismatch between the reported CQI and the actual channel conditions becomes inevitable for all listed CQI options. Nonetheless, for Option 1a, where the CQI is calculated based on target CSI derived from realistic channel measurements, the base station can derive the SINR level experienced by the UE. This information can be very useful and serve as an input for link adaptation processes. For Option 1b, the method for performing CQI adjustment remains unclear and requires further investigation to establish its feasibility. For Options 1c and 2a, the UE must either support a legacy codebook for CSI reporting or implement a proxy model (or a CSI reconstruct model) for CQI calculation. These requirements depend on the UE's capabilities and do not represent a unified design approach. For Option 2b, it involves a two-step procedure for CQI determination, which introduces latency in PDSCH scheduling and may lead to a degradation in system performance.

Table. The analysis on CQI determination

| CQI determination |  | Analysis |
| --- | --- | --- |
| CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation | Option 1a: CQI is calculated based on target CSI with realistic channel measurement | - UE can directly adopt the target CSI with realistic channel measurement for CQI calculation - UE may over-estimate the channel condition, and thus reconstructed PMI and CQI are not matched - Network may always need to make some adjustment on UE reported CQI according to outer loop control |
|  | Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment | - A CQI adjustment table can be constructed according to some channel characteristics or based on some priori information at NW side - UE can calculate the similarity-related metrics between measured channel and the channel characteristics to do corresponding CQI adjustment - Beneficial to the system performance after potential adjustment |
|  | Option 1c: CQI is calculated based on legacy codebook | - UE may not support traditional codebook and AI/ML codebook simultaneously, which will largely increase the UE complexity - Mismatching between PMI and CQI is also unavoidable. If the traditional codebook can already get accurate PMI, it is not necessary to implement AI/ML models. |
| CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation | Option 2a-1: CQI is calculated based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW. | - UE may also be not expected to have CSI reconstruction model as it increases UE computation/storage/power consumption burden to a large extent - The CSI reconstruction model is generally a proprietary design by network side |
|  | Option 2a-2: CQI is calculated based on proxy CSI reconstruction output at UE side, where CSI reconstruction part at the UE is different from the actual CSI reconstruction part used at the NW. | - The accuracy of CQI calculation based on the proxy output CSI at UE is strongly related to the proxy model design, which may be not reliable. - It may impose huge burden on the network side to additionally identify/monitor/manage the UE-side proxy model |
|  | Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. | - UE has to receive a CSI-RS and report the PMI compressed by AI/ML model in the first step. Then, UE will receive a precoded CSI-RS based on the reconstructed PMI at network and report the CQI determined by precoded CSI-RS in the second step. - The two-step procedure increases the time span of the CQI determination process, which may face the channel variation/aging so that the current CQI cannot match the previous CSI - In current specification, this mode is already supported (i.e., when the report quantity is cri-RI-CQI) and less specification impact is foreseen. |

For CSI feedback using two-sided model, for CQI determination in a CSI report, support Option 1a: CQI is calculated based on target CSI with realistic channel measurement.

- RI determination

In the current specification, RI shall be calculated conditioned on the reported CRI. Specifically, if multiple CSI-RS resources corresponding to different beam directions are configured in the CMR, the UE will first indicate a preferred beam indicator, i.e., CRI. Then, for the selected CRI, the UE will determine the associated transmission rank considering both the rank of the propagation channel and the SINR level experienced by the UE. For instance, the UE would evaluate the channel quality for different possible ranks (i.e., different numbers of transmission layers) based on the estimated channel and then select the rank that maximizes a performance metric like achievable throughput and ensures SINR per layer is sufficiently large. For AI/ML CSI feedback using two-side model use case, the CQI determination would be impacted since it’s calculated conditioned on the reported PMI which may be quite different from the reconstructed PMI. However, the RI determination is only conditioned on the reported CRI and thus separate from the CQI/PMI determination. Therefore, we think the legacy RI determination can be directly reused.

For CSI feedback using two-sided model, for RI determination in a CSI report, legacy RI determination can be reused as a starting point.

Payload determination

For AI/ML CSI feedback using two-sided model sub use case with rank ≥ 1, multiple AI/ML model settings have been identified in the study phase to adapt to ranks/layers, e.g., rank specific, rank common, layer specific, layer common. Among those rank/layer adaptation settings for rank>1, layer common and rank common can achieve the best performance-complexity tradeoff, where a unified AI/ML model is applied for each layer under any rank value to perform individual inference, and thus memory storage at the UE side and LCM complexity at the gNB side can be reduced. Besides, it has been taken as the baseline for the evaluation of CSI feedback of 1-on-1 joint training without model generalization/scalability in Rel-18. Therefore, for the standardization of for AI/ML CSI feedback, we support layer common and rank common as the baseline for AI/ML model setting. It’s noted that for a layer common and rank common model with scalability over payload size configurations, layer-specific payload size can still be achieved by varying truncation/masking length of the output linear layer output or quantization parameters for different layers as evaluated in Rel-19. This will not increase the UE computation or storage complexity, but can greatly improve the efficiency of CSI reporting.

Support layer common and rank common as the baseline for AI/ML model setting.

For a layer common and rank common model with scalability over payload size configurations, layer-specific payload size can still be achieved, such as by truncation/masking of the output linear layer output.

Regarding the payload determination issues, the following agreements were achieved in RAN1#122, which clarifies that the CSI payload size is determined by the length of the latent message as well as the quantization codebook configurations. However, there are still some controversial issues, such as the potential down-selection between SQ and VQ, the parameterization of CSI payload, and CSI payload configuration.

Agreement Specify the precoding matrix feedback via two-sided model as follows: For a certain rank , - A precoding matrix is mapped with a latent message , with l = 1, ..., v where l is the layer index. - The mapping is subject to [at least] a pairing ID configured by high-layer signalling. - FFS is common or specific for different layers or ranks - Note: the terminology “latent message” is used for discussion purposes, the detailed name can be discussed further - is quantized and mapped to bit sequence before reporting. Agreement For precoding matrix feedback, for a certain layer of rank , the latent message contains real values - If scalar quantization (SQ) supported, it refers to the quantization of each real value independently, - The payload size for layer and rank is determined by , where is the number of bits per scalar. - If vector quantization (VQ) is supported, it refers to the quantization of a segment of L (L>1) consecutive real values jointly, and the segment(s) are separately quantized. - The payload size for layer and rank is determined by , where is the length of the vector segment, is the number of bits per segment - Note: Combining two real-values as a complex value and performing amplitude-phase quantization is precluded. Agreement: For precoding matrix feedback, consider the following aspects for payload determination - Support only SQ, or only VQ or both - and / or and / or are fixed values or determined from multiple values based on NW configuration or UE reporting. - and / or is layer-common or layer-specific, rank-common or rank-specific. - If configured, and / or and / or above are configured separately from pairing ID, or configured by Pairing ID. - The configuration may be via other parameters which are used to derive , L and . Note: Consider limiting the total number of possible payload sizes considering the inter-vendor training challenges. Note: , and are independent on subband and port configurations, - FFS each combination of , and is applicable to a certain set of subband and port configurations

- Potential down-selection of SQ and VQ

From performance perspective, TR 38.843 has listed some simulation results for the comparison of quantization methods for CSI compression. In general, SQ outperforms VQ with 1%~4.5% gains for quantization aware training Case 2-1, while SQ underperforms VQ with -0.7%~-3.8% degradation for quantization aware training Case 2-2. Additionally, VQ under  quantization aware training Case 2-2 has 0.46%~4% gain over SQ under  quantization aware training Case 2-1. It's also noted in TR 38.843 that more companies observing gain of VQ over SQ than companies observing loss. Therefore, at least VQ shall be supported to achieve better field performance. However, VQ may bring higher complexity for the quantization/dequantization compared with SQ. Given that the training strategy is up to NW implementation, both two quantization aware training cases can be considered to balance the complexity and performance. Accordingly, both SQ and VQ can be supported to be applicable to different quantization aware training cases. From spec wise, SQ and VQ also show very similar spec impacts according to the parameterization of CSI payload.

For payload determination of AI/ML CSI feedback, support both SQ and VQ to be applicable to different quantization aware training cases.

- Parameterization of CSI payload

Regarding the parameterization of CSI payload, the first remaining issue is whether the CSI payload size is common or specific for different layers or ranks. Generally, the precoding matrix is obtained by SVD/EVD decomposition of the MIMO channel matrix. The first layer corresponds to the strongest singular vector capturing the maximum channel energy, and also determines the primary beam direction for spatial multiplexing, providing the majority of beam-forming gains. The higher layers capture residual energy which diminishes rapidly, providing very limited beam-forming gains compared with the first layer. Therefore, it would be quite straightforward to support layer-specific CSI payload size, which is also aligned with the field experience of the legacy eType II. This hierarchy can help ensure efficient resource allocation in AI/ML based CSI feedback, balancing accuracy and overhead.

For payload determination of AI/ML CSI feedback, support layer-specific CSI payload size to balance accuracy and overhead.

According to the study in Rel-19, the model scalability over payload configurations can at least be achieved by: 1) Alt 1: specific output linear layer for each payload configuration; 2) Alt 2: truncation/masking of the output linear layer output; 3) Alt 3: by varying quantization parameters. Alt 1 adopts specific fully-connected layers to compress the features to different output latent vectors exclusively. It would increase the model complexity and storage overhead. Alt 3 lies in altering the quantization part, for example, by adjusting quantization granularity to modify payload sizes. However, changes in the quantization granularity for scalability might have worse performance compared with Alt 1 and Alt 2 where the lengths of encoder output latent can vary. Alt 2 just operates truncation/masking of the maximum linear layer output for each payload configuration, which can achieve the performance-complexity tradeoff. Then, if we consider Alt 2 for scalability over payload configurations, layer-specific CSI payload size generally means layer-specific latent message obtained by truncation/masking of the output linear layer output. And the layer-specific latent message shall be configured separately from the pairing ID.

For payload determination of AI/ML CSI feedback, support layer-specific latent message (i.e., d value) which is configured separately from the pairing ID linked to the CSI reporting setting.

- According to our previous evaluations in Rel-19, if the layer-specific latent message is adopted, the latent message of different layers can reuse the same quantization codebook, as long as the quantization parameters are updated in together with the AI/ML models during training. This layer-common quantization codebook can reduce the UE complexity on quantization at the expense of negligible performance loss compared with layer-specific quantization codebook. Additionally, the quantization codebook can be implicitly conveyed by the pairing ID during model inference phase, given that each pairing ID is only associated with a single quantization codebook. Consequently, the overhead for configuring quantization codebook in the CSI reporting setting for inference can be saved.

For payload determination of AI/ML CSI feedback, support layer-common quantization codebook (i.e., Q and L value), which can be configured by the pairing ID linked to the CSI reporting setting.

- Configuration of CSI payload

Another remaining issue is whether the quantization parameters (e.g., d, Q, or L) are determined based on NW configuration or UE reporting. In eType II codebooks of 5G NR, the CSI payload sizes are determined by the number of reported non-zero coefficients, and there are restrictions on the number of non-zero coefficients per layer to be no more than K0 and the total across all layers to be no more than 2K0. This is primarily driven by some technical considerations that further increasing the number of non-zero coefficients only provides marginal beam-forming gains, but would increase the reporting overhead as well as computational complexity. And the selection of non-zero coefficients can be done by UE implementation easily, such as by simply discarding weak paths based on path power. However, for payload determination of AI/ML CSI feedback, more payloads generally provide better performance according to Rel-19 evaluations, and thus it's not clear how to set a pre-defined restriction like K0 on the maximum payload size. Additionally, from UE implementation perspective, it's also not clear how to select the actual payload size if there are some levels of flexibility allowed, given that the reconstructed CSI is transparent to the UE side. Therefore, we prefer that the quantization parameters (e.g., d, Q, or L) related to the CSI payload size are determined based on NW configuration, which is also the simplest way and provides the least spec impacts.

For payload determination of AI/ML CSI feedback, support the quantization parameters (e.g., d, Q, or L) of each layer to be directly configured by the network.

Quantization configuration/codebook

To achieve alignment between gNB and UE on the quantization/de-quantization method, the quantization codebook shall be transferred to the UE or standardized in the specification, which is related to the inter-vendor training collaboration options. For quantization codebook in inter-vendor collaboration Direction C, since the model specification is up to RAN4 and quantization aware training is assumed, the quantization codebook specification shall also be up to RAN4. However, for inter-vendor collaboration Direction A, whether to dynamic exchange quantization codebook or use a standardized quantization codebook is still quite controversial according to the discussions in RAN1#122 meeting.

Agreement: For quantization codebook used for quantizing the feedback, - For inter-vendor collaboration Direction A, select one of the following: - Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters - Alt2: Use a standardized quantization codebook decided by RAN1.

For inter-vendor collaboration Direction A Options 4-1 and 3a-1, we think that flexible and variable quantization codebook (i.e., Alt1) shall be used for the alignment of quantization/de-quantization between gNB and UE in order to achieve better field performance. Specifically, during model training phase, quantization codebook of standardized configuration(s) is exchanged from NW-side to UE-side along with each exchanged dataset or model parameters. Each exchanged dataset or model parameters will be associated with a pairing ID. Note that the delivery overhead for quantization codebook is negligible since non-OTA solutions are assumed. Then, during model inference phase, the pairing ID will be configured to the UE in the inference reporting setting, implying that the same quantization codebook is directly applied to ensure quantization consistency across training and inference. For standardized quantization codebook (i.e., Alt2 in the agreement above), we think it may not ensure future proofness and would cause too much additional efforts in RAN1 for the quantization codebook standardization. Firstly, all current conclusions or observations on quantization codebook are based on the assumption of transformer model backbone. However, with the development and evolution of AI/ML model, new AI/ML models are constantly emerging and the standardized quantization codebook may not be able to withstand the challenges of the future. Additionally, it may not be feasible to specify a standardized quantization codebook in RAN1 considering the potential challenge of dataset/model alignment.

For quantization codebook for inter-vendor collaboration Direction A, support Alt1 due to its better flexibility and field performance.

For quantization codebook for inter-vendor collaboration Direction A, not support Alt2 since it cannot ensure future proofness and would cause too much additional specification works in RAN1.

UCI mapping

For AI/ML CSI feedback, the CSI payload size can vary quite dramatically depending on the RI selection at the UE side. For instance, the PMI payload for RI=2 is double to that of RI=1 for rank common and layer common model. Since the RI selection is not known to the gNB prior to PUSCH scheduling, it can happen that the actual CSI payload cannot fit in the PUSCH container, e.g., the resulting code rate for UCI is too large. In this case, a portion of the CSI information shall be omitted similar as legacy. Notably, for rank common and layer common/specific model (as adopted by majority companies in previous evaluations), the target CSI of different layers are compressed separately by the UE-side encoder. Therefore, it would be quite straightforward to consider layer-based CSI omission, where the compressed target CSI for lower layer has precedence over that for high layer. However, dropping the entire CSI information of a layer would be quite wasteful. Therefore, on top of the layer-based CSI omission, we can at least consider the following two methods for further CSI omission.

- Target CSI of all configured subbands are compressed together by the AI/ML encoder deployed at the UE side. Then, the encoder output is ordered in a pre-defined manner. For example, the earlier values in the encoder output are given precedence over the latter values. This means that the earlier values in the encoder output are mapped first in the UCI, followed by the latter values in the encoder output. If the resulting code rate of UCI is above a threshold, a portion of the least significant UCI bits corresponding to the latter values in the encoder output will be omitted (i.e., UCI omission from the least significant bits to the most significant bits), until the code rate of UCI falls below the threshold.

- The configured subbands are divided into one or multiple subband groups, and target CSI are compressed group by group by the AI/ML encoder deployed at the UE side. This means that target CSI of different configured subbands or subband groups are ordered in a pre-defined manner, and target CSI of one subband or subband group are compressed each time. The encoder output corresponding to the subbands or subband groups with highest priority are mapped to the most significant bits in UCI, followed by that with lower priority. In this case, a portion of the least significant UCI bits corresponding to the subbands or subband groups with lower priority may be omitted. Notably, based on reconstructed target CSI for the reported subbands, the gNB may be able to estimate the target CSI for the missing subbands by some implementation algorithms like interpolation or extrapolation as legacy.

For CSI feedback using two-sided model, for UCI mapping, the UE may omit a portion of the Part 2 CSI according to pre-defined priority rules.

Consider the following methods for CSI omission for potential down-selection: layer-based CSI omission, subband or subband group based CSI omission, truncation/masking of encoder output.

CSI processing criteria and timeline

In the current specification, the CPU is defined to quantify the simultaneous processing of multiple CSI reports. Each CSI report that is committed for calculation by the UE occupies a number of CPUs from a starting allocation time until the last symbol of the physical channel carrying the CSI report. In Rel-19, to accommodate the adoption of hardware other than CPUs for AI/ML-based CSI processing at the UE side (e.g., GPUs with parallel computing capabilities and impressive speedups), a dedicated APU is further introduced for AI/ML features. Specifically, an AI/ML enabled inference report occupies a number of CPUs and/or a number of APUs. The same occupancy duration is assumed if both CPU and APU are occupied. If any of the unoccupied PU cannot satisfy the corresponding required PU by the inference report, the inference report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied. As far as we are concerned, these CSI processing criteria defined for AI/ML beam management and AI/ML CSI prediction can be directly reused for AI/ML CSI feedback to reduce the spec efforts.

For CSI feedback using two-sided model, dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied for an inference report, where OAPU= 0 or N is reported by UE in UE capability report, and OCPU= 0 or M is reported by UE in UE capability report.

For CSI feedback using two-sided model, for an inference report, Rel-15 CPU occupation time is reused for APU occupation time and/or CPU occupation time.

Apart from the CSI processing criteria, another important issue to be addressed is the UE CSI computation time. In legacy, there are minimum requirements for the time duration between the DCI triggering instance and the CSI report instance (i.e., Z value), and between the last measurement instance and the CSI report instance (i.e., Z'), where Z and Z' are predetermined subject to UE capability. In Rel-19, due to the introduction of AI/ML beam management and CSI prediction, the legacy Z/Z' is extended to be Z+d/Z'+d' to accommodate the AI/ML model processing latency (e.g., model activation latency, model inference latency), where d and d' are reported by UE per SCS in UE capability report. Per our understanding, these enhancements can also be directly applied to the inference report in AI/ML CSI feedback use case.

For CSI feedback using two-sided model, for an inference report, extend legacy Z2 / Z2’ to Z2+d / Z2’+d’, where d and d’ are reported by UE per SCS in UE capability report.

Priority rules for CSI reports

It can happen that multiple CSI reports collide, e.g., two or more CSI reports are scheduled to be transmitted simultaneously, or multiple CSI reports scheduled to be transmitted simultaneously cannot fit in the UCI container. Therefore, a number of priority rules for CSI reports are defined in the current specification. Specifically, CSI reports are prioritized according to their time-domain behaviour, physical channel, CSI content, serving cell index, and CSI report configuration index. In case of CSI report collision, some CSI reports with lower priority are dropped or omitted. In Rel-19, to accommodate the introduction AI/ML beam prediction and CSI prediction, the priority rules are slightly enhanced by defining the k value for different CSI report contents as follows. Apparently, for the AI/ML CSI feedback use case, the priority rules shall also be defined appropriately for the corresponding inference reports and monitoring reports. Considering that AI/ML beam prediction, CSI prediction, and CSI feedback share the same CSI framework, and the beam report is given precedence over regular CSI reports, we suggest to reuse the current priority rules and set the k value to be 1 for all AI/ML CSI feedback related CSI reports.

<omitted text> CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where - for aperiodic CSI reports to be carried on PUSCH for semi-persistent CSI reports to be carried on PUSCH, for semi-persistent CSI reports to be carried on PUCCH and for periodic CSI reports to be carried on PUCCH; - for CSI reports carrying L1-RSRP, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI or L1-SINR and for CSI reports not carrying L1-RSRP, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI or L1-SINR; - c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells; <omitted text>

For the determination of CSI report priority, the existing PriiCSIy,k,c,s is reused, where k = 1 for CSI reports for inference and monitoring.

Conclusions

In this contribution, we discuss the specification enhancements for inference related aspects of AI/ML CSI feedback. We have the following proposals.

Proposals：

- Spec enhancements of inference related aspects should be compatible to all to be specified inter vendor training collaboration options (i.e., Direction A options 3a-1 and 4-1, Direction C) including the associated scalability design.

Spec enhancements specified for AI/ML beam management and CSI prediction use cases should be reused for AI/ML CSI feedback to reduce spec efforts and simplify the spec design, when applicable.

For the target CSI type for model inference, support precoding matrix with spatial-frequency domain representation as a unified solution for different number of antenna port configurations.

For target CSI type for model inference, further study explicit channel matrix as model input for both FDD and TDD scenarios.

For the CSI reporting setting for inference, pairing ID shall be configured to enable the UE to select an encoder compatible with the decoder used by the gNB.

For the CSI reporting setting for inference, support UE to indicate whether the CSI content is generated using the legacy codebook or the deployed AI/ML model.

For CSI feedback using two-sided model, for CQI determination in a CSI report, support Option 1a: CQI is calculated based on target CSI with realistic channel measurement.

For CSI feedback using two-sided model, for RI determination in a CSI report, legacy RI determination can be reused as a starting point.

Support layer common and rank common as the baseline for AI/ML model setting.

For a layer common and rank common model with scalability over payload size configurations, layer-specific payload size can still be achieved, such as by truncation/masking of the output linear layer output.

For payload determination of AI/ML CSI feedback, support both SQ and VQ to be applicable to different quantization aware training cases.

For payload determination of AI/ML CSI feedback, support layer-specific CSI payload size to balance accuracy and overhead.

For payload determination of AI/ML CSI feedback, support layer-specific latent message (i.e., d value) which is configured separately from the pairing ID linked to the CSI reporting setting.

For payload determination of AI/ML CSI feedback, support layer-common quantization codebook (i.e., Q and L value), which can be configured by the pairing ID linked to the CSI reporting setting.

For payload determination of AI/ML CSI feedback, support the quantization parameters (e.g., d, Q, or L) of each layer to be directly configured by the network.

For quantization codebook for inter-vendor collaboration Direction A, support Alt1 due to its better flexibility and field performance.

For quantization codebook for inter-vendor collaboration Direction A, not support Alt2 since it cannot ensure future proofness and would cause too much additional specification works in RAN1.

For CSI feedback using two-sided model, for UCI mapping, the UE may omit a portion of the Part 2 CSI according to pre-defined priority rules.

Consider the following methods for CSI omission for potential down-selection: layer-based CSI omission, subband or subband group based CSI omission, truncation/masking of encoder output.

For CSI feedback using two-sided model, dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied for an inference report, where OAPU= 0 or N is reported by UE in UE capability report, and OCPU= 0 or M is reported by UE in UE capability report.

For CSI feedback using two-sided model, for an inference report, Rel-15 CPU occupation time is reused for APU occupation time and/or CPU occupation time.

For CSI feedback using two-sided model, for an inference report, extend legacy Z2 / Z2’ to Z2+d / Z2’+d’, where d and d’ are reported by UE per SCS in UE capability report.

For the determination of CSI report priority, the existing PriiCSIy,k,c,s is reused, where k = 1 for CSI reports for inference and monitoring.

References

RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Qualcomm (Moderator), RAN#108.