3GPP TSG RAN WG1 Meeting #122bis		                            R1-2506830

Prague, Czech, October 13th – 17th, 2025

Title: 	Discussion on modulation for 6GR

Source: 	ZTE Corporation, Sanechips

Agenda item:	11.4.2

Document for:	Discussion and decision

Introduction

In RAN1#122 meeting, it has been agreed that 5G NR uniform QAM should be supported as basis for study [1].

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Meanwhile, new modulation schemes for 6G are also under discussion. In this contribution, we provide analysis on modulation for 6GR, shaping modulation and low PAPR modulation.

Discussion on QAM modulation for 6GR

In RAN1#122 meeting, it was agreed to support up to 1024QAM in DL and 256QAM in UL as basis for study. In addition to that, for advanced UEs (e.g., FWA or CPE devices) with relaxed form factor limitation, 1024QAM can be supported in UL as well to achieve high throughput. However, further higher modulation order for QAM constellation (e.g., 2048QAM and 4096QAM) is more sensitive on error vector magnitude (EVM) and requires higher signal-to-noise ratio (SNR). Therefore, we think 2048QAM or 4096QAM should not be considered in 6G Day 1.

The maximum modulation order can be considered for 6GR UL is 1024QAM.

Discussion on shaping modulation for 6GR

Metrics for shaping modulation

In RAN1#122 meeting, we discussed the metrics, evaluation assumptions and evaluation methodologies for shaping modulation, which are shown as follows [2]:

Proposal 2.2-7 (for online) For 6GR constellation shaping evaluation for CP-OFDM, (Geometric shaping and probabilistic shaping with optimized MCS), the proposed scheme will be compared with non-shaping with NR MCS table. The evaluation and comparison should consider at least the following: - BLER performance under AWGN channel and fading channel (SISO, and MIMO with rank=1 and rank>1) - Throughput performance under fading channel (SISO, and MIMO with rank=1 and rank>1) - Transmitter and receiver complexity, latency, parallelism implementation, and storage requirements, - PAPR - Expected spec impact - FFS: Simulation assumptions, such as - For MIMO channel evaluation, needs to provide assumptions on MIMO precoder (e.g., open loop MIMO or closed loop MIMO (as baseline)) and receiver assumed (e.g., MMSE or rML, genie channel or realistic channel estimation) - FFS MU-MIMO - For throughput evaluation, needs to provide assumptions on rate adaptation (e.g., target BLER for 1st transmission, maximum # of retransmissions) - Other KPI not excluded, such as EVM, MPR/A-MPR - FFS: System level simulations

For evaluation metrics, we focus on the discussion on BLER, throughput as well as transmitter and receiver complexity.

- BLER performance

Modulation is an essential component in wireless communication. Therefore, the block error rate (BLER) performance of shaping modulation needs to be evaluated for all potential cases with small to large transport block size (TBS) and modulation and coding scheme (MCS) entries in NR MCS table.

- Throughput

Throughput refers to the amount of data successfully transmitted within a unit of time. It reflects the transmission efficiency of the communication system. Throughput is a crucial metric for assessing the performance of an entire communication system, directly reflecting the system's capability to transmit effective data within a unit of time. Higher throughput indicates greater transmission efficiency.

- Complexity

In our opinion, we think complexity analysis includes transmitter complexity, receiver complexity and storage requirement. The receiver complexity is related to power consumption, which is one of important metrics for mobile devices. The overall receiver complexity should be jointly considered, comprising MIMO receiver complexity (for MIMO fading channel), demodulation complexity, decoding complexity and complexity of distribution de-matcher (for PAS). Besides, the transmitter complexity is related to high throughput and latency. Thus, the complexity of shaping modulation should be considered.

The following metrics should be evaluated for shaping modulation:

  - BLER performance

  - Throughput

  - Complexity

For evaluation assumptions, we focus on the discussion on fading channel, MIMO precoder, and MIMO receiver.

- MIMO fading channel model

For MIMO fading channel, we think CDL channel model is more appropriate, since CDL channel introduces the concept of multipath clusters, which is more realistic compared to traditional single-path or simple multipath models. Besides, the evaluation methodologies are discussed in evaluation methodology agenda (i.e., 11.2). In our consideration, the evaluation assumptions for MIMO fading channel should be aligned with the conclusion/agreement in AI 11.2.

- MIMO precoder

For MIMO precoder, we think assumptions on MIMO precoder should be used, including closed-loop MIMO, since it is a practical implementation.

- MIMO receiver

For MIMO receiver, MMSE receiver is the common receiver in current implementation. According to RAN4 requirements, MMSE-IRC receiver is mandatory and R-ML receiver is optional. To this end, we consider at least MMSE receiver should be considered as baseline.

Besides, the evaluation methodologies are discussed in AI evaluation methodology (i.e., 11.2). Therefore, the evaluation assumptions for fading channel should be also aligned with the conclusion/agreement in AI 11.2.

- Throughput performance assumption

Towards throughput performance assumption, hybrid automatic repeat request acknowledgement (HARQ-ACK) based re-transmission is often used when initial transmission fails. In re-transmission, MCS is not necessary the same as initial transmission. Instead, it can be adapted to channel condition or available resource allocation. Therefore, adaptive MCS for both initial transmission and re-transmission should be evaluated.

The following aspects for evaluation assumptions should be considered for shaping modulation:

  - BLER performance for both initial transmission with varied TBS and spectrum efficiency

  - Throughput performance with re-transmission and adaptive MCS assumption

  - MIMO fading channel model

  - Closed loop MIMO for MIMO precoder assumption

  - MMSE receiver for MIMO receiver assumption

Simulation results for GS and PS

Geometric shaping modulation

Geometric shaping employs non-uniform constellations (NUCs) as an alternative to conventional QAM. Unlike uniformly spaced QAM, NUCs approximate a Gaussian distribution in the complex plane, offering potential gains in spectral efficiency and noise resilience.

NUC modulation is a mature technique for performance improvement in coded modulation. NUC modulation has been used in broadcast systems for many years, such as advanced television systems committee standard version 3.0 (ATSC 3.0) [3] and digital video broadcasting (DVB) standard [4].

NUCs include three types: 1D-NUCs (Figure 1(a)), 2D-NUCs (Figure 1(b)) and amplitude-phase shift keying (APSK) modulation (Figure 1(c)).

NUC modulation is a mature technique and has been widely used in broadcast standards.

(a) 1D 64NUC                  (b) 2D 64NUC                 (c) 64APSK

Figure 1 Diagrams for three types of NUCs. Dots are constellation points where digits near the dots are bit labeling corresponding to the constellation points.

Figure 2 shows the coding and modulation chain for NUC modulation. It is shown that the coding and modulation chain is the same as that of 5G NR low density parity check (LDPC) coding and modulation chain [5] except that NUC modulation and demodulation replace QAM modulation and demodulation at the transmitter and the receiver, respectively, where details of channel coding, bit selection, and bit interleaving can be referred to 3GPP TS 38.212 [5].

Figure 2 Coding and modulation chain for NUC modulation.

Introducing NUC for 6GR does not affect the LDPC coding chain in 5G. The LDPC coding chain in 5G can be reused in 6GR.

For NUC modulation, every Qm bits b0, ..., bQm-1 from the output sequence of the bit interleaving are independently and separately mapped to a constellation point of a given NUC using a predefined bit-to-symbol mapping, where Qm is the modulation order of the given NUC.

For NUC demodulation, every received symbol yi in a received sequence y0, ..., yNs-1 is independently and separately demodulated to Qm log-likelihood-ratio (LLR) values l0, ..., lQm-1.

NUC modulation/demodulation is a full parallel process, and it is easier to achieve ultra-high throughput for 6GR.

Figure 3 gives shaping gain of NUC schemes over legacy QAM schemes at a target BLER of 0.01 under AWGN channels, where a TBS of 8424 is used for all simulated MCSs in 256QAM MCS index table [6] while other simulation settings are presented in Table A-1 in the Appendix. In Figure 3, shaping gain G is defined as

G=SNRmin,QAM-SNRmin,NUC,

where SNRmin,QAM is the minimum SNR in dB for a legacy QAM scheme to achieve a target BLER of 0.01, SNRmin,NUC is the minimum SNR in dB for an NUC scheme to achieve a target BLER of 0.01, both the legacy QAM scheme and NUC scheme are of the same SE.

From Figure 3, it can be observed that

- Shaping gain is about 0.15 to 0.25 dB for MCS 5 to 10 (corresponding to Qm = 4).

- Shaping gain is about 0.3 to 0.5 dB for MCS 11 to 19 (corresponding to Qm = 6).

- Shaping gain is about 0.4 to 0.75 dB for MCS 20 to 27 (corresponding to Qm = 8).

Figure 3 Shaping gain for NUC schemes over legacy QAM schemes over AWGN channels at TBS = 8424 for different MCS entries in 256QAM MCS index table [6].

Figure 4 gives BLER curves of legacy QAM schemes and NUC schemes for selected MCS index in Table A-2 in the Appendix over fading channels, where 6 resource blocks are allocated for data transmission while other simulation settings are presented in Table A-3 in the Appendix.

From Figure 4, it can be observed that

- For MCS 18, a shaping gain of about 0.27 dB is obtained at BLER = 0.1 in a setting with 1 transmit antenna and 1 receive antenna with 1 MIMO spatial layer.

- For MCS 18, a shaping gain of about 0.5 dB is obtained at BLER = 0.1 in a setting with 4 transmit antennas and 4 receive antennas with 4 MIMO spatial layers.

| (a)MCS 18, 1T1R, 1 layer | (b)MCS 18, 4T4R, 4 layers |
| --- | --- |

Figure 4 BLER curves of legacy QAM schemes and NUC schemes under MCS 18 over CDL-A channels.

According to our simulation results, NUC can have shaping gain from 0.15 dB to 0.75 dB, where shaping gain of NUC increases as the modulation order increases.

- Shaping gain from 0.15dB to 0.75dB where shaping gain of NUC increases as the modulation order increases

- ~0.27dB shaping gain for MCS level 18 in NR 256QAM table at BLER = 0.1 in the CDL-A 1T1R channel.

- ~0.5 dB shaping gain for MCS level 18 in NR 256QAM table at BLER = 0.1 in the CDL-A 4T4R channel.

NUC modulation for 6GR can be studied and considered.

Probabilistic shaping modulation

Probabilistic shaping approximates the optimal Gaussian distribution by transmitting different QAM symbols at different probabilities [7]. One typical method is probabilistic amplitude shaping (PAS) [7]. Figure 5 gives an example of PAS 64QAM. Figure 6 shows the coding and modulation chain for PAS modulation.

Figure 5 Diagram for PAS 64QAM. Each column represents the coordinates of a constellation point, and the height of the column indicates the probability of the constellation point's occurrence.

Based on our study, we think the following technical issues should be addressed.

- Impact on coding chain

PAS is a serial concatenation architecture of a distribution matcher (DM) and a channel encoder [7], where the DM is applied prior to the channel encoder.

In Figure 6, equiprobable information bits are split into two parts: bits for shaping and bits for non-shaping. The reason for splitting information bits into two parts is to adjust the SE as well as the probability distribution of QAM symbols since the optimal probability distribution for a specific SE restricts the maximum number of bits for shaping, then extra bits exceeding the maximum number of bits for shaping cannot be used for shaping, resulting in the bits for non-shaping. The equiprobable bits for shaping are input to the DM and transformed into non-uniform amplitude levels of amplitude shift keying (ASK) symbols.

Figure 6 Coding and modulation chain for PAS modulation.

The shaped bits, the bits for non-shaping as well as the parity bits generated by channel coding are combined and used for QAM modulation, where the shaped bits converted from the non-uniform amplitude levels by a pre-defined amplitude-to-bit mapping of ASK. Specifically, the shaped bits are mapped to amplitudes of the ASK symbols, the bits for non-shaping and the parity bits are mapped to the equally probable phase components of the ASK symbols. Finally, every two ASK symbols are combined into a QAM symbol and transmitted over the channel. PAS inherently has an interleaver on the coded bits of channel coding output, which is different from the bit interleaving after 5G NR LDPC coding. To this end, we need to evaluate the impact of PAS on bit interleaving after LDPC coding.

PAS modulation has impacts on coding chain in terms of:

- Additional procedures before or after channel coding

- Bit interleaving

- Analysis of DM algorithms

There are many DM algorithms in the literature, such as shell mapping [8], constant composition distribution matching (CCDM) [9], (approximate) enumerative sphere shaping ((A)ESS) [10].

Analysis#1: CCDM algorithm

The most popular DM is CCDM, which is based on the arithmetic coding algorithm. Specifically, arithmetic decoding is used at the transmitter for DM encoding, while arithmetic encoding is used at the receiver for DM decoding. In other words, the encoding and decoding operations for DM are the reverse of those used in conventional arithmetic coding.

CCDM needs an amplitude distribution as input to generate non-uniform amplitude levels [9]. The non-uniform amplitude distribution is the Maxwell-Boltzmann distribution defined as:

Pr(A) = β·exp(-v·|A|2)

where A represents an ASK amplitude in the set {1, 3, ..., 2^m-1} and m is the modulation order of ASK, β is a normalization factor, and v is a non-negative parameter to adjust the probability distribution and need be optimized. The optimal value of v depends on the target SE and the channel state.

There are three aspects that need to be considered in using CCDM based on arithmetic coding for PAS schemes.

The first aspect is that the principle of arithmetic coding is based on probabilities with infinite floating-point precision. In practical, a finite precision for representing probabilities is used. If precision of probabilities is not matched between the transmitter and the receiver, data cannot be recovered correctly.

The second aspect is that arithmetic coding is inherently a serial process for both encoding and decoding. Thus, the CCDM algorithm is a serial process. Serial process is not good for high throughput applications. Support an integrated circuit with a clock frequency of fclk Hz and CCDM processes one ASK symbol in Nclk clock cycles for probability update in arithmetic coding. Thus, if QAM of modulation order Qm is used, the throughput for PAS is

Tmax = (Qm/2 - 1)·fclk/Nclk.

where Qm/2 - 1 is the number of amplitude bits in an ASK symbol.

In current 3GPP specification, the maximum modulation order is Qm = 10 corresponding to 1024QAM. Nowadays, the typical clock frequency for integrated circuits is around 1 GHz. Thus, ideally, if Nclk = 1, the achievable throughput is at most 4 gigabits per second (Gbps), which is far away from the peak data rate even for 5G (e.g., 20Gbps). However, in practical, Nclk is greater than 1. For example, if Nclk = 5, the achievable throughput is only 0.8 Gbps, which is only 1/25 of 20 Gbps.

The third aspect is that the performance of PAS with CCDM degrades for small TBSs. We provide some evaluation for PAS schemes with CCDM and legacy QAM schemes for TBS = 24, 48, 104, 240 under MCS 20 of 256QAM MCS table [6] over AWGN channels. We observe that PAS schemes with CCDM is about 3.5 dB, 2.2 dB, 1.6 dB, and 0.25 dB worse than the legacy QAM schemes at BLER = 0.01 for TBS = 24, 48, 104, and 240, respectively.

The following three aspects need to be considered for CCDM algorithm s:

- Precision mismatch between transmitter and receiver

- Serial process for encoding and decoding

- Performance loss in small TBSs

Analysis#2: (A)ESS and shell mapping algorithm

For other DMs, such (A)ESS or shell mapping, the memory usage is ultra large even though it only consider a bandwidth of one resource block. Detailed analysis is as follows.

The memory usage in bits for (A)ESS or shell mapping is computed as

Nmem=L×NDM×Nprecision,

where L is the number of possible power levels of the amplitude sequence of the DM output, NDM is the DM block length, Nprecision is the computation precision in bits for encoding/decoding. For both (A)ESS and shell mapping combined with 2Qm-QAM, the number of possible power levels L is computed as

L=NDM×Amax2-NDM×Amin28=NDM×2Qm/2×2Qm/2-28,

where Amax=2Qm/2-1 and Amin=1 are the maximum amplitude and the minimum amplitude in the DM output, respectively. However, the computation precision Nprecision of (A)ESS is different from that of shell mapping. For (A)ESS, the typical value for Nprecision is 20 [10]. For shell mapping, the value Nprecision depends on NDM and Qm as Nprecision=NDM×Qm/2-1, where Qm/2-1 is the number of bits corresponding to an amplitude and NDM×Qm/2-1 is the maximum number of data bits input to a DM block.

Assuming a transport block occupies certain number of OFDM symbols (ranging from 2 to 14) in one slot using one resource block, the memory usage in bits is summed over all possible modulation order Qm under the maximum DM block length NDM,max=2×12×14=336, where 2 is the number of amplitudes in one QAM symbol and 12 is the number of QAM symbol in one OFDM symbol of a resource block and 14 is the maximum number of OFDM symbols in a slot. For (A)ESS, the total memory usage in bits summed over modulation orders 4, 6, and 8 is

Nmem,(A)ESStotal=336×24/2×24/2-28×336×20+336×26/2×26/2-28×336×20+336×28/2×28/2-28×336×20=79,027,200,

Thus, the total memory usage for (A)ESS is more than 79 megabits. For shell mapping, the total memory usage in bits summed over modulation orders 4, 6, and 8 is

Nmem,SMtotal=336×24/2×24/2-28×336×336×4/2-1+336×26/2×26/2-28×336×336×6/2-1+336×28/2×28/2-28×336×336×8/2-1=3,679,506,432.

Thus, the total memory usage for shell mapping is more than 3.5 gigabits, which is too large for implementation.

PAS schemes with (A)ESS/shell mapping have an ultra-high memory requirement for encoding/decoding.

- Impact on re-transmission

Besides, the impact on re-transmission should be considered. The probability of amplitudes is chosen to match the MCS of the initial transmission. If the channel changes or the MCS changes in the re-transmission, the probability is no longer matched. The performance of PAS schemes will degrade or even worse than legacy QAM schemes.

IR-HARQ performance may degrade when PAS is used.

Based on analysis above, we think at least these technical issues should be addressed if we consider PAS for 6G modulation.

The following issues for PAS can be studied and evaluated:

  - Impact on channel coding chain

  - Serial process and storage caused by DM algorithms

  - Initial and retransmission performance

Discussion on low PAPR modulation

Background

In OFDM-based system, the Peak-to-Average Power Ratio (PAPR) is a critical parameter that affects the efficiency of the power amplifier (PA). A high PAPR pushes PA into the nonlinear distortion region unless significant back-off is applied, which reduces PA efficiency and increasing power consumption.

Although 5G introduced π/2-BPSK on the uplink to reduce PAPR to mitigate this issue to some extent. 6GR still demands even lower PAPR due to the following reasons:

- According to the 6G SID, bands around 7 GHz are expected to deliver coverage comparable to today’s mid-band (~3.5 GHz). However, higher frequencies suffer greater path loss, so low-PAPR waveforms are needed to preserve PA efficiency and support ubiquitous connectivity.

- Energy efficiency is the design target in 6GR day1. Lower PAPR reduces PA nonlinear distortion, enabling the PA to operate efficiently closer to saturation, which is crucial for reducing UE power consumption and extending battery life.

Therefore, to meet 6G’s coverage and energy efficiency requirements, it remains necessary to investigate low PAPR schemes.

In RAN1#122 meeting, several non-transparent PAPR reduction schemes for DFT-s-OFDM were proposed for 6GR, which can be summarized into the following main directions:

- Spectral extension: Extending the frequency-domain data symmetrically or cyclically to reduce PAPR.

- Frequency-domain truncation: Truncating the π/2-BPSK frequency-domain signal to improve spectral efficiency while preserving its inherent low-PAPR characteristics.

- Time-domain interpolation: Inserting new modulation symbols between adjacent time-domain modulated symbols to reduce phase transitions, thereby lowering PAPR.

- Non-transparent frequency-domain spectral shaping.

These directions can be used individually or jointly to further reduce PAPR. Therefore, non-transparent PAPR reduction schemes need to be investigated.

Non-transparent PAPR reduction schemes should be studied in 6GR.

Discussion on interpolation modulation

An interpolation-based π/2-BPSK modulation scheme (I-π/2-BPSK) can be considered for DFT-s-OFDM waveform to further reduce PAPR. This method introduces an interpolated symbol between each pair of adjacent π/2-BPSK modulation symbols, effectively changing the phase transition between adjacent modulation symbols from π/2 to π/4. After performing a DFT transformation, the bandwidth of the I-π/2-BPSK is consequently expanded to twice that of the original π/2-BPSK due to this interpolation. Following the DFT transformation, the I-π/2-BPSK data undergoes frequency-domain truncation and frequency domain spectral shaping (FDSS). The detailed processing flow is illustrated in Figure 7.

Figure 7 I-π/2-BPSK modulation procedure

Interpolation

The encoded bits are modulated to generate π/2-BPSK modulation symbols. Furthermore, a new modulation symbol is interpolated between every pair of adjacent π/2-BPSK symbols. The interpolated symbol is defined as the sum of these two adjacent π/2-BPSK symbols divided by 2 (for amplitude normalization). Concretely, given a π/2‑BPSK sequence s0,s1,s2,...,sN-1, the generated I-π/2‑BPSK sequence a0,a1,a2,...,a2N-1 can be expressed as

a2n=sn                                   (4-1)

a2n+1=(sn+sn+1)/2 for n<N-1                       (4-2)

a2N-1=(sN-1+s0)/2                            (4-3)

where n = 0, ... , N−1. Consequently, the resulting I-π/2-BPSK sequence can be expressed as illustrated in Figure 8.

Figure 8 I-π/2‑BPSK sequence

In the I-π/2-BPSK sequence, the symbols with even indices are the original π/2-BPSK symbols, while those with odd indices are the interpolated symbols. With the interpolation operation, the phase difference between adjacent I-π/2-BPSK symbols is π/4. Therefore, I-π/2-BPSK modulation approach can achieve a lower PAPR for DFT-s-OFDM transmission compared to conventional π/2-BPSK

By inserting the normalized sum of two adjacent π/2-BPSK symbols between them, I-π/2-BPSK can reduce the phase difference between consecutive symbols from π/2 to π/4.

DFT operation

Next, the time-domain I-π/2-BPSK sequence is transformed into the frequency domain via a DFT operation (i.e., transform precoding). And the DFT data is circularly shifted by half of its length in frequency-domain using DFT-shift, so that the phase of frequency components is rearranged to [-π, π].

Since the interpolation process in I-π/2-BPSK modulation effectively doubles the original π/2-BPSK sequence, the corresponding DFT output occupies twice bandwidth of the original signal.

Figure 9 Bandwidth comparison of π/2-BPSK and I-π/2-BPSK

Due to interpolation process, I-π/2-BPSK occupies twice the bandwidth of the original π/2-BPSK signal, which is equivalent to spectrum extension.

Frequency domain truncation

Since the bandwidth of I-π/2-BPSK is twice that of the original π/2-BPSK (i.e., 2*BW), frequency-domain truncation with a truncation factor F is applied to improve spectral efficiency, where F<1, e.g., 3/4 or 1/2, etc. Specifically, only the central fraction F of the frequency-domain components are retained, while the outer portions are discarded. The corresponding PSD is illustrated in Figure 10. As a result, the occupied bandwidth of I-π/2-BPSK is reduced to 2*BW*F. Consequently, I-π/2-BPSK results in spectral extension if F > 1/2, maintains the same bandwidth as the original π/2-BPSK if F = 1/2, and achieves bandwidth compression if F < 1/2. Different values of truncation factor F can be studied to adapt to various spectrum efficiency requirements.

Figure 10 Frequency-domain truncation for I-π/2-BPSK

Frequency-domain truncation with a truncation factor F (F<1) can be applied for I-π/2-BPSK to adapt to different spectrum efficiency requirements.

- Frequency domain spectral shaping

Additionally, transparent frequency domain spectral shaping can be applied to I-π/2-BPSK modulation after frequency-domain truncation, further reducing the PAPR without causing significant performance degradation. Figure 11 illustrates the normalized power spectral density of π/2-BPSK, I-π/2-BPSK and FDSS filters. It can be observed that the truncated I-π/2-BPSK spectrum aligns more closely with the FDSS envelope.

Figure 11 PSD of π/2-BPSK, I-π/2-BPSK and FDSS filters

Simulation results for interpolation modulation

To evaluate the performance, we present the relevant simulation results in this section. The detailed simulation parameters and assumptions are provided in Appendix Table A-4.

PAPR

Figure 12 shows the PAPR performance for π/2-BPSK and I-π/2-BPSK schemes.

| (a) FDSS: 3-tap with [-0.2, 1, -0.2] | (b) FDSS: Root raised cosine |
| --- | --- |

Figure 12 PAPR for π/2-BPSK and I-π/2-BPSK schemes

From Figure 12, it can be observed that:

Compared to π/2-BPSK without FDSS, I-π/2-BPSK with FDSS provides PAPR reduction of approximately 2.8 dB and 2.3 dB for 3/5 and 1/2 truncation, respectively, at 1% CCDF.

Compared to π/2-BPSK with FDSS, I-π/2-BPSK with FDSS achieves approximately 1 dB and 0.6 dB lower PAPR for 3/5 and 1/2 truncation, respectively, at 1% CCDF.

BLER

Figure 13 and 14 shows the BLER performance for π/2-BPSK and I-π/2-BPSK schemes. The simulations assume the same spectral efficiency across all schemes in each figure. That is, the same transport block size is carried within the same effective transmission bandwidth.

| (a) FDSS: 3-tap with [-0.2, 1, -0.2] | (b) FDSS: Root raised cosine |
| --- | --- |

Figure 13 BLER performance for SE of 0.2344

| (a) FDSS: 3-tap with [-0.2, 1, -0.2] | (b) FDSS: Root raised cosine |
| --- | --- |

Figure 14 BLER performance for SE of 0.1934

From Figure 13 and 14, it can be observed that:

I-π/2-BPSK with FDSS achieves better BLER performance than π/2-BPSK with FDSS under the same spectral efficiency.

I-π/2-BPSK with FDSS shows only a slight performance loss of 0.1~0.2 dB for 1/2 truncation and 0.2~0.3 dB for 3/5 truncation, at 1% BLER, compared to π/2-BPSK without FDSS under the same spectral efficiency.

When the net gain is defined by the sum of PAPR and BLER gain, I-π/2-BPSK with FDSS (1/2 truncation) provides approximately 2.1 dB net gain over π/2-BPSK without FDSS, and approximately 0.9 dB net gain over π/2-BPSK with FDSS.

When the net gain is defined by the sum of PAPR and BLER gain, I-π/2-BPSK with FDSS (3/5 truncation) provides approximately 2.5 dB net gain over π/2-BPSK without FDSS, and approximately 1.1 dB net gain over π/2-BPSK with FDSS.

I-π/2-BPSK modulation scheme should be considered to further reduce PAPR for DFT-s-OFDM.

Conclusion

In this contribution, we have the following observations/proposals:

Observation 1:	NUC modulation is a mature technique and has been widely used in broadcast standards.

Observation 2:	Introducing NUC for 6GR does not affect the LDPC coding chain in 5G. The LDPC coding chain in 5G can be reused in 6GR.

Observation 3:	NUC modulation/demodulation is a full parallel process, and it is easier to achieve ultra-high throughput for 6GR.

Observation 4:	According to our simulation results, NUC can have shaping gain from 0.15 dB to 0.75 dB, where shaping gain of NUC increases as the modulation order increases.

	Shaping gain from 0.15dB to 0.75dB where shaping gain of NUC increases as the modulation order increases

	~0.27dB shaping gain for MCS level 18 in NR 256QAM table at BLER = 0.1 in the CDL-A 1T1R channel.

	~0.5 dB shaping gain for MCS level 18 in NR 256QAM table at BLER = 0.1 in the CDL-A 4T4R channel.

Observation 5:	PAS modulation has impacts on coding chain in terms of:

	Additional procedures before or after channel coding

	Bit interleaving

Observation 6:	The following three aspects need to be considered for CCDM algorithm s:

	Precision mismatch between transmitter and receiver

	Serial process for encoding and decoding

	Performance loss in small TBSs

Observation 7:	PAS schemes with (A)ESS/shell mapping have an ultra-high memory requirement for encoding/decoding.

Observation 8:	IR-HARQ performance may degrade when PAS is used.

Observation 9:	By inserting the normalized sum of two adjacent π/2-BPSK symbols between them, I-π/2-BPSK can reduce the phase difference between consecutive symbols from π/2 to π/4.

Observation 10:	Due to interpolation process, I-π/2-BPSK occupies twice the bandwidth of the original π/2-BPSK signal, which is equivalent to spectrum extension.

Observation 11:	Frequency-domain truncation with a truncation factor F (F<1) can be applied for I-π/2-BPSK to adapt to different spectrum efficiency requirements.

Frequency domain spectral shaping

Observation 12:	Compared to π/2-BPSK without FDSS, I-π/2-BPSK with FDSS provides PAPR reduction of approximately 2.8 dB and 2.3 dB for 3/5 and 1/2 truncation, respectively, at 1% CCDF.

Observation 13:	Compared to π/2-BPSK with FDSS, I-π/2-BPSK with FDSS achieves approximately 1 dB and 0.6 dB lower PAPR for 3/5 and 1/2 truncation, respectively, at 1% CCDF.

Observation 14:	I-π/2-BPSK with FDSS achieves better BLER performance than π/2-BPSK with FDSS under the same spectral efficiency.

Observation 15:	I-π/2-BPSK with FDSS shows only a slight performance loss of 0.1~0.2 dB for 1/2 truncation and 0.2~0.3 dB for 3/5 truncation, at 1% BLER, compared to π/2-BPSK without FDSS under the same spectral efficiency.

Observation 16:	When the net gain is defined by the sum of PAPR and BLER gain, I-π/2-BPSK with FDSS (1/2 truncation) provides approximately 2.1 dB net gain over π/2-BPSK without FDSS, and approximately 0.9 dB net gain over π/2-BPSK with FDSS.

Observation 17:	When the net gain is defined by the sum of PAPR and BLER gain, I-π/2-BPSK with FDSS (3/5 truncation) provides approximately 2.5 dB net gain over π/2-BPSK without FDSS, and approximately 1.1 dB net gain over π/2-BPSK with FDSS.

Proposal 1:	The maximum modulation order can be considered for 6GR UL is 1024QAM.

Proposal 2:	The following metrics should be evaluated for shaping modulation:

•	BLER performance

•	Throughput

•	Complexity

Proposal 3:	The following aspects for evaluation assumptions should be considered for shaping modulation:

•	BLER performance for both initial transmission with varied TBS and spectrum efficiency

•	Throughput performance with re-transmission and adaptive MCS assumption

•	MIMO fading channel model

•	Closed loop MIMO for MIMO precoder assumption

•	MMSE receiver for MIMO receiver assumption

Proposal 4:	NUC modulation for 6GR can be studied and considered.

Proposal 5:	The following issues for PAS can be studied and evaluated:

•	Impact on channel coding chain

•	Serial process and storage caused by DM algorithms

•	Initial and retransmission performance

Proposal 6:	Non-transparent PAPR reduction schemes should be studied in 6GR.

Proposal 7:	I-π/2-BPSK modulation scheme should be considered to further reduce PAPR for DFT-s-OFDM.

References

Chair notes, 3GPP TSG RAN WG1 #122, Bengaluru, India, Aug 25th – 29th, 2025.

R1-2506579, FL summary #2 on modulation, joint channel coding and modulation, Moderator (Qualcomm Incorporated), 3GPP TSG RAN WG1 #122, Bengaluru, India, Aug 25th – 29th, 2025

ATSC, “ATSC Proposed Standard: Physical Layer Protocol (A/322),” Advanced Television Systems Committee, no. S32-230r56, Febuary 2017.

ETSI, "Digital Video Broadcasting (DVB); Second generation framing structure, channel coding and modulation systems for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications; Part 1: DVB-S2,” ETSI EN 302 30701, November 2014, version 1.4.1.

3GPP, “Technical Specification Group Radio Access Network; NR; Multiplexing and channel coding (Release 19),” 3rd Generation Partnership Project (3GPP), Technical Specification (TS) 38.212, June 2025, version 19.0.0.

3GPP, “Technical Specification Group Radio Access Network; NR; Physical layer procedures for data (Release 19),” 3rd Generation Partnership Project (3GPP), Technical Specification (TS) 38.214, June 2025, version 19.0.0.

G. Bocherer, F. Steiner, and P. Schulte, “Bandwidth efficient and rate-matched low-density parity-check coded modulation,” IEEE Trans. Commun., vol. 63, no. 12, pp. 4651–4665, December 2015.

R. Laroia, N. Farvardin, and S. A. Tretter, “On Optimal Shaping of Multidimensional Constellations,” IEEE Trans. Inf. Theory, vol. 40, no. 4, pp. 1044–1056, July 1994.

P. Schulte and G. Bocherer, “Constant composition distribution matching,” IEEE Trans. Inf. Theory, vol. 62, no. 1, January 2016.

Y. C. Gultekin, W. J. van Houtum, A. G. C. Koppelaar, and F. M. J. Willems, “Enumerative sphere shaping for wireless communications with short packets,” IEEE Trans. Wireless Commun., vol. 19, no. 2, pp. 1098–1112, 2020.

- Appendix

Table A-1 Simulation setting for AWGN channels

| Parameter | Value |
| --- | --- |
| Transport block size (TBS ) | 24, 48, 104, 240, or 8424 |
| Number of cyclic redundancy check (CRC) bits (Lcrc) | 16 for TBS = 24, 48, 104, 240; 24 for TBS = 8424 |
| CRC polynomial | gCRC16(D) in TS38.212 for TBS = 24, 48, 104, 240; gCRC24A(D) in TS38.212 for TBS = 8424 |
| Modulation and coding scheme (MCS) | MCS index 5 ~ 27 in TS38.214 MCS table 2, see Table A-2 |
| Distribution matching | CCDM |
| Parameter v for Maxwell-Boltzmann distribution | 0.0192 for MCS 20 |
| Channel coding | 5G NR LDPC coding |
| Redundancy version | 0 |
| Bit selection & Bit interleaving | For NUC schemes and QAM schems, same as in TS38.212; For PAS schemes, see description in Section 3.2.2. |
| Length after bit selection (E) | ceil((TBS+Lcrc)/R/Qm)*Qm, R and Qm are target code rate and modulation order indicated by MCS index in Table A-2, respectively |
| Modulation constellation | QAM or NUC with modulation order indicated by the MCS index |
| Maximum number of transmissions | 1 (including initial transmission) |
| Demodulation | maximum a posterior (MAP) algorithm |
| Channel decoding | layered belief propagation (LBP) algorithm with 25 iterations |
| Metric | minimum SNR at a target BLER of 0.01 |

Table A-2 MCS index table 2 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

Table A-3 Simulation setting for fading channels

| Parameter |  | Value |
| --- | --- | --- |
| System settings | Bandwidth | 100 MHz |
|  | Subcarrier spacing | 30 kHz |
|  | RB number | 6 (allocated continously) |
|  | OFDM symbols | 14 with 0 for PDCCH, 13 for data, and 1 for DM-RS |
|  | Cyclic prefix | normal |
| MIMO settings | Receive Type | LMMSE |
|  | Antenna Type | SU-MIMO |
|  | NB antenna Number (Tx) | 1 for 1 layer, 4 for 4 layers |
|  | UE antenna Number (Rx) | 1 for 1 layer, 4 for 4 layers |
|  | Layer Number (Nlayer) | fixed to 1 or 4 |
|  | MIMO Precoder | closed-loop with SVD |
|  | Code block Frequency Granularity | 24 RBs |
|  | Code block Time Granularity | 5 slots |
|  | Feedback Delay | 5 slots |
| Channel settings | Channel model | CDL-A |
|  | Carrier frequency (fc) | 4 GHz |
|  | UE speed | 3 km/h |
|  | Delay spread | 30 ns |
|  | Channel estimation type | Ideal |
| Coding settings | Channel coding | 5G NR LDPC coding |
|  | Maximum number of transmissions | 1 (including initial transmission) |
|  | LDPC decoding algorithm | scaled layered min-sum with 25 maximum iterations and scalar = 0.75 |
|  | MCS | MCS index 18 for both 1 layer and 4 layers in Table A-2 |
| Modulation setting | Constellation type | 64QAM, 2D 64NUC |
|  | Demodulation algorithm | maximum-logarithmic maximum a posterior (Max-log MAP) |

Table A-4 Simulation parameters for low PAPR modulation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 15 KHz |
| Number of antennas | 1T4R |
| Bandwidth | 2 PRBs |
| Spectral efficiency | 0.1934; 0.2344 |
| Target BLER | 0.01 |
| Channel | TDL-C 300ns |
| UE speed | 3 Km/h |
| Waveform | DFT-s-OFDM |
| Nmuber of DMRS | 2 OFDM symbols |
| FDSS | Opt 1: 3-tap with [-0.2, 1, -0.2]; Opt 2: Root raised cosine with α=1 and 1.25 times length than bandwidth |
| PAPR CCDF | Where x is the data sample, P(·) represents the probability, xpapr is the PAPR decision threshold (dB). |