3GPP TSG RAN WG1 #122bis                       		                                                    R1-2506879

Prague, Czech, Oct 13th – 17th, 2025

Source: 	vivo

Title:	Maintenance on Low band carrier aggregation via switching

Agenda Item:	8.8

Document for:	Discussion and Decision

- Introduction

In the contribution, some remaining issues on low band carrier aggregation via switching will be discussed.

- Discussion

In this section, the remaining issues on the impact on DL and UL transmissions are discussed.

For a UE, it can only perform Tx/Rx on FDD carrier during FDD carrier duration which is not overlapped with switching gap, and can only perform Rx on SDL carrier duration SDL carrier duration which is not overlapped with switching gap. In the last meeting, it was agreed that for Collision 1,2,3,4,5, and 6, the UE does not receive or transmit the corresponding higher layer configured transmissions. This is similar as the behavior when a configured transmission overlaps with the symbols semi-statically configured as TDD UL/DL symbols.

Some remaining issues related with transmission/reception overlapping with switching gap or the slots configured on the other cell are still open. In our view, for DL/UL transmission on a serving cell, collisions with the symbols in switching gap or the slots configured on the other cell can be treated in a similar view as collisions with the symbols semi-statically configured as TDD UL/DL symbols, and the similar UE behaviors can be reused. The details are as following.

  - CSI-RS measurement occasion

For CSI-RS, it was agreed that if a UE is configured by higher layers to receive a CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, or with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the CSI-RS and if a UE is configured by higher layers to receive a CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, or with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the CSI-RS (collision 3,4,5,6).

Currently, when UE is configured with DRX or cell DTX, since UE does not receive some CSI-RS during the inactive period of DRX or cell DTX, the UE reports a CSI report only based on the most recent CSI measurement occasion which is in the active period of DRX or cell DTX, i.e., the UE reports a CSI report only at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell in active periods of DRX or cell DTX of the serving cell no later than CSI reference resource is received, and the UE drops the CSI report otherwise.

If semi-static switching pattern is configured for low band carrier aggregation via switching, the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell should be defined and should occur in valid time of the serving cell for CSI report configured by CSI-ReportConfig.  The UE should only report the CSI report when the UE receives at least one CSI-RS transmission occasion in the valid time of the serving cell no later than CSI reference resource, and the UE drops the CSI report otherwise, wherein the valid time of a cell includes the symbols in a slot configured on the cell by switching pattern and not overlapping with switching gap, i.e., the symbols that are available for CSI-RS receptions as described in Clause 24 of TS 38.213 .

Proposal 1: For periodic CSI-RS and semi-persistent CSI-RS on a serving cell configured in CSI report configuration in CSI-ReportConfig,

- The most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in the symbols available for CSI-RS receptions as described in Clause 24 of TS 38.213.

- The UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell in the symbols available for CSI-RS receptions as described in Clause 24 of TS 38.213, no later than CSI reference resource, and the UE drops the CSI report otherwise.

The related TP for TS 38.214 is given in appendix TP#1.

  - HARQ-ACK feedback

Currently, in HARQ-ACK codebook generation, for the SPS PDSCHs overlapping with symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX if the serving cell is activated with cell DTX, since the UE does not receive these SPS PDSCHs, it does not transmit corresponding HARQ-ACK information when generating SPS PDSCH HARQ-ACK codebook. Similar operation should be applied for SPS PDSCHs overlapping with the symbols that are not available for SPS PDSCHs receptions due to LB CA switching since the UE does not receive these SPS PDSCHs either.

Proposal 2: For HARQ-ACK codebook generation of SPS PDSCHs, the UE does not generate HARQ-ACK information for SPS PDSCHs overlapping with symbols not available for SPS PDSCH receptions as described in Clause 24 of TS 38.213 when the switching pattern is applied.

Currently, for type 1 HARQ-ACK codebook generation, UE will generate the HARQ-ACK information for each serving cell and concatenate the HARQ-ACK information of all serving cells in ascending order of serving cell index for CA case. For LB CA via switching, same rule is applied since it’s based on the CA framework. In addition, for each serving cell, UE needs to determine a set of occasions for candidate PDSCH receptions where UE will exclude some rows of the TDRA table if at least one symbols of the PDSCH time resource derived by the row is configured as semi-static UL symbols by TDD configuration. Here, for LB CA via switching, similar handling can be used. That is, if at least one symbol of the PDSCH time resource derived by a row of the TDRA table on a cell overlaps with the symbols that are not available for PDSCH receptions, the row of  the TDRA table should be excluded for HARQ-ACK codebook generation.

The benefits of reducing redundant HARQ-ACK feedback and UE power consumption is obvious to not feedback HARQ-ACK information for the PDSCH overlapping with invalid time. However, there was a concern that this approach is not aligned with the legacy HARQ-ACK codebook construction for SPS PDSCH or type 1 HARQ-ACK codebook construction, which is based on configured serving cell. Specifically, in the case that Scell is deactivated, wherein the switching pattern is not applied,  the HARQ-ACK information for PDSCH on Pcell overlapping with the switching gap and duration of SDL carrier should not be excluded. To solve this concern, the exclusion operation can be done only when the switching pattern is applied, which is similar as the handling of inactive time of cell DTX. In addition, there is another precedent to generate HARQ-ACK codebook based on Scell activation/deactivation state in legacy operation. That is, it is specified that if a serving cell is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id.

Proposal 3: For type 1 HARQ-ACK codebook generation, a row of the TDRA table overlapping with the symbols not available for PDSCH receptions as described in Clause 24 of TS 38.213 is excluded in determining the set of occasions of candidate PDSCH receptions when the switching pattern is applied.

The related TPs for TS 38.213 for SPS PDSCH HARQ-ACK codebook and type 1 HARQ-ACK codebook are given in appendix TP#2 and TP#3 respectively.

  - PUCCH repetition

In legacy specification, for PUCCH repetition in FDD carrier, a UE determines the NPUCCHrepeat  slots for a PUCCH transmission as the NPUCCHrepeat  consecutive slots starting from a slot indicated to the UE for HARQ-ACK reporting, or a slot determined for SR reporting or for CSI reporting; for PUCCH repetition in TDD carrier, postpone is adopted when a PUCCH repetition overlaps with semi-static DL symbols configured by TDD pattern and/or SS/PBCH block symbols..

When the UE is provided LBCA-SwitchingPattern, for configured PUCCH, it was agreed that the UE does not transmit the PUCCH if the PUCCH overlaps with the symbols configured as switching gap or the slots configured on Scell. However, it is still open whether to count the slots not available for PUCCH repetition due to LB CA switching, i.e., it is not clear that postpone or dropping is adopted when a PUCCH repetition overlaps with switching gap or slots configured on Scell. In order  to guarantee the reliability of PUCCH transmission, postpone should be adopted when a PUCCH repetition overlaps with symbols not available for PUCCH transmission as described in clause 24 of TS 38.213.

Proposal 4: For PUCCH repetition in FDD carrier, when a UE is provided LBCA-SwitchingPattern, the UE postpones a PUCCH repetition in a slot if the PUCCH repetition overlaps with the symbols not available for PUCCH transmission as described in Clause 24 of TS38.213.

The related TPs for TS 38.213 is given in appendix TP#4.

- Conclusion

In this contribution, the following observations and proposals are summarized as follows:

Proposal 1: For periodic CSI-RS and semi-persistent CSI-RS on a serving cell configured in CSI report configuration in CSI-ReportConfig,

- The most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in the symbols available for CSI-RS receptions as described in Clause 24 of TS 38.213.

- The UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell in the symbols available for CSI-RS receptions as described in Clause 24 of TS 38.213, no later than CSI reference resource, and the UE drops the CSI report otherwise.

Proposal 2: For HARQ-ACK codebook generation of SPS PDSCHs, the UE does not generate HARQ-ACK information for SPS PDSCHs overlapping with symbols not available for SPS PDSCH receptions as described in Clause 24 of TS 38.213 when the switching pattern is applied.

Proposal 3: For type 1 HARQ-ACK codebook generation, a row of the TDRA table overlapping with the symbols not available for PDSCH receptions as described in Clause 24 of TS 38.213 is excluded in determining the set of occasions of candidate PDSCH receptions when the switching pattern is applied.

Proposal 4: For PUCCH repetition in FDD carrier, when a UE is provided LBCA-SwitchingPattern, the UE postpones a PUCCH repetition in a slot if the PUCCH repetition overlaps with the symbols not available for PUCCH transmission as described in Clause 24 of TS38.213.

# Appendix

## TP#1 for TS 38.214

Reason for change: The most recent measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on a serving cell is not defined when UE is configured with a DL carrier switching pattern and UE reporting behavior when UE does not receive any CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell is not clear. Summary of change: Specify the most recent measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on a serving cell is not defined when UE is configured with a DL carrier switching pattern and UE reporting behavior when UE does not receive any CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell. Consequences if not approved: The measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on a serving cell is not clear when UE is configured with LBCA-SwitchingPattern.

*********************************Start of TP#1********************************************

5.1.6.1	CSI-RS reception procedure

<Unchanged part omitted>

If the UE is configured with DRX and,

-	if  the UE is configured to monitor DCI format 2_6 or WUS and configured by higher layer parameter ps-TransmitOtherPeriodicCSI or lpwus-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'cri-RSRP-Index', 'ssb-Index-RSRP' and 'ssb-Index-RSRP-Index' when drx-onDurationTimer in DRX-Config [or lpwus_PDCCHMonitoringTimer in [XYZxxx]] is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported;

-	if the UE is configured to monitor DCI format 2_6 or WUS and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP or lpwus-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'cri-RSRP-Index' when drx-onDurationTimer in DRX-Config [or lpwus_PDCCHMonitoringTimer [in XYZxxx]] is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported;

-	otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported.

During non-active periods of cell DTX if cell DTX is activated for a serving cell, the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P'. If the cell DTX is activated for a serving cell [10, TS 38.321], the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in active periods of cell DTX for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P'.

If a UE is provided with LBCA-SwitchingPattern, the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on a serving cell occurs in symbols available for CSI-RS receptions as described in Clause 24 of [6, TS 38.213].

5.2.2.5	CSI reference resource definition

<Unchanged part omitted>

For the CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI', the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell with cell DTX activated [10, TS 38.321] for channel measurement and/or interference measurement in active periods of cell DTX of the serving cell no later than CSI reference resource, and the UE drops the CSI report otherwise.

When the UE is configured with LBCA-SwitchingPattern, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell for channel measurement and/or interference measurement in symbols in the symbols available for CSI-RS receptions as described in Clause 24 of [6, TS 38.213], and no later than CSI reference resource, and the UE drops the CSI report otherwise.

When deriving CSI feedback, the UE is not expected that a NZP CSI -RS resource for channel measurement overlaps with CSI-IM resource for interference measurement or NZP CSI -RS resource for interference measurement.

*********************************End of TP#1********************************************

## TP#2 for TS 38.213

Reason for change: In the periods that a UE is configured to perform TX/RX on FDD carrier, the SPS PDSCHs on SDL carrier are not received by the UE. In the periods that a UE is configured to perform RX on SDL carrier, the SPS PDSCHs on FDD carrier are not received by the UE. There is no need to transmit corresponding HARQACK information of these SPS PDSCHs. Summary of change: Specify that SPS PDSCH HARQ-ACK codebook should exclude HARQ-ACK information the SPS PDSCHs on a cell overlapping with switching gap or the slots configured on the other cell. Consequences if not approved: UE transmits the HARQ-ACK information for the SPS PDSCH on a cell overlapping with switching gap or the slots configured on the other cell.

**********************************Start of TP#2********************************************

9.1.2	Type-1 HARQ-ACK codebook determination

<Unchanged part omitted>

In the following pseudo-code, SPS PDSCH receptions associated with a SPS PDSCH configuration are activated by a DCI format with CRC scrambled by a CS-RNTI or by a DCI format with CRC scrambled by a G-CS-RNTI.

Set NcellsDL to the number of serving cells configured to the UE

Set NcSPS to the number of SPS PDSCH configurations configured to the UE for serving cell c

Set NcDL to the number of DL slots for SPS PDSCH receptions on serving cell c with HARQ-ACK information multiplexed on the PUCCH

Set j=0 – HARQ-ACK information bit index

Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

while c<NcellsDL

Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations

while s<NcSPS

Set nD=0 – slot index

while nD<NcDL

if {

a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols

-	indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, and/or

-	determined as non-active period of cell DTX [11, TS 38.321]

-	not available for SPS PDSCH receptions as described in clause 24 of [6, TS 38.213] when the switching pattern configured by LBCA-SwitchingPattern is applied

where, for unicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-config and, for multicast SPS PDSCHs, NPDSCHrepeat is provided by repetitionNumber if contained in an entry indicated by the time domain resource assignment field in the DCI format scheduling the PDSCH repetition, or provided by pdsch-AggregationFactor-r16 if included in SPS-Config or, otherwise, NPDSCHrepeat=1, and

HARQ-ACK information for the SPS PDSCH is associated with the PUCCH

}

ojACK = HARQ-ACK information bit for this SPS PDSCH reception

j=j+1;

end if

nD=nD+1;

end while

s=s+1;

end while

c=c+1;

end while

**********************************End of TP#2*********************************************

## TP#3 for TS 38.213

Reason for change: In the periods that a UE is configured to perform TX/RX on FDD carrier, the PDSCHs on SDL carrier are not received by the UE, In the periods that a UE is configured to perform RX on SDL carrier, the PDSCHs on FDD carrier are not received by the UE. There is no need to transmit corresponding HARQACK information of the PDSCHs. Summary of change: Specify to exclude the SLIVs on a cell overlapping with switching gap or the slots configured on the other cell when generating type 1 HARQ-ACK codebook. Consequences if not approved: UE transmits the HARQ-ACK information for the candidate PDSCH reception occasion on a cell overlapping with switching gap or the slots configured on the other cell.

**********************************Start of TP#3********************************************

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<Unchanged part omitted>

Set j=0 - index of occasion for candidate PDSCH reception or SPS PDSCH release or TCI state update

Set B=∅

Set MA,c=∅

Set CK1 to the cardinality of set K1

Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 for serving cell c

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while k<CK1

if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

if either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, and timeDomainHARQ-BundlingType1 are provided for serving cell c

R=RT;

R'=RT';

Elseif either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided and timeDomainHARQ-BundlingType1 is not provided for serving cell c

R=RT;

else

Set R to the set of rows

end if

Set CR to the cardinality of R

Set r=0 – index of row in set R

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the serving cell of PUCCH transmission, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k,

nD=nD+1;

else

while r<CR

if the UE is not provided timeDomainHARQ-BundlingType1 and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is

        - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or

        - not available for PDSCH receptions as described in clause 24 of [6, TS 38.213] when the switching pattern configured by LBCA-SwitchingPattern is applied

where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided, and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,k+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided timeDomainHARQ-BundlingType1 and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is

        - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or

        - not available for PDSCH receptions as described in clause 24 of [6, TS 38.213] when the switching pattern configured by LBCA-SwitchingPattern is applied

and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is

        - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or

        - not available for PDSCH receptions as described in clause 24 of [6, TS 38.213] when the switching pattern configured by LBCA-SwitchingPattern is applied

if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

if the UE is not provided with multiPDSCH-perSlotType1-CB and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with multiPDSCH-perSlotType1-CB = 'disabled' and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

Set m to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of R

while R≠∅

Set r=0

while r<CR

if S≤m for start OFDM symbol index S for row r

br,k,nD=j; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row r

R=R\r;

B=B∪br,k,nD;

else

r=r+1;

end if

end while

MA,c=MA,c∪j

j=j+1;

Set m to the smallest last OFDM symbol index among all rows of R;

end while

end if

nD=nD+1;

end if

end while

end if

k=k+1;

end while

else

while k<CK1

if modnU-K1,k+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

if either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, and timeDomainHARQ-BundlingType1 are provided for serving cell c

R=RT;

R'=RT';

Elseif either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided, and timeDomainHARQ-BundlingType1 is not provided for serving cell c

R=RT;

else

Set R to the set of rows

end if

Set CR to the cardinality of R

Set r=0 – index of row in set R

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and slot  n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the serving cell of PUCCH transmission where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0,

nD=nD+1;

else

while r<CR

if the UE is not provided timeDomainHARQ-BundlingType1 and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is

        - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or

        - not available for PDSCH receptions as described in clause 24 of [6, TS 38.213] when the switching pattern configured by LBCA-SwitchingPattern is applied

where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1 or if either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided, and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,k+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided timeDomainHARQ-BundlingType1 and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured

        - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or

        - not available for PDSCH receptions as described in clause 24 of [6, TS 38.213] when the switching pattern configured by LBCA-SwitchingPattern is applied

and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD at least one symbol of the PDSCH time resource derived by row r of set R is

        - configured as UL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or

        - not available for PDSCH receptions as described in clause 24 of [6, TS 38.213] when the switching pattern configured by LBCA-SwitchingPattern is applied

if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2, where d = 0,1,…,C∆K0,r-1.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

if the UE is not provided with multiPDSCH-perSlotType1-CB and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with multiPDSCH-perSlotType1-CB = 'disabled' and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

Set m to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of R

while R≠∅

Set r=0

while r<CR

if S≤m for start OFDM symbol index S for row r

br,k,nD=j; - index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update associated with row r

R=R\r;

B=B∪br,k,nD;

else

r=r+1;

end if

end while

MA,c=MA,c∪j;

j=j+1;

Set m to the smallest last OFDM symbol index among all rows of R;

end while

end if

nD=nD+1;

end if

end while

end if

k=k+1;

end while

end if

**********************************End of TP#3********************************************

## TP#4 for TS 28.213

Reason for change: When a PUCCH repetition configured by higher layers overlaps with at least one symbol not available for PUCCH transmission described in clause 24, it is not clear that the PUCCH repetition is dropped or is postponed. Summary of change: Specify when a PUCCH repetition configured by higher layers overlaps with at least one symbol not available for PUCCH transmission described in clause 24, the PUCCH repetition is postponed. Consequences if not approved: UE behavior on PUCCH repetition configured by higher layers is not clear when UE is configured with LBCA-SwitchingPattern.

**********************************Start of TP#4********************************************

9.2.6	PUCCH repetition procedure

A UE that does not have dedicated PUCCH resource configuration and indicates a capability to transmit with repetitions a PUCCH with HARQ-ACK information [11, TS 38.321], determines a number of NPUCCHrepeat slots for repetitions of a PUCCH transmission with HARQ-ACK information based on an indication by numberOfMsg4HARQ-ACK-Repetitions. If numberOfMsg4HARQ-ACK-Repetitions provides more than one values, the DAI field in a DCI format 1_0 with CRC scrambled by a TC-RNTI scheduling a PDSCH reception that includes a UE contention resolution identity indicates NPUCCHrepeat from the more than one values. The UE transmits any PUCCH with repetitions over NPUCCHrepeat slots before dedicated PUCCH resource configuration is provided. The UE transmits each repetition of the PUCCH using frequency hopping as described in Clause 9.2.1.

In the remaining of this clause, a UE without dedicated PUCCH resource configuration determines a value of a parameter, if applicable, according to Table 9.2.1-1 and/or as specified above in this clause for a PUCCH transmission with repetitions from the UE.

A UE can be indicated to transmit a PUCCH over NPUCCHrepeat slots using a PUCCH resource, where

-	if the PUCCH resource is indicated by a DCI format and includes pucch-RepetitionNrofSlots, NPUCCHrepeat is provided by pucch-RepetitionNrofSlots

-	otherwise, NPUCCHrepeat is provided by nrofSlots

If the UE is provided subslotLengthForPUCCH, a slot for a PUCCH transmission with repetitions over NPUCCHrepeat>1 slots includes a number of symbols indicated by subslotLengthForPUCCH.

For NPUCCHrepeat>1,

-	the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots

-	if the UE is provided multipanelSFN-Scheme and applyIndicatedTCI-State = 'both', a repetition of the PUCCH transmission simultaneously uses first and second spatial domain filters corresponding to first and second TCI-State or TCI-UL-State

-	a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same number of consecutive symbols, as provided by nrofSymbols

-	a repetition of the PUCCH transmission in each of the NPUCCHrepeat slots has a same first symbol, as provided by startingSymbolIndex if subslotLengthForPUCCH is not provided; otherwise mod(startingSymbolIndex, subslotLengthForPUCCH)

-	the UE is configured by interslotFrequencyHopping whether or not to perform frequency hopping for repetitions of the PUCCH transmission in different slots

-	if the UE is configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is not provided pucch-DMRS-Bundling = 'enabled'

-	the UE performs frequency hopping per slot

-	the UE transmits the PUCCH starting from a first PRB, provided by startingPRB or by startingPRB-SBFD if the repetition of the PUCCH transmission is over SBFD symbols and startingPRB-SBFD is provided, in slots with even number and starting from a second PRB, provided by secondHopPRB or by secondHopPRB-SBFD if the repetition of the PUCCH transmission is over SBFD symbols and secondHopPRB -SBFD is provided, in slots with odd number. The slot indicated to the UE for the first repetition of the PUCCH transmission has number 0 and each subsequent slot until the UE transmits the PUCCH in NPUCCHrepeat slots is counted regardless of whether or not the UE transmits the PUCCH in the slot

-	the UE does not expect to be configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot

-	if the UE is configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is provided pucch-DMRS-Bundling = 'enabled'

-	the UE performs frequency hopping per interval of NPUCCHinterval consecutive slots, that start from a slot indicated to the UE and where the UE would transmit a first repetition of the PUCCH, where NPUCCHinterval is the value of pucch-FrequencyHoppingInterval, if provided; otherwise, NPUCCHinterval is the value of pucch-TimeDomainWindowLength

-	the UE transmits the PUCCH over intervals until the UE transmits the PUCCH in NPUCCHrepeat slots, where the first interval has number 0 and each subsequent interval is counted regardless of whether or not the UE transmits the PUCCH in a slot

-	the UE transmits the PUCCH starting from a first PRB, provided by startingPRB or by startingPRB-SBFD if the repetition of the PUCCH transmission is over SBFD symbols and startingPRB-SBFD is provided, in intervals with even number and starting from a second PRB, provided by secondHopPRB or by secondHopPRB-SBFD if the repetition of the PUCCH transmission is over SBFD symbols and secondHopPRB -SBFD is provided, in intervals of frequency hopping intervals with odd number

-	the UE does not expect to be configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot

-	if the UE is not configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot, the frequency hopping pattern between the first PRB and the second PRB is same within each slot

If the UE determines that, for a repetition of a PUCCH transmission in a slot, the number of symbols available for the PUCCH transmission is smaller than the value provided by nrofSymbols for the corresponding PUCCH format, the UE does not transmit the PUCCH repetition in the slot.

A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index corresponding to the SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided dl-OrJointTCI-StateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting.

For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having

-	an UL symbol, as described in clause 11.1, or flexible symbol, or an SBFD symbol as described in clause 11.1, that is not SS/PBCH block symbol provided by startingSymbolIndex as a first symbol, and

-	consecutive UL symbols, as described in clause 11.1, or flexible symbols, or consecutive SBFD symbols as described in clause 11.1, respectively, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols

For paired spectrum or supplementary uplink band, if the UE is not provided LBCA-SwitchingPattern, the UE determines the NPUCCHrepeat slots for a PUCCH transmission as the NPUCCHrepeat consecutive slots starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting. If the UE is provided LBCA-SwitchingPattern, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having the symbols of the PUCCH transmission not overlapping with the symbols not available for PUCCH transmission as described in clause 24.

If a UE would transmit a PUCCH over a first number NPUCCHrepeat>1 of slots and the UE would transmit a PUSCH with repetition Type A or with TB processing over multiple slots over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots.

If a UE would transmit a PUCCH over a first number NPUCCHrepeat>1 of slots and the UE would transmit a PUSCH with repetition Type B over a second number of slots, and the PUCCH transmission would overlap with actual PUSCH repetitions in one or more slots, and the conditions in clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied for the overlapping actual PUSCH repetitions, the UE transmits the PUCCH and does not transmit the overlapping actual PUSCH repetitions.

<Unchanged part omitted>

**********************************End of TP#4********************************************