3GPP TSG RAN WG1 #122bis	R1-2506986

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	10.5.1

Source:	Xiaomi

Title:	Discussion on performance evaluation for ISAC

Document for:	Discussion and Decision

- Introduction

The Rel-20 study item on performance evaluation for integrated sensing and communication (ISAC) has been approved in the RAN#108 meeting, and includes the following objectives [1].

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

Good progress was achieved last meeting on the performance metrics and evaluation assumptions. In this contribution, we provide a detailed discussion on remaining parts for ISAC performance evaluation, including issues for performance metrics, evaluation assumptions, measurements, evaluation results, etc.

- Performance metrics discussion

  - Use case to be considered during the evaluation

Object detection is the most essential feature in sensing and should be studied. Tracking, on the other hand, is more complicated in terms of functionality that needs to be decided and potentially agreed in the group; additionally, it is also more complex to simulate, since for example an object trajectory would need to be agreed.

Given that this is the first time that RAN1 is evaluating sensing and that only 3 meetings are left to complete the study, we suggest to start with focusing on the most basic functionality, i.e., object detection. The evaluation on UAV tracking could be up to company inputs.

Proposal 1: RAN1 discussion focuses on UAV detection

- Up to companies to report the evaluation result for UAV tracking

  - Definition of performance metrics

Following is the agreement for FAP.

Agreement - False alarm probability is agreed as performance metric for NR ISAC. - For cases without true target dropped in simulation area, False alarm probability Type 1 is computed and reported - For cases with true targets dropped in simulation area, False alarm probability Type 2 is computed and reported - Note: both False alarm probability Types are mandatory - KPI values for False alarm probability Type 1 and 2 can be discussed separately. Definitions: - False alarm probability Type 1 (no target dropped in simulation area): An object is detected when there is no target present in simulation area is considered a false alarm. Pf1=n=0N-1QnN Where, - Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0. - N is the total number of drops without targets in the simulation area. - False alarm probability Type 2 (targets dropped in simulation area): An object is detected but not associated with any true targets in the simulation area is considered as a false alarm. Pf2=0≤n<NMn'≠0Dn'Mn'K Where, - Dn' is the number of detected objects but not associated with any true targets in the drop n. - Mn' is the total number of detected objects in the drop n. - FFS: - Option 1: K is number of drops (N) - Option 2: K is number of drops with at least one detected object Note: the number of targets should be reported by companies when providing False alarm probability Type 2

For false alarm, a FAP Type 1 and a FAP Type 2 probability were introduced. Type 1 describes the false alarm probability when no target is dropped in the simulated area and Type 2 the false alarm probability when at least one target is dropped. For FAP Type 1, a given simulation run produces exactly one false alarm if at least 1 target is detected. Since no true target is dropped, it is not possible to compute the misdetection probability or to estimate position/velocity accuracy from this simulation. Therefore, to also get results for the remaining sensing performance metrics, an additional simulation run is needed:

- The first run is performed without any targets to find the needed detection parameters/threshold to meet a given requirement for FAP Type 1, e.g. [3%, 5%, etc].

- The second run is conducted with true targets being present to evaluate other performance metrics, such as FAP Type 2, misdetection probability, position accuracy, and velocity accuracy.

To ensure fairness and consistency in the simulation evaluation, it is recommended to use the same set of parameters and corresponding values for both FAP Type 1 estimation and for the computation of the other performance metrics.

Proposal 2: Same parameters and corresponding values should be applied to derive FAP type1 and FAP type2/misdetection probability/other sensing performance metrics.

For FAP Type 2, any detected object that can’t be paired with any true target should be considered a false alarm. It is assumed that at least one true target is present in each simulation run. FAP Type 2 and other performance metrics can then be derived from the same simulation run.

In the agreement from last meeting, there is an FFS on the choice of the denominator K when calculating Pf2. In option 1, K equals the total number of drops, whereas in Option 2, it represents only the number of drops when at least one target was detected. Some companies argued last meeting for Option 2, and said that from the definition of FAP Type 2, it can only be based on the condition that a target has been detected. Our reading of the agreement is different. Instead, FAP Type 2 says that a true target must have been dropped, not that it must have been detected.

According to Option 2, the following case would not be taken into account when calculating the false alarm probability: 1 true target is dropped, the true target is not detected and no false alarm is detected either. In our view, this case should be counted as “no false alarm (Dn'=Mn'=0)”.

Take the following example to illustrate the impact of Option 2: 10 drops, 1 true target is present in each drop, then

- In drop 1 the target is detected and no false alarm is observed.

- In drop 2 a target is detected but cannot be associated with the true target, e.g., it is too far away from the true target. This is then counted as false alarm.

- In drops 3-10 no target is detected and hence, no false alarm occurs.

With Option 1, this would give a false alarm probability of 10%, which makes sense, we have had 10 simulation runs and only in one we actually had a false alarm. With option 2, on the other hand, this would result into 50% false alarm probability, which obviously is too large.

In another example, assume that two cases can happen in the simulation:

- Case A: targets are detected but they are too far from the true targets and are therefore not associated with true targets

- Case B: no target is detected.

The respective numbers of occurrences of Case A and Case B reveal the impacts of multiple factors, hence should be reflected in certain metrics. They cannot be captured in the missed detection probability since it always is 1 in both cases. Also, since no true target is found in any of the cases, it is not possible to evaluate the position/velocity accuracy, either. Therefore, false alarm is the only possible metric to reflect the occurrences of Case A and Case B.

We provide an analysis on applying Option 1and Option 2, respectively.

- With Option 2 for the FAP Type 2, only Case A would be considered, simulation runs with Case B would have to be excluded from the FAP calculation. That means the FAP will always be 100%, no matter how often Case A or Cased B have occurred.

- With Option 1, on the other hand, the FAP Type 2 would vary depending on the respective numbers how often Case A and Case B have occurred, and can therefore be used.

Additionally, only excluding some simulation runs from the FAP calculation, but not from the misdetection would give an unfair representation of the two metrics versus each other. We should follow same principle to calculate them both. For the misdetection probability it has already been agreed last meeting that “N is total number of drops with at least one target per drop”. It is consistent to use Option1 for FAP type2, which says K is number of drops (N).

Proposal 3: For the FAP type2, K is the number of drops.

Regarding the definition of resolution, SA1 describes it as “the minimum difference in the measured magnitude of target objects (e.g., range, velocity) required to distinguish between distinct objects.” We interpret this to mean the smallest detectable difference in a given metric, such as range/velocity/angle, that enables the discrimination of two separate targets.

The resolution can already be obtained from the simulation settings, e.g., the range resolution is determined by ∆R=c2B for mono-static sensing, and only depending on the simulation bandwidth B. The velocity resolution is determined by ∆v=λ2Tobs for mono-static sensing, i.e., by the carrier frequency and the observation duration Tobs between two adjacent symbols.

Proposal 4: Sensing resolution is not considered as a metric during sensing evaluation in RAN1 for Rel-20 5GA.

  - Requirements of performance metrics

Multiple UAV use cases are identified in SA TR/TS with diverse requirements. It may need a long discussion to agree on the KPI values. It should be noted that sensing evaluation in RAN1 would also be possible without the adaptation of any KPIs. RAN1 could simply evaluate what kind of accuracy is achievable under different assumptions and report the results. When certain target KPI eventually would be defined, then the reported evaluation results can then be interpreted accordingly.

Observation 1: RAN1 evaluation of Sensing performance in Rel-20 for 5GA is possible without defining requirements.

If, however, RAN1 agrees to define a set of basic requirements, then it is our view to use the values from service category 1 from 22.137 as a starting point. This is the most relaxed requirement from SA1 and they could be useful to check the sensing feasibility, i.e., if the 5GA system can support at least one UAV use case or not.

We suggest further to consider 90% as confidence level instead of 95% that are specified for service category 1, because 90% is also what has been used in positioning, and it can be helpful to fairly compare the location estimation obtained from sensing with the results that earlier have been achieved for NR positioning.

As false alarm probability we suggest 5% for an initial evaluation, we believe that the 2% as stated in Service category 1 is still too strong.

Proposal 5: If requirements for the sensing performance metrics should be defined, RAN1 uses the following numbers as starting point for further discussion:

- Confidence level: 90%

- Accuracy of positioning estimate by sensing (for a target confidence level): 10m in horizontal and 10m in vertical plane

- Accuracy of radial velocity estimate by sensing (for a target confidence level): 1m/s

- Missed detection probability: 5%

- False alarm probability: 5%

- Evaluation assumptions

In last meeting, good progress on the evaluation assumptions was achieved in general. Some further details on which RAN1 did not yet have the time to agree on, need to be discussed and finalized in this meeting:

BS antenna configuration:

In the FL summary last meeting the following proposal was made:

[FL3][H] Proposal 6.3-1-rev3 - The following evaluation parameters are agreed for the evaluation on NR ISAC. FFS down-selection between the two options. Parameters; Assumptions. BS antenna configuration; 4GHz, 4.9GHz; 6GHz. For Option 1: - Tx: (12,16, 2,1,1;2,16). - Rx: (12,16,2,1,1;2,16) Option 2: - Tx: (8,8,2,1,1;4,8). - Rx: (8,8,2,1,1;4,8); For Option 1: - Tx: (16,16, 2,1,1;4,16). - Rx: (16,16,2,1,1;4,16) Option 2: - Tx: (8,8,2,1,1;4,8). - Rx: (8,8,2,1,1;4,8).

Given the large number of different proposals that were on the table, we understand it is difficult to converge. However, for the antenna configuration it is quite important to get an aligned setting across companies, so that evaluation results can be fairly compared. It would be good to have as many results as possible collected from different companies for at least one common configuration. For that reason and simplicity, we think that Option 2 should be the baseline. This option makes it also easier to harmonize results because it is common for all carrier frequencies.

Proposal 6: For BS antenna configuration in ISAC evaluation for 5GA in Rel-20, adopt Option 2 as baseline and companies can additionally report other configurations, i.e.,

- Baseline for 4 GHz, 4.9 GHz, 6GHz

- Tx: (8,8,2,1,1;4,8)

- Rx: (8,8,2,1,1;4,8)

BS antenna tilting:

In the last meeting the following proposal was made, for the antenna tilting:

[FL1][H] Proposal 6.3-2-rev1 - The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. BS antenna radiation pattern; Table 9 in Report ITU-R M.2412. BS Mechanic tilt (downtilt angle); Option 1: 90° in GCS (pointing to horizontal direction) Option 2: 102°. BS Electronic tilt; Up to company report. Polarized antenna model; Model-2 in clause 7.3.2 in TR 38.901.

There was about equal support for both Option 1 and Option 2 for the selection of the mechanic tilt. We prefer Option 1 for the purpose of simulation. This is the neutral vertical boresight direction of 900 pointing into the horizon.

It is also worth noting that which mechanic tilt to finally select is not critical for the evaluation. The overall tilt of the main-lobe is composed of both the mechanic and electronic tilt. The latter can for example be realized with analog beamforming using phase shifters. Assuming the electronic tilt is reported by companies, any mechanic tilt could be selected and could be compensated with a properly chosen electric tilt to achieve a desired overall vertical boresight direction of the main lobe.

Observation 2: The overall vertical tilt of the main-lobe is composed of both the mechanic and electronic tilt.

Proposal 7: For BS antenna tilts, RAN1 adopts the following settings for evaluation of ISAC in 5GA for Rel-20

- BS mechanic tilt: 90 degrees (pointing to horizontal direction)

- BS electric tilt: Up to companies to report.

Antenna isolation, RX saturation power, BX Tx power

The following proposal was made in last meeting by the FL:

[FL1][H] Proposal 6.4-1 - The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Antenna isolation; - Option 1: 65dB - Option 2: 80dB. BS Rx saturation power; -28dBm. BS Tx power; When Tx/Rx operates simultaneously, - BS_maxpower = BS Rx saturation power + Isolation, i.e, - Option 1: 37dBm - Option 2: 52dBm Otherwise, max BS Tx power is 56dBm.

The antenna isolation is highly dependent on the implementation. According to some product information from trials in China for low-altitude economy, 65dB can be considered. On the other hand, to get more insight about the impact of advanced implementations or taking into account technical progress in the near future, it is preferable to also simulate certain larger values for antenna isolation, e.g., 70 or 80dB. We are therefore supportive to include both options in the simulations.

As maximum receive power, i.e., the saturation power at the TRP, we would suggest to assume -30dBm, since this is also the value used for NR communication, but we can live with the proposed values from last meeting, i.e., -28 dBm.

The maximum transmit power at the BS can be obtained by the sum of the antenna isolation and maximum receive power. For example, the maximum transmit power is 37dBm/52dBm for antenna isolation of 65 dB/80dB and maximum receive power of -28dBm.

Proposal 8: For antenna isolation, BS Rx saturation power and BS Tx power, RAN1 adopts the following settings for evaluation of ISAC in 5GA for Rel-20

- Antenna isolation: 65dB, 80dB

- BS Rx saturation power: -28dBm

- BS Tx power: RX saturation power + Antenna isolation, e.g.

- 37 dBm (for Rx saturation power -28 dBm and 65 dB antenna isolation)

- 52 dBm (for Rx saturation power -28 dBm and 80 dB antenna isolation)

Concatenation of TX-target and target-RX links

For concatenation in ISAC channel generation for evaluation, we prefer to consider option-3 from the Rel-19 agreements (which is captured as Option 2 in TR 38.901), since it reduces the simulation complexity.

Proposal 9: For concatenation of TX-targe and target-RX links, Option 2 from the ISAC channel modeling in TR 38.901 is adopted.

Interference modeling

The following proposal was made in last meeting.

[FL1][H] Proposal 6.4-2 - The following evaluation parameters are agreed for the evaluation on NR ISAC. - The leakage signal from Tx antenna to the Rx antenna is the same as the Tx signal subject to the attenuation of antenna isolation. Parameters; Assumptions. Co-site inter-sector interference; Not modelled. Inter-site interference; Not modelled. Other interference; I_1+I_2 -I_1 denotes received signal from target specific channel(s) of other target(s) associated same pair of sensing Tx and sensing Rx, i.e., (sensing Tx#1, sensing Rx#1) -I_2 denotes received signal from background channel of the same pair of sensing Tx and sensing Rx, i.e., (sensing Tx#1, sensing Rx#1). BS receiver noise figure; 5dB. [BS Tx noise ]; TBD.

We prefer to not model the co-site interference and inter-site interference to reduce the simulation complexity.

The BS receiver noise figure of 5 dB seems reasonable to us.

For modeling of other interference, it should consist of I1 and I2, where I1 denotes the received signal from target channel(s) of other target(s) in the same TRP, and I2 denotes the received signal from the background channel of the same TRP.

The physical distance between Tx antenna and Rx antenna is tiny for TRP mono-static. Which means that some sensing RS will be directly received by Rx which is not reflected by target and environment. This part signal is self-interference and it’s necessary to model it to make the simulation more realistic.

Proposal 10: For interference modeling, RAN1 adopts the following assumptions for evaluation of ISAC in 5GA for Rel-20

- Co-site inter-sector interference: Not modelled

- Inter-site interference: Not modelled

- Self-interference: Modelled

- Other interference: I1 + I2, where

- I1: I1 denotes received signal from target specific channel(s) of other target(s) associated same pair of sensing Tx and sensing Rx, i.e., (sensing Tx#1, sensing Rx#1)

- I2: denotes received signal from background channel of the same pair of sensing Tx and sensing Rx, i.e., (sensing Tx#1, sensing Rx#1)

- BS receiver noise figure: 5dB

RS overhead

The overhead of the sensing RS is a key factor to represent the mutual impacts of sensing and communication. For example, a resource utilization of about 10% for sensing can be considered in the evaluation. As to sensing RS configuration, it is desired if the exact RE pattern of sensing RS can be aligned in the evaluation. The specific RS pattern for evaluation can be up to companies’ choice when not aligned.

Proposal 11: For sensing RS overhead, RAN1 assumes around 10% for evaluation of ISAC in 5GA for Rel-20.

Overall simulations assumptions:

Based on the above discussion, we propose the following table to be adopted by RAN1 for the ISAC evaluation in 5GA in Rel-20.

Proposal 12: The simulation assumptions in the following table can be used as a starting point for UAV detection based on TRP mono-static.

| Parameter set |  | UMa-AV (FR1) | parameters in R19 calibration |
| --- | --- | --- | --- |
| A. General | Carrier frequency | 4GHz or 4.9GHz, 6 GHz(optional) |  |
|  | System bandwidth | 100 MHz |  |
|  | Numerology | SCS = 30kHz |  |
|  | BS Layout | Hexagonal grid, 7 macro sites, 3 sectors per site 3 sectors with 30, 150, 270 degrees |  |
|  | Wrap-round | No wrap around |  |
|  | Inter-BS (2D) distance | 500m, optional 1000m |  |
|  | BS antenna height | 25m |  |
| B. Target related | Target type | UAV with small size (0.3m x 0.4m x 0.2m) |  |
|  | Target distribution | N = 5 targets per sector in the center site. Optional: N is uniformly distributed from 1 to 10. Horizontal plane: uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally distributed between 1.5m and 300m |  |
|  | Minimum BS-target 3D distance | 10m |  |
|  | Minimum target-target (3D) distance (angular/distance resolution related issue) | 10 m |  |
|  | Target mobility | horizontal speed: uniformly distributed between 0 and 180km/h vertical speed: 0km/h |  |
|  | RCS value | RCS model 1 | NA |
|  | Change of RCS value in simulation | Not changed over CPI | NA |
| C. Transmit power & Interference modelling | Isolation | 65dB/80dB | NA |
|  | BS Rx saturation power | -28dBm | NA |
|  | BS Tx power | Determined by the antenna isolation and maximum receive power BS_maxpower = BS Rx saturation power + Isolation | 56dBm for FR1 |
|  | Self-interference | Modelled | NA |
|  | Co-site inter-sector interference | Not modelled | NA |
|  | Inter-site interference | Not modelled | NA |
|  | Other interference | I_1+I_2 -I_1 denotes received signal from target specific channel(s) of other target(s) associated same pair of sensing Tx and sensing Rx, i.e., (sensing Tx#1, sensing Rx#1) -I_2 denotes received signal from background channel of the same pair of sensing Tx and sensing Rx, i.e., (sensing Tx#1, sensing Rx#1) | NA |
| D. Antenna configurations | BS antenna configuration | Tx:(M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;4,8) Rx:(M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;4,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization | Single dual-pol isotropic antenna |
|  | BS antenna radiation pattern | Table 9 in Report ITU-R M.2412 | Single dual-pol isotropic antenna |
|  | BS Mechanic tilt (downtilt angle) | 90° in GCS (pointing to horizontal direction) | NA |
|  | BS Electronic tilt (Constraints for the range of selective analog beams per TRxP) | Up to companies | NA |
|  | Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |  |
|  | BS receiver noise figure | 5dB |  |
| E. Target Channel model | gNB-target link | gNB-aerial UE of UMa parameters in 36.777 with Alternative 3 |  |
|  | Concatenation of TX-target and target-RX links | Option 2 in TR 38.901 | Option 1/2 |
|  | The power threshold for path dropping after concatenation for target channel | -40dB |  |
|  | XPR of target in target channel | (13.75, 7.07) dB |  |
|  | Absolute time of arrival for target channel | gNB-target: with parameters for UMa in 38.901 |  |
| F. Background Channel model | mono-static: gNB Tx/Rx | TRP based mono-static channel model for UMa in 38.901 |  |
|  | Absolute time of arrival for background channel | With parameters for UMa in 38.901 |  |
| G. Sensing RS | Sensing RS overhead | Around 10% overhead, i.e., All subcarriers in frequency domain 2 symbol per slot | NA |
|  | CPI | 40ms for one sensing process | NA |
|  | Waveform | OFDM | NA |
| H. Others | STX/SRX selection | Based on coupling loss of full calibration | based on coupling loss of large scale calibration |

- Simulation results

In the simulation, non-cooperative sensing is assumed. OS-CFAR (Order sequence-Constant False Alarm Rate) is used to estimate the number of targets in the drop, i.e., value of Mn' in FAP type2 Pf2=0≤n<NMn'≠0Dn'Mn'K. MUSIC algorithm is used to estimate the location and velocity of target with input of value of Mn'.

  - Simulation results for 4.9GHz

| (1) | (2) | (3) |
| --- | --- | --- |

Fig. 1 Simulation results for 4.9 GHz with Tx power 52dBm.

Table 1 Simulation result summary for 4.9 GHz.

| 4.9GHz |  | Performance |
| --- | --- | --- |
| Horizontal localization error [m] | @90% | 0.552m |
|  | @95% | 0.713m |
| Vertical localization error [m] | @90% | 0.480m |
|  | @95% | 0.623m |
| Radial velocity error [m/s] | @90% | 0.043m/s |
|  | @95% | 0.274m/s |
| Missed detection probability [%] |  | 2.5% |
| False alarm probability [%] |  | 3.95% |

  - Simulation results for 6GHz

| (1) |  | (2) |  | (3) |
| --- | --- | --- | --- | --- |

Fig. 2 Simulation results for 6 GHz with Tx power 52dBm.

Table 2 Simulation result summary for 6 GHz.

| 6GHz |  | Performance |
| --- | --- | --- |
| Horizontal localization error [m] | @90% | 0.559m |
|  | @95% | 0.700m |
| Vertical localization error [m] | @90% | 0.402m |
|  | @95% | 0.468m |
| Radial velocity error [m/s] | @90% | 0.009m/s |
|  | @95% | 0.112m/s |
| Missed detection probability [%] |  | 4.00% |
| False alarm probability [%] |  | 1.94% |

  - Observation for the simulation results

With the simulation results for non-cooperative sensing, the requirements of category 1/2 specified in SA1 as below can be meet.

| Table 3 Performance requirements for 5G Wireless sensing in TS22.137Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 2 | 5 | 1 | N/A | 0.1 to 5 | 5 | Outdoor (e.g., detection of human, UAV) |
|  | 3 | 95 | 1 | 1 | 1 | 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian | 1 | 3 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, AGV, vehicle) |

Observation 3: With non-cooperative gNB mono-static sensing, the requirements of category 1/2 of object detection and tracking in TS22.137 could be achieved for the evaluated metrics, i.e., horizontal and vertical location accuracy, velocity, missed detection and false alarm probability.

- Measurements

  - Measurement quantity

To obtain the sensing results in a TRP monostatic sensing system, the sensing receiver should be able to perform measurements on a configured sensing signal, e.g., a legacy DL NR RS, like SSB, CSI-RS, etc., or, in another option, the sensing signal may be up to gNB implementation. The measurement at the sensing receiver can then be used to derive information about the sensing targets, e.g., range, velocity, and horizontal/vertical angle (position and velocity). In the RAN1#122 meeting [2], the following proposal for measurements was given in the FL summary, which we are now discussing further.

### [FL1][H] Proposal 7.1-1-rev3 - The following measurements can be further studied for NR ISAC. - Spectrum of delay, and/or Doppler, and/or angle, i.e., the powers or the amplitudes/phases are reported for at least a range of samples of delay, and/or Doppler, and/or angle - Intermediate sensing results per detected path - Delay, Doppler, angle, and power per path - Intermediate sensing results per detected point - Option 1: Position, velocity, and power per point - Option 2: Range, velocity, angle, and power per point - Final sensing results per detected object/target - Option 1: Position and velocity - Option 2: Range, velocity, and angle - Note: Trajectory report is separate discussion - SINR, RSRP from perspective of sensing

Spectrum of delay, and/or Doppler, and/or angle

In this option, the sensing receiver can first process the channel state information matrix, and then derive delay, Doppler, and angle for reporting. Multiple algorithms are possible how this information can be obtained. In one example, a 2D-DFT algorithm for delay and Doppler can be applied, where the quantized amplitude and phase values of the 2D spectrum are reported. In another example, the MUSIC algorithm can be used to obtain delay/Doppler/angle information which then would be quantized.

A major drawback with approaches to report the entire spectrum, is that they inflict extreme overhead, without adding significant benefit, since most parts of the spectrum actually do not contain information about potential targets and can be filtered out already at the measuring gNB.

Intermediate sensing results per detected path/point

Both options can result in significantly less overhead than reporting of the full spectrum. The number of reported paths/points could be made adjustable (e.g., reporting of Top-K paths/points), then they are both well suited to trade sensing performance vs overhead, and for a very large values of K these options would converge towards the spectrum reporting.

Additionally, both options appear to be very similar in their contents. They both contain information about power, angle, velocity, and range (for the reporting per path, it is expressed in delay but can be converted into range for monostatic sensing). The reporting per point is slightly more inclusive, since the direct report of location and velocity are considered. Therefore, we prefer this option among the two.

Final sensing results per detected object/target

This option has similar contents as the above discussed intermediate sensing results. Without information on power, the possibility to further process the reports from multiple TRPs is limited. As comparison, in the previous case of intermediate results, more information is available to fuse the results from multiple TRPs.

In our view, single TRP measurements should be the baseline, but letting companies combine measurement information from multiple nodes, should not be precluded. The options listed under “intermediate sensing results per detected point” allow for all these possibilities. In some cases, this option could be used as final result and, in another case, it could be an intermediate result for further processing. We are therefore making the following proposal:

Proposal 13: For sensing evaluation in 5G-A-Rel20, RAN1 can consider the following measurement for further study

- Intermediate/Final sensing results per detected point

  - Option 1: Position, velocity, and power per point

  - Option 2: Range, velocity, angle, and power per point

  - The number of detected points is reported by companies

One issue that would need clarification when for reporting the intermediate or finals results is the use of the coordinate system. For intermediate results, the measurement of one TRP would be considered, and the TRP detects the target relative to its own location, thus a local coordinate system (LCS) would be used. Final results, on the other hand, can be based on combined measurement results from multiple TRPs, in such case, the coordinates should be given in the global coordinate system (GCS).

Proposal 14: For sensing evaluation in 5G-A-Rel20, consider different types of coordinate systems for intermediate/final sensing results per detected point/object

- Intermediate results are defined in LCS

- Final results are defined in GCS

  - Measurement quantization

Measurement quantization is necessary when reporting the measurement quantity. The number of needed bits is determined by two aspects, i.e., the range of the reported values for the given metric (minimum/maximum) and the quantity’s granularity. The IEs in existing specifications can be used as reference, e.g., TS 23.032. The proper number of bits for quantization can be verified by simulation. On the other hand, it is expected that RAN1 may only conclude on a range of the bit numbers since different number may be used for different use case or different requirement on performance.

Observation 4: For measurement quantization of a given quantity, the number of needed bits depends both on its range and granularity. Existing TS (e.g., TS 23.032 can be used as a starting point), but the final numbers are highly impacted by the use case and requirements.

- Conclusion

In this contribution, we provide the following observations and proposals.

Observation 1: RAN1 evaluation of Sensing performance in Rel-20 for 5GA is possible without defining requirements.

Observation 2: The overall vertical tilt of the main-lobe is composed of both the mechanic and electronic tilt.

Observation 3: With non-cooperative gNB mono-static sensing, the requirements of category 1/2 of object detection and tracking in TS22.137 could be achieved for the evaluated metrics, i.e., horizontal and vertical location accuracy, velocity, missed detection and false alarm probability.

Observation 4: For measurement quantization of a given quantity, the number of needed bits depends both on its range and granularity. Existing TS (e.g., TS 23.032 can be used as a starting point), but the final numbers are highly impacted by the use case and requirements.

Proposal 1: RAN1 discussion focuses on UAV detection

- Up to companies to report the evaluation result for UAV tracking

Proposal 2: Same parameters and corresponding values should be applied to derive FAP type1 and FAP type2/misdetection probability other sensing performance metrics.

Proposal 3: For the FAP type2, K is the number of drops.

Proposal 4: Sensing resolution is not considered as a metric during sensing evaluation in RAN1 for Rel-20 5GA.

Proposal 5: If requirements for the sensing performance metrics should be defined, RAN1 uses the following numbers as starting point for further discussion:

- Confidence level: 90%

- Accuracy of positioning estimate by sensing (for a target confidence level): 10m in horizontal and 10m in vertical plane

- Accuracy of radial velocity estimate by sensing (for a target confidence level): 1m/s

- Missed detection probability: 5%

- False alarm probability: 5%

Proposal 6: For BS antenna configuration in ISAC evaluation for 5GA in Rel-20, adopt Option 2 as baseline and companies can additionally report other configurations, i.e.,

- Baseline for 4 GHz, 4.9 GHz, 6GHz

- Tx: (8,8,2,1,1;4,8)

- Rx: (8,8,2,1,1;4,8)

Proposal 7: For BS antenna tilts, RAN1 adopts the following settings for evaluation of ISAC in 5GA for Rel-20

- BS mechanic tilt: 90 degrees (pointing to horizontal direction)

- BS electric tilt: Up to companies to report.

Proposal 8: For antenna isolation, BS Rx saturation power and BS Tx power, RAN1 adopts the following settings for evaluation of ISAC in 5GA for Rel-20

- Antenna isolation: 65dB, 80dB

- BS Rx saturation power: -28dBm

- BS Tx power: RX saturation power + Antenna isolation, e.g.

- 37 dBm (for Rx saturation power -28 dBm and 65 dB antenna isolation)

- 52 dBm (for Rx saturation power -28 dBm and 80 dB antenna isolation)

Proposal 9: For concatenation of TX-targe and target-RX links, Option 2 from the ISAC channel modeling in TR 38.901 is adopted.

Proposal 10: For interference modeling, RAN1 adopts the following assumptions for evaluation of ISAC in 5GA for Rel-20

- Co-site inter-sector interference: Not modelled

- Inter-site interference: Not modelled

- Self-interference: Modelled

- Other interference: I1 + I2, where

- I1: I1 denotes received signal from target specific channel(s) of other target(s) associated same pair of sensing Tx and sensing Rx, i.e., (sensing Tx#1, sensing Rx#1)

- I2: denotes received signal from background channel of the same pair of sensing Tx and sensing Rx, i.e., (sensing Tx#1, sensing Rx#1)

- BS receiver noise figure: 5dB

Proposal 11: For sensing RS overhead, RAN1 assumes around 10% for evaluation of ISAC in 5GA for Rel-20.

Proposal 12: The simulation assumptions in the following table can be used as a starting point for UAV detection based on TRP mono-static.

| Parameter set |  | UMa-AV (FR1) | parameters in R19 calibration |
| --- | --- | --- | --- |
| A. General | Carrier frequency | 4GHz or 4.9GHz, 6 GHz(optional) |  |
|  | System bandwidth | 100 MHz |  |
|  | Numerology | SCS = 30kHz |  |
|  | BS Layout | Hexagonal grid, 7 macro sites, 3 sectors per site 3 sectors with 30, 150, 270 degrees |  |
|  | Wrap-round | No wrap around |  |
|  | Inter-BS (2D) distance | 500m, optional 1000m |  |
|  | BS antenna height | 25m |  |
| B. Target related | Target type | UAV with small size (0.3m x 0.4m x 0.2m) |  |
|  | Target distribution | N = 5 targets per sector in the center site. Optional: N is uniformly distributed from 1 to 10. Horizontal plane: uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally distributed between 1.5m and 300m |  |
|  | Minimum BS-target 3D distance | 10m |  |
|  | Minimum target-target (3D) distance (angular/distance resolution related issue) | 10 m |  |
|  | Target mobility | horizontal speed: uniformly distributed between 0 and 180km/h vertical speed: 0km/h |  |
|  | RCS value | RCS model 1 | NA |
|  | Change of RCS value in simulation | Not changed over CPI | NA |
| C. Transmit power & Interference modelling | Isolation | 65dB/80dB | NA |
|  | BS Rx saturation power | -28dBm | NA |
|  | BS Tx power | Determined by the antenna isolation and maximum receive power BS_maxpower = BS Rx saturation power + Isolation | 56dBm for FR1 |
|  | Self-interference | Modelled | NA |
|  | Co-site inter-sector interference | Not modelled | NA |
|  | Inter-site interference | Not modelled | NA |
|  | Other interference | I_1+I_2 -I_1 denotes received signal from target specific channel(s) of other target(s) associated same pair of sensing Tx and sensing Rx, i.e., (sensing Tx#1, sensing Rx#1) -I_2 denotes received signal from background channel of the same pair of sensing Tx and sensing Rx, i.e., (sensing Tx#1, sensing Rx#1) | NA |
| D. Antenna configurations | BS antenna configuration | Tx:(M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;4,8) Rx:(M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;4,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization | Single dual-pol isotropic antenna |
|  | BS antenna radiation pattern | Table 9 in Report ITU-R M.2412 | Single dual-pol isotropic antenna |
|  | BS Mechanic tilt (downtilt angle) | 90° in GCS (pointing to horizontal direction) | NA |
|  | BS Electronic tilt (Constraints for the range of selective analog beams per TRxP) | Up to companies | NA |
|  | Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |  |
|  | BS receiver noise figure | 5dB |  |
| E. Target Channel model | gNB-target link | gNB-aerial UE of UMa parameters in 36.777 with Alternative 3 |  |
|  | Concatenation of TX-target and target-RX links | Option 2 in TR 38.901 | Option 1/2 |
|  | The power threshold for path dropping after concatenation for target channel | -40dB |  |
|  | XPR of target in target channel | (13.75, 7.07) dB |  |
|  | Absolute time of arrival for target channel | gNB-target: with parameters for UMa in 38.901 |  |
| F. Background Channel model | mono-static: gNB Tx/Rx | TRP based mono-static channel model for UMa in 38.901 |  |
|  | Absolute time of arrival for background channel | With parameters for UMa in 38.901 |  |
| G. Sensing RS | Sensing RS overhead | Around 10% overhead, i.e., All subcarriers in frequency domain 2 symbol per slot | NA |
|  | CPI | 40ms for one sensing process | NA |
|  | Waveform | OFDM | NA |
| H. Others | STX/SRX selection | Based on coupling loss of full calibration | based on coupling loss of large scale calibration |

Proposal 13: For sensing evaluation in 5G-A-Rel20, RAN1 can consider the following measurement for further study

- Intermediate/Final sensing results per detected point

  - Option 1: Position, velocity, and power per point

  - Option 2: Range, velocity, angle, and power per point

  - The number of detected points is reported by companies

Proposal 14: For sensing evaluation in 5G-A-Rel20, consider different types of coordinate systems for intermediate/final sensing results per detected point/object

- Intermediate results are defined in LCS

- Final results are defined in GCS

- References

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR, RAN#108, June 9-13, 2025.

- RAN1#122 Chair’s Notes, August 2025.

- RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR, RAN#109, September 15-18, 2025.