3GPP TSG RAN WG1 #122bis		                             R1-2507110

Prague, Czech, October 13th – 17th, 2025

Source:	CATT

Title:	Inter-vendor training collaboration for CSI compression

Agenda Item:	10.1.2

Document for:	Discussion and Decision

Introduction

In RAN#108 meeting, the WID for Rel-20 AI/ML for NR air interface was approved. The objectives of the AI/ML based CSI feedback enhancement related to inter-vendor training collaboration are shown as bellow [1]:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

In this contribution, we provide our views on dataset format/content for inter-vendor collaboration sub-option 4-1.

Discussion

Standardized dataset format/content

As concluded in Rel-19, for inter-vendor sub-option 4-1, NW side sends datasets to UE side for training the actual encoder. The content of the dataset includes data samples and additional information. The data samples comprise target CSI and the corresponding CSI feedback in a specific format. The additional information mainly includes the performance target, quantization codebook and/or pairing ID. Moreover, the inference configuration that are applicable to the dataset may also be provided as additional information, such as the number of Tx ports and sub-bands in the dimensional information of the target CSI, as well as the number of bits of the CSI feedback.

Data samples

In RAN1#122 meeting [2], the following agreement was reached regarding the data samples within the exchanged dataset.

Agreement For Option 4-1 under Direction A in AI/ML based CSI compression, support at least target CSI and CSI feedback in the exchanged dataset(s). - FFS: Target CSI type and format - FFS: CSI feedback type and format - FFS: Association between Target CSI and CSI feedback, including mapping for different number of Tx port, number of sub bands, and CSI payload size.

Target CSI

For sub-option 4-1, target CSI in the data samples in exchanged dataset is used as the input of encoder. In RAN1#122 meeting [2], it was agreed to support at least precoding matrix as target CSI type. During evaluation of Rel-18 and Rel-19, the majority of companies adopted input CSI as spatial-frequency domain eigenvector, while some companies adopted input in the angular-delay domain. In our opinion, SF domain input should be prioritized since its feasibility and performance has been proved by comprehensive evaluation. The other alternative options including domain transformation and utilizing raw channel as target CSI are still lack of study.

Proposal 1: For inter-vendor collaboration sub-option 4-1, spatial-frequency domain input can be prioritized for dataset format standardization.

Another issue to be considered is the format of target CSI in the exchanged dataset. In the evaluations of Rel-18 and Rel-19, most companies used floating point for target CSI quantization to minimize the performance degradation caused by quantization errors. The other solution is using legacy codebook, like eType II, to represent target CSI. Since the dataset exchange is non-OTA based, overhead issue will not be a critical issue. The target CSI in floating point can be adopted to achieve optimal precision and to reduce preprocessing by allowing data to be directly used for model training. If target CSI is reported using eType II codebook for NW-side data collection, the same codebook parameters can be reused for dataset exchange. Comparing to using floating point as the format, using legacy codebook can efficiently reduce the number of bits while still maintaining an acceptable quantization precision.

Proposal 2: For format of target CSI in dataset for sub-option 4-1, the following options can be supported:

Option 1: Rel-19 eType II codebook (with or without enhancement)

Option 2: Floating point

CSI feedback

In RAN1#122[2] the following agreement was reached concerning the decision on whether the CSI feedback should be the quantized in the exchanged codebook.

Agreement For Option 4-1 under Direction A in AI/ML based CSI compression, for CSI feedback type and format, down select one of the following options: - Option 1: The exchanged CSI feedback is the latent vector before quantization. - Option 2: The exchanged CSI feedback is the binary sequence at the output of quantization. - Note: Quantization codebook is exchanged in addition if not specified in the spec. - Note: Whether or not using the Quantization codebook for model training, is up to UE implementation

To narrow down to one option, how the two options will impact UE-side training should be assessed.

If a nominal decoder is trained at UE side, the input of such decoder should be the latent vector with quantization error. The quantization error is introduced by the quantization and de-quantization processes at the encoder and decoder respectively. In such case, the choice between Option 1 and Option 2 become less critical since the CSI feedback always needs to be quantized. After the nominal decoder is obtained, CSI feedback is no longer needed since the actual encoder at UE side can be trained solely with target CSI and nominal decoder. Therefore both Option 1 and Option 2 can be supported if nominal decoder is trained at UE side.

However, if UE side directly trains its actual encoder, the label of the encoder is the latent vector, either with or without quantization. For Option 2, only the quantized latent vector is available, and hence it is not feasible to obtain the original vector from the binary sequence of CSI feedback. When adopting Option 2, the quantization block at encoder should be incorporated into the training processing. While for Option 1, the quantized latent vector can still be obtained with the aligned quantization method. Besides the quantization-aware training as in Option 2, the encoder can be trained without quantization block by using the CSI feedback as label directly. In our opinion, such method is beneficial to ensure the similarity between the actual encoder and the encoder at NW side. By adopting Option 1, UE side obtains greater flexibility on model training. Therefore Option 1 should be supported.

It should be noted that both Option 1 and Option 2 have additional spec impact. For Option 1, a standardized format for representing latent vector is needed. Such format, e.g. floating point, could offer better precision than the quantization method. Considering the dataset exchange is non-OTA based, the overhead will not be a critical issue. If Option 2 is needed, than the mapping order of the quantized bits should be specified.

Proposal 3: For Option 4-1 under Direction A, support Option 1 for CSI feedback type and format, i.e. the exchanged CSI feedback is the latent vector before quantization.

Association between target CSI and CSI feedback

The dataset format should be designed to accommodate multiple configurations to avoid frequent dataset exchange. By integrating data samples for different configurations, such as number of Tx ports, subbands and payload sizes, the data samples can be further aggregated to reduce the size of dataset. In such case, the association between target CSI and CSI feedback may not be a one-to-one mapping. Instead, multiple CSI feedback corresponding to different configurations can be associated with a single target CSI. At the same time, both UE side and NW side have the flexibility to employ scalable models to accommodate different inference configurations. The format of the exchanged dataset should be tailored to support training scalable models. At the same time, it is up to NW side and UE side whether to train a dedicated model for a specific configuration or to train a scalable model for multiple configurations.

We will provide our opinions on the how to support scalability over different number of Tx ports, number of sub bands, and CSI payload sizes in the exchanged dataset in Section 2.2.

Additional information

In addition to data samples, the exchanged dataset for sub-option 4-1 should also include additional information to enable the alignment between the encoder trained at the UE side and the decoder trained at the NW side, thereby achieving the desired performance. In Rel-19, RAN1 has concluded that performance target and quantization method of CSI feedback should be exchanged along with dataset.

In this section, we provide our views on performance target, quantization alignment, and other additional information.

Performance target

As agreed in RAN1#122[2] performance target is confirmed as part of the additional information along with the exchanged dataset, where its type is for further study.

Agreement For Option 4-1 under Direction A in AI/ML based CSI compression, consider the following methods for performance target in the exchanged dataset, - SGCS - FFS:Average SGCS - FFS: SGCS values at X-percentiles - NMSE: - FFS: When the exchanged CSI feedback is the floating-point values at the input of quantization - FFS: When the exchanged CSI feedback is the binary bit sequence at the output of quantization, the binary sequence will be mapped back to the floating-point values via quantization codebook - FFS: Multiple performance targets for different layer when the target CSI type is precoding matrix, different configurations such as antenna ports, subband configuration and payload size configuration

Regarding the performance metric to be used for performance target, both SGCS and NMSE may serve the purpose of UE-side model validation. By comparing the definition of two metrics, NMSE aims to ensure that the actual encoder at UE side is similar to the nominal encoder at NW side, whereas SGCS assesses the end-to-end performance of the two-sided model. At UE side, the actual NMSE can be obtained by comparing the actual encoder output with the CSI feedback using the testing dataset. If the CSI feedback is latent vector, whether the encoder output is quantized to binary sequence or not to calculate NMSE can be up to UE implementation. If nominal decoder is available, UE side can directly obtain the SGCS with two-sided model. On the other hand, NW side needs both encoder and decoder to generate the exchanged dataset. SGCS can be straightforwardly calculated at NW side with the two-sided model. To determine NMSE, the NW side can emulate the UE-side training procedure by retraining a new encoder on the exchanged dataset, or adopt any other implementation-specific approach. Accordingly, both NMSE and SGCS can be chosen as the performance target.

Based on the above analysis, both SGCS and NMSE should be supported as performance target.

Proposal 4: For inter-vendor collaboration sub-option 4-1, both SGCS and NMSE can be supported as performance target.

SGCS can be used for UE-side training with nominal decoder

NMSE can be used for UE-side training without nominal decoder

It should be clarified that the performance target is derived from validation of NW-side data. Therefore UE side should test its own model without using UE-side data. Regarding the detailed format of the performance target, both Option 1 and Option 2 can reflect the performance of the two-sided model. In Option 1, performance target measures the average performance of the AI model. While in Option 2 the performance target is enabled to reflect, to a certain extent, the variations in model performance across different samples. Comparing the two options, Option 1 is both more commonly used and simpler. Option 2, however, ‌may be more susceptible to the influence of low-quality data samples contaminating the testing dataset‌. ‌Therefore, we recommend Option 1 as the baseline method‌, with Option 2 serving as an optional complementary approach.

Proposal 5: Regarding details of the format of the performance target, at least support Option 1.

Option 1: Average performance target, e.g. average SGCS and/or average NMSE

Another issue is whether multiple targets should be provided for different configurations. Based on the evaluation results in Rel-18 and Rel-19, different number of Tx ports and different payload sizes may results in obvious differences in SGCS. Even under the same configuration, the SGCS of layer 1 is always higher than the SGCS of other layers. By providing multiple performance targets for different configurations, UE side can acquire a more precise understanding of the expected performance under various configurations. If NW side expects UE side to differentiate specific configurations when assessing model (actual encoder) performance, it is reasonable to provide corresponding targets separately for these configurations. It should be noted that performance for each specific configuration is not mandatory.

Proposal 6: For inter-vendor collaboration sub-option 4-1, support multiple performance targets corresponding to different configurations, at least for different layers, different number of Tx ports and different payload sizes.

Quantization alignment

The following agreement was achieved in RAN#120bis[3] regarding quantization alignment for different inter-vendor collaboration option/direction:

Agreement For inter-vendor collaboration Direction C, - Use standardized quantization codebook For inter-vendor collaboration Direction A Options 4-1, 3a-1 (with or without NW-side target CSI sharing), - Standardize configuration(s) of quantization codebook, e.g., scalar or vector quantization, segment size of VQ, codebook size. - FFS: applicability of the above for Case 2 - Exchange quantization codebook of (the selected) standardized configuration(s) from NW-side to UE-side along with each exchanged dataset or model parameters. - FFS: whether quantization codebook may be different across different payload size configurations Agreement It is clarified that, for previous agreement on standardize configuration(s) of quantization for inter-vendor collaboration Direction A Options 4-1, 3a-1 (with or without NW-side target CSI sharing), resolve the first FFS with the following clarifications: - The standardized quantization configuration(s) refers to the configuration(s) related to the dimensionality related to the quantization operation (e.g., e.g., scalar or vector quantization, segment size of VQ, codebook size) and does not preclude the use of more advanced quantization algorithms (e.g., using past samples). - Exchanged quantization codebook does not preclude the use of codebook parameterized by parameters, e.g. quantization range and step size determined by some parameterized formula.

For inter-vendor sub-option 4-1, quantization method information should be exchanged along with dataset from NW-side. If Option 4-1 is supported, the specification needs to determine how to describe a specific quantization in the standardized dataset format. As in the agreement, different configuration information is needed for scalar and vector quantization, while both methods were used to evaluate performance of AI/ML based CSI compression in Rel-18 and Rel-19.

Vector quantization aims to achieve optimal performance by utilizing more advanced quantization algorithms and learnable quantization parameters. The standardization of vector quantization configuration can be categorized into two approaches. One approach involves standardizing the quantization algorithm and providing the algorithm parameters within exchanged dataset. Despite the potential performance advantage, one issue with this approach is that advanced quantization algorithms may be proprietary information of NW or UE, and UE may not support various quantization algorithms used by different NW side. The other approach is to directly provide quantization codebook in the exchanged dataset. However, to maintain quantization alignment between the UE side and NW side, the UE side cannot modify the values of the quantization codebook during training. Under such circumstances, the performance benefits of vector quantization will be weakened since the quantization may not be optimal for the actual encoder.

Scalar quantization is a simple solution which needs less specification effort than vector quantization. The simulation results from many companies show that scalar quantization also provide considerable performance gain over baseline. Therefore scalar quantization can serve as the basic option to guarantee an available quantization method at UE side. Considering the distribution of each latent dimension of actual encoder output (before quantization) is unpredictable, the optimal quantization interval is difficult to obtain by NW side without knowledge of the actual encoder. Although the optimal quantization interval can be approximated during NW side training, the quantization configuration still cannot take into consideration the difference between actual encoder at UE side and the nominal encoder at NW side. Uniform quantization can be considered as starting point, where only number of bits per latent dimension is needed for quantization alignment.

Proposal 7: For quantization alignment between UE side and NW side, prioritize uniform scalar quantization.

In RAN1#122[2], the following agreement was achieved regarding quantization alignment when vector quantization is adopted and codebook is not specified.

Agreement For Option 4-1 under Direction A in AI/ML based CSI compression, if the quantization codebook for CSI feedback is not fixed by the specification, support exchange of quantization codebook along with dataset exchange - FFS: Quantization codebook exchange method (e.g, look up table, or how to exchange quantization codebook related information, or other methods) - FFS: Common or different quantization codebook for different CSI payload size - Note: leveraging the discussion in 10.1.1.1 when applied

For vector quantization, the NW side needs to generate the initial quantization codebook and may dynamically update this codebook during training to achieve optimal performance. The generation of codebook may adhere to certain algorithm and the parameters of such algorithm may evolve with the model parameters during training. An alternative way to update the codebook during training is altering some of the codewords directly.

Regarding the first FFS, the exchange method is depended on whether the quantization codebook can be represented by certain algorithm and parameters. If the details of the generating and updating of the codebook cannot be specified, the codebook has to be exchanged by codeword exchanging. Conversely, if one or more algorithm can be agreed upon and standardized for codebook representation, the codebook can be exchanged by indicating a specific algorithm along with its parameters. As of now, the RAN1 discussions have not yet addressed the specifics of codebook generation and updating mechanisms, nor has achieved consensus on a universally accepted algorithm across different companies and vendors. Therefore explicitly exchanging each codeword, e.g., look up table, can be adopted.

If quantization codebook is updated during training, different quantization codebooks may be used to achieve optimal performance under not only different payload sizes, but also different number of Tx ports or subbands. For example, for lager number of Tx ports, the length of each codeword may also increase. Regarding the second FFS, the potential risk of supporting different quantization codebooks may be the increased overhead for multiple codebooks. Such overhead is depended on the format of each codeword. Take an example of a codebook containing 1000 codewords, with each codeword represented in float32. Assuming each codeword is a complex vector with 8 entries, the overhead of exchanging the codebook is 1000*8*2*4Bytes = 64KB. The overhead is marginal compared to the size of the data samples, which can be hundreds of MBs, therefore it is feasible to apply different quantization codebooks for different configurations.

Proposal 8: For quantization codebook exchange method, support explicitly exchanging codebook, e.g., via look up table.

Different quantization codebooks can be applied to different configurations, including different payload sizes, number of Tx ports and subbands.

Other additional information

Besides performance target and quantization method related information, other additional information may also be beneficial for training the model at UE-side.

- Pairing ID

To enable model pairing and ensure the applicability of the AI model trained with the exchanged dataset, the pairing information should also be included within the dataset. In RAN1#122[2], the following agreement was achieved.

Agreement For Option 4-1 under Direction A in AI/ML based CSI compression, support exchange of single paring ID along with dataset exchange - One pairing ID is assigned to one dataset. - The dataset can have different number of Tx port, number of subbands, and CSI payload size configurations (including different quantization codebooks if needed). - From RAN1 perspective, the uniqueness of the paring ID need further studied. - FFS: the impact on pairing ID(s) when additional data samples are added to an exchanged dataset, if supported.

In our opinion, the motivation of introducing pairing ID is to facilitate model training at UE side and NW side. The uniqueness of pairing ID can be maintained in three levels, i.e. cell-specific, PLMN-specific and global. Firstly cell-specific pairing ID requires great efforts for inter-vendor collaboration. The encoder trained with one dataset may only be used in one cell, forcing UE to store a great number of encoders for different cells. However, if the same decoder is deployed at multiple cells, UE side could be able to use the same encoder across these cells. But a cell-specific pairing ID would prevent UE side for determining whether the same encoder can be used across different cells. Conversely, global paring ID can facilitate UE to train an encoder used across different cells, but it has the risk of exposing proprietary information and complex offline collaboration between NW vendors. Moreover, it is difficult to define the global paring ID across different NW vendor or NW entities. A feasible solution may be to ensure the uniqueness of pairing ID within the same operator. In Rel-19, it was agreed that associated ID is PLMN-specific, we think this definition can be reused for pairing ID. Different operators are allowed to establish its own pairing ID independently, while still offering the flexibility to support cell specific definition as needed. At UE side, different encoders are trained for different operators without knowledge of whether the model is provided by the same NW entity. UE can determine which model to be used with the knowledge of operator and paring ID.

Proposal 9: Paring ID is unique within operator, i.e. PLMN-specific pairing ID.

The other issue is how to assign pairing ID when additional data samples are added to an exchanged dataset. One function of additional data samples is to update training dataset at UE side when the NW side model is updated. If a new model is trained at NW side to replace the old model with additional data samples, the pairing ID should indicate that the data samples are for model update, instead of training a separate model. Therefore the same ID corresponding to the original model should be indicated along with the additional data samples. UE can use the additional data samples to update its encoder based on its implementation. Another scenario is NW side training a new model by fine-tuning an existing model with a small amount of data samples. In such case, a new model is available at NW side separate from the original model. The pairing ID should allow UE side to distinguish the two models. Therefore a new value of ID should be assigned to the additional data samples. It should be noted that how to maintain the dataset associated with different pairing ID and how to utilize the dataset to train its encoder should be left for UE side implementation. The pairing ID exchanged along with the dataset is for reference. As long as the corresponding performance target is met, UE side can determine how many data samples should be used. It is also possible that UE side reuse the same model when NW side updates or fine-tunes its decoder.

Proposal 10: For Option 4-1 under Direction A, if additional data samples are added to an exchanged dataset,

Same pairing ID can be reused for updating an existing model.

New pairing ID can be assigned if a new decoder is trained.

- Model structure related information

For inter-vendor collaboration sub-option 4-1, it is beneficial to provide model structure related information along with dataset to UE side. Without the corresponding model structure information, UE side may choose a model structure which is incompatible with decoder at NW side, UE side may not be able to train the encoder that satisfies performance target because of model backbone mismatch in this case. Even if the actual encoder can be trained, UE side may have to add more blocks or layers to the encoder to satisfy the requirement of performance target, resulting in increased model complexity. Since sub-option 3a-1 and Direction C may provide standardized model structure or fully specified model, how to describe the model structure may be reused for sub-option 4-1. Even if no detailed model structure related specification is available, general information can be provided in sub-option 4-1, like whether the model backbone can be classified as Transformer or CNN, along with some critical hyper parameters like the number of blocks. It should be noted that the provided model structure information is only for reference purpose. Implementation details of NW-side model will not be included.

- Configuration related information

The configuration related information may also be included as dataset content, or be implied by the structure/format of dataset. In the first approach, the values of different parameters can be explicitly configured in the dataset. For example, the number of Tx ports and payload sizes supported in the dataset can be configured as dataset content along with the data samples. For each specific number of Tx ports, the corresponding payload sizes are provided separately. In the second approach, some predefined options can be specified and the dataset format/structure may enable UE side to understand some of the configuration parameters, for example, to distinguish the Tx ports and subbands with standardized delimiter. The format of SF domain target CSI in the data samples can be defined as follow for instance. The delimiter1 is used to separate eigen vector for different subbands within a sample. The delimiter2 is used to distinguish different samples. The number of subbands can be configured with the predefined options corresponding to different number of Tx ports.

{subband 1 delimiter1 subband 2…delimiter1 subband n delimiter2 subband 1 delimiter1 subband 2 delimiter1 delimiter1 …  subband n delimiter2 ….}

It should be noted that the configuration information is based on the training data collection configuration. How to represent the specific configuration in the exchanged dataset can be determined after the conclusions on model inference and data collection are reached.

Proposal 11: For the dataset content of inter-vendor collaboration sub-option 4-1, the additional information also includes:

- Pairing ID

- Model structure related information

  - Indicating specified model backbone type, as well as hyper parameters if needed

- Configurations related information

  - E.g. layers, number of Tx ports, payload sizes, number of subbands

Scalability issue for sub-option 4-1

In Rel-19, scalability issue was considered for model structure standardization for inter-vendor collaboration sub-option 3a-1, where various methods were identified to realize scalability over different number of Tx ports, different number of subbands and different feedback bits. The motivation of study scalable model structure is to employ one or limited number of models for different configurations for inference CSI report, avoiding the training and storage of a large number of models at UE side. This motivation also applies to sub-option 4-1. The standardized dataset format is likewise required to enable UE side to train encoders that can be adapted to different configurations for inference CSI report. In RAN1#122, it was agreed to study association between target CSI and CSI feedback, including mapping for different number of Tx ports, number of sub bands, and CSI payload sizes. Therefore the standardized dataset format should support different inference configurations at least regarding different number of Tx ports, different subbands configurations and different payload sizes.

However, if the target CSI and CSI feedback within data samples remains one-to-one mapping, the size of the dataset will increase significantly with the number of configurations. One possible solution is to adapt the standardized dataset format for the training of scalable models. Based on implementation, UE side can independently decide whether to train a scalable model accommodated to various configurations, or to train multiple dedicated non-scalable models each corresponding to specific configuration. To reduce the dataset size, a target CSI can be associated with multiple CSI feedback corresponding to configurations. For a given target CSI, multiple CSI feedback can be provided associated with different payload sizes. To accommodate to different number of subbands, a target CSI including maximum number of subbands can be associated with CSI feedback corresponding to cases where part of the subbands are used as input. For example, given a target CSI representing CSI of 32 Tx and 12 subbands, multiple CSI feedback representing different payload sizes and different patterns of subbands (selected from the 12 subbands of target CSI) can be provided. When data samples from various configurations are included in the dataset, redundant information of target CSI can be avoided comparing to the one-to-one mapping.

Proposal 12: The standardized dataset format should facilitate the aggregation of multiple configurations to reduce the dataset size.

The same target CSI can be associated with different CSI feedback corresponding to different payload sizes.

The same target CSI can be associated with different CSI feedback corresponding to part of subbands included in the target CSI as input.

Another advantage of the aggregated dataset is to assist the scalable model training. On the premise of scalable model are trained at UE side and/or NW side, some additional information may also be needed to assist UE side to train the scalable model. Various methods were identified by RAN1 for model structure scalability study in Rel-19, while the impact on performance caused by different implementation of model scalability at NW side and UE side has not been considered. For instance, considering a scenario where both NW side and UE side support different number of subbands by padding but they use different padding values and the number of subbands after padding is not the same, the performance degradation is unknown. Moreover, UE side and NW side may also employ different methods to realize scalability over the same aspects. For example UE side and NW side may have different choices of token and feature dimension in Transformer structure, or support different payload sizes by multiple adaption layer and padding relatively. In this case, the performance degradation may occur due to different scalability methods adopted by UE side and NW side. Therefore, information sharing from NW side to UE side should also include the indication of model scalability methods. It should be noted that the scalability related information should be a classification of implementation to avoid proprietary information disclosure.

Proposal 13: Method of realizing model scalability can be provided along with dataset for sub-option 4-1 as additional information.

Conclusions

In this contribution, we provided our views regarding specified dataset format/content for inter-vendor training collaboration sub-option 4-1. The following proposals are provided:

Proposal 1: For inter-vendor collaboration sub-option 4-1, spatial-frequency domain input can be prioritized for dataset format standardization.

Proposal 2: For format of target CSI in dataset for sub-option 4-1, the following options can be supported:

Option 1: Rel-19 eType II codebook (with or without enhancement)

Option 2: Floating point

Proposal 3: For Option 4-1 under Direction A, support Option 1 for CSI feedback type and format, i.e. the exchanged CSI feedback is the latent vector before quantization.

Proposal 4: For inter-vendor collaboration sub-option 4-1, both SGCS and NMSE can be supported as performance target.

SGCS can be used for UE-side training with nominal decoder

NMSE can be used for UE-side training without nominal decoder

Proposal 5: Regarding details of the format of the performance target, at least support Option 1.

Option 1: Average performance target, e.g. average SGCS and/or average NMSE

Proposal 6: For inter-vendor collaboration sub-option 4-1, support multiple performance targets corresponding to different configurations, at least for different layers, different number of Tx ports and different payload sizes.

Proposal 7: For quantization alignment between UE side and NW side, prioritize uniform scalar quantization.

Proposal 8: For quantization codebook exchange method, support explicitly exchanging codebook, e.g., via look up table.

Different quantization codebooks can be applied to different configurations, including different payload sizes, number of Tx ports and subbands.

Proposal 9: Paring ID is unique within operator, i.e. PLMN-specific pairing ID.

Proposal 10: For Option 4-1 under Direction A, if additional data samples are added to an exchanged dataset,

Same pairing ID can be reused for model update.

New pairing ID can be assigned if a new decoder is trained.

Proposal 11: For the dataset content of inter-vendor collaboration sub-option 4-1, the additional information also includes:

Pairing ID

Model structure related information

  - Indicating specified model backbone type, as well as hyper parameters if needed

Configurations related information

Proposal 12: The standardized dataset format should facilitate the aggregation of multiple configurations to reduce the dataset size.

The same target CSI can be associated with different CSI feedback corresponding to different payload sizes.

The same target CSI can be associated with different CSI feedback corresponding to part of subbands included in the target CSI as input.

Proposal 13: Method of realizing model scalability can be provided along with dataset for sub-option 4-1 as additional information.

References

- RP-251870, New WI on AIML for NR air interface Ph 2, Qualcomm,RAN#108, Prague, Czech Republic, June 9th-13th, 2025.

- Chair notes RAN1#122, RAN1 #122, Bengaluru, India, Aug 25th – 29th, 2025.

- Chair notes RAN1#120bis, RAN1 #120bis, Wuhan, China, April 7th – 11th, 2025.