3GPP TSG RAN WG1#122bis	R1-2507255

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:	11.3.2

Source:	Samsung

Title:	Discussion on frame structure design for 6GR

Document for:	Discussion and decision

- Introduction

This contribution discusses general frame structure and duplexing aspects for 6GR.

- 6GR general frame structure aspects

In this section, the general aspects on the frame structure are discussed, including the subcarrier spacing (SCS), Max/Min channel bandwidth, Max supported FFT size, CP length, and the basic resource unit.

  - Subcarrier spacing

In RAN1#122 the following agreement was made regarding the SCS design.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. (Strikethrough during RANP) - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision (Strikethrough during RANP) - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

The first three FFS points (highlighted in yellow) in the above RAN1 agreement were pending RAN decision but no resolution was made in RAN#109. Therefore, the clause “Whether to discuss …. RANP decision” was removed. In our understanding, this implies that these FFS points are to be discussed and decided in RAN1.

The first two FFS points are about whether to add new SCS values in addition to the agreed ones (15kHz for FDD, 30kHz for TDD) for sub-6 GHz. First, one important lesson learned from 5G NR is that having flexibility is not always a good design property. Second, a claimed benefit for having additional SCS, e.g., 30kHz for FDD in around 1-2.5GHz is to make it easier to aggregate FDD and TDD bands. This, however, seems to be a matter of convenience yet non-essential for such band aggregation. Third, other claimed benefits include 2x reduction in PHY-layer latency and increased robustness against frequency offset and Doppler shift. Such claimed latency benefit seems to be relevant mainly for L1 DL/UL control signalling. For data transmission/reception, however, the overall latency is dominated by bottlenecks on MAC layer and the overall latency is further dominated by non-RAN components. In addition, there has been no issue reported from commercial deployments due to the use of 15kHz SCS in sub-6GHz (which has been implemented in NR as well as in LTE networks) – especially since a number of mechanisms (spec-transparent or spec-facilitated) are available to mitigate any possible impact of frequency offset and Doppler shifts.

Given the above, there is no compelling reason to complicate the 6GR system design by supporting more than one SCS value per frequency range. Therefore, we propose to remove the two FFS points.

Proposal 1: RAN1 concludes to stop studying additional SCS values for sub-6GHz.

Regarding the SCS values for some of the frequency bands, e.g., around 7 GHz and around 15 GHz, the choices remain to be studied. Bands around 7 GHz can be regarded as an extension of FR1 since channel propagation characteristics are similar. Therefore, 30kHz SCS could be a good choice for bands around 7 GHz. Bands around 15GHz (e.g., 8.4 – 24.25 GHz) are likely to be defined for a new frequency range and 60kHz SCS seems to be a natural choice.

Proposal 2: For around 7GHz, RAN1 supports 30kHz SCS. For around 15GHz, RAN1 supports 60kHz SCS.

In addition, for SSB SCS, the flexibility provided in NR to make it separate from regular channels (e.g., data and control channels) lacks any utility in commercialized products. Thus, to maximize consistency across different channels and avoid useless specifications, the SSB SCS should be the same as the SCS for the band of the SSB transmission.

Proposal 3: The SSB SCS is same as the SCS for the band of the SSB transmission.

  - Maximum channel BW and FFT size

A wider carrier bandwidth should be secured anticipating diverse service classes for the next decades. For the existing frequency bands, taking the NR maximum channel bandwidth would be good enough, i.e., 100 MHz for FR1 and 400 MHz for FR2. In case of new 6G candidate spectrum, i.e., around 7 GHz and 15 GHz, support of bandwidth larger than 100 MHz can differentiate the value of 6GR over NR. On the other hand, it is uncertain how the new spectrum allocation across the globe will progress. With these considerations, targeting 200 MHz as maximum channel bandwidth of the new frequency range/band would be a reasonable assumption while also allowing the possibility of 400 MHz. It appears that spectrum around 7 GHz is getting more interest and is closer to FR1. Then, assuming OFDM-based waveform, 30 kHz subcarrier spacing and 8K-FFT seem to be proper numerology for around 7 GHz. Spectrum around 15 GHz can be regarded as new separate FR, as discussed in previous section, and 400 MHz maximum channel bandwidth can be considered with 60 kHz subcarrier spacing and 8K-FFT.

Proposal 4: For around 7 GHz, RAN1 supports channel bandwidth of 200 MHz with 30 kHz subcarrier spacing and 8K-FFT.

FFS: whether channel bandwidth of 400 MHz is feasible at the UE.

Proposal 5: For around 15 GHz, RAN1 supports maximum channel bandwidth of 400 MHz with 60 kHz subcarrier spacing and 8K-FFT.

  - Minimum channel BW

In Rel-15 NR, most FR1 bands support 5 MHz as the minimum channel bandwidth, except for bands n41, n77, and n78 using 10 MHz, and band n79 using 40 MHz as the minimum channel bandwidth. All FR2 bands support 50 MHz as the minimum channel bandwidth. In Rel-18, 3 MHz minimum channel bandwidth was also introduced for a set of FR1 bands, such as bands n26, n28, n31, n72, n85, n100, and n106. The minimum channel bandwidth may impact the design of the physical layer, especially for signals and channels associated with initial access procedures. Hence, for 6GR, it would be desired to determine the minimum channel bandwidth as early as possible, with potential cooperation within RAN4, to avoid delays in the design of SS/PBCH blocks/etc.

For 6GR, similar minimum channel bandwidth can be considered at least for the bands re-farmed from NR to 6GR (e.g., FR1 and FR2), since existing channelization and band allocation for NR could be likely used for 6GR as the starting point. For new bands around 7GHz and around 15 GHz, a minimum channel bandwidth similar to upper range of FR1 or lower range of FR2 can be considered. For example, the around 7 GHz can be regarded as FR1-extension so its min CBW can be more aligned with upper FR1 TDD band. For around 15GHz, a new FR can apply with either separate SCS (60kHz) or the same SCS (120kHz) as FR2-1, and the minimum CBW can be larger, such as 50MHz.

Proposal 6: For 6GR, RAN1 assumes the following minimum channel bandwidth as a starting point for study, pending RAN4’s confirmation:

- 5 MHz for FR1 FDD bands;

- 10 MHz for lower FR1 TDD bands;

- 20 MHz for upper FR1 TDD bands and around 7GHz bands;

- 50 MHz for around 15GHz bands and FR2 bands.

  - CP length

It was decided in RAN1#122 that the normal CP is supported.

Agreement - 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR. - FFS potential need for other CP

In NR, both normal CP (NCP) and extended CP (ECP) are supported. The motivation to support ECP was to mitigate the multipath delay spread in large cells as the delay spread increases with the cell radius. ECP extends the guard interval (e.g., 4.17 μs for 60 kHz SCS), ensuring delayed multipath components do not interfere with subsequent symbols, thereby suppressing Inter-Symbol Interference (ISI) and Inter-Carrier Interference (ICI). However, usage of ECP was limited only to 60 kHz SCS and it sacrifices spectral efficiency (12 symbols/slot vs. 14 symbols/slot, reducing capacity by ~14.3%) without providing an actual benefit. For that reason and for deployment complexity reasons, ECP was rarely deployed in NR, and an actual need to support ECP in 6GR does not exist.

Regarding the FFS, the exception for PRACH design can be later considered. Therefore, the following proposal is made:

Proposal 7: Except for PRACH, 6GR does not support extended CP length,

- for PRACH, whether or not additional CP length(s) are needed is to be discussed as a part of initial access agenda.

  - Basic resource unit

In NR, the fundamental time-frequency resource structure is built upon scalable units designed to balance flexibility and efficiency. In the time domain, the basic building blocks include symbols, slots, subframes, and frames. An OFDM symbol duration inversely depends on the SCS, ranging from 66.67 µs for 15 kHz SCS to 4.17 µs for 240 kHz SCS. A slot aggregates 14 symbols under normal CP, with slot durations naturally scaling as the symbol duration inversely to the SCS, e.g., 1 ms for 15 kHz SCS and 0.125 ms for 120 kHz SCS. A subframe length remains fixed to 1 ms, while a frame spans 10 ms, maintaining backward compatibility with LTE. In the frequency domain, the smallest schedulable unit is the PRB, comprising 12 consecutive subcarriers. For 15 kHz SCS, a PRB occupies 180 kHz bandwidth. This modular design enables NR to adapt to diverse scenarios: low SCS values (e.g., 15 - 30 kHz) optimize coverage in sub-6 GHz bands, while high SCS values (e.g., 60 - 120 kHz) minimize latency and mitigate phase noise in FR2. The PRB-centric allocation reduces scheduling overhead and ensures efficient spectrum utilization, while the harmonized subframe/frame structure simplifies multi-RAT coexistence. Thus, such design logic is inherited to 6GR as basic resource unit structure.

Observation 1: The basic time-frequency resource unit structure from NR can be reused in 6GR.

Two aspects might be further considered to achieve 6GR design targets. One is the slot structure. In NR Rel-15/16, both “mini-slot” and slot-based scheduling are supported to meet the transmission requirements for URLLC and eMBB applications. In other words, multiple scheduling units exist, corresponding to 2/4/7 symbols and 14 symbols, and are given names of “min-slot” and “slot”. To enhance coverage, NR Rel-18 added a scheduling unit of multiples of 14 symbols and gave it the name “TBoMS”. A flexible duration for the number of symbols in a scheduling unit can be considered in 6GR under the unified framework while also considering, when/if necessary, occasional constraints of at most single CB and using repetition type 1 as time domain resource configuration reference. At least scheduling units of multiples of 14 symbols/1 slot (including one) can be considered. Extending the scheduling unit duration beyond a single slot may also help reduce the associated PDCCH and DMRS overheads, thereby improving the overall spectral efficiency as illustrated in Figure 1. In the context of massive MIMO, extending the scheduling unit beyond 14 symbols/1 slot can also offer implementation benefits. Specifically, for supporting a given number of antenna ports in MU-MIMO, using legacy slot-based scheduling may require larger DCI/DMRS overhead comparing with using longer duration scheduling unit.

Figure 1. Illustration of longer transmission duration (e.g., >14 symbols)

In the frequency domain, a question is whether to support a resource unit smaller than 1 PRB, i.e., sub-PRB allocation, assigning fewer than 12 subcarriers to enable higher coverage (for low data rates) from 6GR day-1. A sub-PRB scheduling granularity could enhance coverage by concentrating transmit power on narrower frequency slices. For cell-edge users or deep-indoor IoT devices, transmitting over a few subcarriers instead of a full PRB provides obvious power gain (e.g., more than 9 dB by simple calculation if down to single tone comparing with full PRB). This is also quite beneficial for non-terrestrial networks (NTN) or rural deployments with limited link budgets. Thus, the introduction of sub-PRB resource unit(s) is justified for study.

Figure 2. Illustration of sub-PRB

Proposal 8: 6GR considers to support the same resource unit structure from NR including: symbol, slot (14 symbols) subframe (1ms)/frame (10ms) in time domain and PRB (12 subcarriers) in frequency domain.

Proposal 9: Additionally, 6GR considers to study the structure for longer transmission duration (e.g., multiples of 14 symbols) in time domain and/or sub-multiples of PRB in frequency domain.

- 6GR duplex

The evolution of wireless communication technologies demands continuous innovation and optimization to meet the growing demands of diverse applications and network scenarios. In this context, the 6GR duplex study presents an opportunity to enhance network performance, reduce interference, improve resource utilization and coverage. This section provides an analysis on FD/HD-FDD, semi-static TDD and semi-static SBFD, while considering key aspects for dynamic TDD and advanced SBFD operations.

  - Supported duplex schemes

In RAN1##122, RAN1 reached the following agreements for 6GR duplex schemes.

Agreement Study and identify the lessons learned from NR duplex modes. On 6GR duplexing study, RAN1 considers at least following duplex types: - FD-FDD - Semi-static TDD - gNB semi-static SBFD - HD-FDD on UE side - Dynamic TDD Study whether to consider following duplexing types - gNB dynamic SBFD - UE SBFD - gNB FD - Note: Other duplex modes are not precluded.

Our preference on the listed duplex schemes remains consistent with the above agreement, focusing on supporting duplex schemes that are most likely to be commercially deployed and justified for potential use cases. For example, as analysed for global commercial deployments in [2], FD-FDD and semi-static TDD are widely used and so 6GR should unquestionably support FD-FDD and semi-static TDD. Additionally, HD-FDD was originally introduced in LTE-M or, in general, for low-cost devices and was also adopted for NR RedCap UEs. The benefits of HD-FDD are clear, i.e., low-cost and small form-factor by removing the duplexer. 6GR will at least support UE types that NR supports, from low-end to high-end [3], and thus HD-FDD should be supported for 6GR.

Observation 2: FD-FDD, semi-static TDD, and HD-FDD on UE side, have been widely commercialized over decades.

On top of the existing well-deployed duplex schemes, Rel-19 successfully specified a new duplex operation, named Sub-Band non-overlapping Full-Duplex (SBFD), to address uplink coverage and latency challenges in TDD bands. Specifically, due to downlink dominant slot patterns, the UL time period is relatively shorter, significantly degrading UL coverage and increasing latency in TDD bands. SBFD enables longer UL time periods within an original DL duration, facilitated by simultaneous transmission and reception at the base station. Notably, SBFD provides more than 5 dB coverage gain over TDD operations in FR1 bands, making it an effective solution to address UL coverage issues. It is important to note that the upper-mid band (e.g., 7.125GHz~8.4GHz) may experience higher pathloss and penetration loss compared to NR FR1 bands, and therefore maintaining a similar UL coverage as for NR FR1 bands will be a particular challenge. To confirm the feasibility and UL performance gain for upper-mid band operation, we evaluated semi-static SBFD operation with Rel-18 system-level simulation assumptions (for 7GHz, the channel model in TR38.901 was used). As shown in Figure 3, semi-static SBFD has a substantial UL performance gain (UPT gain) over semi-static TDD. For example, at 3.5 GHz, SBFD with 128 antenna elements provides ~52%, ~296%, and ~91.8% higher UPT over TDD at 5%, 50%, and 95%tile geometry points, respectively. At 7 GHz, SBFD with 128 antenna elements provides smaller but still substantial gains (~36%, 168%, and 97% at 5%, 50%, and 95%tile points, respectively). The SBFD performance gain at 7 GHz is enhanced in case of more antenna elements, (~34%, 240%, 114% at 5%, 50%, and 95%tile points, respectively). Therefore, SBFD operation should be supported as a day-1 feature for 6GR.

Observation 3: In Rel-18 and Rel-19, semi-static SBFD operation has been extensively evaluated in RAN1 and its feasibility and requirements have been concluded in RAN4.

Figure 3. System-level simulation for 3.5GHz and 7GHz with different number of antenna elements

One controversial point is whether or not to support dynamic TDD operations. The origin of dynamic TDD operation is LTE eIMTA, which was first specified in Rel-12, in 2015. The motivation of dynamic TDD is to adapt to DL/UL traffic ratio by changing the DL/UL slot configuration. However, it is well known that dynamic TDD operation suffers from gNB-gNB cross-link interference (CLI) and UE-UE CLI. Due to the CLI, the feasibility of dynamic TDD operation has been doubtful and was extensively checked in Rel-16. The results in TR38.828 are summarized in Table 1 with highlights. In summary, it is clear that 1) dynamic TDD cannot be deployed in Macro scenarios, 2) dynamic TDD can be used in indoor scenarios, conditionally. But, the condition in indoor scenario requires inter-site coordination/collaboration as well as inter-operator’s coordination/collaboration, which are hardly practical. Because of these reasons, dynamic TDD has not been commercialized over a decade.

Observation 4: Dynamic TDD has not been successfully commercialized due to gNB-gNB CLI and UE-UE CLI. TR38.828 also concludes that dynamic TDD is not practically deployable in Macro scenarios.

Table 1. Summary on dynamic TDD study in TR38.828

| FR1 | Macro-to-Macro scenario | Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios. |
| --- | --- | --- |
|  | Indoor scenarios (Indoor-to-Macro and Indoor-to-Indoor) | Performance degradations were not observed from operating dynamic TDD between an indoor network and a macro network and vice versa if there is sufficient isolation between them. No significant impact from operating dynamic TDD for the indoor scenario was observed as long as the BS and UE powers are similar and the operators co-ordinate so that base station positions are offset. If higher BS power is assumed, some throughput degradation in the indoor scenario was observed due to BS to BS interference. The observations imply that dynamic TDD can be used in indoors as long as care is taken. |
| FR2 | Macro-to-Macro scenario | Some performance degradation was observed from the BS-to-BS interference for macro-macro scenario. The differences in the simulation results imply that operating dynamic TDD in this scenario without impact to neighbor network may be deployment dependent and requires at least careful planning and collaboration between operators to avoid performance impact |
|  | Indoor scenarios (Indoor-to-Macro and Indoor-to-Indoor) | Performance degradations were not observed from operating dynamic TDD between an indoor network and a macro network if there is sufficient isolation between them. Results suggested that to avoid degradation, careful layout and parameterization are necessary for indoor to indoor scenario. Overall, the observations imply that dynamic TDD can be used indoors as long as care is taken. |
|  | Micro-to-Micro scenario | For micro to micro, the differences in the simulation results imply that to avoid BS to BS interference, operators may need to consider the proximity of micro BS in the same area. Overall, the observations imply that dynamic TDD can be used in certain micro deployments as long as care is taken. |

In RAN1#122, RAN1 also identified advanced duplex schemes such as dynamic SBFD, UE-side SBFD, and gNB FD. Advanced SBFD operations represent a potential area for future enhancement. At this stage, we are open to such extensions but a careful study is first needed in terms of the complexity, feasibility, and clarity of associated gains. For example, for dynamic SBFD, same CLI issues as for dynamic TDD are likely to arise. For UE-side SBFD, the performance benefits are questionable while, if feasible,  the UE implementation costs would be substantial as significant changes to the UE RF architecture and filters would be required which may not justify the potential gains. For gNB FD, there can be UL throughput gain in certain scenarios, such as for FR2 and FWA, where the CLI can be effectively controlled or mitigated. In these scenarios, gNB FD can enhance network capacity and reduce latency, making it a viable option for future deployments. However, we also see the limitations on FR1 mobile handheld UEs. For these advanced duplex schemes, RAN4’s feasibility check is also required before any RAN1 work can be considered.

Observation 5: Advanced schemes, such as dynamic SBFD, UE-side SBFD, and gNB-FD require more careful study in both RAN1 and RAN4 in terms of complexity, feasibility, and potential gains.

Overall, we propose to prioritize FD/HD-FDD, semi-static TDD, and semi-static SBFD for 6GR study.

Proposal 10: RAN1 prioritizes FD/HD-FDD, semi-static TDD, and semi-static SBFD for 6GR study.

  - TDD UL/DL configuration

NR TDD configuration includes a cell-common UL/DL configuration and a UE-dedicated UL/DL configuration. More specifically, the cell-common UL/DL configuration is provided in SIB1 and configures the number of DL slots/symbols from the beginning of the TDD pattern and the number of UL slots/symbols from the end of the TDD pattern. Remaining symbols in the TDD pattern are regarded as flexible symbols. The UE-dedicated UL/DL configuration can override flexible symbols to DL symbols or UL symbols, and does not override DL symbols or UL symbols indicated by the cell-common UL/DL configuration. Direction of the flexible symbols can be further indicated by dynamic slot format indicator (SFI) in DCI format 2_0.

The main motivation of UE-dedicated UL/DL configuration and dynamic SFI is to provide a UE-specific traffic optimized slot format. For example, when UL traffic is high, more UL symbols can be configured by UE-dedicated UL/DL configuration or dynamic SFI. So far, this operation is not commercialized since 1) it may result in inter-BS/inter-UE CLI and 2) it may require RRC reconfiguration whenever UE traffic changes and cannot account for fast traffic variations. Also, as previously noted, the Rel-16 CLI study concluded that such operation is not feasible in Macro cell deployments due to UL performance degradation by DL transmissions. Further, dynamic SFI is similar to eIMTA from LTE which was not commercialized for over a decade now. Thus, no real benefits from supporting UE-dedicated UL/DL configuration and dynamic SFI have materialized and those functionalities need not be considered at least for the first release of 6GR.

Another issue is whether or not to specify flexible symbols. As mentioned, flexible symbols are beneficial for traffic adaptation but CLI on flexible symbols cannot be avoidable. More importantly, the current NR deployments use flexible symbols for explicit gap periods from DL-to-UL. However, since specification allows UE transmission and reception on flexible symbols, it is difficult to avoid such transmission/reception. For example, higher-layer configured signals, including SRS/CSI-RS, can be transmitted or received on flexible symbols, even though the flexible symbols are used for explicit gap periods at BS. This prevents a clean operation for 6GR. Therefore, whether to support flexible symbols in 6GR or not should be reconsidered.

Proposal 11: 6GR reconsiders the necessity of flexible symbols and does not support dynamic SFI and UE-specific TDD configuration (e.g., TDD-UL-DL-ConfigDedicated) in the first release.

Figure 4. Illustration of NR TDD-UL-DL configuration

- Conclusion

This contribution discusses frame structure and duplex in 6GR and following observations and proposals are drawn:

For frame structure:

Proposal 1: RAN1 concludes to stop studying additional SCS values for sub-6GHz.

Proposal 2: For around 7GHz, RAN1 supports 30kHz SCS. For around 15GHz RAN1 supports 60kHz SCS.

Proposal 3: The SSB SCS is same as the SCS for the band of the SSB transmission.

Proposal 4: For around 7 GHz, RAN1 supports channel bandwidth of 200 MHz with 30 kHz subcarrier spacing and 8K-FFT.

FFS: whether channel bandwidth of 400 MHz is feasible at the UE.

Proposal 5: For around 15 GHz, RAN1 supports maximum channel bandwidth of 400 MHz with 60 kHz subcarrier spacing and 8K-FFT.

Proposal 6: For 6GR, RAN1 assumes the following minimum channel bandwidth as a starting point for study, pending RAN4’s confirmation:

- 5 MHz for FR1 FDD bands;

- 10 MHz for lower FR1 TDD bands;

- 20 MHz for upper FR1 TDD bands and around 7GHz bands;

- 50 MHz for around 15GHz bands and FR2 bands.

Proposal 7: Except for PRACH, 6GR does not support extended CP length,

- for PRACH, whether or not additional CP length(s) are needed is to be discussed as a part of initial access agenda.

Observation 1: The basic time-frequency resource unit structure from NR can be reused in 6GR.

Proposal 8: 6GR considers to support the same resource unit structure from NR including: symbol, slot (14 symbols) subframe (1ms)/frame (10ms) in time domain and PRB (12 subcarriers) in frequency domain.

Proposal 9: Additionally, 6GR considers to study the structure for longer transmission duration (e.g., multiple of 14 symbols) in time domain and/or sub-multiples of PRB in frequency domain.

For duplex

Observation 2: FD-FDD, semi-static TDD, and HD-FDD on UE side, have been widely commercialized over decades.

Observation 3: In Rel-18 and Rel-19, semi-static SBFD operation has been extensively evaluated in RAN1 and its feasibility and requirements have been concluded in RAN4.

Observation 4: Dynamic TDD has not been successfully commercialized due to gNB-gNB CLI and UE-UE CLI. TR38.828 also concludes that dynamic TDD is not practically deployable in Macro scenarios.

Observation 5: Advanced schemes, such as dynamic SBFD, UE-side SBFD, and gNB-FD require careful study in both RAN1 and RAN4 in terms of complexity, feasibility, and potential gains.

Proposal 10: RAN1 prioritizes FD/HD-FDD, semi-static TDD, and semi-static SBFD for 6GR study.

Proposal 11: 6GR reconsiders the necessity of flexible symbols, and does not support dynamic SFI and UE-specific TDD configuration (e.g., TDD-UL-DL-ConfigDedicated) in the first release.

References:

- RP-252912	Revised SID: Study on 6G Radio	NTT DOCOMO, CMCC, AT&T, Vodafone 	RAN#109

- R1-2505585	Discussion on frame structure design for 6GR	Samsung RAN1#122

- R1-2507252	Design of 6GR air interface	Samsung	RAN1#122bis

Annex: 6G SI Objectives – Frame structure related (RP-252912)

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

- Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - ……